3GPP TSG-RAN WG3 Meeting #127                             	  R3-250114

Athens, GR, 17th – 21st Feb, 2025

Source:	CATT

Title:	(TP to BL CRs) Support of gNB-based AI positioning

Agenda Item:	20.2

Document for:	Discussion and Decision

1 Introduction

In the last RAN3 meeting, we initially discussed the potential RAN3 impact to support AI/ML assisted Positioning, i.e. the Case 3a. According to the discussion [1], some agreements and open issues as below:

The existing LCS framework is re-used for AI/ML positioning. The AI/ML model (inference and training functions) is at the gNB. LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed. FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1). FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified. Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used. FFS if LMF should be aware that the reported measurements are AI/ML generated. Check progress in RAN1 and solution discussion…

In this contribution, we will discuss the potential RAN3 impact to support AI/ML assisted positioning, taking the RAN1 progress/agreements into account. Base on the discussion, we will provide corresponding observations and proposals.

2 Discussion

2.1 NRPPa Transaction

For the Data collection, RAN3 discussed and made a WA that LMF starts NRPPa transaction, as the legacy NRPPa principle. And gNB determines that data collection is needed for its model.

- WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

According to the current Positioning architecture, there’s no direct interface between gNB and LMF. If gNB is allowed to initiate NRPPa transaction towards LMF, how to identify, select and route to proper LMF needs to be addressed.

Observation 1: According to current Positioning architecture, not feasible for a gNB to identify, select and route to proper LMF to initiate a new NRPPa transaction.

Therefore, we understand it’s better to keep the principle of NRPPa, the straightforward way is to turn the WA to an agreement, which means only LMF is allowed to trigger the NRPPa transaction towards the gNB.

Proposal 1: Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to the agreement.

As above, to support AI positioning case 3a, LMF should initiate NRPPa transaction towards the gNBs which hold the AI/ML model for positioning, if the transaction is not exist. Thus, we could define a non-UE associated procedure, e.g. named as AIML Assisted Positioning Enabled. Then, when gNB may determines the data collection is needed, it could request for the label information from the LMF.

Proposal 2: Introduce a new class 2 procedure to initiate NRPPa transaction for AI/ML Assisted Positioning, e.g. AIML Assisted Positioning Enabled.

2.2 Training data collection

On the training data collection, there’re some FFSes to be further considered, as below:

- FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1).

- FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified.

First of all, to avoid extra impact to NGAP, the AI positioning related functions/procedures should be limited to the NRPPa, not extra impact the XnAP.

Proposal 3: AI Positioning related procedures between gNB and LMF should be limited to NRPPa, should not impact the NGAP.

On how the gNB requests for the training data from the LMF and which node selects/decides the UE labels, here’re some options:

- Option 1: LMF selects/decides UE labels for data collection, initiates corresponding positioning measurements and provides gNB Part A + Part B of the labels.

- Option 1bis: LMF selects/decides UE labels for data collection, initiates corresponding positioning measurements and only provide gNB the Part B of the labels.

- Option 2: gNB selects/decides UE labels and provides the UE label IDs to the LMF, LMF initiates positioning measurement and provides Part A and Part B of the labels to the gNB.

- Option 2bis: gNB selects/decides UE labels and provides the UE label IDs to the LMF, LMF initiates positioning measurement, and only provide gNB the Part B of the labels.

For the option 1, gNB request for the data collection towards LMF when needed, which may include the TRP IDs, the intended number of UE labels in the request message. LMF decides the UE labels and triggers the Positioning measurement towards the UE labels. Then LMF provides the information of the labels to the gNB, e.g. both Part A and Part B. With this option, legacy positioning measurement procedures could be fully reused without any enhancement.

Similar to option 1, the option 1bis is a small enhancement on top of option 1, which means gNB could perform the positioning measurements as required and provide the measurement results to the LMF. But here, gNB also keep the measurement results (Part A) of the UE labels, then LMF just need to provide the Part B of the labels. But how to associate the Part A and Part B may require extra enhancement to Positioning procedures, e.g. LMF provides the same label ID in the measurement request, gNB store the Part A associate with the label ID. Then LMF provides the Part B using the same label ID in the NRPPa message. gNB could associate the Part A and Part B with the label ID.

For the option 2 and 2bis, gNB requests for the data collection towards LMF, including the expected UE label IDs. How to select/decide the UE labels in the gNB is not clear, gNB may select part or all of the RRC_Connected UEs. Then, LMF initiates the positioning procedures towards the UEs and the gNB as requested to collect the label information, and provides the label information (Part A + ) Part B to the gNB.

Comparing the options above, the option 1 is most simple and straightforward, data collection is initiated by LMF, and data collection could be performed without any extra impact to the legacy positioning procedures. It’s proposed to go for the option 1to collect the data for the AI assisted positioning (case 3a).

Proposal 4: It’s proposed LMF to select the labels, collect necessary information of the labels, and provides both Part A and Part B of the labels to the gNB.

For Case 3a, since the AI/ML model is deployed at gNB side and the channel measurement entity is also gNB, gNB may request for different information for training, thus, some assistance info should be provided to LMF when request for the training data, e.g. TRP IDs, expected amount of labels, expected measurement type/result (e.g. UL-RTOA, Multi-RTT).

Proposal 5: for Case 3a, gNB should provide some assistance information to LMF when request for training data, e.g. TRP IDs, expected amount of labels, expected measurement type/result (e.g. UL-RTOA, Multi-RTT).

Moreover, since the AI/ML model is deployed at gNB side, gNB may have its preferred UL SRS configuration for training data collection. However, currently, only LMF has the capability of requesting preferred SRS transmission characteristics, e.g., the density and periodicity of SRS resources. During the request process, LMF does not have prior knowledge about gNB-side model, i.e., LMF does not know the SRS configuration most suitable for gNB-side models, no matter from consistency or measurement perspective. To tackle this issue, gNB can provide its preferred SRS configuration to LMF in advance. In this way, LMF can better negotiate with different TRPs to determine the SRS configuration most suitable for gNB side models. Specifically, the report can be performed in explicit (request specific SRS transmission characteristics) or implicit manner (via associated ID).

Proposal 6: For data collection in Case 3a, it’s beneficial for gNB to provide the preferred UL SRS characteristics/configuration for training data collection to LMF.

2.3 Inference

For Model Input/Report for inference, there’s a common understanding that:

- Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used.

We assume NRPPa Measurement procedure could be reused to support the purpose of training data collection, inference and monitoring for Case 3a. If any other information needs to be reported from gNB to LMF is pending to RAN1.

Proposal 7: In Case 3a, existing measurement procedure could be reused for training data collection, inference and monitoring. If any other information needs to be reported from gNB to LMF is pending to RAN1.

There’s an open issue that:

- FFS if LMF should be aware that the reported measurements are AI/ML generated.

Not sure about the purpose of such kind of indication, and what’s the expected behaviors of LMF if reported measurements are AI/ML generated or not. As we’ve already agreed that the performance monitoring for the case 3a is performed in the gNB, gNB may start or stop using the AI model for positioning measurement, not necessary to inform the LMF the reported measurements are AI/ML generated or not.

Proposal 8: For Case 3a, no need to indicate LMF the reported measurements are AI/ML generated or not.

2.4 Performance Monitoring

For performance monitoring, it’s already agreed that:

- RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

In RAN1 agreements, it’s also noted that:

- Note: Exact method to perform monitoring metric calculation is up to implementation.

- Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

How to perform performance monitoring, if any other information need to be provided to NG-RAN to be further discussed, taking RAN1 progress into account.

Proposal 9: How to perform performance monitoring in NG-RAN to be further discussed, taking RAN1 progress into account.

Base on the discussion above, we provide the TPs to introduce Data Collection Information Transfer function to NRPPa:

- TP to BL CR for TS 38.455, see Annex 1.

- TP to BL CR for TS 38.305, see Annex 2.

- TP to BL CR for TS 38.413, see Annex 3.

Proposal 10: Discuss and agree the TPs to BL CRs for TS 38.455, 38.305 and 38.413, as provided in the Annex.

3. Conclusion

In this contribution, discussed the RAN3 impact to support AI/ML assisted positioning for AI assisted Positioning Accuracy enhancements. Based on the discussion above, the following proposals are provided:

Observation 1: According to current Positioning architecture, not feasible for a gNB to identify, select and route to proper LMF to initiate a new NRPPa transaction.

Proposal 1: Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to the agreement.

Proposal 2: Introduce a new class 2 procedure to initiate NRPPa transaction for AI/ML Assisted Positioning, e.g. AIML Assisted Positioning Enabled.

Proposal 3: AI Positioning related procedures between gNB and LMF should be limited to NRPPa, should not impact the NGAP.

Proposal 4: It’s proposed LMF to select the labels, collect necessary information of the labels, and provides both Part A and Part B of the labels to the gNB.

Proposal 5: for Case 3a, gNB should provide some assistance information to LMF when request for training data, e.g. TRP IDs, expected amount of labels, expected measurement type/result (e.g. UL-RTOA, Multi-RTT).

Proposal 6: For data collection in Case 3a, it’s beneficial for gNB to provide the preferred UL SRS characteristics/configuration for training data collection to LMF.

Proposal 7: In Case 3a, existing measurement procedure could be reused for training data collection, inference and monitoring. If any other information needs to be reported from gNB to LMF is pending to RAN1.

Proposal 8: For Case 3a, no need to indicate LMF the reported measurements are AI/ML generated or not.

Proposal 9: How to perform performance monitoring in NG-RAN to be further discussed, taking RAN1 progress into account.

Proposal 10: Discuss and agree the TPs to BL CRs for TS 38.455, 38.305 and 38.413, as provided in the Annex.

4. Reference

- RP-240774 Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RAN1#116/#116bis/#117/#118/118bis/#119 Chair’s Notes

Annex 1. (TP to BL CR for TS38.455) support of AI assisted Positioning

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning and NR E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

-	PRS Information Transfer. This function allows the LMF to exchange PRS related information with the NG-RAN node.

-	Measurement Preconfiguration Information Transfer. This function allows the LMF to request the NG-RAN node to preconfigure and activate measurement gap and/or PRS processing window.

-	Area-specific SRS Information Transfer. This function allows the LMF to notify the NG-RAN node about area-specific SRS configuration information.

- Data Collection Information Transfer. This function allows the gNB to request for training data information for AI based positioning with gNB-side model.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |
| PRS Information Transfer | PRS Configuration Exchange |
| Measurement Preconfiguration Information Transfer | Measurement Preconfiguration Measurement Activation |
| Area-specific SRS Information Transfer | SRS Information Reservation Notification |
| Data Collection Information Transfer | a) Data Collection Notification b) Data Collection Initiation c) Data Collection Reporting |

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

8	NRPPa procedures

8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |
| Data Collection Information Initiation | DATA COLLECTION REQUEST | DATA COLLECTION RESPONSE | DATA COLLECTION FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| Measurement Activation | MEASUREMENT ACTIVATION |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| Data Collection Notification | DATA COLLECTION NOTIFICATION |
| Data Collection Update | DATA COLLECTION UPDATE |

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 8.X	Data Collection Information Transfer

### 8.X.1	Data Collection Notification

#### 8.X.1.1	General

The Data Collection Notification procedure allows the LMF to indicate the NG-RAN node that it’s ready to prepare the data collection information for gNB-based AI model.

#### 8.X.1.2	Successful Operation

Figure 8.5.1.2.1: Data Collection Notification procedure. Successful operation.

The LMF initiates the procedure by sending a DATA COLLECTION NOTIFICATION message to the NG-RAN node. Which allows the NG-RAN node to request for AI Assisted Positioning Information towards the LMF for AI assisted positioning where AI model is gNB-based.

#### 8.X.1.3	Unsuccessful Operation

Not applicable.

#### 8.X.1.4	Abnormal Conditions

Void.

### 8.X.2	Data Collection Initiation

#### 8.X.2.1	General

The Data Collection Information Initiation procedure allows the NG-RAN to request positioning information from the LMF for data collection.

#### 8.X.2.2	Successful Operation

Figure 8.X.2.2.1: Data Collection Information Exchange procedure. Successful operation.

The NG-RAN initiates the procedure by sending a DATA COLLECTION REQUEST message to the LMF.

If the Data Collection Request IE is set to "start", the LMF shall reply with a DATA COLLECTION RESPONSE message and shall initiate the Data Collection Information Update procedure including the requested positioning information in the area of indicated in the TRP Area list IE.

If the Desired amount of UE labels IE is included, the LMF shall, if supported, take it into account when preparing for the UE labels and provide corresponding information to the gNB.

If the UE location required IE is included, the LMF shall, if supported, provide the location of the UE labels to the gNB in the DATA COLLECTION UPDATE message.

If TRP Measurement Quantities IE is included, the LMF shall, if supported, perform corresponding positioning measurement and provide corresponding results to the gNB in the DATA COLLECTION UPDATE message.

If Requested SRS Transmission Characteristics IE is included, the LMF shall, if supported, take it into account when performing Positioning Information Exchange and Measurement procedures for data collection.

If the Periodicity IE is included in the Requested Positioning Information Item IE, the LMF shall, if supported, consider is as the periodicity of reporting the requested Positioning information.

#### 8.X.2.3	Unsuccessful Operation

Figure 8.X.2.3.1: Data Collection Information Exchange procedure. Unsuccessful operation.

If the LMF cannot provide any of the requested positioning information as requested in the DATA COLLECTION REQUEST message, it shall respond with a DATA COLLECTION FAILURE message with an appropriate cause value.

#### 8.x.2.4	Abnormal Conditions

Void.

### 8.X.3	Data Collection Update

#### 8.X.3.1	General

The Data Collection Update procedure allows the LMF to provide positioning information to the NG-RAN for data collection.

#### 8.X.3.2	Successful Operation

Figure 8.X.3.2.1: Data Collection Update procedure.

The LMF initiates the procedure by sending a DATA COLLECTION UPDATE message to the NG-RAN node. The DATA COLLECTION UPDATE message contains the requested positioning information for data collection at NG-RAN.

#### 8.X.3.3	Unsuccessful Operation

Not applicable.

#### 8.X.3.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.1.X	Messages for Data Collection Information Transfer Procedures

#### 9.1.X.1	DATA COLLECTION NOTIFICATION

This message is used to indicate the NG-RAN node that data collection is allowed for gNB-based AI model.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |

#### 9.1.X.2	DATA COLLECTION REQUEST

This message is initiated to request positioning information for data collection.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Data Collection Request | M |  | ENUMERATED(start, stop, …) |  | YES | reject |
| TRP Data Collection List |  | 1 |  |  | YES | ignore |
| >TRP Data Collection Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| Desired amount of UE labels | O |  | INTEGER (1..65536, …) |  | YES | ignore |
| UE location required | O |  | ENUMERATED(true,…) |  | YES | ignore |
| TRP Measurement Quantities |  | 0 .. <maxnoPosMeas> |  |  | YES | ignore |
| >TRP Measurement Type | O |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) |  |  |  |
| >Periodicity | O |  | ENUMERATED(160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, …) |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.27 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

#### 9.1.X.3	DATA COLLECTION RESPONSE

This message is sent to confirm start reporting of the requested positioning information for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.X.4	DATA COLLECTION FAILURE

This message is sent to report failure of initiating data collection of positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.X.5	DATA COLLECTION UPDATE

This message is sent to report positioning information for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| UE label Information list |  | 0..1 |  |  |  |  |
| >UE Label Information Item |  | 1..<maxnoofUELabels> |  |  |  |  |
| >>Location Information | M |  | OCTET STRING | Location of the UE label, as defined in TS 37.355 [14]. | - |  |
| >>Measurement Result | O | FFS |  | Including Part A and Part B. |  |  |
| >>Time Stamp | O |  | 9.2.42 | Location time stamp. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUELabels | Maximum no. of UE labels that can be included within one message. Value is 65536. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

Annex 2. (TP to BL CR for TS38.305) support of AI assisted Positioning

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 7.x	Procedures for Data Collection Information Transfer

### 7.x.1	General

To support AI/ML based positioning with gNB-side model, data collection information is used to enable the data training in gNB-side AI model.

### 7.x.2	Data Collection Information Transfer Procedures

Figure 7.x.2-1 shows the Data Collection Information Transfer Procedure.

Figure 7.x.2-1 Data Collection Information Transfer Procedures

1. LMF initiates the Data Collection Notification procedure towards the gNBs which holds the AI/ML positioning model to establish the NRPPa transaction.

Note: This step could be ignored if NRPPa transaction(s) has already been established between the gNB and LMF for other purposes.

2. When gNB decides to train the model for AI/ML based positioning, it may request for the training data from LMF.

3. The gNB sends the Data Collection Request towards the LMF, with some assistance information, e.g. expected amount of UE labels, expected data type.

4. LMF confirms the requirements of the gNB and replies with the response message.

5. LMF decides the UE labels and initiates proper positioning procedures to collect the training data via the Positioning Information Transfer and Measurement procedures.

6. When the training data is collected, LMF provides the training data to the gNB via the Data Collection Update.

<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

Annex 3. (TP to BL CR for TS38.413) support of AI assisted Positioning

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 8.10	NRPPa Transport Procedures

### 8.10.1	General

The purpose of the NRPPa Transport procedures is to carry NRPPa signalling (defined in TS 38.455 [19]) between the NG-RAN node and the LMF over the NG interface.

The Downlink UE Associated NRPPa Transport procedure and the Uplink UE Associated NRPPa Transport procedure use UE-associated signalling. The UE-associated signalling is used to support E-CID Location Information Transfer, Positioning Information Transfer, Measurement Preconfiguration Information Transfer, and Reporting of General Error Situations due to reception of an NRPPa message that utilized UE-associated signalling.

The Downlink Non UE Associated NRPPa Transport procedure and the Uplink Non UE Associated NRPPa Transport procedure use non-UE associated signalling. The non-UE associated signalling is used to support OTDOA Information Transfer, Assistance Information Transfer, TRP Information Transfer, Measurement Information Transfer, PRS Information Transfer, Area-specific Information Transfer, Data Collection Information Transfer and Reporting of General Error Situations due to reception of an NRPPa message that utilized non-UE associated signalling.

<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>