3GPP TSG RAN WG3 Meeting #127	R3-250119

Athens, Greece, 17 – 21 Feb, 2025

Title: 	Discussion on A-IoT procedures and content

Source: 	ZTE Corporation

Agenda item:	16.2

Document for:	Discussion and Decision

- Introduction

In the previous meeting, we have briefly discussed the content of Inventory, and have achieved some agreement within the latest TR 38.769, as below.

TR 38.769 6.4.1.1 Inventory Inventory can be sent by the AIoT CN for a single device, or a group of devices, or all devices. The Inventory Request from the AIoT CN to the AIoT RAN, may include the following: (1) AIoT Device Identification (to find a single device, a group of devices, or all devices) Note 1: The definition of this identification is out of RAN3 scope. Editor’s Note 1: It is FFS whether AIoT RAN needs to interpret/store/process it. (2) Scope of inventory request (e.g., a certain area in which the inventory is to be triggered) Multiple individual AIoT Device IDs (one ID per device) can be provided to the AIoT CN via a single Inventory Report. Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

This paper further discusses the AIoT procedures and the content.

- Discussion

  - Procedures

A-IoT Inventory procedure

TR38.769 6.5.2.1 Candidate procedures for A-IoT Inventory for Topology 1 Figure 6.5.2.1-1: Message flow for A-IoT Inventory in Topology 1 1a. The A-IoT CN sends an Inventory request message to the A-IoT RAN node, taking into account, among others, the A-IoT transaction scope. 1b. The A-IoT RAN node allocates and coordinates the usage of A-IoT radio resources. 2. The A-IoT RAN node sends an Inventory response message to the A-IoT CN. NOTE 1: In step 2, the A-IoT RAN node may instead send an Inventory failure message to the A-IoT CN indicating that the inventory procedure could not be initiated towards the A-IoT device(s). 3. The A-IoT RAN node performs the inventory procedure towards the A-IoT device(s) over the A-IoT radio interface. 4a/4b. After receiving the inventory result from the A-IoT device(s), the A-IoT RAN node may send one or multiple Inventory reports towards the A-IoT CN including the received inventory result. NOTE 2: Steps 4a/4b may happen in parallel with Step 3 for different A-IoT devices.

Proposal 1: RAN3 to capture above content in 6.5.2.1 in TR38.769 into the WI.

Proposal 1a: RAN3 to agree the following messages/procedures for the support of AIoT Inventory.

- Class 1:  Inventory request procedure

- Inventory request message

- Inventory response message

- Inventory failure message

- Class 2: Inventory report procedure

- Inventory report message

A-IoT Command procedure

6.5.2.2 Candidate procedures for A-IoT Command in Topology 1 Figure 6.5.2.2-1: Message flow for A-IoT Command in Topology 1 1. The Inventory procedures are performed in step 1, which are the same as in Figure 6.5.2.1-1 in step 1-4. NOTE 1: Step 1 is performed for the "A-IoT Inventory and Command" case. Whether and under which conditions step 1 may be skipped in case of "Command-only" and which consequences this would have for the overall message flow depends on discussions led by SA2, SA3 and RAN2. NOTE 2: Whether and which way steps 2a and 2b may be part of step 1 is dependent on discussions led by SA2, SA3 and RAN2. 2a. The A-IoT CN node sends a Command Request message to the A-IoT RAN. 2b. The A-IoT RAN node coordinates the usage of A-IoT radio resources and allocates A-IoT radio resources for the A-IoT command session. 3. The A-IoT RAN node performs A-IoT command procedures towards the A-IoT device over the A-IoT radio interface. 4. The A-IoT RAN node sends a Command Response message to the A-IoT CN, if any command result is received from A-IoT device, the A-IoT RAN node may include the command result in the Command response message. NOTE 3: In step 4, the A-IoT RAN node may instead send a Command Failure message to the A-IoT CN indicating that the command procedure has failed.

In the step 2a in above figure, it is not clear whether one or more AIoT devices are involved in the command procedure, then the time duration is unexpected. So that the AIoT RAN also does not know when it can send the step 4 to the AIoT CN.

From AIoT CN’s point of view, it can configure a timer after sending step 2a to the AIoT RAN node, then before the timer deadline, AIoT CN can think this command procedure is still valid.

However, this method of timer configuration is not good method, because the AIoT CN shall firstly know whether the step 2a has been successfully sent to the RAN node, moreover, it is difficult for AIoT CN to configure a suitable timer, due to different radio condition, the step 3’s handling time interval will be different from different command procedure.

Refer to the AIoT Inventory procedure, it is better to introduce a similar method on command procedure. In detail, after receiving Command request message from AIoT CN, if radio resources can be allocated, the AIoT RAN shall immediately send Command response/failure message to AIoT CN, then after step 3, it sends the Command result message to the AIoT CN.

Figure 1: Modified message flow for A-IoT Command in Topology 1

Proposal 2: After receiving Command request message from AIoT CN and allocating corresponding radio resources, the AIoT RAN shall immediately send Command response/failure message to AIoT CN, then after step 3, it sends the Command result message to the AIoT CN after receiving the command result from AIoT device.

Proposal 2a: RAN3 to agree the following messages/procedures for the support of AIoT Command.

- Class 1:  Command request procedure

- Command request message

- Command response message

- Command failure message

- Class 2: Command result procedure

- Command result message

  - The content of Inventory request message

- correlation identifier

TR 23.700-13 V1.3.0 4). Constructs a request for an Inventory operation using the determined A-IoT Device Identification information page the AIoT Devices, and a correlation identifier for the AIOTF to correlate the inventory responses to the request. The Inventory request is routed to the Readers determined by the initial reader selection.

For RAN side, in case that multiple parallel inventory request procedures are initiated by AIoT-CN to the same reader, the reader needs to use different identifier to differentiate them.

Proposal 3.1: correlation identifier is needed.

- reader id

TR 23.700-13 V1.3.0 3. For the requested operation the AIOTF: 1). Performs initial reader selection by either: - The AF providing information to identify readers to include in the initial reader selection network. The information can identify multiple or an individual Reader. NOTE 1: Which readers are identified by the information from the AF is up to the network deployment, configuration or implementation. - If a single UE Reader ID is provided by the AF via the NEF for the operation, then that is used as the selected Reader. - If reader selection information or UE Reader ID is not provided, then how the AIOTF determines which readers to use is based on implementation. The AIOTF may be e.g. preconfigured with which readers to use, or take the requested target AIoT Devices last known location into account, etc. If no readers can be selected then the request is rejected.

Before initiating Inventory procedure, the AIoT CN shall know which gNB(s) has AIoT capability by e.g., OAM.

For the initial inventory procedure, the AIoTF has no idea of the association between reader and device(s), it shall send some duplicated Inventory request messages including the same device(s) to different readers.

After initial inventory procedure, AIoT CN can acquire that each device to be inventory/command, is controlled by which reader, i.e., the CN can establish/store association between each device and reader.

This association is included in device context, and stored in the CN, e.g., device context = {device id list, reader id}. The reader id can be RAN/gNB/BS/TRP id.

AIoTF shall establish, update and store AIoT Device context, which includes association between device id(s) and reader id.

Proposal 3.2: reader id is needed for AIoTF to initially select reader.

- area information

TR 38.769 (2) Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

Proposal 3.3: area information is needed for AIoTF to indicate a certain area in which the inventory is to be triggered.

- device id

TR 38.769 6.4.1.1 Inventory Inventory can be sent by the AIoT CN for a single device, or a group of devices, or all devices. The Inventory Request from the AIoT CN to the AIoT RAN, may include the following: (1) AIoT Device Identification (to find a single device, a group of devices, or all devices) Note 1: The definition of this identification is out of RAN3 scope. Editor’s Note 1: It is FFS whether AIoT RAN needs to interpret/store/process it.

We have already agreed that device id(s) are optional present in the Inventory request message. If it is NULL, it means that the selected reader shall inventory all of the AIoT devices.

Proposal 3.4: device identification is needed.

- Assistance information from AIOTF

For the device id included in the Inventory request message, some companies think the device id is transparent to reader. In this case, estimated Number of devices is needed, e.g., to enable CFRA, to assists resource allocation.

TR 23.700-13 V1.3.0 e. The AIOTF may provide the following assistance information to AIoT RAN/UE Reader: - AIoT service type (e.g. Inventory, Command); - approximate number of AIoT devices based on AF request; - approximate D2R message size based on AF request.

Incoming LS (R3-250069 / S2-2501241) from SA2 Q1: Is it feasible for CN to provide the following information to the reader? - the service type of A-IoT (e.g. inventory, command) - whether the service is targeted for a single or multiple devices; - the approximate number of target devices (if available). SA2 answer: CN can provide the information listed above to the reader if available.

Proposal 3.5: AIoT service type (e.g. Inventory, Command) is needed.

Proposal 3.6: Indication to indicate a single or multiple devices is needed.

Proposal 3.7: Approximate number of AIoT devices is needed.

Proposal 3.8: Approximate D2R message size is needed.

- Conclusion

Based on the above analysis, we provide the final proposal.

Proposal 1: RAN3 to capture above content in 6.5.2.1 in TR38.769 into the WI.

Proposal 1a: RAN3 to agree the following messages/procedures for the support of AIoT Inventory.

- Class 1:  Inventory request procedure

- Inventory request message

- Inventory response message

- Inventory failure message

- Class 2: Inventory report procedure

- Inventory report message

Proposal 2: After receiving Command request message from AIoT CN and allocating corresponding radio resources, the AIoT RAN shall immediately send Command response/failure message to AIoT CN, then after step 3, it sends the Command result message to the AIoT CN after receiving the command result from AIoT device.

Proposal 2a: RAN3 to agree the following messages/procedures for the support of AIoT Command.

- Class 1:  Command request procedure

- Command request message

- Command response message

- Command failure message

- Class 2: Command result procedure

- Command result message

Proposal 3: Inventory request message may include the following IEs.

- correlation identifier

- reader id

- area information

- device identification

- AIoT service type (e.g. Inventory, Command)

- Indication to indicate single or multiple devices

- Approximate number of AIoT devices

- Approximate D2R message size