3GPP TSG RAN WG3 Meeting #127	R3-250120

Athens, Greece, 17 – 21 Feb, 2025

Title: 	Discussion on reader selection

Source: 	ZTE Corporation

Agenda item:	16.2

Document for:	Discussion and Decision

- Introduction

In the RAN #106 plenary meeting, the WID (New Work Item: Solutions for Ambient IoT (Internet of Things) in NR) has approved in RP-243326.

The RAN3’s scope is as below.

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

This paper addresses to discuss the AIoT reader selection.

- Discussion

  - Background

TR38.769 Reader selection may need coordination between A-IoT RAN node and A-IoT CN. NOTE 5: How to perform reader selection needs further discussions. NOTE 6: In Topology 1, whether the AIoTF needs to be aware of the readers served by the A-IoT RAN node and their location and how this information is provided to the AIoTF needs further discussion.

TR 23.700-13 V1.3.0 The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work: 1. A new core network function is introduced to support Ambient IoT (e.g. AIOTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality. a. The AIOTF registers itself in the NRF with its NF profile, this is to enable the discovery of AIOTF instances e.g. by an NEF. The NF profile at least includes the AIOTF ID/address, NF type, information used to allow the NEF to discover AIOT instances, e.g. based on the target area information in AF request. Other details of the NF profile, if needed, will be completed in normative phase. b. For topology 1, the AIOTF selects AIOT RAN nodes and optionally a list of the BS readers. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN. NOTE 1: Providing the UE Reader list to the RAN only applies to the RRC-based option. For topology 1, NGAP is terminated at an AIOT RAN node. An AIOT RAN node that supports one or more BS readers, may report its supported BS reader information e.g., BS reader IDs and their corresponding service areas. The OAM may configure the AIOTF with BS Reader information including e.g., their corresponding service area when the BS Readers are integrated in the network. NOTE 2: Whether RAN3 will specify BS reader information (e.g. BS reader ID, service area) reporting to AIOTF over NGAP depends on RAN3 decision and if so SA2 may include this feature in SA2 specifications. NOTE 3: If the request from the AIOTF to the AIOT RAN does not include a BS reader list, then AIOT RAN can use all available BS readers. NOTE 4: The BS reader’s service area needs to coordinate with RAN WG(s). c. The AIOTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s). d. The AIOTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF. e. The AIOTF may provide the following assistance information to AIoT RAN/UE Reader: - AIoT service type (e.g. Inventory, Command); - approximate number of AIoT devices based on AF request; - approximate D2R message size based on AF request. NOTE 5: If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation. NOTE 6: The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3. NOTE 7: Further assistance information can be added during the normative phase and in cooperation with other WGs if necessary.

  - Candidate methods on AIoT reader selection

There are possible three methods to perform AIoT reader selection.

- Method 1: AIoTF selects reader, AIoT RAN is not involved into reader selection and obeys the selection.

- Method 2: AIoT RAN selects reader, AIoTF is not involved into reader selection

- Method 3: Both AIoTF and AIoT RAN select reader by coordination

    - Method 1

AIoTF knows that a set of AIoT devices are within an Inventory area, e.g., within a warehouse, meanwhile, it also knows that the warehouse is controlled by a certain AIoT reader, i.e., AIoTF has stored the area-reader pair.

AIoTF sends Inventory/command message AIoT RAN including selected AIoT reader. The AIoT RAN forwards the received Inventory/command message to the AIoT reader indicated by AIoTF.

Figure 1: AIoTF selects reader based on its stored association between AIoT reader and area

Observation 1:  For method 1, the AIoTF sends Inventory/command message including the selected AIoT reader, based on its stored association between AIoT reader and area information.

    - Method 2

AIoTF sends Inventory/command message to AIoT RAN including the area.

AIoT RAN selects one or more AIoT readers and then forwards the received Inventory/command message to the selected reader(s), taking radio resource into account.

Figure 2: AIoT RAN selects reader based on its stored association between AIoT reader and area

Observation 2:  For method 2, the AIoTF sends Inventory/command message including area information but excluding reader id, AIoT RAN select AIoT reader based on its stored association between AIoT reader and area, taking radio resource into account.

    - Method 3

AIoTF sends Inventory/Command message to AIoT RAN including suggested AIoT reader(s) and area information.

AIoT RAN finally selects the AIoT reader and forward the received Inventory/Command message to the selected reader, taking radio resource into account.

Figure 3: Both AIoTF and AIoT RAN select reader by coordination, based on their stored association between AIoT reader and area

Observation 3:  For method 3, both AIoTF and AIoT RAN shall store the association between reader and area.

Observation 3a: For method 3, based on their stored association, AIoTF sends Inventory/command message including the suggested AIoT reader and area information, then AIoT RAN finally select the AIoT reader taking radio resource into account.

  - Comparison among three candidate methods

In order to compare with above three candidate methods, RAN3 shall firstly consider the following issues.

Issue 1: Whether a certain AIoT device can be inventoried/commanded by more than one T1 reader?

Normally, the operator shall deploy only one T1 reader within a special Inventory area, then, when inventory/command request message is sent to AIoT RAN node, only one T1 reader can be selected in this area and is responsible to perform Inventory/command procedure.

Proposal 1: In a special AIoT handling area, RAN3 shall consider whether only one T1 reader is deployed and can be selected to perform AIoT procedure.

If only one T1 reader is deployed within a special AIoT area, it is not essential for AIoT RAN node to select reader based on radio resource.

Proposal 1a: If only one T1 reader deployed within a special AIoT handing area, AIoT RAN will not consider radio resource condition to select reader.

Proposal 1b: Assuming only one T1 reader is deployed within a special AIoT handling area, both AIoTF and AIoT RAN will select the same target reader regardless of radio resource condition.

Issue 2: Whether AIoTF can always know and store the association between reader and AIoT handing area?

The pre-condition for method 1 is that, AIoTF shall know and store the association between reader and AIoT handing area, so that, we shall know whether reader’s location information shall be always provided to the AIoTF.

In the scope of WID, it indicates that “Device location reporting at reader ID granularity”. It implies that the position of each reader shall be pre-acquired by AIoTF, otherwise, the AIoTF cannot locate AIoT device to satisfy the WI’s requirement.

However, according to the latest TR23.700-13, it means that reader id and its corresponding serving area are allowed to NOT be transferred from AIoT RAN to AIOTF.

An AIOT RAN node that supports one or more BS readers, may report its supported BS reader information e.g., BS reader IDs and their corresponding service areas.

Proposal 2: RAN3 shall consider whether AIoTF can acquire the position of each reader and can store the association between reader and AIoT handling area.

If AIoTF cannot pre-acquire the position of reader, it cannot know the association between reader and AIoT handling area, then method 1 is not workable.

Proposal 2a: If AIoTF cannot pre-acquire the position of each reader, method 1 is unworkable.

Proposal 2b: Assuming AIoTF can always acquire each reader’s position, method 1 is workable (i.e., AIoTF selects target reader based on the association between reader and AIoT handling area).

Issue 3: When down selecting above candidate methods, shall we consider T2 reader for future proof?

In the SI phase, we agreed that in an Inventory area, both T1 reader and T2 reader can be deployed.

For both method 2 and method 3, the AIoT RAN shall store the UE context, otherwise, the AIoT RAN cannot select T2 reader.

Both RAN1 and RAN2 are discussing the RRC states (IDLE/Inactive/Connected mode) of UE as reader, the AIoT RAN has no idea of IDLE UE as reader, then it cannot select IDLE UE as target reader. On the other hand, the AIoTF can store all of the UE context, it can select both T1 reader and T2 reader.

Proposal 3: When down selecting candidate methods, RAN3 shall decide whether to consider T2 reader for future proof, especially the IDLE UE as reader?

If considering T2 reader (e.g., IDLE UE as reader) for future proof, both method 2 and method 3 are unworkable.

Proposal 3a: Assuming to consider T2 reader (e.g., IDLE UE as reader) for future proof, method 2 and method 3 are unworkable (i.e., AIoT RAN cannot select target reader if IDLE UE as reader).

  - Compromised method

In our view, in the further release, the UE as T2 reader will be consider, for the IDLE UE or for the UP based solution, only the AIoTF can select reader, so we have the following proposal.

Proposal 4:  RAN3 to agree Method 1 in this R19 WI, i.e., the AIoTF sends Inventory/command message including the selected AIoT reader, based on its stored association between AIoT reader and area information.

Additional, we have agreed to transfer area information from AIoTF to AIoT RAN, as below.

TR38.769 The Inventory Request from the A-IoT CN to the A-IoT RAN, may include the following: (1) A-IoT Device Identification (to find a single device, a group of devices, or all devices) NOTE 1: The definition of this identification is out of RAN3 scope. For topology 1, A-IoT RAN needs to store the A-IoT Device Identification received from the inventory request. NOTE 2: Whether to interpret and process AIoT Device Identification received from the inventory request needs further discussion. (2) Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

Based on above agreement, and considering only T1 reader in R19 WI, we can use a compromised method, as below.

Upon reception of Inventory request message, the AIoT RAN shall:

1) if reader id is included, forward the Inventory request message to the indicated reader;

2) if scope of inventory request is included, select the target reader, then forward the Inventory request message to this reader selected by AIoT RAN.

Proposal 4a: Otherwise, RAN3 to agree the following method.

Upon reception of Inventory request message, the AIoT RAN shall:

1) forward the Inventory request message to the reader selected by AIoTF, if reader id is included; or

2) select the target reader, then forward the Inventory request message to this reader selected by AIoT RAN, if scope (i.e., area information) of inventory request is included but reader id is not included.

- Conclusion

Based on the above analysis, we provide the following observations and proposals.

Observation 1:  For method 1, the AIoTF sends Inventory/command message including the selected AIoT reader, based on its stored association between AIoT reader and area information.

Observation 2:  For method 2, the AIoTF sends Inventory/command message including area information but excluding reader id, AIoT RAN select AIoT reader based on its stored association between AIoT reader and area, taking radio resource into account.

Observation 3:  For method 3, both AIoTF and AIoT RAN shall store the association between reader and area.

Observation 3a: For method 3, based on their stored association, AIoTF sends Inventory/command message including the suggested AIoT reader and area information, then AIoT RAN finally select the AIoT reader taking radio resource into account.

Proposal 1: In a special AIoT handling area, RAN3 shall consider whether only one T1 reader is deployed and can be selected to perform AIoT procedure.

Proposal 1a: If only one T1 reader deployed within a special AIoT handing area, AIoT RAN will not consider radio resource condition to select reader.

Proposal 1b: Assuming only one T1 reader is deployed within a special AIoT handling area, both AIoTF and AIoT RAN will select the same target reader regardless of radio resource condition.

Proposal 2: RAN3 shall consider whether AIoTF can acquire the position of each reader and can store the association between reader and AIoT handling area.

Proposal 2a: If AIoTF cannot pre-acquire the position of each reader, method 1 is unworkable.

Proposal 2b: Assuming AIoTF can always acquire each reader’s position, method 1 is workable (i.e., AIoTF selects target reader based on the association between reader and AIoT handling area).

Proposal 3: When down selecting candidate methods, RAN3 shall decide whether to consider T2 reader for future proof, especially the IDLE UE as reader?

Proposal 3a: Assuming to consider T2 reader (e.g., IDLE UE as reader) for future proof, method 2 and method 3 are unworkable (i.e., AIoT RAN cannot select target reader if IDLE UE as reader).

Proposal 4:  RAN3 to agree Method 1 in this R19 WI, i.e., the AIoTF sends Inventory/command message including the selected AIoT reader, based on its stored association between AIoT reader and area information.

Proposal 4a: Otherwise, RAN3 to agree the following method.

Upon reception of Inventory request message, the AIoT RAN shall:

1) forward the Inventory request message to the reader selected by AIoTF, if reader id is included; or

2) select the target reader, then forward the Inventory request message to this reader selected by AIoT RAN, if scope (i.e., area information) of inventory request is included but reader id is not included.