3GPP TSG-RAN WG3 Meeting #127	R3-250129

Athens, GR, 17-21 Feb, 2025	Revision of R3-247402

Title: 	(TP to BLCR for TS 38.473) Discussion on on-demand SSB SCell operation

Source: 	Huawei

Agenda item:	17.2

Document Type:	 Other

# 1. Introduction

Previous RAN3-126 meeting discussed the on-demand SSB SCell operation, with the following agreement.

- For RRC based signalling, the DU can decide activation/deactivation state of OD-SSB configuration per SCell and convey it to the CU included in the RRC container. There is no RAN3 impact.

And the following issues were discussed but without conclusions.

- The gNB-CU indicates the allowed cell list for on-demand SSB to gNB-DU?

- Whether the MAC CE based on-demand SSB indication should be notified to CU?

In this contribution, we provide our considerations on on-demand SSB SCell operation, based on RAN1 and RAN2 progress.

# 2. Discussion

## 2.1 Case 1 – No Always-on SSB Transmission

The operation of on-demand SSB SCell involves three potential triggering methods as detailed in [1]:

- Using an existing signal or channel for UE uplink wake-up signals,

- Cell on/off status communicated via backhaul,

- SCell activation/deactivation signaling.

For Case #1, the SSB is not continuously transmitted. When the SSB is not transmitted, this Cell can be regarded as deactivated cell due to the energy saving by the neighbouring nodes. According to the current XnAP specification, the procedure for such cell activation/deactivation via backhaul is already supported, allowing an NG-RAN node to request the activation of a cell by a neighbouring NG-RAN node. For the case discussed, the existing cell activation message can be re-used without changes to the specification.

With regards to the SMTC in case the SCell SSB broadcast status changes, RAN3 can be pending to RAN2 further progress, then decide if there is any impact.

For Case #1, with no always-on SSB transmission, RAN3 to agree no additional modifications to the Xn specification for cell (de)activation.  With regards to the SMTC or L3 measurement, RAN3 can be pending to other WG progress.

For the CU/DU split architecture, the general on-demand SSB procedure over F1 is illustrated in Figure 1 below.

Figure 1: The general on-demand SSB procedure

In our view, the DU itself can decide whether and when to broadcast SSB transmission as well as its broadcast periodicity. For example, when the SCell load is low, the DU could transit into the on-demand SCell state and stop SSB broadcast. Conversely, it can exit the on-demand SSB Cell state and return to the normal Cell state.

On the other hand, given that the CU has a general system view, it is beneficial for the CU to provide Cells allowed to be in on-demand SSB operation to the DU especially considering the on-demand SSB broadcast could be quite dynamic.  This approach aligns with the R18 inter-node beam activation, where the CU can provide a list of cells allowed to be deactivated.

Companies also discussed whether to reuse the existing Cells Allowed to be Deactivated List IE introduced in R18 to support the function of OD-SSB SCell operation. The advantage is that no more standard work is needed to introduce a new IE. But there may have backward compatibility issue – the DU may be confused to know either to initiate inter-node beam activation or on-demand SSB SCell operation or both. In this sense, a separate allowed cell list is preferred.

The CU can send the allowed cells for on-demand SSB operation to the DU. And for those indicated allowed cells, it is up to the DU to determine whether and when to broadcast SSB transmission as well as broadcast periodicity.

Secondly, for the RRC and MAC CE based signaling to indicate on-demand SSB transmission, below provides the latest progress in RAN1 and RAN2. Also, previous RAN3 meeting agreed that it is up to to DU to decide the activation/deactivation state of OD-SSB configuration for RRC based signalling.

Agreement@RAN1#119 For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 FFS: Whether the value of N can be implicitly determined using a timer Agreement at RAN2#127bis meeting 1. No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. 2. Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. 3. NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

For MAC CE based signalling, the DU can send the MAC CE to the UE. Since such signalling is dynamic to trigger the activation/deactivation of OD-SBB transmission, it could be the DU to decide the activation/deactivation of OD-SSB transmission.

For MAC CE based signalling, it is the DU to decide to send the On-demand SSB activation (even the deactivation) MAC CE to the UE.

Due to the fact that the OD-SSB transmission could be used for synchronization, cell measurement, SCell activation etc in different scenarios. The notification to CU should be discussed by considering different scenarios’ requirement. For example, if the OD-SSB transmission is for L3 measurement, there is a need to notify CU so that the CU may be anticipated to receive L3 measurement results from the UE.

When the DU sends on-demand SSB MAC CE to the UE, the CU should be notified by the DU (e.g., CU may be anticipated to receive L3 measurement results from the UE).

- In addition, when the DU sends the on-demand SSB activation MAC CE to the UE, RAN3 can discuss how the DU decides and sends the SCell activation MAC CE to the UE. One possible way is to rely on the L1 measurement based on on-demand SSB, as agreed in RAN1 to use periodic, semi-persistent, and aperiodic L1 measurement reports by using existing CSI framework. Another way is to rely on the L3 measurement result. In this case, the DU may need the assistance from the CU to decide whether to activate the SCell or not.

RAN3 to discuss how the DU decides the SCell activation based on the L1 measurement and/or the L3 measurement result after sending the on-demand SSB activation MAC CE.

## 2.2 Case 2 - Always-on SSB is periodically transmitted on the cell

For Case #2, the always-on SSB is periodically transmitted on the SCell. From RAN3 perspective, this cell can be considered as a normal working cell. For example, this SCell can be already activated for one UE while being deactivated for another. Also, RAN1 agreed that the always-on SSB could be multiplexed with on-demand SSB as follows.

Agreement at RAN1-118bis meeting For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain Agreement at RAN1#119 Response to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?) of Obj.1: - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Further update to be made based on RAN1#119 progress. Agreement at RAN1#119 Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1: - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

Over Xn interface, neighbouring NG-RAN nodes can treat this cell as a normal served cell since there is always on SSB transmission, i.e., without deactivation indication. As a result, there is no need to activate such cell. The difference of AO-SSB and OD-SSB mainly are the time/frequency locations, for which RAN1 continues its discussion.

Similarly, over F1 interface, the CU can treat this cell as an activated and “in service” cell. Similar to case 1, RAN3 can wait for RAN2 further progress of the SMTC when the SSB periodicity is changed.  For the exact F1 impact, it is suggested that RAN3 can focus on the case1 first, then decide if any agreements can be applied to case 2 as well.

For case #2 (Always-on SSB is periodically transmitted on the cell) on-demand SSB, RAN3 to agree that no Xn specification impact for cell (de)activation.  With regards to the SMTC or L3 measurement, RAN3 can be pending to other WG progress. The rest of F1 impact can be considered after case 1.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

Case#1: No always-on SSB transmission

- For Case #1, with no always-on SSB transmission, RAN3 to agree no additional modifications to the Xn specification for cell (de)activation.  With regards to the SMTC or L3 measurement, RAN3 can be pending to other WG progress.

The CU can send the allowed cells for on-demand SSB operation to the DU. And for those indicated allowed cells, it is up to the DU to determine whether and when to broadcast SSB transmission as well as broadcast periodicity.

For MAC CE based signalling, it is the DU to decide to send the On-demand SSB activation (even the deactivation) MAC CE to the UE.

When the DU sends on-demand SSB MAC CE to the UE, the CU should be notified by the DU (e.g., CU may be anticipated to receive L3 measurement results from the UE).

RAN3 to discuss how the DU decides the SCell activation based on the L1 measurement and/or the L3 measurement result after sending the on-demand SSB activation MAC CE.

- Case#2: Always-on SSB is periodically transmitted on the cell

For case #2 (Always-on SSB is periodically transmitted on the cell) on-demand SSB, RAN3 to agree that no Xn specification impact for cell (de)activation.  With regards to the SMTC or L3 measurement, RAN3 can be pending to other WG progress. The rest of F1 impact can be considered after case 1.

- The TP for TS 38.473 to implement the Proposal 2 is provided in Annex.

# 4. Reference

- RP-241650 Revised WID: Enhancements of network energy savings for NR, Ericsson

- Chair notes RAN1#117 eom4.

# 5. TP for TS 38.473

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the Cells Allowed to On-Demand SSB List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to initiate on-demand SSB for the indicated cells for network energy saving purpose.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List | O |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. | YES | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | ignore |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cells Allowed to On-Demand SSB List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be On-Demand SSB List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>