3GPP TSG-RAN WG3 Meeting #127	R3-250150

Athens, Greece, 17 – 21 February 2025

Agenda item:	16.3

Source:	Xiaomi

Title:	Discussion on location of the device

Document for:	Other

# 1	Introduction

According to the discussion in study phase in [1], device location inference by the AIoTF relies on gNB ID and reader ID (to be defined), combined with reader location awareness. This contribution addresses the following issues:

- Reader definition

- AIoTF knowledge of gNB ID

- AIoTF knowledge of reader location

# 2	Discussion

Reader definition

An AIoT RAN comprises geographically distributed antenna sets (readers), where each reader corresponds to a Transmission/Reception Point (TRP). An AIoT TRP is defined as:

- A set of co-located antennas covering a distinct area.

- Supporting AIoT transmission/reception functionalities.

Observation 1, The TRP of a reader can be regarded a set of geographically co-located antennas supporting AIoT Transmission Point and/or AIoT Reception Point functionalities.

Proposal 1, Reuse TRP ID to uniquely identify a reader with in a gNB.

AIoTF Knowledge of gNB ID

The mechanism varies by architecture:

| Architecture | gNB ID Acquisition |
| --- | --- |
| Direct Path | AIoTF obtains gNB ID during NG setup. |
| Indirect Path | AIoTF selects gNB via AMF/OAM, with implicit gNB awareness. |

It seems that no additional information is required in subsequent communications, as the association is pre-established.

Observation 2, for both direct path or indirect path, explicit inclusion of gNB ID in operational messages (e.g., inventory reports) is redundant, as the AIoTF inherently identifies the gNB through prior procedures.

Proposal 2, there’s no need to introduce explicit gNB ID in the inventory report or command response message.

AIoTF Knowledge of Reader Location

The reader location mapping can be realized in AIoTF via:

- OAM pre-reconfiguration

- Reader information exchange procedures

Figure 1 device location information

The possible call flows can be as below:

1. AIoTF obtains the reader information via reader information exchange.

2-4. AIoTF selects a TRP (reader) for AIoT operations.

5. TRP ID is included in AIoT response/report messages (e.g., inventory reports).

6. AIoTF maps TRP ID to pre-acquired location data.

Proposal 3, RAN3 agree to include TRP ID in the inventory response message, inventory report message and command response message.

# 3	Conclusion

Based on the above discussion, we have the following observations and proposals.

Observation 1, The TRP of a reader can be regarded a set of geographically co-located antennas supporting AIoT Transmission Point and/or AIoT Reception Point functionalities.

Proposal 1, Reuse TRP ID to uniquely identify a reader with in a gNB.

Observation 2, for both direct path or indirect path, explicit inclusion of gNB ID in operational messages (e.g., inventory reports) is redundant, as the AIoTF inherently identifies the gNB through prior procedures.

Proposal 2, there’s no need to introduce explicit gNB ID in the inventory report or command response message.

Proposal 3, RAN3 agree to include TRP ID in the inventory response message, inventory report message and command response message.

# 4	Reference

[1] TR 38.769: Study on Ambient IoT Solutions.

# 5	Annex discussion reference from TR 38.769

## 6.9	Locating ambient IoT devices

### 6.9.1	General

A-IoT device location information may be used for the following purposes:

(a) improving the A-IoT operation itself, e.g., by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the A-IoT service.

Locating an Ambient IoT device at "reader ID granularity" is useful for both purposes and is to be supported.

NOTE: "reader ID granularity" is likely not sufficient for purpose (b) and may be refined in the future.

### 6.9.2	Topology 1

For support of A-IoT device location, the A-IoT RAN node may report the A-IoT RAN node ID to the A-IoT CN in the Inventory Report or Command Response messages.

The A-IoT RAN node may additionally report a Reader ID to the A-IoT CN in the Inventory Report and/or the Command Response messages, which corresponds to the reader to which the A-IoT device responded.

NOTE 1: Whether multiple Reader ID(s) can be included by the A-IoT RAN node in the Inventory Report/Command Response message needs further discussion.

NOTE 2: Definition and format of the A-IoT RAN node ID and Reader ID needs further discussion.

NOTE 3: Whether and how to have awareness of Reader location in A-IoT CN needs further discussion.