3GPP TSG-RAN WG3 Meeting #127	R3-250154

Athens, Greece, 17 – 21 February 2025

Agenda item:	15.2

Source:	Xiaomi

Title:	Support of full eNB as regenerative payload

Document for:	Discussion

# 1	Introduction

In this contribution, we’d like to discuss how to support full eNB as regenerative payload for IoT NTN according to the latest WID [1]

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

The following topics are discussed:

- Without S&F satellite operation

- S&F satellite operation

# 2	Discussion

### Without S&F satellite operation

For the case that eNB on board without store and forward operation, it’s possible for the eNB onboard to have S1 connection with the MME on ground, when the feeder link is not available e.g., due to the satellite moving, the S1 connection between them may not be available, similar to NG connection management, it’s proposed to introduce S1 removal procedure for this case.

Regarding the stage 2 specification in TS 36.300, the content specified in TS 38.300 can be used as reference, currently, the NR stage 2 is not stable, the LTE stage2 can wait until the NR stage 2 is stable.

Proposal 1, RAN3 agree to introduce S1 removal procedure for full eNB as regenerative payload.

### S&F satellite operation

According to RAN3 agreement “RAN3 decides to work on Split MME architecture and Full CN on board architecture. Whether eNB only on board architecture is feasible or not can be discussed later.”, for split MME or full CN onboard architecture, if the gNB and CN part are deployed in a same satellite, the S1 interface signalling can be implementation, RAN3 can conclude that there’s no RAN3 impacts to support this scenario.

Observation 1, if gNB and CN part are deployed in a same satellite, the S1 interface is an internal interface, the signalling can be satellite implementation.

Proposal 2, RAN3 agree that there’s no RAN3 impact if eNB and CN part are deployed in a same satellite.

One possible deployment option is that the onboard eNB and CN parts are deployed on different satellites within the same orbit, the distance and connection between the CN and eNB are relatively stable, communicating via inter-satellite links as shown in Figure 1.

Figure 1 example arch for eNB and CN in different satellites

if the onboard eNB and onboard CN deployed in different satellites, we believe that the SA2’s solution can be used and there will be no additional SA2 impacts. One consideration is that the eNB needs to be aware of whether the MME on the other satellite has an available feeder link and whether store and forward (S&F) operation is supported, so that it can decide whether to serve UE or whether to broadcast the store and forward operation to UE.

The following scenarios may exist (assuming the eNB and CN can communicate via inter-satellite link):

Case 1: The eNB has no feeder link, while the MME has a feeder link. The eNB can provide normal service to the UE.

Case 2: Both the eNB and CN have no feeder link, and both support S&F. The eNB can broadcast an S&F indication to the UE.

Case 3: The eNB has a feeder link and can connect to the ground CN, allowing the eNB to provide normal service to the UE.

It is evident that the eNB’s behaviour may vary depending on the feeder link status of both the eNB and the MME. Therefore, it would be beneficial for the eNB to be aware of the feeder link status of the CN on other satellites.

Proposal 3, RAN3 discuss whether to support the deployment option that the onboard eNB and onboard CN are on different satellites but in same orbit.

Proposal 4, if the scenario that onboard eNB and onboard CN are deployed on different satellites is supported, eNB needs to be aware of the feeder link status of CN on other satellites.

# 3	Conclusion

Based on the discussion, we have the following observations and proposals.

### without S&F satellite operation

Proposal 1, RAN3 agree to introduce S1 removal procedure for full eNB as regenerative payload.

### S&F satellite operation

Observation 1, if gNB and CN part are deployed in a same satellite, the S1 interface is an internal interface, the signalling can be satellite implementation.

Proposal 2, RAN3 agree that there’s no RAN3 impact if eNB and CN part are deployed in a same satellite.

Proposal 3, RAN3 discuss whether to support the deployment option that the onboard eNB and onboard CN are on different satellites.

Proposal 4, if the scenario that onboard eNB and onboard CN are deployed on different satellites within the same orbit is supported, eNB needs to be aware of the feeder link status of CN on other satellites.

# 4	Reference

[1] RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

# 5	Annex (reference from TS 23.401)

### 4.13.9	Support of Store and Forward Satellite Operation

#### 4.13.9.1	General

The Store and Forward Satellite Operation applies in E-UTRAN with satellite access and is suitable for delay-tolerant communication services (e.g. CIoT/MTC, SMS, etc.). If the satellite does not support Store and Forward Satellite operation and there is no feeder link connection to the ground network, then the satellite cannot provide any service to any UEs. When both the service link and feeder link are available, all services can be provided. To support Store and Forward Satellite operation, some network functionality needs to be deployed on the satellite payload.

Example deployments are described in Annex O.

Figure 4.13.9.1-1: Store and Forward (S&F) Satellite operation

NOTE 1:	In the Figure 4.13.9.1-1, the NTN GW belongs to the satellite deployment and is not subject to 3GPP specifications.

In Store and Forward Satellite operation, the end-to-end exchange of signalling/data traffic is handled in a sequence of steps reflecting the intermittent availability of the service link when the satellite can exchange data with the UE and of the intermittent availability of the ground link when the satellite can exchange data with the core network. This is depicted in Figure 4.13.9.1.1.

For example, a simple sequence of events for the transmission of data from the UE to a server on the ground may involve two steps: firstly, when service link is available (e.g. location L1 in Figure 4.13.9.1.1) the UE sends the data to the satellite. Subsequently (e.g. location L2 in Figure 4.13.9.1.1), the satellite carrying the payload connects to the ground network and delivers the UE data to the network.

Downlink data can be stored onto the same or a different satellite and provided to the UE later using the first step and the process is repeated.

When a satellite supports Store and Forward Satellite operation and is operating in S&F Mode, the on-board eNodeB broadcasts an indication of operating in S&F Mode as described in TS 36.300 [5], which the UE uses to determine the current operation mode of the satellite. If a satellite is operating in S&F Mode and if a UE is enabled for Store and Forward Satellite operation, then the UE indicates the S&F capability during Attach and Tracking Area Update.

NOTE 2:	Some deployments cannot support all system features, for example they cannot support user plane establishment and user plane data transfer, when operating in S&F Mode in the split MME deployment described in Annex O.

The MME may adapt periodic update timer, mobile reachable timer and Implicit Detach timer to the fact that the UE is served in S&F mode.

Editor's note:	How to handle the specification of the "List of satelliteID(s)" and "S&F Monitoring List" as per conclusions in TR 23.700-29 [96] is FFS.

When an MME is operating in S&F Mode:

-	If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, if the UE supports Store and Forward Satellite operation, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation.

-	If a UE indicates Store and Forward capability, the MME may provide to the UE a S&F Wait Timer when accepting or rejecting a NAS procedure.

NOTE 3:	How the MME determines the S&F Wait Timer is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, etc.

When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause.

If the UE did not indicate Store and Forward capability in NAS signalling, if the network determines to reject the UE, the network shall reject the UE request with a cause non-specific to S&F Mode.

The MME may indicate to UE the Uplink S&F estimated delivery time in NAS messages (Attach accept or TAU accept message or service accept). How UE uses this information is left for UE implementation.

NOTE 4:	The Uplink S&F estimated Delivery time is the estimated/expected time required to deliver the data to the ground network element from the time the data is sent by the UE.

The MME may trigger a Update Location procedure with the HSS along with the authentication procedure to fetch subscription information from the HSS. The MME may indicate the timestamp information to HSS during the Update Location procedure. This timestamp information shall be used by the HSS to ensure that newer location for that UE is not cancelled. If MME makes an Update Location Request before the completion of the authentication procedure, it shall include an indication that this Location Update is provisional, i.e. the HSS shall not consider the UE as registered until it receives the final Update Location Request (without indication that the Location Update is provisional). The timestamp information is the time when the on-board of satellite MME part has received the NAS procedure from the UE. The HSS compares the timestamp received in the Update Location Request with any stored timestamp of a previous Update Location Request and determines whether to accept or reject the request. If the received Update Location Request does not include a timestamp, the HSS assumes the present time as the timestamp of the received Update Location Request. The HSS shall reject the Update Location Request, if the timestamp associated with this request is older than the stored timestamp. If the HSS accepts the Update Location Request, the HSS shall store the timestamp associated with the latest Update Location Request. If the HSS does not support the timestamp, the MME shall reject a UE Attach request for S&F Mode.

NOTE 5:	The timestamp information is used by the HSS to ensure that interactions with MMEs for a UE are handled in the correct order.

NOTE 6:	The absence of timestamp in the Update Location Request can occur e.g. when the UE is connecting from a network that does not operate in S&F mode.

The EPS may expose whether a UE is in S&F Mode and provide to the SCS/AS related timing information to guide the SCS/AS decision when to try to contact the UE. This is further described in clause 5.6.3.10 of TS 23.682 [74].