3GPP TSG-RAN WG3 Meeting #127	R3-250155

Athens, Greece, 17 – 21 February 2025

Agenda item:	20.2

Source:	Xiaomi

Title:	(TP to BL CR for TS 38.455) Support of gNB-side model (case 3a)

Document for:	Other

# 1	Introduction

In this contribution, we’d like to discuss the data collection of ground truth label info and performance moniotring for case 3a based on the following progress in RAN3 and provied the corresponding TP in the Annex.

Case 3a: The AI/ML model (inference and training functions) is at the gNB. LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1. WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. RAN3#126 Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed. FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1). FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified. Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used. FFS if LMF should be aware that the reported measurements are AI/ML generated. Check progress in RAN1 and solution discussion…

### Key Issues for Case 3a

- Data Collection for Ground Truth Labels

  - Challenges in UE selection and correlating input data (Part A: channel measurements) and labels (Part B: ground truth).

- LMF Awareness of AI-Assisted Positioning

  - Need for LMF to control or be informed about AI-generated measurements.

# 2	Discussion

## Data Collection for Ground Truth Labels

RAN3 discussed data collection for the gNB-side model to support case 3a, i.e., gNB-assisted positioning. Companies provided various solutions with different understandings. Before discussing the detailed messages for data collection, we should focus on the basic positioning procedures and issues to establish a common understanding of the purpose of data collection.

For case 3a, the AI is applied to UL related positioning in gNB side, the following call flow in TS 38.305 illustrates the UL only positioning as an example, it can be noted that the gNB/TRP involved in the positioning may use AI to generate the intermediate positioning measurements, i.e., in step 7. In other words, the gNB that perform AI-assisted positioning are not only the serving gNBs of the measured UE, but also the neighbour gNBs. Thus, for AI training data collection, the gNB that has AI training data collection requirement should also collect the data from UE in the neighbours for better model generalization.

Figure 8.13.3.4-1: UL-TDOA positioning procedure

Observation 1, gNBs with AI assisted positioning capability require data from UEs in neighboring gNBs, not just their own served UEs.

When we have above positioning pictures in mind, the following issues can be further discussed for AI training data collection.

1. Scope of UE data collection

As we discussed above, the AI training is for the gNB that participate the UL related positioning, multiple TRPs around the UE will perform the measurements, not only the TRP from the serving cell but also the TRP from the neighbours. For gNB with AI capability to provide UL positioning measurements, it can collect data not only from the UEs served by itself but also from the UEs served by the neighbours.

2. UE Selection Mechanism

If we refer to LMF-side model for data collection as shown in Annex C, it should be noted that the UE(s) whose data can be collected are either PRU or UE(s) with user consent. The PRU(s) are known by LMF, and the user consent info is stored in UDM, if LMF wants to collect data, it checks whether UE has user consent with UDM. Before that, LMF may inquiry AMF for the candidate UE list in a specific area.

For gNB-side model, we think the same principles should be followed, i.e., the UE whose data can be collected should either be PRU or has user consent, and LMF is the entity to perform the UE selection by using the same mechanism as LMF-side model data collection.

3. Correlating Part A and Part B Data

For AI model training, shown in the figure 1, there are multiple data sets to be used as training data input. One data set includes the part A parameters (i.e., channel measurement related info) and the part B parameters (i.e., ground truth label related info), and the part A paraments and part B parameters in one data set should be correlated from UE and timing perspective, considering the part A are from TRP, part B are from LMF, RAN3 needs to discuss how to correlate part A parameters and part B parameters.

Figure 1 AI/ML model training

According to UL related positioning procedures, part A data are configured by LMF via measurement procedures. The simplest way is that the part B info can be provided at the same time when configuring the measurement for part A. An alternative way is configuring measurement for part A and providing part B in different messages, gNB can use time stamps and transaction ID in each message to corelate the data.

Proposal 1, RAN3 discuss the following issues for data collection:

- Scope of UE Data Collection.

- UE Selection Mechanism

- Correlating Part A and Part B Data

Proposal 2, gNB with AI capability to provide UL positioning measurements is able to collect data not only from the UEs served by itself but also from the UEs served by the neighbours.

Proposal 3, LMF should select UEs (PRUs or consented UEs) using the same mechanism as LMF-side models.

Proposal 4, Part B (ground truth label) can be provided at the same time when configuring the measurement for part A (measurements) for data correlation.

Back to our previous discussion, there’re mainly three solutions discussed.

Solution 1, AMF is involved in data collection in [1].

In this solution, the UE is selected by gNB, before UE selection, either the LMF proactively provides the list of UEs or NG-RAN request available UEs, the UEs provided by the CN is already verified. i.e., the UEs either are PRU or UE(s) with user consent. But this solution can only collect the data of UEs served by the NG-RAN node, which limit the generalization of the collected data, thus the performance of the trained model will be affected, as it’s very common in UL positioning that a gNB measures the signal of UE from neighbours. Another drawback of this solution is that it has too much impacts on AMF.

Solution 2, Xn-like data collection over NRPPa in [2].

The same as solution 1, the UE is selected by gNB, only the data of the UEs served by the gNB can be collected, which limit the generalization of the collected data.

Solution 3, enhancement on existing positioning procedures

For solution 3, if gNB wants to collect data for training, it indicates the needs to the LMF. There may be multiple gNB in a desired area want to collect training data. The figure above shows an example that gNB 1 and gNB 2 want data collection. LMF can have the comprehensive requirements from all gNB(s), it performs the UE selection considering the UE information and TRP information in step 1, same mechanism as in LMF-side data collection is used, LMF can select PRU or UE with user consent, and the LMF initiates UL related positioning procedures by reusing existing mechanism, E.g. LMF may initiate UL SRS configuration for the selected UE in step 2, the selected UE may be or may be not served by the gNB that collects data. LMF sends measurement request including the ground truth label for the gNB(s) indicate the request for data collection, the gNB perform the measurement to obtain part A, and then correlate part A and part B for model training.

We think solution 3 is more efficient with less spec impacts and follows the same principle of existing positioning mechanism.

Observation 2, in solution 1 and solution 2, the gNB can only collect data from the UE it serves, which may impact the performance of the trained model as they limit the generalization of training data.

Observation 3, solution 1 and solution 2 have much more spec impacts than solution 3.

Observation 4, in solution 3, LMF aggregates gNB requests, select UEs and triggers UL-SRS configurations. One UE’s data can be used for multiple gNBs’ training, it not only supports better generalization on data collection, but also improve the resource efficiency for data collection.

Proposal 5, Solution 3 is recommended for its efficiency, minimal spec impact, and alignment with existing positioning frameworks. RAN3 is invited to adopt the proposed TP in the Annex for Case 3a.

## Performance Monitoring

RAN3 agrees that NG-RAN node performs monitoring metric calculation for its own model. For performance data collection, if label-based performance monitoring is used, the NG-RAN node needs to collect ground truth label from LMF, we think the same mechanism as training data collection can be used.

Observation 4, performance monitoring data includes input data, output data from gNB, and ground truth label from LMF, where the same mechanism as training data collection can be used.

Proposal 6, for ground truth label-based performance monitoring, same procedure for training data collection can be used.

## LMF Awareness of AI

FFS if LMF should be aware that the reported measurements are AI/ML generated.

In our understating, the LMF should be aware of whether AI is used for the measurement results (at least for LOS/NLOS, UL RTOA, gNB Tx-Rx time difference), as LMF knows the LCS requirements, LMF can decide wither to use AI or not. We think the following extent awareness of LMF can be considered

- LMF Controls AI Usage: Aligns with LMF’s role as positioning manager.

- gNB Control AI usage with Notification: Not aligned with LMF’s role as the positioning manager; LMF cannot override decisions.

We think option 1 is more algin with LCS principle, that LMF is the manager of the positioning, option 2 only notify LMF, but LMF can do nothing.

Proposal 7, RAN3 agree that AI-based positioning measurement should be aware by LMF

## RAN split architecture

If RAN split architecture is supported, we think the AI model training and inference can be deployment in gNB-CU, which is aligned with AI/ML for NG-RAN.

Proposal 8, if RAN split architecture is supported, the AI/Model training, inference and management should be deployed in gNB-CU.

# 3	Conclusion

## Data collection for ground truth label

Observation 1, gNBs with AI assisted positioning capability require data from UEs in neighboring gNBs, not just their own served UEs.

Proposal 1, RAN3 discuss the following issues for data collection:

- Scope of UE Data Collection.

- UE Selection Mechanism

- Correlating Part A and Part B Data

Proposal 2, gNB with AI capability to provide UL positioning measurements is able to collect data not only from the UEs served by itself but also from the UEs served by the neighbours.

Proposal 3, LMF should select UEs (PRUs or consented UEs) using the same mechanism as LMF-side models.

Proposal 4, Part B (ground truth label) can be provided at the same time when configuring the measurement for part A (measurements) for data correlation.

Observation 2, in solution 1 and solution 2, the gNB can only collect data from the UE it serves, which may impact the performance of the trained model as they limit the generalization of training data.

Observation 3, solution 1 and solution 2 have much more spec impacts than solution 3.

Observation 4, in solution 3, LMF aggregates gNB requests, select UEs and triggers UL-SRS configurations. One UE’s data can be used for multiple gNBs’ training, it not only supports better generalization on data collection, but also improve the resource efficiency for data collection.

Proposal 5, Solution 3 is recommended for its efficiency, minimal spec impact, and alignment with existing positioning frameworks. RAN3 is invited to adopt the proposed TP in the Annex for Case 3a.

## Performance Monitoring

Observation 4, performance monitoring data includes input data, output data from gNB, and ground truth label from LMF, where the same mechanism as training data collection can be used.

Proposal 6, for ground truth label-based performance monitoring, same procedure for training data collection can be used.

## Awareness of AI by LMF

Proposal 7, RAN3 agree that AI-based positioning measurement should be aware by LMF

## RAN split architecture

Proposal 8, if RAN split architecture is supported, the AI/Model training, inference and management should be deployed in gNB-CU.

# 4	Reference

[1] R3-247459, AI/ML based positioning accuracy enhancements (Qualcomm Incorporated)

[2] R3-247578, Discussion on NRPPa signalling design for AI/ML for PHY (Ericsson)

# 5	Annex B	Text Proposal to BL BR (Solution 3)

<<<<<<<<<<<<<<<<<<<< Change Start >>>>>>>>>>>>>>>>>>>>

### 8.2.8	TRP Information Exchange

#### 8.2.8.1	General

The purpose of the TRP Information Exchange procedure is to allow the LMF to request the NG-RAN node to provide detailed information for TRPs hosted by the NG-RAN node. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.8.2	Successful Operation

Figure 8.2.8.2-1: TRP Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a TRP INFORMATION REQUEST message. The NG-RAN node responds with a TRP INFORMATION RESPONSE message that contains the requested TRP information.

If the TRP List IE is included in the TRP INFORMATION REQUEST message, the NG-RAN node should include in the TRP INFORMATION RESPONSE message, the requested information for all TRPs included in the TRP List IE.

If the TRP List IE is not included in the TRP INFORMATION REQUEST message, the NG-RAN node should include the requested information for all TRPs hosted by the NG-RAN node in the TRP INFORMATION RESPONSE message

If the PRS Muting IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account as the muting information for the given PRS resource set.

If the QCL Info IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account for the given PRS resource list.

If the DL-PRS Resource Coordinates IE is included in the Geographical Coordinates IE in the TRP Information IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account as the DL PRS Resource Coordinates relative to the TRP coordinate.

If the Mobile IAB-MT UE ID IE is included in the TRP Information IE in the TRP INFORMATION RESPONSE message, the LMF shall, if supported, use this information to determine an updated location of the Mobile TRP as specified in TS 23.273 [20].

If the TRP Information Type Item IE is set to 'mobile trp location info', the NG-RAN node shall, if supported, derive the location of the Mobile TRP as specified in TS 23.273 [20] and include the Mobile TRP Location Information in the TRP INFORMATION RESPONSE message.

If the Data Collection Available Indication IE is included in the TRP INFORMATION REQUEST message and the NG-RAN node identifies a requirement for data collection, the NG-RAN node shall include the Data Collection Required IE in the TRP INFORMATION RESPONSE message to inidate its data collection requirement to the LMF.

If the Data Collection Required IE is included in the TRP Information IE in the TRP INFORMATION RESPONSE message, the LMF may initiate procedures for the required data  collection UE selection and provide the required data to the NG-RAN node.

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

#### 9.1.1.14	TRP INFORMATION REQUEST

This message is sent by an LMF to request information for TRPs hosted by an NG-RAN node.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP List |  | 0 ..1 |  |  | YES | ignore |
| >TRP Item |  | 1 .. <maxnoTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>PRS Bandwidth Aggregation Request Indication | O |  | ENUMERATED(true, …) |  | YES | ignore |
| TRP Information Type List |  | 1 |  |  | YES | reject |
| >TRP Information Type Item |  | 1 .. <maxnoTRPInfoTypes> |  |  | EACH | reject |
| >>TRP Information Type Item | M |  | ENUMERATED (nr pci, ng-ran cgi, nr arfcn, prs config, ssb config, sfn init time, spatial direction info, geo-coordinates, …, trp type, on-demand prs, trp tx teg, beam antenna info, mobile trp location info, common ta) |  | - |  |
| Data Collection Available Indication | O |  | ENUMERATED(ground truth label, …, FFS) |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

#### 9.1.1.15	TRP INFORMATION RESPONSE

This message is sent by an NG-RAN node to convey TRP information to an LMF.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP Information List |  | 1 |  |  | YES | ignore |
| >TRP Information Item | M | 1 .. <maxnoTRPs> |  |  | - |  |
| >>TRP Information | M |  | 9.2.25 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| Data Collection Required | O |  | ENUMERATED(ground truth label, …, FFS) |  | YES | ignore |

Editor note: FFS on whether gNB provide more information for data collection requirements.

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

## 8.5	Measurement Information Transfer

### 8.5.1	Measurement

#### 8.5.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall include the Measurement Beam Information IE in the TRP Measurement Result IE of the MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF may take it into account as the TRP estimate of the measurement quality. If the Measurement Quality IE includes the Zenith Quality IE, the LMF may take it into account within the angle measurement quality.

If the Timing Reporting Granularity Factor IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring measurements including UL RTOA and gNB Rx-Tx Time Difference.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, return the corresponding measurement results in the MEASUREMENT RESPONSE message within the indicated time.

If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account as the number of SRS measurement time occasions for a measurement instance.

If the Time Window Information Measurement List IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, measure the UL SRS resources from the UE within the indicated time window(s).

If the Label Information IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use the information for gNB-side model life cycle management, .e.g, model training and performance monitoring.

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5 and so on, see TS 38.133 [16] | - |  |
| >>Ground Truth Label Information | O |  | FFS |  |  |  |

<<<<<<<<<<<<<<<<<<<< Change End >>>>>>>>>>>>>>>>>>>>

# 6	Annex C	 Reference(data collection for LMF side model from TS 23.273 S2-2410978)

### 6.x.3	Data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements

The procedure for data collection from NG-RAN for LMF-based AI/ML positioning is used to e.g. train the AI/ML Model.

Figure 6.x.3-1: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements

- The LMF determines that data collection is required e.g. to train an AI/ML Model for UE positioning for a number of UEs or to monitor the AI/ML Model performance. The LMF may also initiate the data collection upon the request of an NWDAF containing MTLF as described in step 3 in figure 6.x.4-1.

- The LMF may know the SUPI of these UEs, e.g. when training a AI/ML model using a PRU, which are associated to this LMF that executes data collection, the LMF may optionally invoke an Nnrf_NFDiscovery Request service operation to an NRF in order to identify the LMF(s) which has PRU associated to a list of TAIs. The LMF may send an Nlmf_Location_MeasurementData Request service operation to one or more of the other PRU serving LMFs in order to get the data collection and the known PRU location in this case step 6 follows.

If the LMF does not know the SUPIs of those UEs, the LMF try to get the list of SUPIs from the AMF, before that the LMF finds the AMFs via NRF using Nnrf_NFDiscoveryRequest.

- For each AMF serving a specific area. the LMF subscribes to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

- The AMF send Namf_EventExposure_Subscriber_Response or Namf_EventExposure_Notify ("list of SUPIs in/out the area of interest").

For each SUPI provided by the AMF that is in the area of interest then steps 5 to step 6 are performed.

- The LMF checks whether the SUPI provided user consent for data collection for the purpose of training with UDM. If user consent is provided, then step 6 follows. if user consent is not provided, the LMF does not perform step 6 for this UE, that will not be part of the data collection, and its data is not used for the LMF-based AI/ML Model positioning.

Editor's note:	Further details on user consent for data collection for model training or for performance monitoring for LMF-based AI/ML positioning will be aligned with SA WG3.

- The LMF request input data for the target UE from NG-RAN.

- The LMF receives ground truth data from the UE.

The measurements and ground truth data are used for training. The UE location is derived from the measurements data by using LMF-based AI/ML Positioning. The derived UE location and ground truth data are used for performance monitoring.

The LMF may initiate data collection for multiple UEs simultaneously, as such step 8 and 9 may occur in paralell for a number of SUPIs as determined by the LMF.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

Editor's note:	Whether and how the LMF takes the RAN load into account in this procedure is FFS.

### 6.x.3	Data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements

The procedure for data collection from NG-RAN for LMF-based AI/ML positioning is used to e.g. train the AI/ML Model.

Figure 6.x.3-1: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements

- The LMF determines that data collection is required e.g. to train an AI/ML Model for UE positioning for a number of UEs or to monitor the AI/ML Model performance. The LMF may also initiate the data collection upon the request of an NWDAF containing MTLF as described in step 3 in figure 6.x.4-1.

- The LMF may know the SUPI of these UEs, e.g. when training a AI/ML model using a PRU, which are associated to this LMF that executes data collection, the LMF may optionally invoke an Nnrf_NFDiscovery Request service operation to an NRF in order to identify the LMF(s) which has PRU associated to a list of TAIs. The LMF may send an Nlmf_Location_MeasurementData Request service operation to one or more of the other PRU serving LMFs in order to get the data collection and the known PRU location in this case step 6 follows.

If the LMF does not know the SUPIs of those UEs, the LMF try to get the list of SUPIs from the AMF, before that the LMF finds the AMFs via NRF using Nnrf_NFDiscoveryRequest.

- For each AMF serving a specific area. the LMF subscribes to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

- The AMF send Namf_EventExposure_Subscriber_Response or Namf_EventExposure_Notify ("list of SUPIs in/out the area of interest").

For each SUPI provided by the AMF that is in the area of interest then steps 5 to step 6 are performed.

- The LMF checks whether the SUPI provided user consent for data collection for the purpose of training with UDM. If user consent is provided, then step 6 follows. if user consent is not provided, the LMF does not perform step 6 for this UE, that will not be part of the data collection, and its data is not used for the LMF-based AI/ML Model positioning.

Editor's note:	Further details on user consent for data collection for model training or for performance monitoring for LMF-based AI/ML positioning will be aligned with SA WG3.

- The LMF request input data for the target UE from NG-RAN.

- The LMF receives ground truth data from the UE.

The measurements and ground truth data are used for training. The UE location is derived from the measurements data by using LMF-based AI/ML Positioning. The derived UE location and ground truth data are used for performance monitoring.

The LMF may initiate data collection for multiple UEs simultaneously, as such step 8 and 9 may occur in paralell for a number of SUPIs as determined by the LMF.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

Editor's note:	Whether and how the LMF takes the RAN load into account in this procedure is FFS.