3GPP TSG-RAN WG3 Meeting #127	R3-250170

Athens, Greece, 17th – 21st February 2025

Agenda Item:		11.3

Source:		NEC

Title:	(TP to BLCR TS38.423 and TS38.473) Open issues for AI/ML-based CCO

Document for:		Discussion and decision

# Introduction

For CCO, the following two timing information were introduced:

Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1.

Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node config Update message over Xn.

In RAN3#126 meeting, CR to TS 38.423 and CR to TS 38.473 [2] were agreed to exchange Future Coverage Modification List over Xn and Future Coverage Modification Notification and predicted CCO assistance information over F1 respectively.

However, two issues regarding to whether to send time information related to coverage modification cause over Xn and what UE performance feedback can be used for CCO are still FFS. Besides, IE values are still FFS.

In this contribution, we further discuss the abovementioned open issues.

# Discussion

2.1	Time information over Xn

In XnAP NG-RAN NODE CONFIGURATION UPDATE message, a Time for Future Coverage State IE can be included for each Future Coverage Modification Item. As explained in the semantics description, this time information indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the NG-RAN node1 relative to the time of receiving this information [1].

As for timing information for predicted CCO issue, it indicates the time when the predicted CCO issue will happen in the gNB who performed this prediction. gNB uses this timing information and the predicted CCO issue to generate the corresponding future coverage configuration and timing information for future coverage state in legacy way or using AI/ML model. If in CU/DU split, gNB-CU will send this timing information together with the predicted CCO issue to gNB-DU, which can assist gNB-DU to select corresponding future coverage configuration and time for future coverage modification.

As described above, timing information for predicted CCO issue is used to generate corresponding future coverage configuration and time for future coverage state. For neighbour nodes, this timing information for predicted CCO issue is useless. The neighbour nodes only need to know when the future coverage state(s) will be applied, which can allow the neighbour nodes to adapt to this CCO coverage change.

Proposal 1: Timing information for predicted CCO issue is NOT needed to exchange over Xn.

2.2	UE performance feedback for CCO

UE performance feedback is used to evaluate the AI/ML model by comparing UE performance before and after a CCO action that is triggered based on the output from AI/ML Model Inference.

CCO is to optimize the network by adjusting cell coverage due to coverage and capacity problems. For AI/ML based CCO, since the predicted CCO issue can be avoided or mitigated by advance CCO action triggered by the AI/ML Model Inference output, UE performance should be not (much) impacted due to the coverage adjustment, especially for the UE in the cell border area.

As specified in TS38.423, legacy UE performance measurements collected by gNB includes Average UE throughput DL, Average UE throughput UL Average packet delay and Average packet loss DL. gNB should get and evaluate UE performance for UEs that may be affected by the CCO action, e.g. UEs in the cell border area. Therefore, UE location information, if available, is suggested to be reported together with UE performance in the DATA COLLECTION UPDATE message, or neighbour gNBs perform UE selection based on UE location and only report the UE performance of the UEs in the cell border area.

Proposal 2: Legacy UE performance measurement metrics can be reused for CCO.

Proposal 3: RAN3 to discuss the following two options for UE performance feedback for CCO:

- To report UE location information together with UE performance in the DATA COLLECTION UPDATE message.

- Neighbour gNBs perform UE selection based on UE location and only report the UE performance of the UEs in the cell border area.

2.3	IE values

In last meeting, we agreed that Time interval for predicted CCO issue and future CCO state can be in the granularity of seconds. But the maximum value for the time interval is still FFS.

Considering this time interval is one of the predicted information, we suggest to follow the defined value for Requested Prediction Time IE, i.e. 1 to 60 seconds.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |

Proposal 4: The maximum value of the Time interval for predicted CCO issue and future CCO state is 60 seconds.

Regarding the new added Future Cell Coverage State IE and Future SSB Coverage State IE, the IE format is referred to the legacy Cell Coverage State IE. The legacy IE can be used to notify a neighbour gNB about dynamic coverage configuration switch among the configured coverage configurations, in which no coverage (inactive) is included. However, the purpose of CCO function is to provide optimal coverage and capacity to address insufficient coverage or capacity problems, and considering this, value ‘0’ which indicates that the cell or SSB will be inactive is not applicable for CCO.

Proposal 5: To remove value ‘0’ for Future Cell Coverage State IE and Future SSB Coverage State IE.

TP to TS38.423 and TS38.473 respectively, mirroring the above proposals, are available in Annex.

Proposal 6: To agree the TP to TS38.423 and TP to TS38.473 in Annex.

# Proposals

In this contribution, we provide our analysis on open issues for AI/ML-based CCO and made the following proposals:

Proposal 1: Timing information for predicted CCO issue is NOT needed to exchange over Xn.

Proposal 2: Legacy UE performance measurement metrics can be reused for CCO.

Proposal 3: RAN3 to discuss the following two options for UE performance feedback for CCO:

- To report UE location information together with UE performance in the DATA COLLECTION UPDATE message.

- Neighbour gNBs perform UE selection based on UE location and only report the UE performance of the UEs in the cell border area.

Proposal 4: The maximum value of Time interval for predicted CCO issue and future CCO state is 60 seconds.

Proposal 5: To remove value ‘0’ for Future Cell Coverage State IE and Future SSB Coverage State IE.

Proposal 6: To agree the TP to TS38.423 and TP to TS38.473 in Annex.

# Reference

[1] R3-247928, (BL CR to TS 38.423) Support of enhancements on AI/ML for NG-RAN

[2] R3-247929, (BL CR to TS 38.473) Support of enhancements on AI/ML for NG-RAN

# Annex TP to BLCR TS38.423

<Start of 1st change>

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| TAI Support List | O |  | 9.2.3.20 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| Local NG-RAN Node Identifier Removal | O |  | Local NG-RAN Node Identifier 9.2.2.101 |  | YES | ignore |
| Future Coverage Modification List |  | 0..1 |  | List of cells whose coverage will be modified. | YES | ignore |
| >Future Coverage Modification Item |  | 0..<maxnoofCellsinNG-RAN node> |  |  | - |  |
| >>Global NG-RAN Cell Identity | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of the cell whose coverage will be modified. | - |  |
| >>Future Cell Coverage State | M |  | INTEGER (1..63, ...) | Indicates the future coverage configuration of the concerned cell. | - |  |
| >>Future SSB Coverage Modification List |  | 0..1 |  | List of SSB beams whose coverage will be modified. | - |  |
| >>>Future SSB Coverage Modification Item |  | 0..<maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB index | M |  | INTEGER (0..63) | Identifier of the SSB beam whose coverage will be modified. | - |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (1..15, ...) | Indicates the future coverage configuration of the concerned SSB beams. | - |  |
| >>Predicted Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the reason for the predicted coverage modification in NG-RAN node1. | - |  |
| >>Time for Future Coverage State | O |  | INTEGER (1..60, …) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the NG-RAN node1 relative to the time of receiving this information, in seconds. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the NG RAN nodes. Value is 32. |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |
| maxnoofNeighbourNG-RAN nodes | Maximum no. of neighbour NG-RAN nodes. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifCellDeploymentStatusIndicatorPresent | This IE shall be present if the Cell Deployment Status Indicator IE is present. |

<End of 1st change>

<Start of 2nd change>

### 9.3.5	Information Element definitions

<<<<<<<<<<<<<<<<<<<< Unchanged part omitted>>>>>>>>>>>>>>>>>>>>

#### -- F

Future-Coverage-Modification-List ::= SEQUENCE (SIZE (0..maxnoofCellsinNG-RANnode)) OF Future-Coverage-Modification-Item

Future-Coverage-Modification-Item ::= SEQUENCE {

globalNG-RANCell-ID						GlobalCell-ID,

future-cellCoverageState				INTEGER (1..63, ...),

future-SSB-Coverage-Modification-List	Future-SSB-Coverage-Modification-List		OPTIONAL,

predicted-Coverage-Modification-Cause	Predicted-CoverageModificationCause			OPTIONAL,

timeForFutureCoverageState				INTEGER (1..60, ...),						OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Future-Coverage-Modification-Item-ExtIEs} }	OPTIONAL,

...

}

Future-Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Future-SSB-Coverage-Modification-List ::= SEQUENCE (SIZE (0..maxnoofSSBAreas)) OF Future-SSB-Coverage-Modification-Item

Future-SSB-Coverage-Modification-Item ::= SEQUENCE {

sSBIndex						INTEGER(0..63),

future-SSBCoverageState			INTEGER (1..15, ...),

iE-Extension					ProtocolExtensionContainer { { Future-SSB-Coverage-Modification-Item-ExtIEs} }	OPTIONAL,

...

}

Future-SSB-Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<End of 2nd change>

# Annex TP to BLCR TS38.473

<Start of 1st change>

#### 9.3.1.A	Predicted CCO Assistance Information

This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO issue | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the type of predicted CCO issue. |
| Predicted Affected Cells and Beams | O |  | 9.3.1.212 |  |
| Time for Predicted CCO Issue | O | INTEGER (1..60, ...) |  | Indicates the time when the predicted CCO issue will happen from the time of receiving this information, in seconds. |

#### 9.3.1.B	Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by a gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Future Coverage Modification Notification List |  | 1 |  |  |
| >Future Coverage Modification Notification Item |  | 1..<maxCellingNBDU> |  |  |
| >>NG CGI | M |  | 9.3.1.12 |  |
| >>Future Cell Coverage State | M |  | INTEGER (1..63, ...) | Indicate the future coverage configuration of the concerned cell. |
| >>Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (1..15, ...) | Indicates the future coverage configuration of the concerned SSB beams. |
| >>Time for Future Coverage Modification | O |  | INTEGER (1..60, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum numbers of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

<End of 1st change>

<Start of 2nd change>

### 9.4.5	Information Element Definitions

<<<<<<<<<<<<<<<<<<<< Unchanged part omitted>>>>>>>>>>>>>>>>>>>>

#### -- T

TimeforFutureCoverageModification ::= INTERGER (1..60, ...)

TimeforPredictedCCOIssue ::= INTEGER (1..60, ...)

<End of 2nd change>