3GPP TSG-RAN WG3 Meeting #127	R3-250171

Athens, Greece, 17th – 21st February 2025

Agenda Item:		11.4

Source:		NEC

Title:			Continuous MDT collection for AI/ML

Document for:		Discussion and decision

# Introduction

Regarding continuous MDT collection, two problems for continuous MDT collection, as below, were identified in SI and captured in the TR38.743 [1].

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

In last meeting, RAN3 agreed the following requirement for Continuous MDT functionality:

In R19, introducing the Continuous MDT functionality with a solution being applicable to legacy UEs and not only to Rel-19 UEs.

Based on the above agreement, this contribution further discusses the above two issues, the possible solutions and the corresponding impacts on RAN3 specification.

# Discussion

According to TR37.817 [2] and TR38.743 [1], the following data need to be collected from UE for AI/ML:

- UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

- UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

- UE Mobility History Information

In order to limit the impact on UE power consumption and processing, continuous collection and logging of UE measurement should as much as possible rely on the measurements that are already supported in UE and so we only consider to continuous collect the above data.

## 2.1	Problem A (measurement continuity)

We already agreed that “Continuous collection of MDT traces is beneficial only for AI/ML training in OAM”. Training data is the initial dataset used to train machine learning algorithms on how to make predictions. In order to increase the precision of a ML model, the process of training an ML model needs large volume of input data and a rather long training time. And the provision of training data is not time critical.

As in legacy, immediate MDT can be configured to UE for measurements in RRC Connected, while logged MDT in – RRC Idle and RRC Inactive. Regarding to the gap during RRC state transition, it is a extremely short period of time comparing to the long training time and the measurements during the transition can be negligible.

Besides, the collected raw data may still need to be processed and translated into usable information for training by data processing technology, which can also handle the sparse missing data issue, due to e.g. RRC state transition or other reasons.

Therefore, we think that, for AI/ML training, the measurement continuity during the same RRC state can be guaranteed in legacy way and the measurement discontinuity during RRC state transition can be negligible.

Observation 1: Measurement continuity during the same RRC state can be guaranteed in legacy way.

Observation 2: Measurement discontinuity during RRC state transition is extreme short and can be negligible.

## 2.2	Problem B (trace correlation)

RAN3 sent an LS to SA5 [3] to enquire whether S-TMSI as a “Measured UE Identifier” in TS28.558 that is included in all the UE level measurements can be used to identify which UE sends the measurement report and to do the trace correlation in TCE.

In the reply LS [4], SA5 clarified that

“So, the “Measured UE Identifier” parameter specified in TS28.558 is not applicable to NG-RAN UE level (MDT) measurements. Therefore, SA5 has agreed to add a clarification in TS28.558 that the measurement bullet g) is only applicable for management-based activation of 5GC UE level measurement collection, and the bullet g) in NG-RAN UE level measurements are set to “N/A” instead of “S-TMSI…”

Besides, SA5 also mentioned that

“SA5 agree that there is a benefit to have a (temporary) identifier in the management-based MDT measurement report. This identity shall be unique within the lifetime of the measurement session. So, some data analytics function (e.g., AIML related function) can have a better understanding that the collected data sets are measured from the same source. Therefore, it would be good to provide some kind of identifier to indicate the measurement was done in the same or different UE.”

As confirmed by SA5, an identifier that is unique within the lifetime of the measurement session is needed to indicate the measurement was done in the same or different UE.

In last meeting, one solution, i.e. using TR and TRSR for trace correction was proposed. However, Trace Reference and Trace Recording Session Reference are configured only for Logged MDT, and so gNB needs to recover these information from Logged MDT reports if available and attach it in the Immediate MDT records from the same UE. This brings complexity to gNB and may not be feasible if no Logged MDT report available.

Therefore, to address this issue, we prefer OAM or AMF to allocate a temporary UE identifier for continuous MDT collection. For example, OAM can allocate to gNB a temporary UE identifier for Continuous MDT collection in Trace Session Activation message, or AMF can allocate it to gNB after receiving its Cell Traffic Trace message. Then gNB includes this temporary UE identifier in the MDT records and reports to TCE. TCE uses this temporary UE identifier included in MDT records to combine MDT records from a same UE.

This UE temporary identifier can also be included in logged MDT measurement configuration, which to implicitly request UE to perform continuous MDT, and then UE will continue the measurements across RRC states and even during RRC states transition.

Proposal 1: OAM or AMF allocates a temporary UE identifier for continuous MDT collection, and gNB includes it in MDT records to TCE for trace correlation.

As we agreed in last meeting, “RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN”, we suggest to send an LS to SA5 and to inform them this agreement and ask them to support the solution for continuous MDT collection, which needs OAM e.g. to allocate a temporary UE identifier or to introduce a new Job Type value for continuous MDT, etc.

## 2.3 User consent

Comparing to legacy MDT, continuous MDT collection needs to set new requirements for UEs, e.g. continuous measurements collection capability, UE memory size, high energy consumption, etc, and users need to be asked and confirm the use of this function. Therefore, a new user consent is preferred to be used for continuous MDT or for specific use cases or specific UE measurements of continuous MDT.

Proposal 2: A new user consent is needed for continuous MDT collection.

A draft LS to SA5 capturing the abovementioned issues is included in Annex.

Proposal 3: To agree the LS to SA5 on continuous MDT collection in Annex.

# Proposals

In this contribution, we provide our further analysis on continuous MDT collection for AI/ML and the following observation and proposals were drawn from the analysis.

Observation 1: Measurement continuity during the same RRC state can be guaranteed in legacy way.

Observation 2: Measurement discontinuity during RRC state transition is extreme short and can be negligible.

Proposal 1: OAM or AMF allocates a temporary UE identifier for continuous MDT collection, and gNB includes it in MDT records to TCE for trace correlation.

Proposal 2: A new user consent is needed for continuous MDT collection.

Proposal 3: To agree the LS to SA5 on continuous MDT collection in Annex.

# Reference

[1] TR38.743, Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

[2] TR37.817, Study on enhancement for Data Collection for NR and EN-DC

[3] R3-243941, Reply LS to SA5 on improved KPIs involving end-to-end data volume transfer time analytics, RAN3

[4] R3-245021, Reply LS on improved KPIs involving end-to-end data volume transfer time analytics, SA5

# Annex LS to SA5

3GPP TSG-RAN WG3 Meeting #127	R3-25xxxx

Athens, Greece, 17th – 21st February 2025

Title:	[draft] LS on Continuous MDT collection

Response to:

Release:	Rel-19

Work Item:	NR_AIML_NGRAN_enh

Source:	NEC (to be RAN3)

To:	SA5

Cc:

Contact person:	Hui Ma

hui.ma@emea.nec.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

In the Rel-18 and Rel-19 AI/ML for NG-RAN, RAN3 discussed how to support continuous MDT collection targeting the same UE across RRC states.

RAN3 reached the following agreements:

The existing MDT framework is used as baseline for data collection from the UE.

Continuous collection of MDT traces is beneficial only for AI/ML training in OAM. Continuous MDT collection is to enable the continuous collection of MDT data from the same UE across RRC state changes (RRC_Connected, RRC_Idle, RRC_Inactive).

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

And two problems for continuous MDT collection, as below, were captured in the TR38.743.

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

To address the above issues, some proposed enhancements may have OAM impacts. However, RAN3 lacks knowledge on whether such designs are possible and seeks to understand the following questions:

Question 1: For continuous MDT, whether OAM can allocate a temporary UE identifier for UE to includes in measurement reports for trace correlation in TCE?

Question 2: As assumed by RAN3 that OAM configures Management Based MDT for Continuous MDT towards the NG-RAN, whether a new Job Type value for continuous MDT needs to be defined?

Question 3: Comparing to legacy MDT, continuous MDT collection sets new requirements for UEs, e.g. continuous measurements collection capability, UE memory size, high energy consumption, etc. Whether a new user consent is needed for continuous MDT or for specific use cases or specific UE measurements of continuous MDT?

ACTION: RAN3 kindly asks SA5 to provide answers to the questions above.

# 2	Actions

To SA5:

ACTION: RAN3 kindly asks SA5 to provide answers to the questions above.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#127bis		2025-04-07 - 2025-04-11		China, CN

RAN3#128		2025-05-19 - 2025-05-23		Malta, MT