3GPP TSG-RAN WG3 Meeting #127	R3-250174

Athens, Greece, 17th – 21st February 2025

Agenda Item:		12.3

Source:		NEC

Title:			Open issues for NR Femto

Document for:		Discussion and decision

# Introduction

In RAN3#126 meeting, three CRs [1, 2, 3] were agreed to introduce NR Femto architecture, functional split, protocol stacks and access control in TS38.300, necessary procedure changes in TS 38.413 and NNSF by NR femto GW in TS38.410.

In this contribution, we further discuss the open issues for NR femto, including functional split, local service, security and mobility.

# Discussion

## 2.1	Functional split

In TS38.300, additional functions hosted by a NR Femto comparing to a gNB is included, but functions for NR Femto GW and AMF are still missing, therefore, we propose to add the possible functions for NR Femto GW and AMF listed as below to TS 83.300.

The NR Femto GW, if deployed, hosts the following functions:

-	Relaying UE-associated NG application part messages between the AMF serving the UE and the NR Femto serving the UE.

-	Terminating non-UE associated NG application part procedures towards the NR Femto and towards the AMF.

NOTE:	If a NR Femto GW is deployed, non-UE associated procedures shall be run between NR Femtos and the NR Femto GW and between the NR Femto GW and the AMF.

-	Optionally terminating NG-U interface with the NR Femto and with the UPF.

-	Supporting TAC and PLMN ID used by the NR Femto.

-	Xn interfaces shall not be established between the NR Femto GW and other nodes.

-	Routing the PATH SWITCH REQUEST message towards the AMF based on the GUAMI of the source AMF received from the NR Femto.

The AMF hosts the following functions in addition to legacy functions:

-	Access control for UEs that are members of CAG.

-	Providing the NG-RAN node with the list of CAGs allowed for the UE and, whether the UE is allowed to access non-CAG cells.

-	In case of a NR Femto directly connected:

- verifying that the identity used by the NR Femto is valid when receiving the NG SETUP REQUEST message;

-	The AMF may send two transport layer addresses of different versions only in case of NR Femto GW which does not terminate user plane.

Proposal 1: To capture the abovementioned functions hosted by NR Femto GW and AMF in TS38.300.

## 2.2	Local services

As concluded in TR38.799, local services support by NR femto can be achieved by reusing LADN and edge computing functionality defined in SA2 and will have no RAN impact. However, in TS38.300, one separate section to describe this item is still essential. Therefore, we suggest to capture the following sentence in a sub-session under 4.X Support of NR Femtos in TS38.300.

“In order to support access to local services, NR Femtos reuse LADN and edge computing functionality as specified in TS 23.501 [x] and TS 23.548 [y].

The local UPF may be either stand-alone or co-located with the NR Femto node.”

Proposal 2: To add one separation session for local services support under session 4.X Support of NR Femtos in TS38.300, capturing the description above.

## 2.3	Security aspects

As agreed in last meeting, there is no need to introduce definitions of “open”, “closed”, and “hybrid” access mode, but just to reference existing definitions and specification for access control with CAG. Instead, to realized “hybrid” like mode, the NR Femto may activate a cell shared by both PLMN and PNI-NPN, and this cell is accessible to both UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell and legacy UEs not supporting CAG. Therefore, no access mode check is needed in AMF.

Regarding verification of that the identity used by the NR Femto and verification of UE authorization to request local service in NR Femto, SA3 needs to be involved to check whether the verifications can be supported. Besides, we also need to consult SA3 whether there is no security issue if NR Femto activates a cell shared by both PLMN and PNI-NPN.

Proposal 3: LS SA3 on security issues listed below:

- Verification of that the identity used by the NR Femto by AMF.

- Verification of UE authorization to request local service in NR Femto by AMF.

- Any security issue if NR Femto activates a cell shared by both PLMN and PNI-NPN.

## 2.4	Mobility optimization

5G Femto is analogous to the LTE concept of HeNB and is deployed to provide NR access at home or at enterprise premises. Considering large scale femto deployment, the same problem for measurement configuration for handover as in HeNB still exists.

While the UE is in RRC_CONNECTED state, the UE performs normal measurement and mobility procedures based on configuration provided by the network. But UEs cannot continuously preform measurements and SI acquisition of a large number of femto cells. In order to allow the UE to perform measurements efficiently and to support proper handover to a NR femto, it is preferred to introduce a proximity mechanism to let the gNB know that the UE is entering or leaving the proximity of one or more CAG member cells and configure UEs in an efficient way.

Proposal 4: RAN3 to study a proximity mechanism for NR Femto.

In the agreed CR [1], a Note is captured to explain how to use CAG mechanism to realize hybrid access as below:

-	The NR Femto may activate a cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

In legacy, during the handover procedure, if a UE is tried to handover to a CAG member cell but the membership check is failed, AMF or target NR femto shall reject this handover preparation procedure and inform the source node with a cause. However, if the target cell is a shared cell as the abovementioned new deployment scenario we just agreed, which can accept a UE as either a member or a non-member, so in this case, we think the AMF or the target NR femto can directly decide whether to continue handover preparation procedure with an acknowledgement message instead of a failure message, which can increase the handover success rate and improve UE experience.

Proposal 5: When handover a UE to a shared cell, even if the membership check for the UE is failed, AMF or target NR femto can accept the UE as a non-member and continue the handover preparation procedure with an acknowledgement message.

## 2.2	DC support

As RAN3 agreed that an NPN cell can be activated in a NR Femto, and considering in legacy DC is supported for PNI-NPN and may involve both PNI-NPN and PLMN cells, NR Femto can also support DC, e.g. a NR Femto cell can be used as a PCell or can be added as a SCell.

Proposal 6: DC can be supported in NR Femto.

# Proposals

In this contribution, we provided considerations on functional split, protocol stacks and access control, and the following proposals were drawn:

Proposal 1: To include the description on functional split and protocol stacks for NG User Plane and Xn User/Control Plane in section 2.1 and section 2.2 to BLCR TS38.300.

Proposal 2: Whether to define “open, hybrid and closed access modes” can depend on operators’ deployment requirements.

Proposal 3: LS SA3 on security issues listed below:

- Verification of that the identity used by the NR Femto by AMF.

- Verification of UE authorization to request local service in NR Femto by AMF.

- Any security issue if NR Femto activates a cell shared by both PLMN and PNI-NPN.

Proposal 4: RAN3 to study a proximity mechanism for NR Femto.

Proposal 5: When handover a UE to a shared cell, even if the membership check for the UE is failed, AMF or target NR femto can accept the UE as a non-member and continue the handover preparation procedure with an acknowledgement message.

Proposal 6: DC can be supported in NR Femto.

# Reference

[1] R3-247936, (BL CR to 38.300) Introduction of NR Femto Architecture and Protocol Aspects

[2] R3-247938, (BL CR to TS 38.413) Support of NR Femto architecture with NR Femto Gateway

[3] R3-247937, (BL CR to 38.410 for Femto) Introduction of NR Femto in NGAP list of function