3GPP TSG-RAN WG3 Meeting #127	R3-250179

Athens, Greece, February 17th – 21st, 2025

Agenda item:		20.2

Title:		Discussion on AI/ML based Positioning Accuracy 		Enhancements

Source:		NEC

Document for:		Discussion and Decision

- Introduction

The work items pertaining on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface were officially approved in RAN #102 and updated in RAN #105. The details concerning the positioning use case after RAN #105 have been finalized as follows [1].

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Based on the previous progress, this contribution addresses the following topics:

- Signalling on data collection

- Signalling on model output

- LS for model performance monitoring of Case 3a

- Discussion

  - Signalling on data collection

For case 3a, the AI/ML model is deployed at gNB side, thus training data can be collected at gNB directly to train a model. However, the existing principle is NRPPa transaction should be initiated by LMF, and gNB cannot start an NRPPa procedure proactively. During last meeting, there trigger an issue of whether the gNB should be allowed to initiate the NRPPa procedure for the purpose of generating training dataset in Rel-19. After several rounds of discussions, there reached a WA as [2]:

WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

According to the existing principle, the positioning requirement is initiated by the AMF, UE, or some entity in 5GC(e.g., GMLC), rather than by the NG-RAN node. Subsequently, if the NG-RAN is to participate in the positioning procedure, the LMF will trigger the NRPPa procedure upon receiving this requirement. Similarly, this principle applies analogously for data collection: the LMF triggers the NRPPa procedure if LMF deems an NR-RAN-assisted/LMF-based positioning procedure is necessary. Furthermore, if the gNB determines that data collection is needed, it can request data collection procedure after LMF has triggered the NRPPa procedure, Consequently, the current principle remains valid without any change even if AI/ML model is employed for positioning. Therefore, this WA should be turned to agreement.

Confirm the WA that:

- The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

  - Signalling on model output

During last RAN3 meeting, there triggered an issue on whether LMF should be aware that the reported measurements are generated using AI/ML model, as well as the potential impact on further specification effect if this information should be known at LMF side.

- Option 1: dedicated IEs (e.g., AI/ML UL-RTOA) in the Measurement request and response message. - Option 2: new flag, AI/ML Positioning measurement ENUMERATED (true,…) in the TRP Measurement Result IE.

The similar issue was discussed in RAN1, and there is no agreement until now partly since the LCM procedure is unspecified currently. The same situation is faced in RAN3 also. For example, if LMF is the only entity to preform LCM operation, the LMF may be aware of whether the AI/ML is activated, or even know the reported output is derived from which model, thus it is unnecessary to report that the measurements is generated by AI/ML model. Therefore, this issue should be postponed until more discussion of LCM on RAN2 and RAN1.

If LMF knows whether AI/ML positioning is activated, LMF is also aware of that whether the reported measurement is generated by AI/ML.

Postpone the discussion of whether the specification needs to distinguish if the measurements are generated by AI/ML until more discussion on LCM operation in RAN2/RAN1.

  - LS for model performance monitoring of Case 3a

In the context of model monitoring of case 3a, RAN1 #118 has agreed two options for model performance metric calculation for label-based monitoring, and the final selection of Option A and Option B is pending RAN3 confirmation. The last meeting has agreed that NG-RAN node is the entity to perform monitoring metric calculation for case 3a in RAN3.

Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa.

This issue is triggered by RAN1, and it may relate to the determination of model monitoring for case 1. Therefore, an LS to RAN1 is necessary for subsequent work.

Send an LS to RAN1 to inform RAN3 agreement on model performance monitoring for case 3a.

- Conclusion

In this contribution, we discussed the issues of AI/ML for positioning accuracy enhancement. Observation and proposals are summarized as following:

Observation 1:	If LMF knows whether AI/ML positioning is activated, LMF is also aware of that the reported measurement is generated by AI/ML.

Proposal 1:	Confirm the WA that:

The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

Proposal 2:	Postpone the discussion of whether the specification needs to distinguish if the measurements are generated by AI/ML until more discussion on LCM operation in RAN2/RAN1.

Proposal 3:	Send an LS to RAN1 to inform RAN3 agreement on model performance monitoring for case 3a.

# References

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #105.

- RAN3_126_agenda_20241122_EOM1, 3GPP RAN3 #126.

3GPP TSG-RAN WG3 Meeting #127	R3-xxxxxx

Athens, Greece, February 17th – 21st, 2025

Title:	[draft] LS on AI/ML based positioning accuracy enhancement

Response to:	-

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	NEC (to be RAN3)

To:	RAN1

Cc:	RAN2

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall Description:

RAN3 has discussed the issue for the selection between Option A and Option B for model performance metric calculation of case 3a according to the following agreement from RAN1.

##### Agreement @RAN1#118 For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

RAN3 concluded that Option B should be supported in normative work.

# 2. Actions:

To RAN1

ACTION: 	RAN3 kindly ask RAN1 to take the above information into account.

3. Date of Next RAN3 Meetings:

TSG RAN WG3 Meeting #127bis           7th – 11st April 2025, CN.

TSG RAN WG3 Meeting #128             19th – 23rd May 2025, Malta, MT.