3GPP TSG-RAN WG3 #127	R3-250182

Greece, Athens, 17th – 21st February 2025

Agenda item:		14.3

Title:		(TP for TS 38.300) Discussion on regenerative payload enhancement for NR NTN

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the previous two meetings, RAN3 made the following progress on NR NTN regenerative payload enhancement:

RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Inactive UE support: Option 1, UE context is forwarded to the next satellites Option 2, UE context is kept on a fixed node (on-ground gNB or AMF) Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used Whether to support NG suspend/Resume? Continue the discussion on above open issues…

There are some open issues left in RAN3. And this paper will discuss them.

# 2. Discussion

2.1 Whether and how to support inactive UE

In earlier meetings, we talked about how to support inactive UEs in the NR NTN. We came to an agreement that "RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment." However, some companies raise it again because if the new serving GNB payload cannot retrieve the UE context, there will be some issues, and they propose the following solutions:

Option 1, UE context is forwarded to the next satellites.

Option 2, UE context is kept on a fixed node (on-ground gNB or AMF).

Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used.

In option 1, the gNB payload has to calculate where the inactive UE is so that it can send the UE context to the next satellites before the gNB stops covering the inactive UE. Also, the UE context is forwarded via the Xn interface, but we cannot always guarantee the ISL is established between neighbouring on-board gNBs all of the time, so the forwarding may not be transmitted successfully. Additionally, the serving on-board gNB may forward the UE context to multiple neighbouring on-board gNBs, thereby increasing the signalling cost.

Observation 1: Option 1 requires more calculation resources of on-board gNB and increases the Xn interface signalling exchange overhead. Additionally, we cannot always guarantee the successful transmission of the UE context through the ISL between neighbouring on-board gNBs.

Option 2: If the UE context is stored in fixed nodes, like on-ground gNB/AMF, the necessary information exchange between the on-board gNB and on-ground gNB/AMF will introduce significant extra signalling overhead and latency.

Observation 2: Option 2 brings a significant increase in signal overhead and latency.

At least two AMFs and two on-board gNBs will be used in the UE retrieve procedure and path switch procedure for Option 3. This has a big effect on the NG interface and CN. In addition, considering the two on-board gNBs are far away from each other and many necessary signalling need to be transmitted between them, the overall latency of these procedures is unacceptably long.

Observation 3: Option 3 has a significant impact on the NG interface and the CN. And the overall latency of the procedures is unacceptably long.

Overall, we do not see any obvious advantage of these three options above compared to the legacy method (i.e., RRC connection resume and RRC connection re-establishment).

Proposal 1: The three options above have obvious drawbacks and RAN3 is suggested to use the legacy method to handle the inactive UE in regenerative payload architecture.

2.2 NG interface management

In the previous meeting, RAN3 discussed how to handle when gNB leaves and revisit the serving area of the AMF. RAN3 agreed to go first with the NG Removal/Setup for the NG interface management, but the Suspend/Resume NG interface management is not precluded since many companies believe it has benefits and is worth further discussion.

Some objection companies are concerned about the Suspend/Resume NG interface management because the onboard gNB needs to store all the interface context for the AMF in its orbit, and the AMF needs to store all the interface context for all onboard gNB connected to it. These interface contexts will take up much storage for onboard gNB and on-ground AMF.

As we mentioned in the last meeting discussion, the part of the NG interface configurations and context (e.g., AMF/gNB IDs and TNL association) can be stored and are enough to suspend/resume NG interface management. In addition, considering the limited number of on-ground AMFs, the NG interface configurations and context may not take many storages of onboard gNB, even if the gNB needs to store all the interface context. In terms of the storage of AMF, it is on-ground, so the storage budget also should be sufficient.

Moreover, we think the gNB and AMF should be able to store the NG interface context, which does not mean they must store all of the NG interface context instances they established before and keep them all the time. The gNB and AMF can release some of them based on implementation. The NG Removal/Setup could be used if the NG interface context instances are released.

Observation 4: the suspend/resume NG interface management has no obvious storage problem for NTN gNB and AMF.

The Suspend/Resume NG interface management can make the NG interface quick resume with smaller overhead, which was also challenged in the last meeting. Suppose the NG interface configuration/context is not stored in the gNB and AMF. In that case, these configurations must be transmitted at every NG setup procedure between AMF and gNB, which is a burden for the wireless feeder link channel. When the suspend/resume NG interface management is adopted since the CN knows the AMF's serving area and the coverage of onboard gNB, the NG interface can be resumed automatically when the gNB revisits the AMF's serving area since the NG-interface configurations and TNL association related configuration/parameters are kept in the gNB and the AMF—no extra NG signaling needed. The supported TA list may be updated after the NG interface is resumed. Also, the NG Removal/Setup requires the TNL establishment, NG setup, and response message transmission. Therefore, the Suspend/Resume NG interface management can quickly resume with less overhead than NG Removal/Setup.

Observation 5: The NG interface can automatically resume when the gNB revisits the AMF's serving area.

Observation 6: the Suspend/Resume NG interface management can quickly resume with less overhead than NG Removal/Setup.

Considering the Suspend/Resume NG interface management has no obvious storage problem, clear benefits for quick resumes, and few overheads, RAN3 is suggested to agree to it.

Proposal 2: RAN3 is suggested to agree on the Suspend/Resume NG interface management method and the TP 38.300 in Annex A.

2.3 TAI update of on-board gNB

This is an agreement of the previous meeting:

The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration.

Since the on-board gNB keeps moving, the on-board gNB supported tracking area also keeps changing with gNB movement. RAN3 agrees that the AMF can know the gNB’s TAIs via OAM pre-configuration since the OAM can calculate the gNB supported TAI. Regarding the on-board gNB, it has to calculate its supported TAIs by itself. We cannot ensure the OAM calculated TAIs in AMF and on-board gNB calculated TAIs are perfectly aligned. It may lead to TAIs misalignment between AMF and on-board gNB, which may cause the AMF to send paging messages to the wrong gNBs. Therefore, we propose that the OAM also provide supported TAIs to the corresponding on-board gNBs to make the TAIs align between AMF and on-board gNB.

Proposal 3: The OAM also provides supported TAIs to the corresponding on-board gNBs to make TAIs alignment between AMF and on-board gNB.

2.4 The feeder link switchover impacts on NG interface.

In Rel-17/18, we defined hard feeder link switchover and soft feeder link switchover, where the hard feeder link switchover requires the satellite to maintain only one feeder link at a time. This definition is also applicable to the regenerative payload in Rel-19.

We defined the regenerative payload as terminating the NG interface toward the 5GC via the feeder link. Since the NG interface is a logic interface carried by TNL, the feeder link switchover only changes the TNL while the NG interface is unchanged. So, the hard feeder link switchover between gNBs and NTN gateways may lead to an interruption of the NG interface (including NG-C and NG-U), and it may not have a direct impact on the Uu interface, which is different from the case in the transparent payload.

Observation 7: The hard feeder link switchover in the regenerative payload will interrupt the NG interface.

Since the OAM can provide the time window (or the schedule) of the successive switchovers of the feeder link, the NG interface may be suspended in the feeder link switchover time window (or the schedule).

Proposal 4: The regenerative payload should suspend the NG interface pointed by the switchover time window or schedule given by the OAM. The TP 38.300 is provided in Annex A.

# 3. Conclusion

Observation 1: Option 1 requires more calculation resources of on-board gNB and increases the Xn interface signalling exchange overhead. Additionally, we cannot always guarantee the successful transmission of the UE context through the ISL between neighbouring on-board gNBs.

Observation 2: Option 2 brings a significant increase in signal overhead and latency.

Observation 3: Option 3 has a significant impact on the NG interface and the CN. And the overall latency of the procedures is unacceptably long.

Observation 4: the suspend/resume NG interface management has no obvious storage problem for NTN gNB and AMF.

Observation 5: The NG interface can automatically resume when the gNB revisits the AMF's serving area.

Observation 6: the Suspend/Resume NG interface management can quickly resume with less overhead than NG Removal/Setup.

Observation 7: The hard feeder link switchover in the regenerative payload will interrupt the NG interface.

Proposal 1: The three options above have obvious drawbacks and RAN3 is suggested to use the legacy method to handle the inactive UE in regenerative payload architecture.

Proposal 2: RAN3 is suggested to agree on the Suspend/Resume NG interface management method and the TP 38.300 in Annex A.

Proposal 3: The OAM also provides supported TAIs to the corresponding on-board gNBs to make TAIs alignment between AMF and on-board gNB.

Proposal 4: The regenerative payload should suspend the NG interface pointed by the switchover time window or schedule given by the OAM. The TP 38.300 is provided in Annex A.

# References

# Annex A: TP to BLCR for TS 38.300

--------------------------------------------------------------Start of the change-----------------------------------------------------------

## 16.14	Non-Terrestrial Networks

### 16.14.4	Switchover

#### 16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for a specific NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in NTN.

#### 16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption may occur during the transition between the feeder links.

#### 16.14.4.3	Procedures

The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over between two gNBs is performed. The transfer of the affected UE(s)' context between the two gNBs at feeder link switch over is performed by means of either NG based or Xn based handover, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

The hard feeder link switchover may cause an NG interface interruption between the NGSO regenerative payload and 5GC. The OAM provides the time window or schedule of the feeder link switchover; then, the regenerative payload should suspend the NG interface pointed by the switchover time window or schedule.

/******* Unchanged part skipped ******/

### 16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

Two types of NG interface management methods are supported when the regenerative NTN payload leaves and revisits an AMF service area as defined in TS 38.413 [26]:

-	NG Removal/Setup;

-	NG Suspend/Resume.

Editor’s Note: The details of two NG interface management methods are FFS.

--------------------------------------------------------------End of the change----------------------------------------------------------