3GPP TSG-RAN WG3 Meeting #127	R3-250194

Athens, Greece, 17 – 21 February 2025

Agenda Item:		19.2

Source:		Charter Communications

Title:	CLI mitigation procedures

Document for:		Discussion and decision

# Introduction

In previous meetings RAN3 has agreed to exchange information over Xn interface and F1 interface regarding:

- Semi-static cell-specific SBFD time and frequency location configuration with cell granularity

- Measurement resource configuration

- Strongest DL beam information

- CLI mitigation request

Even though there is mention of CLI mitigation request, there is an express statement, that a gNB that receives a CLI mitigation request is not obligated to act on it.  We believe this is akin to “sweeping the dirt under the rug”.  Therefore nothing has been done so far other than simply request.  Even if a gNB is not obligated to respond to a CLI mitigation request, we should have the procedures, if in a particular deployment or situation, the gNB wants to respond to the CLI mitigation request and to cooperate to improve the interference situation. This contribution addresses CLI mitigation procedures.

# Discussion

We believe that CLI mitigation and testing procedures, must be flexible enough to allow the action to be started / continued / stopped at any particular point in time, to provide whatever testing parameters are necessary, and to provide the result of testing to the requesting entity.

The existing Xn messages RESOURCE STATUS REQUEST, RESOURCE STATUS UPDATE, RESOURCE STATUS RESPONSE and RESOURCE STATUS FAILURE and the corresponding F1 messages with appropriate modifications specific to the CLI mitigation procedure would support that functionality.

The existing RESOURCE STATUS REQUEST message already contains the infrastructure to:

- identify one or more types of requests.

- start / stop a particular request;

- which cell should report the results of a request,

- which SSB should report the results of a request;

- a field to specify the periodicity of reporting, if so desired

  - CLI-mitigation request

Proposal 1: A victim gNB can request an aggressor gNB to start CLI mitigation action by sending a RESOURCE STATUS REQUEST with:

- the existing “Registration Request” IE set to “start”,

- the existing “ReportCharacteristics” IE set to the new value to indicate “Mitigation Request”,

- the existing “Cell to Report List” IE including the list of cells of the neighboring gNB that is causing the unwanted CLI that the victim gNB wants mitigated,

- the existing “SSB to Report List” IE including the list of SSBs of the neighboring gNB that is causing the unwanted CLI that the victim gNB wants mitigated,

- the new “CLI Mitigation Information” IE which includes

  - in the case of cell-level testing, the affected cell ID, the CLI power measured by the victim gNB cell, and the CRI of the NZP-CSI RS Configuration information.

  - In the case of SSB-level testing, the cell ID which contains the SSB, the affected SSB index, and the corresponding CLI Power.

Proposal 2: A gNB receiving a RESOURCE STATUS REQUEST that requests the start of “CLI Mitigation” shall respond with either a RESOURCE STATUS RESPONSE, if it in fact starts mitigation actions, or a RESOURCE STATUS FAILURE, if it does not start any mitigation action.

Proposal 3: A gNB that has previously requested start of CLI mitigation, and received acknowledgment that CLI mitigation has been started, but which still detects unacceptable level of CLI can send a RESOURCE STATUS REQUEST with

- the existing “Registration Request” IE set to the new value of “continue”,

- the existing “Report Characteristics” IE set to the new value to indicate “Mitigation Request”

- the existing “Cell to Report List” and/or existing “SSB to Report List” IE, including the appropriate cells/SSBs for which it wants the next level of mitigation to be applied.

Proposal 4: A gNB that has received a RESOURCE STATUS REQUEST requesting that CLI mitigation be continued, shall respond with a RESOURCE STATUS RESPONSE, if it can or is willing to apply the next level of CLI mitigation, or with a RESOURCE STATUS FAILURE otherwise.

Proposal 5: A victim gNB that has requested CLI mitigation, and that eventually is satisfied with the level of CLI mitigation applied by the aggressor, shall send a RESOURCE STATUS REQUEST indicating that the CLI mitigation is to be stopped.

Proposal 6: A gNB receiving a RESOURCE STATUS REQUEST that requests the stop of “CLI Mitigation” shall stop the CLI mitigation actions and respond with a RESOURCE STATUS RESPONSE.

  - CLI testing

Proposal 7: If a gNB1 wants to be proactive and wants to start a CLI testing with a neighboring gNB2 in order to find out how a particular configuration of its power level, antenna setting, etc might impact neighboring gNB2, it can request a CLI testing by sending a RESOURCE STATUS REQUEST with

- the existing “Registration Request” IE set to the existing value of “start”,

- the existing “Report Characteristics” IE set to the new value of “CLI Testing”,

- the existing “Cell to Report List” IE including the list of cells of the neighbor gNB2 that it is requesting to measure and report the CLI interference,

- the existing “SSB to Report List” IE including the list of SSBs, if any, that it is requesting to measure and report the CLI interference.

- including the new “CLI measurement configuration” IE to indicate to gNB2, the testing/interfering signal it will be using for the test.

Proposal 8: If a gNB2 receives a RESOURCE STATUS REQUEST message from gNB1 requesting a CLI test, it shall respond with a RESOURCE STATUS RESPONSE, if it accepts the test request, or with a RESOURCE STATUS FAILURE, if it refuses the test request.

Proposal 9: If gNB1 receives a RESOURCE STATUS RESPONSE from gNB2 in response to a previously sent request for testing, gNB1 shall start the testing with the test signal that had been previously indicated.

Proposal 10: If gNB1 receives a RESOURCE STATUS FAILURE from gNB2 in response to a previously sent request for testing, gNB1 shall not start any testing.

Proposal 11: When gNB2 completes the measurement of CLI interference which was requested by gNB1, it shall reply to gNB1 with a RESOURCE STATUS UPDATE message that includes the new IE “Cell CLI Resource Status”, which contains the test results.

Proposal 12: If gNB1 receives a RESOURCE STATUS UPDATE from gNB2 containing test results for a previously requested CLI testing campaign sent to gNB2, it shall stop that test campaign and send to gNB2 the corresponding RESOURCE STATUS REQUEST with the existing “Registration Request” IE set to the existing “stop” value.

  - Handling disaggregated architecture

Existing F1 messages, with the modifications proposed in Annex B are used to cater for the case in which a disaggregated gNB is used. Namely:

In the case of a disaggregated gNB, a CU that has received a Xn RESOURCE STATUS REQUEST requesting the start or stop of an action shall send to the proper DU the corresponding F1 RESOURCE STATUS REQUEST, with the IEs of the messages set to the appropriate values.

In the case of a disaggregated gNB, a DU that has received a F1 RESOURCE STATUS REQUEST message requesting the start of an action shall respond to the sending CU with either a F1 RESOURCE STATUS RESPONSE, if it in fact starts the action, or a F1 RESOURCE STATUS FAILURE, if it does not start any action, with the IEs of the message set to the appropriate values.

In the case of a disaggregated gNB, a DU receiving a F1 RESOURCE STATUS REQUEST that requests the stop of an action shall stop the action and respond with a F1 RESOURCE STATUS RESPONSE, with the IEs of the message set to the appropriate values.

In the case of a disaggregated gNB, a CU that has received a F1 RESOURCE STATUS RESPONSE, or F1 RESOURCE STATUS FAILURE, or F1 RESOURCE STATUS UPDATE message from a DU, should send to the neighboring gNB, the corresponding Xn RESOURCE STATUS RESPONSE or Xn RESOURCE STATUS FAILURE or Xn RESOURCE STATUS UPDATE message, respectively, with the IEs of the message set to the appropriate values.

# Annex A – Changes to XnAP (TS38.423)

#### 9.1.3.18	RESOURCE STATUS REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested measurement according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, continue, configure,…) | Type of request for which the resource status is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the NG-RAN node2 is requested to report. First Bit = PRB Periodic, Second Bit = TNL Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit =Number of Active UEs Periodic, Fifth Bit =RRC connections Periodic, Sixth Bit = NR-U Channel List Periodic. Seventh Bit = CLI Testing Eighth Bit = CLI Mitigation Other bits shall be ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>CLI Mitigation Information | C-ifMitigationRequestStartCont |  | 9.2.2.x |  | -- |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | – |  |
| >>>SSB To Report Item |  | 1 .. < maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB-Index | M |  | INTEGER (0..,63..) |  | – |  |
| >>>>CLI Mitigation Information | C-ifMitigationRequestStartCont |  | 9.2.2.x |  | -- |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | – |  |
| >>>Slice To Report Item |  | 1 .. < maxnoofBPLMNs > |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | – |  |
| >>>>S-NSSAI List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of indicated measurements. Also used as the averaging window length for all measurement object if supported. This IE is ignored if the Registration Request IE is set to "add". | YES | Ignore |
| CLI Measurement Configuration | C-ifConfigStartCLITesting |  | 9.2.2.y |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |
| ifConfigStartCLITesting | This IE shall be present if the RegistrationRequest has the values “Configure”, or “Start”, and the ReportCharacteristics IE is set to the value “CLI Testing” |
| ifMitigationRequestStartCont | This IE shall be present if the Report Characteristics IE is set to “CLI Mitigation” AND the Registration Request IE is set to either “start” or “continue”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

--------------------------------Next Change--------------------------------

#### 9.1.3.21	RESOURCE STATUS UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >> CLI Resource Status | O |  | 9.2.2.z |  |  |  |
| >>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>TNL Capacity Indicator | O |  | 9.2.2.49 |  | – |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.51 |  | – |  |
| >>Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| >>Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>RRC Connections | O |  | 9.2.2.56 |  | – |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | – |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UchannelIDs, …) | The NR-U channel utilised in the last reporting period | – |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 indicates that the channel resources have been utilized for DL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | – |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | – |  |
| >>>>Channel Occupancy Time Percentage UL | O |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for UL traffic served by the corresponding NR-U Channel of the serving cell for UEs that transmit to the serving cell. Value 100 indicates that the channel resources have been utilized for UL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | YES | ignore |
| >>>>Energy Detection Threshold UL | O |  | INTEGER (-100..-50,…) | Indicates the average of the maximum ED Threshold configured by the gNB for UL channel sensing. Value is in dBm. | YES | ignore |
| >>>>Radio Resource Status NR-U | O |  | 9.2.2.104 | Indicates the radio resource status per NR-U channel. | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofNR-UchannelIDs | Maximum no. NR-U channel IDs in a cell. Value is 16. |

--------------------------------Next Change--------------------------------

#### 9.2.2.x	CLI Mitigation Information

This IE contains information that may be used to mitigate CLI across gNBs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CLI Information |  |  |  |  |
| >CLI Power detected | M | [0..X] | INTEGER (0..X) | Number of dBm of CLI power |
| >CRI | O |  | INTEGER (1..64) | Includes the CSI-RS Resource Indicator (CRI) value. CRI is an index indicating a CSI-RS resource within a set of resources |
| >SSB Index | O |  | INTEGER (0..63) | Identifier of the SSB beam. |

--------------------------------Next Change--------------------------------

9.2.2.y CLI Measurement Configuration

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CLITestType | M | ENUMERATED(gNB-to-gNB, …) |  |  |
| ServedCellList | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |
| >ServedCellItem | M |  |  |  |
| >>NZP-CSI-RS Configuration | M |  | 9.2.2.w |  |
| >>SSBCLIList | 0..1 |  |  |  |
| >>SSBCLIItem | 1.. <maxnoofSSBAreas> |  |  |  |
| >>SSB-Specification | M |  |  | Coding to be agreed |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a gNB node cell. Value is 64. |

--------------------------------Next Change--------------------------------

9.2.2.z	CLI Resource Status

This IE contains information about the CLI test result

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Report Level |  |  |  |  |
| >cell_based |  |  |  |  |
| >>CLI Power | M | [0..X] | INTEGER (0..X) | Number of dBm of CLI power |
| >>CRI | M |  | INTEGER(1..64) | The CSI-RS Resource Indicator (CRI) value, i.e the index of a CSI-RS resource within a set of resources of the NZP-CSI-RS configured by the aggressor. |
| >SSB_based |  |  |  |  |
| >> SSB Area CLI List | 0..1 |  |  |  |
| >>> SSB Area CLI Item | 1 .. <maxnoofSSBAreas> |  |  |  |
| >>>>SSB Index | M |  | INTEGER(0..63) |  |
| >>>>CLI Power | M | [0..X] | INTEGER (0..X) | Number of dBm of CLI power |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a gNB node cell. Value is 64. |

--------------------------------Next Change--------------------------------

#### 9.2.2.w	NZP-CSI-RS Configuration

This IE contains information describing the NZP-CSI-RS configuration for a cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NZP-CSI-RS-ResourceSet | M |  | OCTET STRING | Includes the NZP-CSI-RS-ResourceSet IE, as defined in TS 38.331 [10]. |
| NZP-CSI-RS-Resource List |  | 1 |  | List of CLI mitigation assistance information per cell |
| >NZP-CSI-RS-Resource Item |  | 1 .. <maxnoofNZP-CSI-RS-RESOURCE> |  |  |
| >>NZP-CSI-RS-Resource | M |  | OCTET STRING | Includes the NZP-CSI-RS-Resource IE, as defined in TS 38.331 [10]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofNZP-CSI-RS-RESOURCE | Maximum no. of NZP-CSI-RS-Resoures. Value is 64. |

# Annex B – Changes to F1 AP (TS38.473)

#### 9.2.1.20	RESOURCE STATUS REQUEST

This message is sent by gNB-CU to gNB-DU to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestStoporAdd |  |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Registration Request | M |  |  | ENUMERATED(start, stop,add, continue, configure, …) | Type of request for which the resource status is required. | YES | ignore |
| Report Characteristics | C-ifRegistrationRequestStart |  |  | BIT STRING (SIZE(32)) | Each position in the bitmap indicates measurement object the gNB-DU is requested to report. First Bit = PRB Periodic, Second Bit = TNL Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit = HW LoadInd Periodic, Fifth Bit = Number of Active UEs Periodic, Sixth Bit = NR-U Channel List Periodic. Seventh Bit = CLI Testing Eight Bit = CLI Mitigation Other bits shall be ignored by the gNB-DU. | YES | ignore |
| Cell To Report List |  |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  |  | 1 .. <maxCellingNBDU> |  |  | - |  |
| >>Cell ID | M |  |  | NR CGI 9.3.1.12 |  | - |  |
| >>CLI Mitigation Information |  |  | C-ifMitigationRequestStartCont |  |  |  | 9.2.2.x |  |  |  | -- |  |  |  |
| >>SSB To Report List |  |  | 0..1 |  | SSB list to which the request applies. | - |  |
| >>>SSB To Report Item |  |  | 1 .. < maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB index | M |  |  | INTEGER (0..63) |  | - |  |
| >>>>CLI Mitigation Information |  | C-ifMitigationRequestStartCont |  | 9.2.2.x |  | -- |  |
| >>Slice To Report List |  |  | 0..1 |  | S-NSSAI list to which the request applies. | - |  |
| >>>Slice To Report Item |  |  | 1..< maxnoofBPLMNsNR> |  |  | - |  |
| >>>>PLMN Identity | M |  |  | 9.3.1.14 | Broadcast PLMN | - |  |
| >>>>S-NSSAI List |  |  | 1 |  |  | - |  |
| >>>>>S-NSSAI Item |  |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  |  | 9.3.1.38 |  | - |  |
| Reporting Periodicity | O |  |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of indicated measurements. Also used as the averaging window length for all measurement object if supported. This IE is ignored if the Registration Request IE is set to “add”. | YES | ignore |
| CLI Measurement Configuration | C-ifCLITesting |  |  | 9.3.1.y |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |
| ifConfigStartCLITesting | This IE shall be present if the RegistrationRequest has the values “Configure”, or “Start”, and the ReportCharacteristics IE is set to the value “CLI Testing” |
| ifMitigationRequestStartCont | This IE shall be present if the Report Characteristics IE is set to “CLI Mitigation” AND the Registration Request IE is set to either “start” or “continue”. |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a gNB node cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

--------------------------------Next Change--------------------------------

#### 9.2.1.23	RESOURCE STATUS UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Hardware Load Indicator | O |  | 9.3.1.136 |  | YES | ignore |
| TNL Capacity Indicator | O |  | 9.3.1.128 |  | YES | ignore |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellingNBDU > |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >> CLI Resource Status | O |  | 9.3.1.z |  | -- |  |
| >>Radio Resource Status | O |  | 9.3.1.129 |  | - |  |
| >>Composite Available Capacity Group | O |  | 9.3.1.130 |  | - |  |
| >>Slice Available Capacity | O |  | 9.3.1.134 |  | - |  |
| >>Number of Active UEs | O |  | 9.3.1.135 |  | - |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs) | Identifies a portion of the NR-U Channel Bandwidth on which channel access procedure in shared spectrum has been performed in the last reporting period. | - |  |
| >>>>Channel Occupancy Time Percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 indicates that the channel resources have been utilized for DL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | - |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | - |  |
| >>>>Channel Occupancy Time Percentage UL | O |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for UL traffic served by the corresponding NR-U Channel of the serving cell for UEs that transmit to the serving cell. Value 100 indicates that the channel resources have been utilized for UL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | YES | ignore |
| >>>>Radio Resource Status NR-U | O |  | 9.3.1.295 | Indicates the radio resource status per NR-U channel. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |

--------------------------------Next Change--------------------------------

#### 9.3.1.x	CLI Mitigation Information

This IE contains information that may be used to mitigate CLI across gNBs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CLI Information |  |  |  |  |
| >CLI Power detected | M | [0..X] | INTEGER (0..X) | Number of dBm of CLI power |
| >CRI | O |  | INTEGER (1..64) | Includes the CSI-RS Resource Indicator (CRI) value. CRI is an index indicating a CSI-RS resource within a set of resources |
| >SSB Index | O |  | INTEGER (0..63) | Identifier of the SSB beam. |

--------------------------------Next Change--------------------------------

9.3.1.y CLI Test Parameters

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CLITestType | M | ENUMERATED(gNB-to-gNB, …) |  |  |
| ServedCellCLIList | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |
| >ServedCellCLIItem | M |  |  |  |
| >>NZP-CSI-RS Configuration | M |  | 9.3.1.w |  |
| >>SSBCLIList | 0..1 |  |  |  |
| >>SSBCLIItem | 1..<maxnoofSSBAreas> |  |  |  |
| >>SSB-Configuration | M |  |  | Coding to be agreed |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a gNB node cell. Value is 64. |

--------------------------------Next Change--------------------------------

9.3.1.z	CLI Resource Status

This IE contains information about the CLI test result

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Report Level |  |  |  |  |
| >cell_based |  |  |  |  |
| >>CLI Power | M | [0..X] | INTEGER (0..X) | Number of dBm of CLI power |
| >>CRI | M |  | INTEGER(1..64) | The CSI-RS Resource Indicator (CRI) value, i.e the index of a CSI-RS resource within a set of resources of the NZP-CSI-RS configured by the aggressor. |
| >SSB_based |  |  |  |  |
| >> SSB Area CLI List | 0..1 |  |  |  |
| >>> SSB Area CLI Item | 1 .. <maxnoofSSBAreas> |  |  |  |
| >>>>SSB Index | M |  | INTEGER(0..63) |  |
| >>>>CLI Power | M | [0..X] | INTEGER (0..X) | Number of dBm of CLI power |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a gNB node cell. Value is 64. |

--------------------------------Next Change--------------------------------

#### 9.3.1.w	NZP-CSI-RS Configuration

This IE contains information describing the NZP-CSI-RS configuration for a cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NZP-CSI-RS-ResourceSet | M |  | OCTET STRING | Includes the NZP-CSI-RS-ResourceSet IE, as defined in TS 38.331 [10]. |
| NZP-CSI-RS-Resource List |  | 1 |  | List of CLI mitigation assistance information per cell |
| >NZP-CSI-RS-Resource Item |  | 1 .. <maxnoofNZP-CSI-RS-RESOURCE> |  |  |
| >>NZP-CSI-RS-Resource | M |  | OCTET STRING | Includes the NZP-CSI-RS-Resource IE, as defined in TS 38.331 [10]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofNZP-CSI-RS-RESOURCE | Maximum no. of NZP-CSI-RS-Resoures. Value is 64. |