3GPP TSG-RAN WG3 Meeting #127	R3-250258

Athens, GR, 17th–21nd February, 2025

Title: 	Support for Slice level UE performance

Source: 	Tejas Networks,CEWiT

Agenda item:	11.2 AI/ML enabled Slicing

Document Type:	Discussion

# 1. Introduction

Companies have different understandings on if per slice per UE performance (e.g., throughput) is really supported by SA5 specification.

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

There is no consensus if UE performance can be measured in per UE per S-NSSAI and reported from target node to source node.

This TDoc presents our view on the [1]chair's notes(R3-247785) from the last meeting regarding UE performance granularity.

# 2. Discussion

2.1 UE performance granularity

One of the main objectives of introducing AI/ML in gNBs is to enable optimal resource management, which is typically evaluated at the slice level. RAN3 has also agreed to assess the radio resource status and available capacity at the slice level. Since all the analysis we are measuring at the slice level, the UE performance metrics should also be evaluated at the slice level. This will provide a comprehensive overview of the UE's performance after handover.

#### 6.3.1.4	UE throughput

##### 6.3.1.4.1	Average DL UE throughput in gNB

a)	This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and subcounters per supported S-NSSAI. This measurement is also referred to as DL M5 in TS 37.320 [9].

If we consider per DRB per slice which was put forward by some companies, after the handover, there is a possibility that the QoS flows in DRB1 of the PDU session in the source node will be remapped to another DRB in the target node. This means that the source node cannot directly compare the UE performance in DRB1 between node 1 and node 2. Therefore, evaluating performance at the DRB level becomes irrelevant.

From the [2]TS 28.558(UE level measurements for 5G system (Release 19) clause 6.3.1.4 as shown above. It is clearly mentioned that the UE performance is filterable by per PLMN ID and per QoS level and per supported slice. So, it is clear that UE performance can be measured at slice level.

Proposal 1: Agree with the current WA that per slice level performance feedback is sufficient.

Aspect#1: How UE performance per slice granularity can be discussed/decided by RAN3, but has some dependency on how UE performance per UE granularity is calculated which was discussed in Rel-18 correction and an LS to SA5 is triggered, e.g.,

  - Or if it is an average over time among all packets via all DRBs within the same slice/UE, e.g., R3-247412

- AvgTputDLUE-SA5=DV1+DV2∆T1+∆T2=5+10Mbit500+200ms≅21.43Mbps

  - Or if it is an average of DRB level metric within the same slice/UE, e.g., R3-247412

- AvgTputDLUE-RAN3=TputDRB#1+TputDRB#22=10+502Mbps=30Mbps

- E///, Huawei, ZTE: SA5 has defined and supported the per slice per UE performance. RAN3 does not need to further discuss.

- Nokia, CATT: it is unclear from SA5 spec, how per slice per UE performance is calculated. What’s supported in SA5 spec now is per slice, but not per slice per UE.

In the last meeting’s summary of offline discussion(R3-247785) [1] which is shown below, there was a discussion regarding the formula/equation that need to be used to measure the average UE throughput.

The key point is that regardless of the formula/equation used, as long as the same formula is applied at both the source and target to measure the UE performance, the result from the target will indicate the source, whether the UE's performance in a particular slice has increased or decreased after handover.

Let the above shown calculations of Average UE throughput of DRB1(DV1=5Mbit and ∆T1 =500ms,Throughput=10Mbps) and DRB2(DV2=10Mbit and ∆T2 =200ms,Throughput=50Mbps) be the values of UE performance measured at source node before handover. Suppose, after handover it took 400ms to transmit 10Mbit by the DRB2.
The results of UE throughput at Target node are:
AvgTputDLUE-SA5=DV1+DV2∆T1+∆T2=5+10Mbit500+400ms≅16.66Mbps 
                
The new Throughput of the DRB2 is 10Mbit400ms =25Mbps

AvgTputDLUE-RAN3=TputDRB#1+TputDRB#22=10+252Mbps=17.5Mbps 

It is clear that the average UE throughput decreases when using both equations. Therefore, the source can ultimately conclude that the performance of the UE has reduced after the handover, regardless of which equation is used.

Observation 1: Since the same equation is used at both the source and target nodes to measure UE performance, regardless of the chosen equation, it will provides sufficient information for the source node to assess the UE's performance.

Since SA5 has already provided the equation to calculate the average throughput per UE in sections 6.3.1.4.1 and 6.3.1.4.2, we request that RAN3 adhere to the same equation for UE throughput metrics.

Proposal 2: RAN3 can adhere to the same equation for UE throughput metrics as specified by SA5

Since TS 28.558 only mentions that slice-level UE performance is possible, it does not specify how it is calculated.

Observation 2: Since TS 28.558 only mentions that slice-level UE performance is possible, it does not specify how it is calculated.

All gNBs are aware of the DRBs and the slices to which they are mapped.

In order to find the Slice level average UE throughput, the equation can be modified into:

Average UE throughput per slice=∑(Data volume  for each DRB in the slice)∑(Time for each DRB in the slice)

Proposal 3:  RAN 3 should consider following equation to calculate the slice level Average UE throughput,

Average UE throughput per slice=∑(Data volume  for each DRB in the slice)∑(Time for each DRB in the slice)

3.Conclusions

Proposal 1: Agree with the current WA that per slice level performance feedback is sufficient.

Observation 1: Since the same equation is used at both the source and target nodes to measure UE performance, regardless of the chosen equation, it provides sufficient information for the source node to assess the UE's performance.

Proposal 2: RAN3 can adhere to the same equation for UE throughput metrics as specified by SA5

Observation 2: Since TS 28.558 only mentions that slice-level UE performance is possible, it does not specify how it is calculated.

Proposal 3:

RAN 3 should consider following equation to calculate the slice level Average UE throughput,

Average UE throughput per slice=∑(Data volume  for each DRB in the slice)∑(Time for each DRB in the slice)

4.References

[1]R3-247785

[2]TS 28.558