3GPP TSG-RAN WG3 #127								R3-250398

Athens, Greece, 17th – 21st Feb 2025

Agenda Item:	12.2

Source:	China Telecom

Title: 	Discussion on Multi-hop Prevention and Functionalities for WAB

Document for:	Discussion

Introduction

The new WI on additional topological enhancements for NR was approved in RAN#105 meeting [1]. The following candidate solutions to avoid multi-hop WAB topology were discussed at last RAN3 meeting:

- Solution 1: The WAB-gNB uses dedicated frequencies and/or PCIs. FFS on any other legacy OTA parameters.

- Solution 2: Use the slice dedicated for backhauling, i.e. use a list of S-NSSAIs in RRCsetupcomplete to do access control and/or use a list of S-NSSAIs in handover signalling. No CN upgrade is needed.

- Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

- Solution5: In case of handover for a WAB-node, the new WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node.

During the outcome of online discussion, Solution 2 for handover scenario was agreed and the following agreement was achieved:

Agree on Solution 2:

For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling.

For initial access procedure, the solution 1 and Solution 3 still need to be further discussed. This contribution aims to further discuss on multi-hop prevention and functionalities for WAB.

Discussion

Down Selection of Multi-hop Prevention Solutions

- Solution 1

During the last online discussion, some companies proposed that the multi-hop prevention issue could be addressed by preconfiguring a list of CGIs for WAB gNBs. Additionally, Solution 1 was found to have no impact on RAN2. Regarding the impact on RAN2, an example can be observed from NCR-MTNCR-MT may be configured with allowed and/or forbidden cell list for cell reselection from OAM server (specified in TS 38.300) while allow- and/or exclude-listed cells in SIB3/4 may be signaled in the NCR-MT’s camped cell. Since ones in SIB3/4 are not only for NCR-MT but mainly for access UEs, NCR-MT should not use these lists if OAM already configured NCR-MT with allowed and/or forbidden cell lists. The expected operation of NCR-MT would be like below:

| Cases | NCR-OAM configured forbidden cell list | NCR-OAM configured allowed cell list | Expected NCR-MT behaviour |
| --- | --- | --- | --- |
| 1 | not configured | not configured | NCR-MT follows the “excluded cell list” and “allowed cell list” broadcasted in SIB. |
| 2 | configured | not configured | NCT-MT cannot access the cell that belongs to either “OAM configured forbidden cell list” or “excluded cell list” broadcasted in SIB. If “allowed cell list” is broadcasted in SIB, the NCR-MT can only access those cells. |
| 3 | not configured | configured | NCR-MT can only access the cell that belongs to the OAM configured allowed cell list. |
| 4 | configured | configured | NCR-MT can only access the cell that belongs to the OAM configured allowed cell list. |

Therefore, the following stage 3 texts are included in TS38.304 to determine which cell lists should be used for cell reselection, if allowed cell list and/or forbidden cell lists, and exclude-listed cells and/or allow-listed cells are signaled:

NCR-MT may be configured with additional allowed cell list and/or forbidden cell list, see TS 38.300 [2]. The NCR-MT shall consider only the allowed cell list, if configured by OAM as candidates for cell reselection (ignore above exclude-listed cells and/or allow-listed cells). The NCR-MT shall not consider the cells for cell reselection in the forbidden cell list, if configured by OAM.

From the perspective of WAB-MT, the similar RAN2 impact still exist in Solution 1.

Observation 1: Solution 1 has impact on RAN2 specification.

Solution 1 implies that WAB can only operate within a dedicated network with exclusive resources. Currently, a dedicated network has been deployed solely for railway applications. However, according to the justification provided in the WID, WAB must support multiple application scenarios, including aircraft, cruise ships, helicopters, and vehicles [1]. It is evidently impractical to configure a dedicated network for each scenario. Moreover, operators have already allocated all NR frequencies and PCI for the 5G commercial network. If Solution 1 is adopted, operators would need to adjust their network planning strategies, which is not feasible at this stage. Therefore, Solution 1 is not viable for commercial deployment.

Observation 2: Solution 1 is not feasible in commercial network.

- Solution 3

Solution 3 can be considered a straightforward approach to avoiding multi-hop; however, it impacts RAN2, as it requires introducing a new indication in the SIB1 message and defining the corresponding behavior of WAB-MT in the inactive and idle states. Compared to Solution 2, access control in Solution 3 is performed during the cell (re)selection process, where WAB-MT treats a cell broadcasting an access indicator as a forbidden cell. During the handover procedure, the UE may not verify the SIB1 information provided by the target cell via the handover command. If a handover failure occurs, WAB-MT reselects other cells to initiate the RRC establishment procedure. Additionally, this air interface enhancement does not affect existing commercial UEs, as they simply ignore the new indicator. Therefore, based on the above analysis, Solution 3 affects RAN2 specifications, such as TS38.331 and TS38.304. To accommodate certain special cases, it is proposed that OAM configure a forbidden/allowed cell list for WAB-MT, similar to NCR-MT in the IDLE/INACTIVE state. The following texts on the access control mechanism for NCR-MT are quoted below:

NCR-MT may be configured with additional allowed cell list and/or forbidden cell list, see TS 38.300 [2]. The NCR-MT shall consider only the allowed cell list, if configured by OAM as candidates for cell reselection (ignore above exclude-listed cells and/or allow-listed cells). The NCR-MT shall not consider the cells for cell reselection in the forbidden cell list, if configured by OAM.

Observation 3: Solution 3 is a straightforward solution to avoid multi-hop.

Proposal 1: agree to introduce a new indicator in SIB for access control.

WAB authorization

According to the conclusion of SA2, the WAB-gNB informs the AMFs that it is an MWAB when establishing an NG connection. Therefore, a new indicator must be introduced in the NG Setup Request/RAN Configuration Update message to explicitly identify the node as a WAB-gNB. This indicator remains unchanged after the NG interface is established. Without this indicator in the NG Setup Request/RAN Configuration Update message, the AMF will interpret the node as a legacy gNB.

Proposal 2: The WAB-gNB informs the AMFs that it is an MWAB when establishing the NG connection.

According to RAN3 agreements from the last meeting, the authorization of the WAB-MT differs from the authorization, configuration, and activation of the WAB-gNB, which are managed through mechanisms such as OAM and SeGW. If the WAB-MT authorization status changes from "authorized" to "not authorized," the WAB-MT functions as a normal UE, and the WAB-gNB can no longer provide service to any UEs. Consequently, it must cease broadcasting and remove the Xn/NG connection. Whether the SCTP connection for the Xn and NG interfaces is maintained depends on pre-configuration or OAM settings. Conversely, if the WAB-MT authorization status changes from "not authorized" to "authorized," the BH PDU session and its corresponding DRB resources can be reallocated to the WAB-gNB, allowing it to perform the standard WAB-gNB setup procedure.

Proposal 3: If the WAB-MT authorization status changes from "authorized" to "not authorized," the WAB-MT functions as a normal UE, and the WAB-gNB can no longer provide service to any UEs. Consequently, it must cease broadcasting and remove the Xn/NG connection.

If the SeGW triggers a change in the WAB-gNB authorization status from "authorized" to "not authorized," the authorization information can be conveyed via an NGAP message, such as an NG Setup Response or RAN Configuration Update ACK message. In this scenario, the WAB-gNB can no longer provide service to any UEs, must cease broadcasting, and remove the Xn/NG connection. Additionally, the SCTP connection for the Xn and NG interfaces should be removed. If the authorization status change is triggered by the OAM, the WAB-gNB also ceases providing service to any UEs and must stop receiving and transmitting signals to and from UEs while removing the Xn/NG connection. However, since the BH PDU session can still be maintained, the SCTP connection may also be preserved.

Proposal 4: If SeGW trigger the WAB-gNB authorization status changes from “authorized” to “not authorized”, the WAB-gNB could not continue to provide service to any UEs and needs to ceases to receive/send signal from/to UE and remove Xn/NG connection. If the authorization status changes is triggered by CN, the SCTP connection shall be removed.

Xn interface

According to the objective of the WID and the SA2 conclusion, the WAB-gNB can establish an Xn interface with neighboring cells. Additionally, the Xn-based handover procedure can be performed via the Xn interface, following legacy mechanisms. In conventional stationary base stations, the Xn interface and cell relationship configurations are typically fixed. Operators can configure neighboring cell relationships and interfaces manually or utilize tools such as Automatic Neighbor Relations (ANR) for automatic configuration. As specified in TS23.501, the WAB, functioning as a mobile base station, may be mounted on a moving vehicle and serve UEs both inside and outside the vehicle. Consequently, the Xn interface between the WAB-gNB and the BH gNB may not always be maintained. Both the WAB-gNB and BH-gNB may remove the cell relationship and Xn interface when the base station moves out of the coverage area of the neighboring BH gNB. To address this issue, a new indicator can be included in the Xn Setup/Modification message to specify the survival time of the Xn interface.

Proposal 5: A new indicator can be included in the Xn Setup message to specify the survival time of the Xn interface.

NG interface

According to the objectives of the WID, NG connection management should be considered during the WI phase. As specified in TS23.501, the WAB-gNB and MWAB-MT are authorized and controlled separately. When the WAB-gNB attempts to establish a connection with the core network (CN), it must provide the WAB-gNB Indicator via the NG Setup Request message. The CN must then verify the authorization status and return the result in the NG Setup Response or Failure message.

If the authorization status changes from 'authorized' to 'not authorized,' the AMF can signal the updated status in the AMF CONFIGURATION UPDATE message. Subsequently, the NG connection removal procedure must be initiated by the WAB-gNB after all served UEs have been handed over to neighbouring cells.

Proposal 6: A new WAB-gNB Indicator should be included in the NG Setup Request message.

Proposal 7: The AMF can signal 'authorized' or 'not authorized' in the AMF CONFIGURATION UPDATE message.

Proposal 8: The NG connection removal procedure must be triggered by the WAB-gNB if the authorization status changes from 'authorized' to 'not authorized'.

Proposal

Based on the above analysis, we have the following observations and proposals:

Observation 1: Solution 1 has impact on RAN2 specification.

Observation 2: Solution 1 is not feasible in commercial network.

Observation 3: Solution 3 is a straightforward solution to avoid multi-hop.

Proposal 1: agree to introduce a new indicator in SIB for access control.

Proposal 2: The WAB-gNB informs the AMFs that it is an MWAB when establishing the NG connection.

Proposal 3: If the WAB-MT authorization status changes from "authorized" to "not authorized," the WAB-MT functions as a normal UE, and the WAB-gNB can no longer provide service to any UEs. Consequently, it must cease broadcasting and remove the Xn/NG connection.

Proposal 4: If SeGW trigger the WAB-gNB authorization status changes from “authorized” to “not authorized”, the WAB-gNB could not continue to provide service to any UEs and needs to ceases to receive/send signal from/to UE and remove Xn/NG connection. If the authorization status changes is triggered by CN, the SCTP connection shall be removed.

Proposal 5: A new indicator can be included in the Xn Setup message to specify the survival time of the Xn interface.

Proposal 6: A new WAB-gNB Indicator should be included in the NG Setup Request message.

Proposal 7: The AMF can signal 'authorized' or 'not authorized' in the AMF CONFIGURATION UPDATE message.

Proposal 8: The NG connection removal procedure must be triggered by the WAB-gNB if the authorization status changes from 'authorized' to 'not authorized'.

Reference

- RP-242395，New WID on additional topological enhancements for NR，NTT DOCOMO, INC., AT&T