3GPP TSG RAN WG3 Meeting #127	R3-250583

Athens, Greece, 17 - 21 February, 2025

Agenda Item:	14.3

Source:	ETRI, Ajou University

Title:	Discussion on Support of Inactive UE in NTN

Document for:	Discussion

- Introduction

The Rel-19 NR NTN WID[1] includes the following objectives to support regenerative payload with full gNB on board:

Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In the previous meeting, RAN3 got the following progresses on NR NTN regenerative payload:

RAN3#126: Inactive UE support: Option 1, UE context is forwarded to the next satellites Option 2, UE context is kept on a fixed node (on-ground gNB or AMF) Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used Whether to support NG suspend/Resume? Continue the discussion on above open issues…

In this contribution, we propose to further discuss the issues on the support of inactive UE in NTN.

- Discussion

  - Potential Solutions for the Support of Inactive UE

Motivated by that re-using the existing mechanism designed for TN cannot fully and well support inactive UE in NTN, we have discussed multiple potential solutions in the last meeting[2] as follows:

(Solution 1) Forwarding UE context to the next satellites

(Solution 2) Keeping UE context in a fixed node (on-ground gNB or AMF)

(Solution 3) Keeping UE context in the last serving gNB and Retrieving the context using NG-based retrieval procedure

  - (Solution 1) Forwarding UE context to the next satellites

In Solution 1, the context of inactive UE is forwarded to the next satellites so that the UE context can be retrieved from the nearby gNB when the UE resumes its RRC connection.

    - Possible Options

- Option 1-1: Paging & Relocation (Network-triggered)

The last serving gNB initiates RAN paging for the inactive UE when its Xn interface with the candidate gNBs relevant to the UE’s RNA is about to be disconnected. After paging, the UE context can be relocated to the current serving gNB.

- Option 1-2: Periodic RNA Update & Relocation (UE-triggered)

The UE can be configured to initiate Periodic RNA update when the serving gNB changes. After receiving RRCResumeRequest for RNA update, the current serving gNB can retrieve the UE context from the last serving gNB.

- Option 1-3: Proactively moving UE context to the next satellite

When the serving gNB changes or the Xn interface is about to be disconnected, the last serving gNB pushes the UE context to the incoming candidate gNB proactively.

    - Problems and Issues

(1) Signaling Storm for each relocation

In Option 1-1, all inactive UEs in the coverage of current serving gNB should be paged. Moreover, in the mega-constellation environment, there may be many candidate gNBs relevant to the UE’s RNA. Thus, one or more gNBs would be involved in each RAN paging for the inactive UE.

In Option 1-2, a lot of inactive UEs need to initiate Periodic RNA update simultaneously when the serving gNB changes.

In Option 1-3, the UE contexts of all inactive UEs should be pushed to a candidate gNB proactively. In case of mega-constellation, we additionally have some issues including whether/how to select one or more candidate gNBs for pushing the UE contexts.

Therefore, these Options in Solution 1 can cause heavy signaling storm for each relocation/pushing and there are new issues that we need to consider.

(2) Significant Signaling Overhead due to the repeated relocation/pushing

We’d like to note that this discussion on the support of inactive UE in NTN was raised from the case study in which the inactive UE resumes its RRC connection after several serving gNBs have passed.

Therefore, the UE context relocation or proactively pushing should be repeated until the inactive UE actually resumes the RRC connection for UL/DL data transmission as shown in Figure 1.

So, these Options in Solution 1 may have significant signaling overhead due to the repeated relocation/ pushing.

Figure 1: Example of repeated UE context relocation/pushing (in Option 1-1 and 1-3)

(3) Security Issues and Xn Standard Impacts

When an inactive UE resumes its RRC connection, it should be verified by the serving gNB. Option 1-3 may have security issues with the UE verification based on the UE context pushed by the last serving gNB. Moreover, Option 1-3 has some standard impacts on Xn interface to select the target gNB and forward the UE context.

Observation 1: (Solution 1)

| Solution | (Solution 1) Forwarding UE context to the next satellites |
| --- | --- |
| Possible Options | - Option 1-1: Paging & Relocation (Network-triggered) - Option 1-2: Periodic RNA Update & Relocation (UE-triggered) - Option 1-3: Proactively moving UE context to the next satellite |
| Problems & Issues | - Signaling Storm for each relocation - Significant Signaling Overhead due to the repeated relocation/pushing - New issues including target gNB selection, security, and so on |
| Standard Impacts | - Xn interface in Option 1-3 |

  - (Solution 3) Keeping UE context in the last serving gNB and Retrieving the context using NG-based retrieval procedure

In Solution 3, the last serving gNB keeps the UE context of inactive UE, and when the inactive UE resumes the RRC connection, the current serving gNB initiates NG-based UE context retrieval procedure to get the UE context from last serving gNB via AMF on the ground.

    - Problems and Issues

(1) Partial Solution not working in some cases

In Solution 3, the UE context is retrieved from the last serving gNB via AMF on the ground. Thus, the NG connection between the AMF and the last serving gNB should be maintained.

However, as shown in Figure 2, the Solution 3 does not work in the following cases:

- Released NG interface

When the last serving gNB leaves the serving area of AMF, the NG interface between the AMF and the last serving gNB is released.

- Unreachable from the AMF

The last serving gNB is not reachable from the AMF when the last serving gNB is flying over the area(e.g. ocean) in which there is no feeder link connected with a GW on the ground.

Figure 2: Example of the cases in which the Solution 3 does not work

(2) NG Standard Impacts

The Solution 3 has some standard impacts on NG interface to retrieve the UE context from the last serving gNB via AMF.

Observation 2: (Solution 3)

| Solution | (Solution 3) Keeping UE context in the last serving gNB and Retrieving the context using NG-based retrieval procedure |
| --- | --- |
| Problems & Issues | - Partial Solution not working in some cases (e.g. the last serving gNB is not reachable from the AMF as it released NG connection with the AMF or it has no feeder link to the ground) |
| Standard Impacts | - NG interface |

  - (Solution 2) Keeping UE context in a fixed node (on-ground AMF or gNB)

In Solution 2, the UE context is stored in a fixed node (e.g. AMF or gNB) on the ground.

    - Possible Options

- Option 2-1: Storing UE context in the AMF

The last serving gNB forwards the UE context to the AMF on the ground, when its Xn interface with the candidate gNB(s) or its NG interface with AMF is about to be disconnected. When receiving RRCResumeRequest from the inactive UE, if the current serving gNB fails to retrieve the UE context from the last serving gNB, it can fetch the UE context from the AMF on the ground.

- Option 2-2: Storing UE context in the on-ground gNB

The last serving gNB forwards the UE context to the gNB on the ground, when its Xn interface with the candidate gNB(s) or its NG interface with AMF is about to be disconnected. When receiving RRCResumeRequest from the inactive UE, if the current serving gNB fails to retrieve the UE context from the last serving gNB, it can fetch the UE context from the gNB on the ground.

    - Problems and Issues

(1) New function of a fixed node on the ground

In Option 2-1 and 2-2, it is required to introduce new function for keeping the UE context in the AMF / gNB on the ground.

Additionally, the Option 2-2 has some issues including the selection of available gNB on the ground, the Xn interface management between on-board gNB and on-ground gNB, and the mobility support of inactive NTN UEs through on-ground gNB, and so on.

(2) Xn / NG Standard Impacts

The Option 2-1 and 2-2 have some standard impacts on NG / Xn interface to forward & retrieve the UE context to/from on-ground AMF / gNB.

Observation 3: (Solution 2)

| Solution | (Solution 2) Keeping UE context in a fixed node (on-ground AMF or gNB) |
| --- | --- |
| Possible Options | - Option 2-1: Storing UE context in the AMF - Option 2-2: Storing UE context in the on-ground gNB |
| Problems & Issues | - Requiring new function of a fixed node on the ground - New issues including the selection and the management of Xn interface with gNB on the ground in Option 2-2 |
| Standard Impacts | - NG interface in Option 2-1 - Xn interface in Option 2-2 |

  - Preferred Solution : Solution 2 (Option 2-1 Storing UE context in the AMF)

As mentioned above, the Solution 1 and 3 have some critical problems as follows:

- Solution 1(Forwarding UE context to the next satellites) has significant signaling overhead caused by each UE context relocation, and it becomes more severe with repeated relocation.

- Solution 3(Retrieving UE context using NG-based retrieval procedure) does not work in some cases (e.g. the last serving gNB is not reachable from the AMF as it released NG connection with the AMF or it has no feeder link to the ground)

And all Solutions have some standard impacts (except Option 1-1 and 1-2) and some Options have open issues that require further consideration.

Therefore, even though the Option 2-1 in Solution 2 has NG standard impacts and requires new function of AMF, it is beneficial in terms of reduced signaling overhead with minimized context relocation, not relying on the Xn interface, short service latency, and for a mobile terminated data/signaling, the AMF can initiate RNA-based paging and the current serving gNB can get the UE context from the AMF effectively as shown in the example of call flows in Figure 3.

(a) Relocation of UE context from the last serving gNB

(b) Resuming RRC Connection for a mobile originated data/signaling

(c) Resuming RRC Connection for a mobile terminated data/signaling

Figure 3: Example of call flows in Option 2-1 of Solution 2

Observation 4: The Option 2-1 in Solution 2 is beneficial in terms of reduced signaling overhead with minimized context relocation, not relying on the Xn interface, short service latency, and for a mobile terminated data/signaling, the AMF can initiate RNA-based paging and the current serving gNB can get the UE context from the AMF effectively.

Proposal 1: RAN3 to consider the Option 2-1 in Solution 2 as a basic solution for the support of Inactive UE in NTN.

- Conclusion

Based on the discussion in section 2, we have the following observations and proposals:

Observation 1: (Solution 1)

| Solution | (Solution 1) Forwarding UE context to the next satellites |
| --- | --- |
| Possible Options | - Option 1-1: Paging & Relocation (Network-triggered) - Option 1-2: Periodic RNA Update & Relocation (UE-triggered) - Option 1-3: Proactively moving UE context to the next satellite |
| Problems & Issues | - Signaling Storm for each relocation - Significant Signaling Overhead due to the repeated relocation/pushing - New issues including target gNB selection, security, and so on |
| Standard Impacts | - Xn interface in Option 1-3 |

Observation 2: (Solution 3)

| Solution | (Solution 3) Keeping UE context in the last serving gNB and Retrieving the context using NG-based retrieval procedure |
| --- | --- |
| Problems & Issues | - Partial Solution not working in some cases (e.g. the last serving gNB is not reachable from the AMF as it released NG connection with the AMF or it has no feeder link to the ground) |
| Standard Impacts | - NG interface |

Observation 3: (Solution 2)

| Solution | (Solution 2) Keeping UE context in a fixed node (on-ground AMF or gNB) |
| --- | --- |
| Possible Options | - Option 2-1: Storing UE context in the AMF - Option 2-2: Storing UE context in the on-ground gNB |
| Problems & Issues | - Requiring new function of a fixed node on the ground - New issues including the selection and the management of Xn interface with gNB on the ground in Option 2-2 |
| Standard Impacts | - NG interface in Option 2-1 - Xn interface in Option 2-2 |

Observation 4: The Option 2-1 in Solution 2 is beneficial in terms of reduced signaling overhead with minimized context relocation, not relying on the Xn interface, short service latency, and for a mobile terminated data/signaling, the AMF can initiate RNA-based paging and the current serving gNB can get the UE context from the AMF effectively.

Proposal 1: RAN3 to consider the Option 2-1 in Solution 2 as a basic solution for the support of Inactive UE in NTN.

- Reference

RP-240775	Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

R3-247316	Support of Inactive UE mobility in NTN (Xiaomi, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, China Telecom, Lenovo)