3GPP TSG-RAN WG3 #127	          R3-250634

# Athens, Greece, 17th – 21th Feb 2025

Agenda item:	14.3

Source: 	Samsung

Title: 	Further discussion on support of regenerative payload for NR NTN

Document for:	Discussion and decision

Introduction

Last meeting further discussed the support of regenerative payload for NR NTN, and the following consensus is achieved,

Inactive UE support:

Option 1, UE context is forwarded to the next satellites

Option 2, UE context is kept on a fixed node (on-ground gNB or AMF)

Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

Whether to support NG suspend/Resume?

Continue the discussion on above open issues…

In this contribution, we further discuss the support of regenerative payload.

Discussion

Last RAN3 meeting continued to discuss the NG interface management. And the following open issue is captured,

RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators to avoid frequent TNL disconnection and re-connection?

And another alternative is to introduce new NG suspend/resume procedures.

According to the discussion during last several RAN3 meetings, the proponents of NG suspend/resume mentioned that the introduction of NG suspend/resume will save transmission energy when recovering the connection between gNB and AMF since less information is expected to be transmitted compared to NG setup as well as the following up RAN/AMF Configuration Update procedures.

On the other hand, the introduction of NG suspend/resume will require much more storage on NG interface management related information for gNB on board, especially after several orbits, the gNB on board may need to store the NG interface related information with every AMF along the orbits. In addition, there’s also a concern that whether the original suspended information is still valid when the gNB on board connects back to a specific AMF.

In our understanding, considering that the supported TA list of an on-board gNB could be provided to AMF by OAM, the main information that is needed to be stored at the gNB onboard is the TNL association related information, which actually does not require too much storage space, even if the gNB onboard needs to maintain the interface related information with every AMF along the orbits.

In addition, we observe that the addition/update/removal of the TNLA is initiated by the AMF, and both AMF and gNB is aware of the ephemeris of the regenerative payload as well as the supported TA information for a gNB on board. As a consequence, we think it is also worth considering whether to introduce suspend/resume indicators to AMF Configuration Update procedure instead.

And we can immediately see a benefit when we introduce a resume indicator to AMF CONFIGURATION UPDATE message that it will potentially save the number of interface management messages to be exchanged when the feeder link recovers. More specifically,

- Reusing the NG Setup approach to recover the feeder link: Needs one Class 1 NG Setup procedure + one Class 1 AMF Configuration Update procedure if there are TNLAs that are needed to be added/updated/removed.

- Adding resume indicator in RAN CONFIGURATION UPDATE message approach to recover the feeder link: Needs one Class 1 RAN Configuration Update procedure + one Class 1 AMF Configuration Update procedure if there are TNLAs that are needed to be added/updated/removed.

- Our proposed approach: Only needs one Class 1 AMF Configuration Update procedure to add/update/remove TNLAs and recover the feeder link at the same time.

We think it is the simplest approach we can identify.

Proposal 1: Introduce Suspend/Resume indicators in AMF CONFIGURATION UPDATE message, to support NG suspend/resume.

The next open issue is about whether the Xn interface can be established over feeder link, as well as whether the NG interface can be established over ISL.

And we take whether the NG interface can be established over ISL as an example.

From connection perspective, the current discussion only assumes that the feeder link is always available between NTN GW and gNB onboard. However, in potential deployment (which has also been mentioned by operators), it is not always guaranteed that the feeder link is always available for a specific operator. As a consequence, when the direct feeder link between the gNB onboard and the NTN GW is unavailable, we need to provide means to ensure that the NG interface can still be maintained in some cases e.g. when the atmospheric attenuation is too severe due to the bad weather. As an alternative approach, we can trmporarily use ISL to carry NGAP messages in order to maintain the connection.

Note that in our understanding, if the feeder link is available, it is more reasonable for the gNB onboard to use the direct feeder link to carry NGAP messages.

Proposal 2: It is possible to use ISL to carry NGAP messages.

The next open issue is about the LS received by RAN3 from SA2 and SA5. RAN3 agreed that the information of supported TAIs for NTN gNB is provided to AMF by OAM, in order to avoid frequent NGAP signalling for configuration update. And RAN3 sent an LS to SA2 and SA5 to ask for their opinion on such OAM based solution.

According to the reply LS from SA5 in R3-250030, SA5 has agreed to support the OAM based solution and will further work on this in R19 WI.

While according to the reply LS from SA2 in R3-250023, SA2 still has concerns on the OAM based solution and would like to confirm the following understanding with RAN3,

- SA2 discussed whether the OAM solution would have impacts to the existing procedures related to paging and sequencing of OAM updates and RAN3 updates towards the AMF, e.g. in NGAP Setup/configuration update etc. However, it was unclear to SA2 how the OAM solution would work based on the description in the incoming LS to be able to determine whether there are impacts.

- The frequency of updates by OAM was not clear to SA2 and whether the information provided to the AMF is a timetable of which TAIs the gNB will support at what time, or whether the updates are provided frequently as the satellite moves updating the AMF each time a change occurs.

SA2 would like to confirm whether RAN3 and SA5 are aligned on the frequency and contents of the OAM updates to the AMF.

In our understanding, RAN3’s agreement implies that after NG Setup procedure, the NG-RAN node will not provide the update of supported TAIs via NGAP signalling; instead, the update of supported TAIs will be totally up to the OAM based solution. So there will be no issue on the sequencing of OAM updated and RAN3 updates towards the AMF.

Regarding the frequency of updated by OAM, in our understanding, the information provided to the AMF is better to be a timetable, but it is totally up to SA5 to decide the detailed solution since SA5 will work on this in R19.

Regarding the content of the OAM updates to the AMF, our understanding is that it is clear enough that the content of the update is on the supported TAIs for the gNB.

Proposal 3: No issue is identified to support the OAM based solution on the update of supported TAIs by the gNB.

Another open issue is on the support of RRC Inactive UEs.

For the options,

Option 1, UE context is forwarded to the next satellites

Option 2, UE context is kept on a fixed node (on-ground gNB or AMF)

Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

Remember that we’ve already agreed the following agreement in RAN3,

RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment.

So basically, the current mechanism has already been workable without supporting any of the options on the table for enhancement.

Firstly, we think Option 1 is not a good choice, since it introduces too much signalling overhead over Xn/NG interface, especially for those NGSO satellites.

Secondly, for NGSO satellites, the Option 3 does not work very well especially when the feeder link between the anchor gNB and the CN on ground is unavailable; under such condition, the context retrieve will still be failed so the benefit looks limited to introduce extra NG context retrieve procedure only.

Thirdly, for Option 2, the solution to keep UE context on an on-ground gNB will introduce other open issues including e.g. the need to define a special on-ground gNB to store the UE context, the forwarding mechanism, etc., which introduces too much spec impact. As a result, compared to keeping the UE context on an on-ground gNB, keeping the UE context on AMF looks more reasonable. On the other hand, the feasibility to keep the UE context on AMF needs coordination with SA2, and the primary observation is that storing all the RAN side UE context (including RRC context and NG context for all the RRC inactive UEs) will require an extra huge amount of storage space from the CN.

Proposal 4: No enhancement is pursued to support inactive UEs.

Conclusion

In this paper, we provide primary discussion on support of regenerative payload for NR NTN. The following proposals are provided,

Proposal 1: Introduce Suspend/Resume indicators in AMF CONFIGURATION UPDATE message, to support NG suspend/resume.

Proposal 2: It is possible to use ISL to carry NGAP messages.

Proposal 3: No issue is identified to support the OAM based solution on the update of supported TAIs by the gNB.

Proposal 4: No enhancement is pursued to support inactive UEs.

References