3GPP TSG-RAN WG3 Meeting #127	R3-250681

Athens, Greece, 17th – 21th Feb 2025

Title: 	Discussion on Support of regenerative payload for NR NTN

Source: 	CMCC

Agenda item: 	14.3

Document Type:	Discussion

- Introduction

In the 3GPP TSG RAN Meeting #103, the work plan for Non-Terrestrial Networks (NTN) for NR Phase 3 has been approved in RP-240775. One of the objectives is support of regenerative payload [RAN3, RAN2, RAN4]:

- Specify the support of gNB on board in TS 38.300

- Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3]

- Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

Based on it, in RAN3 #125 meeting, the following conclusions are captured:

NG Interface management Issue:

Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management.

Further discussions on benefits to introduce Option 3 NG Suspend/Resume for NG interface management are not precluded but there are oppositions of some companies to have 2 methods for NG interface management.

Inactive UE support

RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment.

Feeder link switch issue

RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

And During RAN3 #125-bis, the following have been captured in chairlady’s notes:

The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration.

The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch.

Legacy NG HO procedure can be reused for inter AMF HO for NTN regenerative payload.

RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators to avoid frequent TNL disconnection and re-connection?

Also TP has been accepted to introduce NG Removal procedures in TS38.410 and TS38.413.

During RAN3 #126, no agreements were achieved. But companies are encouraged to think more pros and cons on RRC_INACTIVE connections resume and supporting NG Suspend/Resume.

2	Discussion

In RAN3 #125bis, it is proposed by some companies that RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators to avoid frequent TNL disconnection and re-connection.

Also in RAN3 #126, related discussions continued. And companies are encouraged to think more pros and cons on RRC_INACTIVE connection resume and supporting NG Suspend/Resume. So in this draft proposal, we will focus on these 2 topics: whether introduce NG Suspend/Resume Procedures and whether introduce the enhancement for RRC_INACTIVE connection resume.

2.1 NG Suspend/Resume Procedures Enhancement

To recover the UE connections quickly, some companies suggest to introduce the NG Suspend/Resume Procedures.  When the satellite is moving out of the service area of AMF, it switches UEs over to the new gNB through Xn or through AMF. After UE Handover execution, the gNB can sends NG Suspend procedures to AMF and restore the UE context such as UE ID, PDU session ID and some others in the satellite (gNB on board), NG connections will be suspended. And when the satellite moves back to the previous AMF, it can quickly set up NG through NG Resume messages, and then resume RRC connections for the UE.

Proposal 1: It is beneficial to introduce NG Suspend/Resume procedures in some scenarios, to help the NG connection recover quickly.

The NG-RAN node ID, UE ID, PDU Session ID shall be carried in NG Resume message from NG-RAN node to AMF, to quickly recover the NG connection.

To improve the efficiency of NGAP messages over the feeder link, a list of UE Context can be carried in one NG suspend/resume message. The NG Suspend/Resume shall be designed as non-UE associated procedures.

When AMF receive this NG Resume message, it shall check the UE ID and PDU session ID in its own UE context information, and know which PDU sessions are still active and shall be resumed.

Proposal 2: It is proposed to carry the NG-RAN node ID, UE ID, PDU Session ID in NG Resume message from NG-RAN node to AMF. Using these information, AMF can check with its own UE context information and know which PDU sessions shall be resumed.

Scenario 3: UE transfer from Satellite1(gNB1 on board) to Satellite2(gNB2 on board)  through Xn when it is leaving the AMF serving area, NG Suspend/Resume Procedures

Scenario 4: UE transfer from Satellite1(gNB1 on board) to Satellite2(gNB2 on board)  through AMF when it is leaving the AMF serving area, NG Suspend/Resume Procedures

And for RRC_INACTIVE UE, RRC suspend procedures shall finish before NG suspend, the UE Context still restores in the old gNB. And after NG Resume, RRC may resume for the Inactive UEs, the new gNB may retrieve the UE context through UE Context Resume message from the old gNB. These can reuse the current procedures.

However, some satellite companies think the storage and energy resource is very expensive and limited in satellite, store the so many UE context in satellite for such a long time cost too much.

Also some other companies think the UE won’t stay so long under the same coverage area for after the satellite moves back.  It is meaningless to store the UE context in satellite for such a long time.

So it is better to let the satellite decides whether it shall active the NG Suspend/Resume Procedures for quick recovery according to its own resource status and its moving trajectory.

Proposal 3：It is up to satellite (gNB on board) to make decision whether and when to activate NG Suspend/Resume Procedures.

2.2 RRC_Inactive connections Resume

After some discussion, RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment.

Meanwhile, it is still beneficial for UE if the RRC_Inactive connections can resume quickly from the satellite moving out to the satellite moving in.

When the satellite (regenerative payload) moves out of the service area of UE, the ideal scenario is to handover all the UEs to the new gNB on satellite(regenerative payload) or to the ground. However, no matter the handover through ISL or through feeder link, it is more risky than on the ground that the handover can’t succeed. And also it is high cost to handover through satellites.

Meanwhile, for the RRC_Inactive UEs, the UE contexts can’t be HO to the new serving gNB. And when UE RRC resume with the new gNB, the new gNB needs to retrieve the UE contexts from the old gNB. However in the old gNB, the UE state may still be Inactive and so the retrieve fail. And it is a waste of ISL or feeder link resource to send the retrieve UE context request from the new gNB to the old gNB.

After the new gNB fails to retrieve UE contexts from the old gNB, it will reject the UE resume. And then, after some time, UE can try to set up new connections with AMF and new gNB. And the UE context will exist in the old satellite (gNB on board) for a longer time.

And during this period, if DL data comes, AMF will first reach the old gNB and then old gNB pages to the new gNB. It probably fails due to the unstable ISL or feeder link. Even if it succeeds, it takes a long time to finally pages to the UE. All this process is long, high cost and risky. In this solution, UE has a bad experience and satellite is not efficient for keeping the UE context for a long time and too much un-necessary signal interworking with 5GC.

It seems a simple and better solution that if we can store the UE context in AMF and use AMF paging for DL data to new gNB after the old satellite moves out.

And the old gNB can release the RRC_INACTIVE connections, turn UE into RRC_IDLE and  remove the corresponding UE context if no NG Suspend/Resume procedures triggered. It also avoid other conflicts or un-useful paging signals if UE turns into RRC_IDLE state from RRC_INACTIVE state when the satellite moves out.

Proposal 4: When the satellite (regenerative payload) moves out, it is proposed to release the RRC connections for all the RRC_INACTIVE connections, and turn them into RRC_IDLE connections, then release the corresponding UE context. In this way to avoid the cost for gNB to restore useless information.

Proposal 5: When the satellite (regenerative payload) moves out, it is proposed to save UE context into AMF before NG Removal. When DL data or signalling arrives for the UE, 5GC is responsible for paging them to the new gNB.

Proposal 6: It is proposed to clarify in stage2 that all the UE contexts shall be removed before NG Removal procedures for RRC_CONNECTED and RRC_INACTIVE UE.

Meanwhile if the NG Suspend/Resume procedures will be introduced, similar scenario can happen if the HO is not successful for RRC_CONNECTED UEs with RRC_INACTIVE UEs. To quickly resume the NG connections when the satellite(gNB on board) moves back, when satellite(gNB on board) moves out, RRC_INACTIVE UE context can be restored in the CN through NG Suspend message. When the new satellite comes as the serving gNB, it can quickly resume RRC connection with UE and retrieve UE Context from CN through UE Context Resume Request.

2.3 NG Pre-Set Up Procedures Enhancement

In above procedures, to guarantee the continuity of UE service, the new satellite (gNB on board) needs to set up NG connection with AMF before the previous satellite (gNB on board) is leaving the AMF serving area.

Usually, if the ephemeris is provisioned, AMF know when to initiate the setup of NG connections with satellites in advance. If not, satellite can be triggered to setup NG with AMF in advance according to time and location condition which is provisioned, before the previous satellite leaves the AMF serving area.

The procedures for NG pre-setup we proposed can refer to the NTN time or location triggered CHO procedures via Xn. Time-based or location-based triggerment may be configured independently for NG pre-setup in NTN in at least hard satellite switch case where the service discontinuity gap time length is assumed to be zero or negligible.

Here the satellite (gNB on board) still keeps the NG connections with the previous AMF when it sets up the NG connections with the new AMF. The old NG connections will be released or deleted after UE HO, as procedures proposed in chapter 2.1 and 2.2.  Since at this moment, there is no UE context in the new satellite (gNB on board), so the pre-set up NG interfaces are gNB and AMF pair level.

Proposal 7: It is proposed to introduce the time and location-triggered NG pre-setup before the UE HO to the new satellite (gNB on board), to guarantee the UE service continuity.

3	Summary

In this contribution, we provide considerations on the NTN regenerative payload in RAN3, following observations and proposals are made:

Proposal 1: It is beneficial to introduce NG Suspend/Resume procedures in some scenarios, to help the NG connection recover quickly.

Proposal 2: It is proposed to carry the NG-RAN node ID, UE ID, PDU Session ID in NG Resume message from NG-RAN node to AMF. Using these information, AMF can check with its own UE context information and know which PDU sessions shall be resumed.

Proposal 3：It is up to satellite (gNB on board) to make decision whether and when to activate NG Suspend/Resume Procedures.

Proposal 4: When the satellite (regenerative payload) moves out, it is proposed to release the RRC connections for all the RRC_INACTIVE connections, and turn them into RRC_IDLE connections, then release the corresponding UE context. In this way to avoid the cost for gNB to restore useless information.

Proposal 5: When the satellite (regenerative payload) moves out, it is proposed to save UE context into AMF before NG Removal. When DL data or signalling arrives for the UE, 5GC is responsible for paging them to the new gNB.

Proposal 6: It is proposed to clarify in stage2 that all the UE contexts shall be removed before NG Removal procedures for RRC_CONNECTED and RRC_INACTIVE UE.

Proposal 7: It is proposed to introduce the time and location-triggered NG pre-setup before the UE HO to the new satellite (gNB on board), to guarantee the UE service continuity.

4	Reference

[1] 	RP-240775, work plan for NTN

[2]       TS 38.300, NG-RAN stage2

[3]       TS 38.423, Xn application protocol (XnAP)

[4]       TS 38.413, NG application protocol (NGAP)

[5]       TS 38.331, Radio Resource Control Protocol (RRC)

[6]       TR 38.821, Solutions for NR to support NTN

[7]       RAN3_126_agenda_20241122_1444_EOM, chairlady’s note