3GPP TSG RAN WG3 Meeting #127	R3-250753

Athens, Greece, 17 - 21 February, 2025

Agenda Item:	31.2

Source:	Huawei

Title:	Response to R3-250203 and R3-250567

Document type:	Response

# 1	Introduction

This paper is a response to the discussion paper in [1], where it is proposed to enhance the NGAP to include the 2 octets TAC for E-UTRAN as a forbidden area information for NR to LTE mobility restriction.

# 2	Discussion

Currently, in NGAP, the Forbidden Area Information is included in the mobility restriction list carrying the three Octet TAC, and is referring to the TS 23.501.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Serving PLMN | M |  | PLMN Identity 9.3.3.5 |  | - |  |
| <<<<<<<<<<<<<<<<<<<< Irrelevant Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Forbidden Area Information |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains Forbidden Area information as specified in TS 23.501 [9]. | - |  |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >Forbidden TACs |  | 1..<maxnoofForbTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | The TAC of the forbidden TAI. | - |  |

The discussion in [1] had the following analysis:

In TS 29.503 sec. 6.1.6.2.4, which specifies the forbiddenAreas exchanged via SBI in 5GC, describes this IE as "List of forbidden areas in 5GS," and the array(Area) referred to by this IE is defined as 2 or 3 octets in TS 29.571 sec. 5.4.2. Our understanding is that NR TAC is defined as 3 octets in TS 23.003, so it can be implied as EPS TAC when forbiddenAreas is 2 octets and NR TAC when it is 3 octets. On the other hand, these specifications do not explicitly specify that EPS TAC can be configured for forbiddenAreas

First, when referring to TS 23.501, the forbidden area is specified as follows.

Mobility Restrictions consists of RAT restriction, Forbidden Area, Service Area Restrictions, Core Network type restriction and Closed Access Group information as follows: <<<<<<<<<<<<<<<<<<<< Irrelevant Text Omitted >>>>>>>>>>>>>>>>>>>> - Forbidden Area: In a Forbidden Area, the UE, based on subscription, is not permitted to initiate any communication with the network for this PLMN. The UE behaviour in terms of cell selection, RAT selection and PLMN selection depends on the network response that informs the UE of Forbidden Area. A Forbidden Area applies either to 3GPP access or to non-3GPP access. Further description on Forbidden Area when using wireline access is available in TS 23.316 [84]. Support for Forbidden Area with NR satellite access is described in clause 5.4.11.8. Forbidden Areas should not be used for Untrusted or Trusted non-3GPP access. NOTE 1: If a UE receives that the UE is accessing from a forbidden tracking area when registering over untrusted non-3GPP access or trusted non-3GPP access, the UE cannot determine the corresponding TAI and thus needs to consider that access to untrusted non-3GPP access and to trusted non-3GPP access in this PLMN is forbidden until the forbidden area list is removed as described in TS 24.501 [47]. NOTE 2: The UE reactions to specific network responses are described in TS 24.501 [47].

Though the TS 23. 501 does not provide details on the TAC encoding details, the analysis in [1] is saying that the TS 29.571 provides the 2 or 3-octet string of TAC, as excerpted as follows.

| Tac | String | 2 or 3-octet string identifying a tracking area code as specified in clause 9.3.3.10 of 3GPP TS 38.413 [11], in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the TAC shall appear first in the string, and the character representing the 4 least significant bit of the TAC shall appear last in the string. Examples: A legacy TAC 0x4305 shall be encoded as "4305". An extended TAC 0x63F84B shall be encoded as "63F84B" |
| --- | --- | --- |

Hence we think the analysis is correct to add the 4G TAC as the forbidden area information into the mobility restriction list in the NGAP. And we agree it is better to sends LS to SA2/CT4 for potential check.

Observation 1:  The analysis in [1] is correct to add the 4G TAC as the forbidden area information into the mobility restriction list in the NGAP. And it is better to send LS to SA2/CT4 for potential specification check.

On the other hand, we think that the handover restriction list in S1AP should also be considered, as copied follows.

<<<<<<<<<<<<<<<<<<<< Excerpt begins >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.22	Handover Restriction List

This IE defines roaming or access restrictions for subsequent mobility action for which the eNB provides information about the target of the mobility action towards the UE, e.g., handover and CCO, or for SCG selection during dual connectivity operation. If the eNB receives the Handover Restriction List IE, it shall overwrite previously received restriction information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Serving PLMN | M |  | 9.2.3.8 |  |  |  |
| Equivalent PLMNs |  | 0..<maxnoofEPLMNs> |  | Allowed PLMNs in addition to Serving PLMN. This list corresponds to the list of “equivalent PLMNs” as defined in TS 24.301 [24]. This list is part of the roaming restriction information. Roaming restrictions apply to PLMNs other than the Serving PLMN and Equivalent PLMNs. |  |  |
| >PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| Forbidden TAs |  | 0..<maxnoofEPLMNsPlusOne> |  | Intra LTE roaming restrictions. |  |  |
| >PLMN Identity | M |  | 9.2.3.8 | The PLMN of forbidden TACs. |  |  |
| >Forbidden TACs |  | 1..<maxnoofForbTACs> |  |  |  |  |
| >>TAC | M |  | 9.2.3.7 | The TAC of the forbidden TAI. |  |  |
| Forbidden LAs |  | 0..<maxnoofEPLMNsPlusOne> |  | Inter-3GPP RAT roaming restrictions. |  |  |
| >PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| >Forbidden LACs |  | 1..<maxnoofForbLACs> |  |  |  |  |
| >>LAC | M |  | OCTET STRING (SIZE(2)) |  |  |  |
| Forbidden inter RATs | O |  | ENUMERATED(ALL, GERAN, UTRAN, CDMA2000, …, GERAN and UTRAN, CDMA2000 and UTRAN) | Inter-3GPP and 3GPP2 RAT access restrictions. “ALL” means that all RATs mentioned in the enumeration of this IE are restricted. |  |  |
| NR Restriction in EPS as Secondary RAT | O |  | ENUMERATED(NRrestrictedinEPSasSecondaryRAT, …) | Restriction to use NR when the NR is used as secondary RAT in EN-DC. | YES | ignore |
| Unlicensed Spectrum Restriction | O |  | ENUMERATED(UnlicensedRestricted, …) | Restriction to use unlicensed spectrum in the form of LAA or LWA/LWIP or NR-U as described in TS 23.401 [11]. | YES | ignore |
| Core Network Type Restrictions |  | 0..<maxnoofEPLMNsPlusOne> |  | Includes any of the Serving PLMN or any PLMN of the Equivalent PLMNs listed in the Mobility Restriction List IE for which Core network type restriction applies as specified in TS 23.501 [46]. |  |  |
| >PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| >Core Network Type | M |  | ENUMERATED(5GCForbidden, …,EPCForbidden) | Indicates whether the UE is restricted to connect to 5GC or to EPC for this PLMN. |  |  |
| NR Restriction in 5GS | O |  | ENUMERATED(NRrestrictedin5GS, …) | Restriction to use NR when the NR connects to 5GS. | YES | ignore |
| Last NG-RAN PLMN Identity | O |  | 9.2.3.8 | Indicates the NG-RAN PLMN ID from where the UE formerly handed over to EPS and which is preferred in case of subsequent mobility to 5GS. | YES | ignore |
| RAT Restrictions |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains RAT restriction related information as specified in TS 23.401 [11]. | YES | ignore |
| >PLMN Identity | M |  | 9.2.3.8 |  | - |  |
| >RAT Restriction Information | M |  | BIT STRING { LEO (0), MEO (1), GEO (2), OTHERSAT (3), NR-LEO (4), NR-MEO (5), NR-GEO (6), NR-OTHERSAT (7)} (SIZE(8, …)) | Each position in the bitmap represents a RAT. If a bit is set to "1", the respective RAT is restricted for the UE. If a bit is set to "0", the respective RAT is not restricted for the UE. | - |  |

<<<<<<<<<<<<<<<<<<<< Excerpt Ends >>>>>>>>>>>>>>>>>>>>

First, it can be observed that the Forbidden TAs only contains the LTE two-octet TAC for the intra-LTE-system mobility. Also, it can be seen that the inter-working with 5GS is already supported, e.g., by the Core Network Type Restrictions containing the 5GCForbidden, NR Restriction in 5GS etc. However, the 5GS forbidden area containing the 3-octet string TAC is missing.

Observation 2: In S1AP Handover Restriction List, the forbidden TAs only contains the LTE two-octet TAC. And the inter-working with 5GS is already supported, e.g., by the Core Network Type Restrictions containing the 5GCForbidden, NR Restriction in 5GS etc. However, the 5GS forbidden area containing the 3-octet string TAC is missing.

Second, in TS 36.331, as excerpted below, the UE can report the 5GC CGI information, where the 3-octet string 5GC TAC is included. This means the eNB can acquire those TACs supported by the neighbor gNBs.

MeasResultEUTRA ::= SEQUENCE { physCellId PhysCellId, cgi-Info SEQUENCE { cellGlobalId CellGlobalIdEUTRA, trackingAreaCode TrackingAreaCode, plmn-IdentityList PLMN-IdentityList2 OPTIONAL } OPTIONAL, <<<<<<<<<<<<<<<<<<<< Irrelevant Text Omitted >>>>>>>>>>>>>>>>>>>> [[ cgi-Info-5GC-r15 SEQUENCE (SIZE (1..maxPLMN-r11)) OF CellAccessRelatedInfo-5GC-r15 OPTIONAL ]] } } CellAccessRelatedInfo-5GC-r15 ::= SEQUENCE { plmn-IdentityList-r15 PLMN-IdentityList-r15, ran-AreaCode-r15 RAN-AreaCode-r15 OPTIONAL, -- Need OR trackingAreaCode-5GC-r15 TrackingAreaCode-5GC-r15, cellIdentity-5GC-r15 CellIdentity-5GC-r15 } CellIdentity-5GC-r15 ::= CHOICE{ cellIdentity-r15 CellIdentity, cellId-Index-r15 INTEGER (1..maxPLMN-r11) } <<<<<<<<<<<<<<<<<<<< Irrelevant Text Omitted >>>>>>>>>>>>>>>>>>>> TrackingAreaCode-5GC-r15 ::= BIT STRING (SIZE (24))

Observation 3: in TS 36.331, the UE can report the 5GC CGI information, where the 3-octet string 5GC TAC is included. This means the eNB can acquire those TACs supported by the neighbor gNBs.

Hence, if the NGAP CR is to be considered for the NR to LTE mobility, then the companion S1AP CR should also be considered for the LTE to NR mobility, to include the 5GS TAC list as the forbidden TAs 5GS. Also, other CRs for other interfaces including Xn, X2 can be considered together.

If the above is agreed, RAN3 can send the LS to SA2/CT4 attached with the two agreed CRs for any potential specification update in other WGs.

Proposal 1: RAN3 to agree the NGAP CR and S1AP CR together to support the forbidden area for interworking between 4G and 5G. Also, other CRs for other interfaces including Xn, X2 can be considered together.

Proposal 2: RAN3 to agree the LS to SA2/CT4 attached with agreed CRs for any potential specification update in other WGs.

The draft S1AP CR can be found in the Annex.

# 3	Conclusion

Based on the discussion wherein, we have the following proposal.

Observation 1:  The analysis in [1] is correct to add the 4G TAC as the forbidden area information into the mobility restriction list in the NGAP. And it is better to send LS to SA2/CT4 for potential specification check.

Observation 2: In S1AP Handover Restriction List, the forbidden TAs only contains the LTE two-octet TAC. And the inter-working with 5GS is already supported, e.g., by the Core Network Type Restrictions containing the 5GCForbidden, NR Restriction in 5GS etc. However, the 5GS forbidden area containing the 3-octet string TAC is missing.

Observation 3: in TS 36.331, the UE can report the 5GC CGI information, where the 3-octet string 5GC TAC is included. This means the eNB can acquire those TACs supported by the neighbor gNBs.

Proposal 1: RAN3 to agree the NGAP CR and S1AP CR together to support the forbidden area for interworking between 4G and 5G. Also, other CRs for other interfaces including Xn, X2 can be considered together.

Proposal 2: RAN3 to agree the LS to SA2/CT4 attached with agreed NGAP/S1AP CRs for any potential specification update in other WGs.

The draft S1AP CR can be found in the Annex.

# 4	References

- R3-250203 NR to LTE mobility restriction (NTT DOCOMO INC., Nokia)

- R3-250567 Enhancement of mobility restriction [Enhanced Mobility] (Nokia)

3GPP TSG-RAN WG3 Meeting #127	R3-25xxxx

Athens, GR, 17-21 Feb, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR |  | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement of handover restriction list [Enhanced Mobility] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-02-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently in the S1AP Handover Restriction List IE, the forbidden TAs only contain the LTE two-octet TACs. And the inter-working with 5GS is already supported, e.g., by the Core Network Type Restrictions containing the 5GCForbidden, NR Restriction in 5GS etc. However, the 5GS three-octet TACs as the forbidden area are missing for LTE to 5GS mobility. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the S1AP Handover Restriction List IE, introduce the 5GS three-octet TACs as the forbidden area for interworking with 5GS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The inter-system handover from LTE to 5GS based on the 5GS forbidden area is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.22, 9.3.4, 9.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.22	Handover Restriction List

This IE defines roaming or access restrictions for subsequent mobility action for which the eNB provides information about the target of the mobility action towards the UE, e.g., handover and CCO, or for SCG selection during dual connectivity operation. If the eNB receives the Handover Restriction List IE, it shall overwrite previously received restriction information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Serving PLMN | M |  | 9.2.3.8 |  |  |  |
| Equivalent PLMNs |  | 0..<maxnoofEPLMNs> |  | Allowed PLMNs in addition to Serving PLMN. This list corresponds to the list of “equivalent PLMNs” as defined in TS 24.301 [24]. This list is part of the roaming restriction information. Roaming restrictions apply to PLMNs other than the Serving PLMN and Equivalent PLMNs. |  |  |
| >PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| Forbidden TAs |  | 0..<maxnoofEPLMNsPlusOne> |  | Intra LTE roaming restrictions. |  |  |
| >PLMN Identity | M |  | 9.2.3.8 | The PLMN of forbidden TACs. |  |  |
| >Forbidden TACs |  | 1..<maxnoofForbTACs> |  |  |  |  |
| >>TAC | M |  | 9.2.3.7 | The TAC of the forbidden TAI. |  |  |
| Forbidden LAs |  | 0..<maxnoofEPLMNsPlusOne> |  | Inter-3GPP RAT roaming restrictions. |  |  |
| >PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| >Forbidden LACs |  | 1..<maxnoofForbLACs> |  |  |  |  |
| >>LAC | M |  | OCTET STRING (SIZE(2)) |  |  |  |
| Forbidden inter RATs | O |  | ENUMERATED(ALL, GERAN, UTRAN, CDMA2000, …, GERAN and UTRAN, CDMA2000 and UTRAN) | Inter-3GPP and 3GPP2 RAT access restrictions. “ALL” means that all RATs mentioned in the enumeration of this IE are restricted. |  |  |
| NR Restriction in EPS as Secondary RAT | O |  | ENUMERATED(NRrestrictedinEPSasSecondaryRAT, …) | Restriction to use NR when the NR is used as secondary RAT in EN-DC. | YES | ignore |
| Unlicensed Spectrum Restriction | O |  | ENUMERATED(UnlicensedRestricted, …) | Restriction to use unlicensed spectrum in the form of LAA or LWA/LWIP or NR-U as described in TS 23.401 [11]. | YES | ignore |
| Core Network Type Restrictions |  | 0..<maxnoofEPLMNsPlusOne> |  | Includes any of the Serving PLMN or any PLMN of the Equivalent PLMNs listed in the Mobility Restriction List IE for which Core network type restriction applies as specified in TS 23.501 [46]. |  |  |
| >PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| >Core Network Type | M |  | ENUMERATED(5GCForbidden, …,EPCForbidden) | Indicates whether the UE is restricted to connect to 5GC or to EPC for this PLMN. |  |  |
| NR Restriction in 5GS | O |  | ENUMERATED(NRrestrictedin5GS, …) | Restriction to use NR when the NR connects to 5GS. | YES | ignore |
| Last NG-RAN PLMN Identity | O |  | 9.2.3.8 | Indicates the NG-RAN PLMN ID from where the UE formerly handed over to EPS and which is preferred in case of subsequent mobility to 5GS. | YES | ignore |
| RAT Restrictions |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains RAT restriction related information as specified in TS 23.401 [11]. | YES | ignore |
| >PLMN Identity | M |  | 9.2.3.8 |  | - |  |
| >RAT Restriction Information | M |  | BIT STRING { LEO (0), MEO (1), GEO (2), OTHERSAT (3), NR-LEO (4), NR-MEO (5), NR-GEO (6), NR-OTHERSAT (7)} (SIZE(8, …)) | Each position in the bitmap represents a RAT. If a bit is set to "1", the respective RAT is restricted for the UE. If a bit is set to "0", the respective RAT is not restricted for the UE. | - |  |
| Forbidden TAs 5GS |  | 0..<maxnoofEPLMNsPlusOne> |  | 5GS roaming restrictions. | YES | ignore |
| >PLMN Identity | M |  | 9.2.3.8 | The PLMN of forbidden TACs. | - |  |
| >Forbidden TACs |  | 1..<maxnoofForbTACs> |  |  | - |  |
| >>5GS TAC | M |  | 9.2.3.53 | The TAC of the forbidden TAI. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofEPLMNs | Maximum no. of equivalent PLMN Ids. Value is 15. |
| maxnoofEPLMNsPlusOne | Maximum no. of equivalent PLMN Ids plus one. Value is 16. |
| maxnoofForbTACs | Maximum no. of forbidden Tracking Area Codes. Value is 4096. |
| maxnoofForbLACs | Maximum no. of forbidden Location Area Codes. Value is 4096. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.3.4	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-M6ReportAmount,

id-M7ReportAmount,

id-TimeBasedHandoverInformation,

id-Bearers-SubjectToDLDiscarding-Item,

id-Bearers-SubjectToDLDiscardingList,

id-ForbiddenTAs5GS,

maxnoofCSGs,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

ForbiddenInterRATs ::= ENUMERATED {

all,

geran,

utran,

cdma2000,

...,

geranandutran,

cdma2000andutran

}

ForbiddenTAs5GS ::= SEQUENCE (SIZE(1.. maxnoofEPLMNsPlusOne)) OF ForbiddenTAs5GS-Item

ForbiddenTAs5GS-Item ::= SEQUENCE {

pLMN-Identity		PLMNidentity,

forbiddenTACs		ForbiddenTACs5GS,

iE-Extensions		ProtocolExtensionContainer { {ForbiddenTAs5GS-Item-ExtIEs} } OPTIONAL,

...

}

ForbiddenTAs5GS-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

ForbiddenTAs ::= SEQUENCE (SIZE(1.. maxnoofEPLMNsPlusOne)) OF ForbiddenTAs-Item

ForbiddenTAs-Item ::= SEQUENCE {

pLMN-Identity		PLMNidentity,

forbiddenTACs		ForbiddenTACs,

iE-Extensions		ProtocolExtensionContainer { {ForbiddenTAs-Item-ExtIEs} } OPTIONAL,

...

}

ForbiddenTAs-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

ForbiddenTACs ::= SEQUENCE (SIZE(1..maxnoofForbTACs)) OF TAC

ForbiddenTACs5GS ::= SEQUENCE (SIZE(1..maxnoofForbTACs)) OF FiveGSTAC

ForbiddenLAs ::= SEQUENCE (SIZE(1..maxnoofEPLMNsPlusOne)) OF ForbiddenLAs-Item

ForbiddenLAs-Item ::= SEQUENCE {

pLMN-Identity		PLMNidentity,

forbiddenLACs		ForbiddenLACs,

iE-Extensions		ProtocolExtensionContainer { {ForbiddenLAs-Item-ExtIEs} } OPTIONAL,

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

HandoverRestrictionList ::= SEQUENCE {

servingPLMN					PLMNidentity,

equivalentPLMNs				EPLMNs					OPTIONAL,

forbiddenTAs				ForbiddenTAs			OPTIONAL,

forbiddenLAs				ForbiddenLAs			OPTIONAL,

forbiddenInterRATs			ForbiddenInterRATs		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {HandoverRestrictionList-ExtIEs} }	OPTIONAL,

...

}

HandoverRestrictionList-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NRrestrictioninEPSasSecondaryRAT		CRITICALITY ignore	EXTENSION NRrestrictioninEPSasSecondaryRAT		PRESENCE optional}|

{ ID id-UnlicensedSpectrumRestriction		CRITICALITY ignore	EXTENSION UnlicensedSpectrumRestriction	PRESENCE optional}|

{ ID id-CNTypeRestrictions			CRITICALITY ignore	EXTENSION CNTypeRestrictions	PRESENCE optional}|

{ ID id-NRrestrictionin5GS			CRITICALITY ignore	EXTENSION NRrestrictionin5GS 	PRESENCE optional}|

{ ID id-LastNG-RANPLMNIdentity		CRITICALITY ignore	EXTENSION PLMNidentity			PRESENCE optional}|

{ ID id-RAT-Restrictions			CRITICALITY ignore	EXTENSION RAT-Restrictions		PRESENCE optional}|

{ ID id-ForbiddenTAs5GS				CRITICALITY ignore	EXTENSION ForbiddenTAs5GS		PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.3.6	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-TimeBasedHandoverInformation						ProtocolIE-ID ::= 350

id-Bearers-SubjectToDLDiscarding-Item				ProtocolIE-ID ::= 351

id-Bearers-SubjectToDLDiscardingList				ProtocolIE-ID ::= 352

id-CoarseUELocationRequested						ProtocolIE-ID ::= 353

id-CoarseUELocation									ProtocolIE-ID ::= 354

id-ForbiddenTAs5GS									ProtocolIE-ID ::= aaa

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Change Ends