3GPP TSG-RAN WG3 Meeting #93	R3-161985

Gothenburg, Sweden, 22 - 26 August 2016

Agenda item:	25

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Extending maximum number of cells in eNB – Solution 1

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we provide a text proposal to resolve the following comeback related to R3-161767 [2]:

CB (Nokia) in R3-161985 TP on solution 1 for TR

Solution 1 clarification:

Text for solution clarification of 1767 agreed

+ move the evaluation text on signaling to the “solution description” section with some rewording on signaling gain

Solution 1 evaluation:

Then evaluation of signaling impact -> no impact

excluding the conclusion

# 2	Text Proposal

Beginning of Text Proposal for TR 36.896

### 6.2.1	Solution 1: Integrate multiple logical eNBs into a single physical eNB

This solution is to integrate multiple logical eNBs into a single physical eNB, which is an implementation choice available today. The physical eNB can serve 256 cells per logical eNB that is integrated.

Each logical eNB will follow current standards, and establish a separate S1 interface with the MME and a separate X2 interface with logical eNBs that are not integrated into the physical eNB. There is no need for an X2 interface between logical eNBs that are integrated.

The Path Switch Request procedure is still required for handovers between cells of the physical eNB when the source and target belong to different logical eNBs.

Next Change

## 6.3	Evaluations

### 6.3.x	Evaluation on “Integrate multiple logical eNBs into a single physical eNB”

-	Impact on existing nodes: None.

-	Impact on specification, existing features and procedures: None.

-	Impact on interworking with legacy nodes: None.

-	Impact on signalling: None.

-	Impact on network planning: None.

-	Impact on CAPEX/OPEX: None.

End of Text Proposal for TR 36.896

# 3	Conclusion

It is proposed to agree to the above Text Proposal for TR 36.896.

# References

- 3GPP TR 36.896 V1.0.0, Study on Flexible eNB-ID and Cell-ID in E-UTRAN

- R3-161767, Extending maximum number of cells in eNB – Solution 1, Nokia, Alcatel-Lucent Shanghai Bell