3GPP TSG-RAN WG3 Meeting #93bis	R3-162533

# Sophia Antipolis, France, 10th – 14th Otober 2016

Source: 	Ericsson

Title:	RAN architecture scenarios requiring flexible design

Agenda Item:	10.6.1.1

Document for:	Discussion and Decision

# Introduction

During RAN3-93bis discussions on the text proposal in R3-162421 were carried out.

In this document a revised version of the TP is provided.

# Text Proposal for TR38.801

-----------------------------------------Start of Changes-----------------------------------------

### 6.1.2	Functional split between central and distributed unit

#### 6.1.2.1	General description of split options

In the study item for a new radio access technology, 3GPP is expected to study different functional splits between central and distributed units. E-UTRA protocol stack is taken as a basis for further discussion, with the understanding that the conclusions may need to be revisited, once RAN2 defines the protocol stack for NR. The following functional splits between central and distributed unit are possible, as illustrated in Figure 6.1.2.1-1.

Figure 6.1.2.1-1: Function Split between central and distributed unit

Option 1 (1A-like split)

-	The function split in this option is similar as 1A architecture in DC. RRC is in the central unit. PDCP, RLC, MAC, physical layer and RF are in the distributed unit.

Option 2 (3C-like split)

-	The function split in this option is similar as 3C architecture in DC. RRC, PDCP are in the central unit. RLC, MAC, physical layer and RF are in the distributed unit.

Option 3 (intra RLC split)

-	Low RLC (partial function of RLC), MAC, physical layer and RF are in distributed unit. PDCP and high RLC (the other partial function of RLC) are in the central unit.

Option 4 (RLC-MAC split)

-	MAC, physical layer and RF are in distributed unit. PDCP and RLC are in the central unit.

Option 5 (intra MAC split)

-	RF, physical layer and some part the MAC layer (e.g. HARQ) are in the distributed unit. Upper layer is in the central unit.

Option 6 (MAC-PHY split)

-	Physical layer and RF are in the distributed unit. Upper layers are in the central unit.

Option 7 (intra PHY split)

-	Part of physical layer function and RF are in the distributed unit. Upper layers are in the central unit.

Option 8 (PHY-RF split)

-	RF functionality is in the distributed unit and upper layer are in the central unit.

Editor’s note: The options represented consist of a non-exhaustive list. The work in other working groups on protocols and functions definition shall be monitored and further split options based on such progress shall be added or removed if needed.

Flexible functional split

Some of the benefits of a New RAN architecture with the deployment flexibility to split and move functions between central and distributed units are below:

-	Flexible HW implementations allows scalable cost effective solutions

-	A split architecture (between central and distributed units) allows for coordination for performance features, load management, real-time performance optimization, and enables NFV/SDN

-	Configurable functional splits enables adaptation to various use cases, such as variable latency on transport

The choice of how to split functions in the New RAN architecture depends on some factors related to radio network deployment scenarios, constraints and intended supported services. 
Some examples of such factors are:

- Need to support specific QoS settings per offered services (e.g. low latency, high throughput)

- Need to support specific user density and load demand per given geographical area (which may influence the level of RAN coordination)

- Need to be able to function with transport networks with different performance levels, from Ideal to non-ideal

The NR design should support the flexibility to move RAN functions between the central unit and distributed unit depending on the factors above, and should be studied.

-----------------------------------------End of Changes-----------------------------------------