3GPP TSG-RAN3 Meeting #93bis	R3-162541

# Sophia Antipolis, France, 10 – 14 October 2016

Agenda Item:	10.6.1.1

Source: 		Fujitsu

Title: 	Description of intra-MAC functional split

Document for:		Discussion and decision

# Introduction

During RAN3#93bis the intra-MAC functional split was discussed in [1] and [2]. The following notes were captured during the session:

CB: # 40_Intra-MAC

- TP merger with   R3-162232

- check scheduler aspect more sentence?

Therefore this contribution proposes a Text Proposal introducing a description of option 5, taking into account online discussion.

# References

[1]	R3-162232, RAN functional split: intra MAC split description and benefits, Intel Corporation

[2] R3-162472, Description of intra-MAC functional split, Fujitsu

# Text Proposal for TR 38.801

##### 6.1.2.2.5	Option 5 (intra MAC split)

----------------------------------------------Start of Changes----------------------------------------------

Description:

Option 5 assumes the following distribution:

- RF, physical layer and lower part of the MAC layer (Low-MAC) in the Distributed Unit

- Higher part of the MAC layer (High-MAC), RLC and PDCP in the Central Unit

Therefore by splitting the MAC layer into 2 entities (e.g. High-MAC and Low-MAC), the services and functions provided by the MAC layer will be located in the Central Unit (CU), in the Distributed Unit (DU), or in both. An example of this distribution and its justification is given below.

In High-MAC sublayer:

The centralized scheduling in the High-MAC sublayer will be in charge of the control of multiple Low-MAC sublayers. It takes high-level centralized scheduling decision.

The inter-cell interference coordination in the High-MAC sublayer will be in charge of interference coordination methods such as JP/CS CoMP.

In Low-MAC sublayer:

Time critical functions in the Low-MAC sublayer include the functions with stringent delay requirements (e.g. HARQ) or the functions where performance is proportional to latency (e.g. radio channel and signal measurements from PHY, random access control). It reduces the delay requirements on the fronthaul interface.

Radio specific functions in the Low-MAC sublayer can for perform scheduling-related information processing and reporting. It can also measure/estimate the activities on the configured operations or the served UE’s statistics and report periodically or as requested to the High-MAC sublayer.

Benefits and justification:

Depending on the different implementations of the intra-MAC functional split, the following pros and cons can be defined:

Pros:

•	Reduce the bandwidth needed on fronthaul, dependent on the load of RAN-CN interface;

•	Reducing latency requirement on fronthaul (if HARQ processing and cell-specific MAC functionalities are performed in the DU);

•	Efficient interference management across multiple cells and enhanced scheduling technologies such as CoMP, CA, etc., with multi-cell view;

Cons:

•	Complexity of the interface between CU and DU;

•	Difficulty in defining scheduling operations over CU and DU;

•	Scheduling decision between CU and DU will be subject to fronthaul delays, which can impact performances in case of non-ideal fronthaul and short TTI;

•	Limitations for some CoMP schemes (e.g. UL JR).

----------------------------------------------End of Changes----------------------------------------------

##### 6.1.2.2.6	Option 6 (MAC-PHY split)