3GPP TSG-RAN WG3 #96						             R3-171595

Hangzhou, P. R. China, 15th – 19th May 2017

Source:	CATT

Title:	Stage 3 TP for SCG change procedure

Agenda Item:	10.9.1

Document for:	Discussion and Decision

Introduction

This contribution provides a stage3 TP for SCG change procedure.

TP for stage3 TS36.423

///////////////////////////////////////////////////////Text Proposal/////////////////////////////////////////////////////////////////////////////

- 8	X2AP procedures

- 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

# Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| X2 Setup | X2 SETUP REQUEST | X2 SETUP RESPONSE | X2 SETUP FAILURE |
| eNB Configuration Update | ENB CONFIGURATION UPDATE | ENB CONFIGURATION UPDATE ACKNOWLEDGE | ENB CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| SeNB Addition Preparation | SENB ADDITION REQUEST | SENB ADDITION REQUEST ACKNOWLEDGE | SENB ADDITION REQUEST REJECT |
| MeNB initiated SeNB Modification Preparation | SENB MODIFICATION REQUEST | SENB MODIFICATION REQUEST ACKNOWLEDGE | SENB MODIFICATION REQUEST REJECT |
| SeNB initiated SeNB Modification | SENB MODIFICATION REQUIRED | SENB MODIFICATION CONFIRM | SENB MODIFICATION REFUSE |
| SeNB initiated SeNB Release | SENB RELEASE REQUIRED | SENB RELEASE CONFIRM |  |
| X2 Removal | X2 REMOVAL REQUEST | X2 REMOVAL RESPONSE | X2 REMOVAL FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| SgNB Change | SgNB Change Required | SgNB Change Confirm | SgNB Change Failure |

# Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Load Indication | LOAD INFORMATION |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| UE Context Release | UE CONTEXT RELEASE |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Error Indication | ERROR INDICATION |
| Radio Link Failure Indication | RLF INDICATION |
| Handover Report | HANDOVER REPORT |
| X2 Release | X2 RELEASE |
| X2AP Message Transfer | X2AP MESSAGE TRANSFER |
| SeNB Reconfiguration Completion | SENB RECONFIGURATION COMPLETE |
| MeNB initiated SeNB Release | SENB RELEASE REQUEST |
| SeNB Counter Check | SENB COUNTER CHECK REQUEST |

///////////////////////////////////////////////////////Next of Change /////////////////////////////////////////////////////////////////////////////

- 8.x	Procedures for E-UTRAN-NR Dual Connectivity

- 8.x.y	SgNB initiated SgNB Change

- 8.x.y.1	General

The purpose of the SgNB initiated SgNB Change procedure is to request the MeNB to trigger the change of secondary node for multi-RAT dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

- 8.x.y.2	Successful Operation

# Figure 8.x.y.2-1: SgNB initiated SgNB Change, successful operation

The SgNB initiates the procedure by sending the SGNB CHANGE REQUIRED message to the MeNB. When the SgNB sends the SGNB CHANGE REQUIRED message, it shall start the timer TDCoverall.

The SGNB CHANGE REQUIRED message may contain

-	the SgNB to MeNB Container IE.

If the MeNB is able to perform the change requested by the SgNB, the MeNB shall send the SGNB CHANGE CONFIRM message to the SgNB. For each E-RAB configured with the SCG bearer option or SCG split bearer, the MeNB may include the DL Forwarding GTP Tunnel Endpoint IE and the UL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE to indicate that it requests data forwarding of uplink and downlink packets to be performed for that bearer. For each E-RAB configured with the split bearer option, the MeNB may include the DL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE to indicate that it requests data forwarding of downlink packets to be performed for that bearer.

The SgNB may start data forwarding and stop providing user data to the UE and shall stop the timer TDCoverall upon reception of the SGNB CHANGE CONFIRM message.

# Interaction with the MeNB initiated SgNB Addition Preparation procedure:

As specified in TS 37.740, the MeNB shall send, after having received the SgNB initiated SgNB Change procedure, the SGNB ADDITION REQUEST message including the SgNB to MeNB Container IE generated by the source SgNB towards the target SgNB, upon which the target SgNB shall provide respective information in the SgNB to MeNB Container IE within the SGNB ADDITION REQUEST ACKNOWLEDGMENT message.

- 8.x.y.3	Unsuccessful Operation

# Figure 8.6.1.3-1: SgNB initiated SgNB Change, unsuccessful operation

In case the request change cannot be performed successfully, the MeNB sends the SGNB CHANGE FAILURE message with an appropriate cause value to the SgNB.

- 8.x.y.4	Abnormal Conditions

If the timer TDCoverall expires before the SgNB has received the SGNB CHANGE CONFIRM or the SGNB CHANGE FAILUE message, the SgNB shall regard the requested change as failed and may take further actions like triggering the SgNB initiated SgNB Release procedure to release all SgNB resources allocated for the UE.

# Interactions with the SgNB Reconfiguration Completion and SgNB initiated SgNB Release procedure:

If the timer TDCoverall expires before the SgNB has received the SGNB RECONFIGURATION COMPLETE or the SGNB RELEASE REQUEST message, the SgNB shall regard the requested RRC connection reconfiguration as being not applied by the UE and shall trigger the SgNB initiated SgNB Release procedure.

///////////////////////////////////////////////////////Next of Change /////////////////////////////////////////////////////////////////////////////

- 9	Elements for X2AP Communication

- 9.1.x	Messages for MR-DC Procedures

- 9.1.x.y1	SGNB CHANGE REQUIRED

This message is sent by the SgNB to the MeNB to request the change of secondary node for multi-RAT dual connectivity operation for a specific UE

Direction: SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source SgNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Candidate Target SgNB Cell List |  | 1 .. <maxnoofCandidateCells> |  |  | EACH | ignore |
| >Candidate SgNB Cell ID | M |  | ECGI 9.2.14 |  |  |  |
| SeNB to MeNB Container | M |  | OCTET STRING | Includes the SCG-Config message as defined in TS 38.331 | YES | reject |

- 9.1.x.y2	SGNB CHANGE CONFIRM

This message is sent by the MeNB to inform the SgNB about the successful change.

Direction: MeNB  SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNN | YES | ignore |
| SgNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source SgNB | YES | ignore |
| E-RABs to be Released List |  | 1 |  |  | YES | reject |
| >E-RABs To Be Released Item |  | 1 .. <maxnoof Bearers> |  |  | – | – |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – | – |
| >>>SCG Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – | – |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – | – |
| >>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – | – |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – | – |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

- 9.1.x.y3	SGNB CHANGE FAILURE

This message is sent by the MeNB to inform the SgNB that the SgNB initiated SgNB Change has failed.

Direction: MeNB  SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SgNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source SgNB | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

///////////////////////////////////////////////////////End of Change /////////////////////////////////////////////////////////////////////////////