3GPP TSG-RAN WG3 Meeting # 96	R3-171813

Hangzhou, China, May 15 – 19, 2017

Agenda item:		10.10.1

Source:	Huawei, Hisilicon.

Title:					Discussions on fundamental F1 AP procedures

Document for:		Discussion

1. Introduction

In last RAN3 meeting, it was agreed to capture (with FFS) some management functions for F1AP. This paper propose how to proceed to remove these FFS and complete the messages.

2. Discussion

2.1 	Reset

It seems reasonable that both the CU and DU should be allowed to initiate a reset. One thing that may be worth considering is if we should allow the possibility to release all allocated resources on F1 and radio resources related to indicated UE association. We see no strong reason in not allowing this possibility and therefore suggest to keep this possibility.

We propose to simply remove the FFS in section 8.2.3 except for the already identified FFS regarding node naming and APID definition.

In addition to this we propose to populate the tabular based on S1 with an FFS related to the APID that is used.

2.2 	Error indication

It seems reasonable to also support error indication. We propose to remove the general FFS from 8.2.2 except for the FFS related to APID. We also propose to populate the tabular based on S1AP but with an added FFS for the APID.

2.3 	F1 setup

The current S1 setup is based on the S1AP. We believe that a similar assumption can be made for F1, i.e. that the gNB-DU is configured with the TNL endpoint of the gNB-CU and that the gNB-DU may trigger the setup once operational. It is assumed that the most frequent scenario is adding a gNB-DU to an already operational gNB-CU.

Unlike S1 and X2 setup, there is however very limited need to transfer any additional information (e.g. TA or served cell) since it is assumed that the gNB-CU and gNB-DU must be well configured and dimensioned in order to cooperate. It is not similar to X2 setup, where interfaces are established on the fly based on varying radio conditions. Therefore, we propose that the contents of the F1 setup message can be minimised and that OAM is used to configure both nodes in a consistent way. With this assumption we also suggest that the need for update of this information over F1AP is negligible.

Another aspect to consider is whether there is a need to maintain F1 UE related context in case sending a subsequent F1 setup. Here we believe that this would be beneficial.

We propose to remove the general FFS from 8.2.1 except for the FFS related to APID. We also propose to populate the tabular based on S1AP but with an added FFS for the APID.

3. Conclusion

This text proposal analyse some remaining topics for the captured procedures and propose to remove the FFS (apart from the one related to APID) and to populate the tabular as indicate in the text proposal.

Annex – Text proposal 38.473

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |

## 8.2	Interface Management procedures

### 8.2.1	Reset

#### 8.2.1.1	General

The purpose of the Reset procedure is to initialise or re-initialise the F1AP UE-related contexts, in the event of a failure in the gNB-CU or gNB-DU. This procedure does not affect the application level configuration data exchanged during, e.g., the F1 Setup procedure.

The procedure uses non-UE associated signalling.

#### 8.2.1.2	Successful Operation

##### 8.2.1.2.1	Reset Procedure Initiated from the gNB-CU

Figure 8.2.1.2.1-1: Reset procedure initiated from the gNB-CU. Successful operation.

In the event of a failure at the gNB-CU, which has resulted in the loss of some or all transaction reference information, a RESET message shall be sent to the gNB-DU.

At reception of the RESET message the gNB-DU shall release all allocated resources on F1 and radio resources related to the UE association(s) indicated explicitly or implicitly in the RESET message and remove the indicated UE contexts including F1AP ID.

After the gNB-DU has released all assigned F1 resources and the UE F1AP IDs for all indicated UE associations which can be used for new UE-associated logical F1-connections over the F1 interface, the gNB-DU shall respond with the RESET ACKNOWLEDGE message. The gNB-DU does not need to wait for the release of radio resources to be completed before returning the RESET ACKNOWLEDGE message.

##### 8.2.1.2.2	Reset Procedure Initiated from the gNB-DU

Figure 8.2.1.2.2-1: Reset procedure initiated from the gNB-DU. Successful operation.

In the event of a failure at the gNB-DU, which has resulted in the loss of some or all transaction reference information, a RESET message shall be sent to the gNB-CU.

At reception of the RESET message the gNB-CU shall release all allocated resources on F1 related to the UE association(s) indicated explicitly or implicitly in the RESET message and remove the F1AP ID for the indicated UE associations.

After the gNB-CU has released all assigned F1 resources and the UE F1AP IDs for all indicated UE associations which can be used for new UE-associated logical F1-connections over the F1 interface, the gNB-CU shall respond with the RESET ACKNOWLEDGE message.

#### 8.2.1.3	Abnormal Conditions

### 8.2.2	Error Indication

#### 8.2.2.1	General

The Error Indication procedure is initiated by a node in order to report detected errors in one incoming message, provided they cannot be reported by an appropriate failure message.

If the error situation arises due to reception of a message utilising UE associated signalling, then the Error Indication procedure uses UE associated signalling. Otherwise the procedure uses non-UE associated signalling.

#### 8.2.2.2	Successful Operation

Figure 8.2.2.2-1: Error Indication procedure, gNB-CU originated. Successful operation.

Figure 8.2.2.2-2: Error Indication procedure, gNB-DU originated. Successful operation.

When the conditions defined in clause 10 are fulfilled, the Error Indication procedure is initiated by an ERROR INDICATION message sent from the receiving node.

The ERROR INDICATION message shall contain at least either the Cause IE or the Criticality Diagnostics IE. In case the Error Indication procedure is triggered by utilising UE associated signalling the gNB-CU UE F1AP ID IE shall be included in the ERROR INDICATION message. If the gNB-CU UE F1AP ID IE are not correct, the cause shall be set to appropriate value, e.g., “Unknown or already allocated gNB-CU UE F1AP ID” or “Unknown or inconsistent pair of UE F1AP ID”.

#### 8.2.2.3	Abnormal Conditions

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received and clears gNB-CU overload state information at the gNB-DU. If the gNB-DU and gNB-CU do not agree on retaining the UE Contexts this procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation.

The gNB-DU initiates the procedure by sending a F1 SETUP REQUEST message including the appropriate data to the gNB-CU. The gNB-CU responds with a F1 SETUP RESPONSE message including the appropriate data.

The exchanged data shall be stored in respective node and used for the duration of the TNL association. When this procedure is finished, the F1 interface is operational and other F1 messages can be exchanged.

#### 8.2.3.3	Unsuccessful Operation

Figure 8.2.3.3-1: F1 Setup procedure: Unsuccessful Operation.

If the gNB-CU cannot accept the setup, it should respond with a F1 SETUP FAILURE and appropriate cause value.

#### 8.2.3.4	Abnormal Conditions

#### <<Unchanged text omitted>>

### 9.x1.y	Interface Management messages

#### 9.x1.y.1	RESET

This message is sent by both the gNB-CU and the gNB-DU and is used to request that the F1 interface, or parts of the F1 interface, to be reset.

Direction: gNB-CU  gNB-DU and gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y1.v1 |  | YES | reject |
| Cause | M |  | 9.x2.y1.v2 |  | YES | ignore |
| CHOICE Reset Type | M |  |  |  | YES | reject |
| >F1 interface |  |  |  |  |  |  |
| >>Reset All | M |  | ENUMERATED (Reset all,…) |  | - |  |
| >Part of F1 interface |  |  |  |  |  |  |
| >>UE-associated logical F1-connection list |  | 1 |  |  | - |  |
| >>>UE-associated logical F1-connection Item |  | 1 .. <maxnoofIndividualF1ConnectionsToReset> |  |  | EACH | reject |
| >>>> gNB-CU UE F1AP ID | O |  | 9.x2.y1.v4 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofIndividualF1ConnectionsToReset | Maximum no. of UE-associated logical F1-connections allowed to reset in one message. Value is 256. |

#### 9.x1.y.2	RESET ACKNOWLEDGE

This message is sent by both the gNB-CU and the gNB-DU as a response to a RESET message.

Direction: gNB-DU  gNB-CU and gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y1.v1 |  | YES | reject |
| UE-associated logical F1-connection list |  | 0..1 |  |  | YES | ignore |
| >UE-associated logical F1-connection Item |  | 1 .. <maxnoofIndividualF1ConnectionsToReset> |  |  | EACH | ignore |
| >>gNB-CU UE F1AP ID | O |  | 9.x2.y1.v4 |  | - |  |
| Criticality Diagnostics | O |  | 9.x2.y1.v3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofIndividualF1ConnectionsToReset | Maximum no. of UE-associated logical F1-connections allowed to reset in one message. Value is 256. |

#### 9.x1.y.3	ERROR INDICATION

This message is sent by both the gNB-CU and the gNB-DU and is used to indicate that some error has been detected in the node.

Direction: gNB-CU  gNB-DU and gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y1.v1 |  | YES | ignore |
| gNB-CU UE F1AP ID | O |  | 9.x2.y1.v4 |  | YES | ignore |
| Cause | O |  | 9.x2.y1.v2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.x2.y1.v3 |  | YES | ignore |

#### 9.x1.y.4	F1 SETUP REQUEST

This message is sent by the gNB-DU to transfer information for a TNL association.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y1.v1 |  | YES | reject |
| gNB-DU ID | M |  | 9.x2.y1.v5 |  | YES | reject |

#### 9.x1.y.5	F1 SETUP RESPONSE

This message is sent by the gNB-CU to transfer information for a TNL association.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y1.v1 |  | YES | reject |

9.x1.y.6		F1 SETUP FAILURE

This message is sent by the gNB-CU to indicate F1 Setup failure.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y1.v1 |  | YES | reject |
| Cause | M |  | 9.x2.y1.v2 |  | YES | ignore |
| Time to wait | O |  | FFS |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.x2.y1.v3 |  | YES | ignore |

### <<Unchanged text omitted>>

### 9.x2.y1	Radio Network Layer Related IEs

#### 9.x2.y1.v1	Message Type

The Message Type IE uniquely identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type |  |  |  |  |
| >Procedure Code | M |  | INTEGER (0..255) |  |
| >Type of Message | M |  | CHOICE (Initiating Message, Successful Outcome, Unsuccessful Outcome, …) |  |

9.x2.y1.v2		Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the F1AP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, …) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Unspecified, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Not enough User Plane Processing Resources, Hardware Failure, O&M Intervention, Unspecified, …) |  |

The meaning of the different cause values is described in the following table. In general, “not supported” cause values indicate that the related capability is missing. On the other hand, “not available” cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |

| Transport Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Transport Network Layer related. |

| Protocol cause | Meaning |
| --- | --- |
| Transfer Syntax Error | The received message included a transfer syntax error. |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerning criticality indicated “reject”. |
| Abstract Syntax Error (Ignore And Notify) | The received message included an abstract syntax error and the concerning criticality indicated “ignore and notify”. |
| Message Not Compatible With Receiver State | The received message was not compatible with the receiver state. |
| Semantic Error | The received message included a semantic error. |
| Abstract Syntax Error (Falsely Constructed Message) | The received message contained IEs or IE groups in wrong order or with too many occurrences. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related. |

| Miscellaneous cause | Meaning |
| --- | --- |
| Control Processing Overload | Control processing overload. |
| Not Enough User Plane Processing Resources Available | No enough resources are available related to user plane processing. |
| Hardware Failure | Action related to hardware failure. |
| O&M Intervention | The action is due to O&M intervention. |
| Unspecified Failure | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer, NAS or Protocol. |

9.x2.y1.v3		Criticality Diagnostics

The Criticality Diagnostics IE is sent by the gNB-DU or the gNB-CU when parts of a received message have not been comprehended or were missing, or if the message contained logical errors. When applicable, it contains information about which IEs were not comprehended or were missing.

For further details on how to use the Criticality Diagnostics IE, (see clause 10).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Procedure Code | O |  | INTEGER (0..255) | Procedure Code is to be used if Criticality Diagnostics is part of Error Indication procedure, and not within the response message of the same procedure that caused the error. |
| Triggering Message | O |  | ENUMERATED(initiating message, successful outcome, unsuccessful outcome) | The Triggering Message is used only if the Criticality Diagnostics is part of Error Indication procedure. |
| Procedure Criticality | O |  | ENUMERATED(reject, ignore, notify) | This Procedure Criticality is used for reporting the Criticality of the Triggering message (Procedure). |
| Information Element Criticality Diagnostics |  | 0 .. <maxnoof Errors> |  |  |
| >IE Criticality | M |  | ENUMERATED(reject, ignore, notify) | The IE Criticality is used for reporting the criticality of the triggering IE. The value ‘ignore’ shall not be used. |
| >IE ID | M |  | INTEGER (0..65535) | The IE ID of the not understood or missing IE. |
| >Type of Error | M |  | ENUMERATED(not understood, missing, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofErrors | Maximum no. of IE errors allowed to be reported with a single message. The value for maxnoofErrors is 256. |

#### 9.x2.y1.v4 gNB-CU UE F1AP ID [FFS]

The gNB-CU UE F1AP ID uniquely identifies the UE association over the F1 interface within the gNB-CU and the associated gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-CU UE F1AP ID | M |  | INTEGER (0 .. 232 -1) |  |

#### 9.x2.y1.v5 gNB-DU ID [FFS]

The gNB-DU ID identifies the gNB-DU uniquely among gNB-DU connected to the same gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-DU ID | M |  | INTEGER (0 .. 232 -1) |  |