3GPP TSG-RAN3 Meeting #96	R3-171842

Hangzhou, China, 15 – 19 May, 2017

Title: 	TP on Paging message delivery over F1 to 38.473

Source: 	Huawei

Agenda item:	10.10.1

Document for:	Discussion

Introduction

This paper provides text proposal of paging message delivery over F1 to TS 38.473.

Text proposal

----------------------------------------------------1st Change ---------------------------------------------------------------

7	Functions of F1AP

Editor’s note: The text highlighted below is FFS.

The F1AP protocol has the following functions:

-	F1 interface management functions comprise the:

-	Reset function to initialize the peer entity after node setup and after a failure event occurred.

-	Error indication function to indicate to the gNB-CU that a logical error has occurred.

-	F1 setup function to exchange (respectively update) application level data needed for the gNB-DU and gNB-CU.

-	Paging delivery function enables that gNB-CU deliver paging information to gNB-DU.

----------------------------------------------------Next Change ---------------------------------------------------------------

8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

Editor’s note: The naming of the procedures and messages below is FFS.

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| Paging Delivery | PAGING DELIVERY |

----------------------------------------------------Next Change ---------------------------------------------------------------

### 8.2.x	Paging Delivery

#### 8.2.x.1	General

The Paging Delivery procedure is initiated by gNB-CU for delivering paging information to gNB-DU in order that gNB-DU sending paging message to UE.

#### 8.2.x.2	Successful Operation

Figure 8.2.x.2-1: Paging Delivery. Successful operation.

Upon receiving PAGING DELIVERY message, the gNB-DU shall send paging message to the UE based on the information provided in Cell ID IE and Paging Occasion IE as specified in TS 38.300 [x].

----------------------------------------------------Next Change ---------------------------------------------------------------

- x.x.x.1	PAGING DELIVERY

This message is sent by the CU and is used to request the DU to send paging message to UE.

Direction: CU  DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <reference> |  | YES | reject |
| Paging to Be Delivered List |  | 1 |  |  | YES | reject |
| >Paging to Be Delivered Item IEs |  | 1 .. <maxnoof PDUSessions> |  |  | EACH | reject |
| >>Paging message | M |  | OCTET STRING | <paging message as defined in TS 38.331> | - |  |
| >> Cells to Be Delivered List | M |  | <reference> |  |  |  |
| >>>Cells to Be Delivered Item IEs |  | 1 .. <maxnoof Cells> |  |  |  |  |
| >>>>Cell ID | M |  | <reference> |  | - |  |
| >>>>Paging Occasion | M |  | <reference> | <paging message as defined in TS 38.331> | - |  |