3GPP TSG-RAN WG3 #96	R3-171962

Hangzhou, China, 15th – 19th May 2017

Agenda Item:	10.11.3

Source:	Ericsson, Vodafone, SK Telecom, AT&T

Title:	TP for CP and UP split

Document for:	pCR

# TP for 38.401

Start of Text Proposal for TS 38.401

#### 10.2.1.2 F1 interface general principles

The general principles for the specification of the F1 interface are as follows:

-	the F1 interface shall be open;

-	the F1 interface shall support the exchange of signalling information between the endpoints, in addition the interface shall support data transmission to the respective endpoints;

-	from a logical standpoint, the F1 is a point-to-point interface between the endpoints.

NOTE:	A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the endpoints.

-	the F1 interface shall support control plane and user plane separation;

-	the F1 interface shall separate Radio Network Layer and Transport Network Layer;

-	the F1 interface shall enable exchange of UE associated information and non-UE associated information;

-	the F1 interface shall be future proof to fulfil different new requirements, support new services and new functions;

-	One gNB-CU and set of gNB-DUs are visible to other logical nodes as a gNB. The gNB terminates X2, Xn, NG and S1-U interfaces;

-	The CU may be separated in control plane (CP) and user plane (UP).

Editor’s Note: how to realize the CP and UP separation (e.g., by implementation) is FFS.

End of Text Proposal for TS 38.401