3GPP TSG-RAN WG3 #96	Tdoc R3-171997

Hangzhou, China, 15th – 19th May 2017

Agenda Item:	10.11.3

Source:	Ericsson

Title:	On the functional distribution over the F1 interface

Document for:	Approval

# Text Proposal

*************** START OF MODIFIED SECTION***********

## 10.2	 F1 interface

Editor Note: This subclause shows overall Fs interface e.g. function description

### 10.2.1	General aspects

This clause captures the architecture for gNB-CU and gNB-DU, F1 interface principles.

#### 10.2.1.1	Architecture for gNB-CU and gNB-DU

Editor’s note: Whether one gNB-DU can be connected with multiple gNB-CUs is FFS (e.g. by pooling concept), thus architecture for gNB-CU and gNB-DU is also FFS.

#### 10.2.1.2	F1 interface general principles

The general principles for the specification of the F1 interface are as follows:

-	the F1 interface shall be open;

-	the F1 interface shall support the exchange of signalling information between the endpoints, in addition the interface shall support data transmission to the respective endpoints;

-	from a logical standpoint, the F1 is a point-to-point interface between the endpoints.

NOTE:	A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the endpoints.

-	the F1 interface shall support control plane and user plane separation;

-	the F1 interface shall separate Radio Network Layer and Transport Network Layer;

-	the F1 interface shall enable exchange of UE associated information and non-UE associated information;

-	the F1 interface shall be future proof to fulfil different new requirements, support new services and new functions;

-	One gNB-CU and set of gNB-DUs are visible to other logical nodes as a gNB. The gNB terminates X2, Xn, NG and S1-U interfaces;

gNB-CU is mainly responsible for UE handling, while gNB-DU is mainly responsible for cell handling. Each logical node owns its resources.

Editor’s note: How to split the functionality between gNB-CU and gNB-DU is FFS .

### 10.2.2	Functions of the F1 interface protocols and functional split

Editor’s note: Naming of each function is FFS.

#### 10.2.2.1	General

The following clauses describe the functions supported over F1-C and F1-U.

#### 10.2.2.2	F1-C functions

#### 10.2.2.2.1	GTP-U tunnel management function (FFS)

This function is used to establish and release GTP-U (3GPP TS 29.281 [x3]) tunnels between the gNB-CU and the gNB-DU upon a bearer service request. This involves assigning a tunnel identifier for each direction.

Editor’s note: Whether this section is kept or moved to bearer management function or removed is FFS.

#### 10.2.2.2.2	F1 interface management function

The error indication function is used by the gNB-DU or gNB-CU to indicate to the gNB-CU or gNB-DU that an error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the gNB-DU and gNB-CU.

The F1 setup (respectively the gNB-DU and gNB-CU configuration update) function allows to exchange (respectively update) application level data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. The F1 setup is initiated by the gNB-DU.

Editor’s note: The space of NR Cell ID, and how/whether to map them in the gNB-CU are FFS.

#### 10.2.2.2.3	gNB-DU management function (FFS)

Editor’s note: Whether gNB-DU is managed by O&M or by gNB-CU is FFS. The impact on F1 interface is FFS.

### 10.2.2.2.4	System Information management function

Scheduling of system broadcast information is carried out in the gNB-DU. The gNB-DU is responsible for transmitting the system information according to the scheduling parameters available.

Editor’s note: How to support this function is FFS.

### 10.2.2.2.5	gNB-DU and gNB-CU measurement reporting function (FFS)

The gNB-DU and gNB-CU measurement reporting functions are used to report the measurements of gNB-DU and gNB-CU.

### 10.2.2.2.6	Load management function (FFS)

The load management function allows the gNB-CU and gNB-DU to request the reporting of load information to the gNB-DU.

### 10.2.2.2.7	Paging function (FFS)

The gNB-DU is responsible for transmitting the paging information according to the scheduling parameters available.

### 10.2.2.2.8	F1 UE context management function

The F1 UE context management function supports the establishment of the necessary overall initial UE context.

The establishment of the F1 UE context is initiated by the gNB-CU and accepted or rejected by the gNB-DU based on admission control criteria.

The F1 UE context management function also supports the release of the context previously established in the gNB-DU. The release of the context is triggered by the gNB-CU either directly or following a request received from the gNB-DU.

Editor’s note: What is UE context needs to be clarified.

### 10.2.2.2.9	Bearer management function

The bearer management function is responsible for triggering establishing, modifying, and releasing radio bearer resources for user data transport once an F1UE context is available in the gNB-DU. The establishment and modification of radio bearer resources are triggered by the gNB-CU and accepted/rejected by the gNB-DU based on resource reservation information and QoS information to be provided to the gNB-DU.

Editor’s note: Whether mobility functions such as intra-CU/inter-DU handover can be supported by bearer management function is FFS.

Editor’s note: The relationship of bearer management and simultaneous transmission from two DUs is FFS.

Editor’s note: How to split and configure RRM functions between gNB-CU and gNB-DU is FFS.

### 10.2.2.2.10	Transfer of RRC message

This function allows to transfer RRC messages between gNB-CU and gNB-DU.

Editor’s note: Whether RRC message is transferred over F1-C or F1-U or both is FFS.

#### 10.2.2.3	F1-U functions

### 10.2.2.3.1	Transfer of user data

This function allows to transfer of user data between gNB-CU and gNB-DU.

### 10.2.2.3.2	Flow control function

This function allows to control the downlink user data flow to the gNB-DU. The detailed protocol is specified in TS 38.475 [4x].

Editor’s note: Whether to support uplink flow control is FFS.

### 10.2.3	F1 interface protocols and protocol structure

## 10.2.3.1	F1 Control Plane Protocol (F1-C)

Figure 10.2.3.1-1 shows the protocol structure for F1-C. The TNL is based on IP transport, comprising the SCTP on top of IP. The application layer signalling protocol is referred to as F1AP (F1 Application Protocol).

Figure 10.2.3.1-1: Interface protocol structure for F1-C

Editor’s note: Whether other protocol stacks are standardized for F1-C is FFS.

## 10.2.3.2	F1 User Plane Protocol (F1-U)

Figure 10.2.3.2-1 shows the protocol structure for F1-U. The TNL is based on IP transport, comprising the UDP and GTP-U on top of IP.

Figure 10.2.3.2-1: Interface protocol structure for F1-U

Editor’s note: Whether other protocol stacks are standardized for F1-U is FFS.

***************END OF MODIFIED SECTION***************

# References

TS 38.401 ‘NG RAN; Architecture Description’, May 2017.

R3-171727, ‘On the functional distribution over the F1 interface’, Ericsson, May 2017.