3GPP TSG RAN WG3 Meeting #97	R3-173292

Berlin, Germany, August 21 – 25, 2017

Agenda item:		10.8.3

Source:	Altiostar Networks, Intel Corporation

Title:	TP to TS 38.475 for Flow Control

Document for:		Decision

# 1	Introduction

In this paper, we provide Text Proposal to TS38.475 for F1 User Plane enhancements for flow control [1]. Discussion on flow control principles and comparison of different flow control approaches are available in our paper [2,3].

# Text Proposal to TS 38.475

Beginning of Text Proposal to TS 38.475

# 5		F1 user plane protocol

## 5.1	General

The F1 UP protocol layer is using services of the transport network layer in order to allow flow control of user data packets transferred over the F1 interface.

## 5.2	F1 user plane protocol layer services

The following functions are provided by the F1 UP protocol:

-	Provision of F1 UP specific sequence number information for user data transferred from the gNB-CU to the gNB-DU for a specific data radio bearer;

-	Information of successful in sequence delivery of PDCP PDUs to the UE from gNB-DU for user data associated with a specific data radio bearer;

-	Information of PDCP PDUs that were not delivered to the UE;

-	Information of the currently desired buffer size at the gNB-DU for transmitting to the UE user data associated with a specific data radio bearer;

-	Information of the currently minimum desired buffer size at the gNB-DU for transmitting to the UE user data associated with all data radio bearers.

-	Information of the average buffer size at the gNB-DU for transmitting to the UE user data associated with a specific data radio bearer,

-	Information of the buffer drain rate at the gNB-DU for transmitting to the UE user data associated with a specific data radio bearer.

Editor’s note: whether more information is needed is FFS.

## 5.3	Services expected from the F1 Transport Network Layer

The F1 user plane protocol layer expects the following services from the Transport Network Layer:

-	Transfer of user data.

## 5.4	Elementary procedures

### 5.4.1	Transfer of Downlink User Data

#### 5.4.1.1	Successful operation

The purpose of the Transfer of Downlink User Data procedure is to provide F1-U specific sequence number information at the transfer of user data carrying a DL PDCP PDU from the gNB-CU to the gNB-DU via the F1-U interface.

An F1 user plane instance making use of the Transfer of Downlink User Data procedure is associated to a single data radio bearer only. The Transfer of Downlink User Data procedure is invoked whenever user data for that particular data radio bearer needs to be sent across the F1-U interface.

The gNB-CU shall assign consecutive F1-U sequence numbers to each transferred F1-U packet.

The gNB-DU shall detect whether an F1-U packet was lost and memorise the respective sequence number after it has declared the respective F1-U packet as being "lost".

The gNB-DU shall transfer the remaining PDCP PDUs towards the UE and memorise the highest PDCP PDU sequence number of the PDCP PDU that was successfully delivered in sequence towards the UE.

NOTE:	The Transfer of Downlink User Data procedure and the associated feedback of lost F1-U packets assist the gNB-CU in avoiding PDCP HFN de-synchronisation. If an NG-RAN deployment decides to not use the Transfer of Downlink User Data procedure, PDCP HFN synchronization should be ensured by other means.

Figure 5.4.1.1-1: Successful Transfer of Downlink User Data

#### 5.4.1.2	Unsuccessful operation

### 5.4.2	Downlink Data Delivery Status

#### 5.4.2.1	Successful operation

The purpose of the Downlink Data Delivery Status procedure is to provide feedback from the gNB-DU to the gNB-CU to allow the gNB-CU to control the downlink user data flow via the gNB-DU for the respective data radio bearer. The gNB-DU may also transfer uplink user data for the concerned data radio bearer to the gNB-CU together with a DL DATA DELIVERY STATUS frame within the same GTP-U PDU.

When the gNB-DU decides to trigger the Feedback for Downlink Data Delivery procedure it shall report:

a)	the highest PDCP PDU sequence number successfully delivered in sequence to the UE among those PDCP PDUs received from the gNB-CU;

d)	the desired buffer size in bytes for the concerned data radio bearer;

c)	the minimum desired buffer size in bytes for the UE;

d)	the F1-U packets that were declared as being "lost" by the gNB-DU and have not yet been reported to the gNB-CU within the DL DATA DELIVERY STATUS frame.

e)	the average buffer size in bytes at gNB-DU for the concerned bearer; average over the time duration between consecutive reports is baseline.

f) 	the buffer drain rate in the number of RLC SDUs was taken by MAC for the concerned bearer between consecutive reports.

NOTE:	If an NG-RAN deployment has decided not to use the Transfer of Downlink User Data procedure, d) above is not applicable.

Editor’s note: whether enhancement is needed is FFS.

The DL DATA DELIVERY STATUS frame shall also include an indication whether the frame is the last DL status report received in the course of releasing a data radio bearer from the gNB-DU. When receiving such indication, if applicable, the gNB-CU considers that no more UL data is to be expected from the gNB-DU.

The gNB-CU, when receiving the DL DATA DELIVERY STATUS frame:

-	regards the desired buffer size under b) and c) above as the amount of data desired from the gNB-DU being declared

-	from the PDCP sequence number reported under a) above within the same frame, as well as from the most recently reported PDCP sequence number(s) of all data radio bearers established for the UE;

-	as the momentary desired buffer sizes, independent of buffer sizes indicated in the past.

-	is allowed to remove the buffered PDCP PDUs according to the feedback of successfully delivered PDCP PDUs;

-	decides upon the actions necessary to take for PDCP PDUs reported other than successfully delivered.

After being reported to the gNB-CU, the gNB-DU removes the respective PDCP sequence numbers.

Figure 5.4.2.1-1: Successful Downlink Data Delivery Status

#### 5.4.2.2	Unsuccessful operation

## 5.5	Elements for the F1 user plane protocol

### 5.5.1	General

In the present document the structure of frames are specified by using figures similar to figure 5.5.1-1.

| Bits |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 |  | 1 | 0 |  |
| Field 1 |  |  |  | Field 2 |  |  |  |  | 1 | Octet 1 |
| Field 3 |  |  |  |  |  | Field 4 |  |  | 2 | Octet 2 |
| Field 4 continue |  |  |  | Spare |  |  |  |  |  | Octet 3 |
| Field 6 |  |  |  |  |  |  |  |  | 2 | Octet 4 Octet 5 |
| Field 6 continue |  |  |  | Padding |  |  |  |  |  |  |
| Spare extension |  |  |  |  |  |  |  |  | 0-m |  |

Figure 5.5.1-1: Example frame format

Unless otherwise indicated, fields which consist of multiple bits within an octet have the most significant bit located at the higher bit position (indicated above frame in figure 5.5.1-1). In addition, if a field spans several octets, most significant bits are located in lower numbered octets (right of frame in figure 5.5.1-1).

On the F1 interface, the frame is transmitted starting from the lowest numbered octet. Within each octet, the bits are sent according to decreasing bit position (bit position 7 first).

Spare bits should be set to “0” by the sender and should not be checked by the receiver.

The header part of the frame is always an integer number of octets. The payload part is octet aligned (by adding ‘Padding’ when needed).

The receiver should be able to remove an additional spare extension field that may be present at the end of a frame. 
See description of Spare extension field.

### 5.5.2	Frame format for the F1 user plane protocol

#### 5.5.2.1	DL USER DATA (PDU Type 0)

This frame format is defined to allow the gNB-DU to detect lost F1-U packets and is associated with the transfer of a Downlink PDCP PDU over the F1-U interface when the length of the PDCP SN is 18 bits (The length of PDCP SN is pending to RAN2).

The following shows the respective DL USER DATA frame.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  | spare |  |  |  | 1 |
| F1-U Sequence Number |  |  |  |  |  |  |  | 3 (FFS) |
| Spare extension |  |  |  |  |  |  |  | 0-4 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

#### 5.5.2.2	DL DATA DELIVERY STATUS (PDU Type 1)

This frame format is defined to transfer feedback to allow the receiving gNB-CU to control the downlink user data flow via the gNB-DU when the length of the PDCP SN is 18 bits(The length of PDCP SN is pending to RAN2).

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  | Spare |  | Final Frame Ind. | Lost Packet Report | 1 |
| Highest successfully delivered PDCP Sequence Number |  |  |  |  |  |  |  | 3(FFS) |
| Desired buffer size for the data radio bearer |  |  |  |  |  |  |  | 4 |
| Minimum desired buffer size for the UE |  |  |  |  |  |  |  | 4 |
| Average buffer size for the data radio bearer |  |  |  |  |  |  |  | 4 |
| Buffer drain rate for the data radio bearer |  |  |  |  |  |  |  | 4 |
| Number of lost F1-U Sequence Number ranges reported |  |  |  |  |  |  |  | 1 |
| Start of lost F1-U Sequence Number range |  |  |  |  |  |  |  | 4* (Number of reported lost F1-u SN ranges) (FFS) |
| End of lost F1-U Sequence Number range |  |  |  |  |  |  |  |  |
| Spare extension |  |  |  |  |  |  |  | 0-4 |

Figure 5.5.2.2-1: DL DATA DELIVERY STATUS (PDU Type 1) Format

Editor’s note: whether enhancement is needed is FFS.

### 5.5.3	Coding of information elements in frames

#### 5.5.3.1	PDU Type

Description: The PDU Type indicates the structure of the F1 UP frame. The field takes the value of the PDU Type it identifies; i.e. "0" for PDU Type 0. The PDU type is in bit 4 to bit 7 in the first octet of the frame.

Value range: {0=DL USER DATA, 1=DL DATA DELIVERY STATUS, 2-15=reserved for future PDU type extensions}

Field length: 4 bits

#### 5.5.3.2	Spare

Description: The spare field is set to "0" by the sender and should not be interpreted by the receiver. This field is reserved for later versions.

Value range: (0–2n-1).

Field Length: n bits.

#### 5.5.3.3	F1-U Sequence Number

Description: This parameter indicates the F1-U sequence number as assigned by the gNB-CU when the length of PDCP SN is 18 bits (FFS).

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.4	Lost Packet Report

Description: This parameter indicates the presence of a list of lost F1-U packets in the respective F1 UP frame.

Value range: {0=Lost Frame List not present, 1=Lost Frame List present}.

Field length: 1 bit.

#### 5.5.3.5	Final Frame Indication

Description: This parameter indicates whether the frame is the last DL status report as described in clause 5.4.2.1.

Value range: {0=Frame is not final, 1= Frame is final}.

Field length: 1 bit.

#### 5.5.3.6	Highest successfully delivered PDCP Sequence Number

Description: This parameter indicates feedback about the in-sequence delivery status of PDCP PDUs at the gNB-DU towards the UE when the length of the PDCP SN is 18 bits (FFS).

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.7	Desired buffer size for the data radio bearer

Description: This parameter indicates the desired buffer size for the concerned data radio bearers as specified in clause 5.4.2.1.

Value range: {0..232-1}.

Field length: 4 octets.

#### 5.5.3.8	Minimum desired buffer size for the UE

Description: This parameter indicates the minimum desired buffer size for all data radio bearers established for the UE as specified in clause 5.4.2.1.

Value range: {0..232-1}.

Field length: 4 octets.

#### 5.5.3.9	Average buffer size for the data radio bearer

Description: This parameter indicates the average buffer size for the concerned data radio bearer as specified in clause 5.4.2.1.

Value range: {0..232-1}.

Field length: 4 octets.

#### 5.5.3.10	Buffer drain rate for the data radio bearer

Description: This parameter indicates the number of RLC SDUs taken by MAC for the concerned data radio bearer specified in clause 5.4.2.1.

Value range: {0..232-1}.

Field length: 4 octets.

#### 5.5.3.11	Number of lost F1-U Sequence Number ranges reported

Description: This parameter indicates the number of F1-U Sequence Number ranges reported to be lost.

Value range: {1..256}.

Field length: 1 octet.

#### 5.5.3.12	Start of lost F1-U Sequence Number range

Description: This parameter indicates the start of an F1-U sequence number range when the length of the PDCP SN is 18 bits (FFS).

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.13	End of lost F1-U Sequence Number range

Description: This parameter indicates the end of an F1-U sequence number range when the length of the PDCP SN is 18 bits (FFS).

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.14	Spare extension

Description: The spare extension field shall not be sent. The receiver should be capable of receiving a spare extension. The spare extension should not be interpreted by the receiver, since in later versions of the present document additional new fields might be added in place of the spare extension. The spare extension can be an integer number of octets carrying new fields or additional information; the maximum length of the spare extension field (m) depends on the PDU type.

Value range: 0–2m*8-1.

Field Length: 0–m octets. For the PDU Types defined in the present document m=4.

### 5.5.4	Timers

## 5.6	Handling of unknown, unforeseen and erroneous protocol data

End of Text Proposal to TS 38.475

# References

- R3-172606, TS 38.475 v0.1.1, F1 interface user plane protocol, Samsung

- R3-172546, “F1 User Plane Enhancement for Flow Control,” Altiostar, Vodafone, Orange, SKT, Intel

- R3-173291, “F1 User Plane Enhancement for Flow Control,” Altiostar, Intel Corporation