3GPP TSG-RAN WG3 Meeting #99	R3-181365

Athens, Greece, Feb. 26th – Mar. 2nd, 2018

Agenda item:		10.8.2.5

Source:	LG Electronics Inc.

Title:	Issue on bearer type selection for NGEN-DC DRB options

Document for:		Approval

1. Introduction

In the past meetings, we have achieved some high level agreements on bearer and flow handling for NGEN-DC DRB options. But still there are many open issues left. This paper is to investigate more on the bearer type selection. The corresponding proposals are also provided.

2. Discussion

In last RAN3 Ad-hoc meeting, we have achieved a high level agreement on bearer type selection for MR-DC, which is given as follows:

## 8.2 Bearer type selection … In MR-DC with 5GC, the following principles apply: - The MN decides per PDU session the location of the SDAP entity, i.e. whether it shall be hosted by the MN or the SN or by both. - If the MN decides to host an SDAP entity it may decide some of the related QoS flows to be realized as MCG bearer, some as SCG bearer, and others to be realized as split bearer. - If the MN decides that an SDAP entity shall be hosted in the SN, some of the related QoS flows may be realized as SCG bearer, some as MCG bearer, while others may be realized as split bearer. The SN may remove or add SCG resources for the respective QoS flows, as long as the QoS for the respective QoS flow is guaranteed.

The agreement above means that SN may add SCG resources for some QoS flows based on its load or other UE’s situation served by it. For example, SN is using DRB 4 to serve flow 1 and flow 2 for a UE. But with the time going, SN decides to add a DRB 5 for flow 2. Flow 1 is still served by DRB 4.

Observation 1): SN may add a new DRB for the flows served by it.

In the example above, in cast that DRB 4 and 5 are both SCG Split bearer, the SN needs to trigger the SN Modification procedure to tell MN on the new DRB 5. Currently, the SN triggered SN Modification Required procedure has only “ PDU sessions To Be Released List” IE. Therefore, the “PDU sessions To Be Modified List” IE is needed.

Observation 2): In case that SN adds SCG resources for the QoS flows, e.g., mapping the existing flows to a new DRB, SN should trigger the SN Modification Required procedure with “PDU sessions To Be Modified List” IE.

Proposal 1): To add “PDU sessions To Be Modified List” IE into the SN triggered SN Modification Required message and also the corresponding change in response message.

3. Conclusion

In this contribution, the issues on bearer type decision and change were investigated. The following proposals are suggested to RAN3:

Proposal 1): To add “PDU sessions To Be Modified List” IE into the SN triggered SN Modification Required message and also the corresponding change in response message.

Proposal 2): To adopt the Stage 3 change for TS 38.423.

4. References

- RP-170855, “NR Radio Access Technology WI”, NTT DOCOMO

- TS 37.340 v.1.2.0

- R3-180575, “Further Discussion and Stage2 pCR for SN Initiated SN Modification Procedure” ZTE

Text proposal for TS 38.423

************************* First change, the unchange text is skipped********************************

8.3.4	S-NG-RAN node initiated S-NG-RAN node Modification

Editor’s Note: All the text below is FFS.

8.3.4.1	General

This procedure is used by the S-NG-RAN node to modify the UE context in the S-NG-RAN node.

The procedure uses UE-associated signalling.

8.3.4.2	Successful Operation

Figure 8.3.4.2-1: S-NG-RAN node initiated S-NG-RAN node Modification, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUIRED message to the M-NG-RAN node.

When the S-NG-RAN node sends the S-NODE MODIFICATION REQUIRED message, it shall start the timer TDCoverall.

The S-NODE MODIFICATION REQUIRED message may contain

-	the S-NG-RAN node to M-NG-RAN node Container IE.

-	PDU sessions to be modified within the PDU sessions To Be Modified Item IE;

-	PDU sessions to be released within the PDU sessions To Be Released Item IE;

-	the SCG Change Indication IE.

If the M-NG-RAN node receives a S-NODE MODIFICATION REQUIRED message containing the SCG Change Indication IE, the M-NG-RAN node shall act as specified in TS 37.340 [8].

If the M-NG-RAN node is able to perform the modifications requested by the S-NG-RAN node, the M-NG-RAN node shall send the S-NODE MODIFICATION CONFIRM message to the S-NG-RAN node. The S-NODE MODIFICATION CONFIRM message may contain the M-NG-RAN node to S-NG-RAN node Container IE.

Upon reception of the S-NODE MODIFICATION CONFIRM message the S-NG-RAN node shall stop the timer TDCoverall.

8.3.4.3	Unsuccessful Operation

Figure 8.3.4.3-1: S-NG-RAN node initiated S-NG-RAN node Modification, unsuccessful operation.

In case the request modification cannot be performed successfully the M-NG-RAN node shall respond with the S-NODE MODIFICATION REFUSE message to the S-NG-RAN node with an appropriate cause value in the Cause IE.

The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE.

8.3.4.4	Abnormal Conditions

Void.

************************* Second change, the unchange text is skipped********************************

9.1.2.8	S-NODE MODIFICATION REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to request the modification of S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

Editor’s Note: The tabular below is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <reference> |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | <reference> |  | YES | ignore |
| SCG Change Indication | O |  | <reference> |  | YES | ignore |
| PDU sessions To Be Modified List |  | 0..1 |  |  | YES | ignore |
| > PDU sessions To Be Modified Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | <reference> |  | – | – |
| >>Bearer Configurations To Be Modified |  | 1 .. <maxnoofBearerConfigs> |  |  | EACH | reject |
| >>>CHOICE Bearer Configuration | M |  |  |  |  |  |
| >>>>SN terminated Bearer |  |  |  |  |  |  |
| >>>>>PDU Session Modification Info – SN terminated |  |  | 9.2.p5 |  | - |  |
| >>>>MN terminated Bearer |  |  |  |  |  |  |
| >>>>>PDU Session Modification Info – MN terminated |  |  | 9.2.p7 |  | - |  |
| PDU sessions To Be Released List |  | 0..1 |  |  | YES | ignore |
| >PDU sessions To Be Released Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>PDU session ID | M |  | <reference> |  | – | – |
| >>Cause | M |  | <reference> |  | – | – |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Includes the SCG-Config message as defined in TS yy.xxx [x] Editor’s Note: to be checked with RAN2 | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

9.1.2.9	S-NODE MODIFICATION CONFIRM

This message is sent by the M-NG-RAN node to inform the S-NG-RAN node about the successful modification.

Direction: M-NG-RAN node  S-NG-RAN node.

Editor’s Note: The tabular below is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <reference> |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the S-NG-RAN node | YES | ignore |
| PDU sessions Admitted To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >PDU sessions Admitted To Be Modified Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>PDU session ID | M |  | <reference> |  | – |  |
| >>Bearer Configurations Admitted To Be Modified |  | 1 .. <maxnoofBearerConfigs> |  |  | EACH | ignore |
| >>>CHOICE Bearer Configuration | M |  |  |  |  |  |
| >>>>SN termianted Bearer |  |  |  |  |  |  |
| >>>>> PDU Session Modification Response Info – SN terminated | M |  | 9.2.p6 |  | – |  |
| >>>>MN terminated Bearer |  |  |  |  |  |  |
| >>>>> PDU Session Modification Response Info – MN terminated | M |  | 9.2.p8 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the SCG-ConfigInfo message as defined in TS yy.xxx [x] Editor’s Note: FFS if same RRC message is applicable for SCG bearer. | YES | ignore |
| Criticality Diagnostics | O |  | <reference> |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |