3GPP TSG-RAN WG4 Meeting #100e                	R4-2115323

Electronic Meeting, Aug.16 - 27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.133 draftCR on the CSI-RS resource periodicity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CSIRS_L3meas-Core |  |  |  |  |  | Date: |  |  | 2021-8-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. In RAN4 96e meeting, the CSI-RS resource periodicity was agreed as in WF R4-2012178. This needs to be added to applicability sections when single 5ms window is configured. - CSI-RS periodicities for L3 measurement: 10, 20, 40 ms 2. The section numbers of Annex B in 9.10.2.2 and 9.10.3.2 are missing. 3. Some editorial corrections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CSI-RS resource periodicity was added to the applicability sections for intra-frequency and inter-frequency measurements. And the section numbers of Annex B are fixed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE CSI-RS based measurement requirements are not clear enough. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.10.1, 9.10.2.2 and 9.10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of changes >>

<Start of Change 1>

## 9.10	CSI-RS based L3 measurements

### 9.10.1	Introduction

This clause contains general requirements on the UE regarding CSI-RS based measurement reporting in RRC_CONNECTED state. The requirements are split in intra-frequency and inter-frequency measurements requirements.

The requirements in this clause apply, provided:

-	Only one MO is configured per CSI-RS frequency layer, and

-	all CSI-RS resources in the same MO are configured with the same csi-rs-MeasurementBW, and

-	all CSI-RS resources in the same MO are configured with the same periodicity, and

-	associatedSSB is configured in CSI-RS-Resource-Mobility and detectable, and

-	the associated SSB is QCLed with the corresponding CSI-RS resources in FR2, and

-	for intra-frequency layers, the starting point of the first 5ms window is the slot boundary of the serving cell, where the corresponding slot contains the configured L3 CSI-RS resource of the serving cell in the servingCellMO with the smallest offset, and

-	for intra-frequency layers, the starting point of the second 5ms window, if applicable, is determined by an offset of half of the CSI-RS periodicity in slots with regards to the starting point of the first 5ms window, and

-	the number of CSI-RS resources in any duration that equals to the length of a slot is no larger than UE capability maxNumberCSI-RS-RRM-RS-SINR.

### 9.10.2	CSI-RS based intra-frequency measurements

#### 9.10.2.1	Introduction

A measurement is defined as a CSI-RS based intra-frequency measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for measurement, and

-	the CP type of the CSI-RS resource of neighbour cell configured for measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for measurement, and

-	It is applied for SCS = 60KHz

-	the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for measurement

The UE shall be able to identify new intra-frequency cells and perform CSI-RSRP, CSI-RSRQ and CSI-SINR measurements of identified intra-frequency cells if carrier frequency information is provided by PCell or the PSCell.

No measurement gap is needed for intra-frequency CSI-RS resources measurements.

For intra-frequency CSI-RS based measurements, UE may cause scheduling restriction as specified in clause 9.10.2.6.

Note:	Extended CP for CSI-RS based measurement is not supported in this release.

#### 9.10.2.2	Requirements applicability

The measurement of the associated SSB follows the same requirements as SSB based measurements defined in 9.2.

The requirements in clause 9.10.2 apply, provided:

-	Only one intra-frequency CSI-RS layer per serving cell is configured, and

-	The BW of the CSI-RS on the intra-frequency neighbor cell is within the active BWP of the UE, and

-	The associated SSB of the CSI-RS resources being identified or measured are detectable, and the CSI-RS resources configured for CSI-RS based L3 measurements are measurable, and

-	The bandwidth of CSI-RS resources of intra-MO is the same as that of the CSI-RS resources configured for the serving cell, and

-	All CSI-RS resources on one intra-frequency layer are configured within up to two separate windows where each window is up to 5ms, and

- for the case of single window further provided

-	The periodicity of the configured CSI-RS resources is 10ms, 20ms or 40ms

- for the case of two separate windows further provided

-	The two windows are either both fully non-overlapped with MG or both partially overlapped with MG

-	The periodicity of the configured CSI-RS resources is 20ms or 40ms, and

-	The gap between two 5ms windows is half of the CSI-RS periodicity.

-	Numerology for intra-frequency CSI-RS and data of serving cell are the same.

Editor’s note: two separate 5ms windows for intra-frequency measurement is supported at least for FR1, and FFS for FR2.

Editor’s note: FFS whether the conditions for two separate 5ms windows apply also to the case when all CSI-RS resources on one intra-frequency layer are configured within one single windows.

An intra-frequency cell shall be considered detectable when for each relevant associated SSB:

SS-RSRP related side conditions given in clauses 10.1.2.1 and 10.1.3.1 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses 10.1.7.1 and 10.1.8.1 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses 10.1.12.1 and 10.1.13.1 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

A CSI-RS resource shall be considered measurable when for each relevant CSI-RS resource:

-	CSI-RSRP related side conditions given in clauses 10.1.2.3 and 10.1.3.3 for FR1 and FR2, respectively, for a corresponding Band,

-	CSI-RSRQ related side conditions given in clauses 10.1.7.2 and 10.1.8.2 for FR1 and FR2, respectively, for a corresponding Band,

-	CSI-SINR related side conditions given in clauses 10.1.12.2 and 10.1.13.2 for FR1 and FR2, respectively, for a corresponding Band,

-	CSI_RP and CSI-RS Ês/Iot according to Annex B.2.12 for a corresponding Band.

<End of Change 1>

<Start of Change 2>

### 9.10.3	CSI-RS based Inter-frequency measurements

#### 9.10.3.1	Introduction

A measurement is defined as a CSI-RS based inter-frequency measurement provided it is not defined as an intra-frequency measurement according to clause 9.10.2.

If a UE is configured with the higher layer parameter CSI-RS-Resource-Mobility and the higher layer parameter associatedSSB is configured, the UE shall be able to identify inter-frequency cells indicated for measurement and perform CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements of identified inter-frequency cells.

When measurement gaps are needed, the UE is not expected to detect the associated SSB nor perform measurement of the CSI-RS resource configured in CSI-RS-Resource-Mobility on an inter-frequency measurement object which start earlier than the gap starting time + switching time, and ends later than the gap end – switching time. When the inter-frequency cells are in FR2 and the per-FR gap is configured to the UE in EN-DC, SA NR, NE-DC and NR-DC, or the serving cells are in FR2, the inter-frequency cells are in FR2 and the per-UE gap is configured to the UE in SA NR and NR-DC, the switching time is 0.25ms. Otherwise the switching time is 0.5ms.

#### 9.10.3.2	Requirements applicability

The associated SSB layer of the CSI-RS follows the same requirements as SSB based measurements defined in 9.3.

The requirements in clause 9.10.3 apply, provided:

-	The associated SSB of the cell being identified or measured is detectable, and

-	All CSI-RS resources on one inter-frequency layer are configured within a window of up to 5ms, and

-	The periodicity of the configured CSI-RS resources is 10ms, 20ms or 40ms, and

-	CSI-RS resources for measurements and the associated SSB for cell identification are configured within measurement gap.

An inter-frequency cell shall be considered detectable when for each relevant associated SSB:

-	SS-RSRP related side conditions given in clauses 10.1.4.1 and 10.1.5.1 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses 10.1.9.1 and 10.1.10.1 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses 10.1.14.1 and 10.1.15.1 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.3 for a corresponding Band.

A CSI-RS resource shall be considered measurable when for each relevant CSI-RS resource:

-	CSI-RSRP related side conditions given in clauses 10.1.4.3 and 10.1.5.3 for FR1 and FR2, respectively, for a corresponding Band,

-	CSI-RSRQ related side conditions given in clauses 10.1.9.2 and 10.1.10.2 for FR1 and FR2, respectively, for a corresponding Band,

-	CSI-SINR related side conditions given in clauses 10.1.14.2 and 10.1.15.2 for FR1 and FR2, respectively, for a corresponding Band,

-	CSI _RP and CSI-RS Ês/Iot according to Annex B.2.13 for a corresponding Band.

<End of Change 2>