3GPP TSG RAN WG4 Meeting #100-e 		R4-2115368

Electronic Meeting, August 16 – 27, 2021

Title:	Reply LS on UE/TRP Tx/Rx timing error mitigation

Response to:	R4-2107610 (R1-2104111)

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact person:

Name:	Qiuge Guo

E-mail Address:	guoqiuge@catt.cn

Attachments: None

1	Overall description

In LS R4-2107610 (R1-2104111), RAN1 provides the following agreements on mitigating TRP/UE Tx/Rx timing error for TDOA together with the definition of UE/TRP Tx/Rx timing errors and TEGs.

Agreement: - Support the following for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA - Support a UE to provide the association information of RSTD measurements with UE Rx TEG(s) to the LMF when the UE reports the RSTD measurements to the LMF if the UE has multiple TEGs - Support a TRP providing the association information of DL PRS resources with Tx TEGs to the LMF if the TRP has multiple TEGs - Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs - FFS: the details of the signalling, procedures, and UE capability - Send an LS to RAN4 to check if there is any issue to support the above enhancements

RAN4 discussed the agreements above and the definition of UE/TRP Tx/Rx/RxTx TEG provided in the LS, and the following views are provided from RAN4 side:

- It is RAN4 understanding that TEG framework enables association information without limiting implementation to ensure that the timing error difference between measurements/transmissions associated to the same TEG are within a certain margin.

- It is not necessary to know the absolute timing error for UE/TRP Tx/Rx TEG.

- The above mechanism for timing error mitigation defined by RAN1 is feasible for both UE Rx/Tx and TRP Rx/Tx.

- UE/TRP may group the timing errors for UE/TRP Rx/Tx (e.g., based on RF chains and antenna panel) such that timing error difference in the same group is within a certain margin

- RAN4 will further study on RRM requirements for timing error mitigation mechanism, timing error grouping method, criteria and margin.

- RAN4 will further study if any specific UE behavior will be defined.

- RAN4 will further discuss the time variation of TEGs.

2	Actions

To RAN WG1

ACTION: 	RAN4 kindly asks RAN1 to take the information above into consideration.

3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #101-e		Nov. 1 – 12, 2021		Electronic Meeting