3GPP TSG-RAN WG4 Meeting#100-e                               R4-2113925

E-meeting,16th Aug– 27th Aug, 2021

Agenda item:	9.18.2

Source: 	ZTE Corporation

Title: 	Discussion on reply LS on NR coverage enhancement

Document for:	Approval

Introduction

In the previous RAN1 meeting, there was one reply LS [1] sent to RAN4 with some additional questions raised recently. In this contribution, we want to share some initial understandings on that reply LS.

Discussion

- For joint channel estimation, is there a maximum duration during which UE is able to maintain power consistency and phase continuity under certain tolerance level? If any, how long is it?

- Proposed reply: yes, there is a maximum duration, the tolerance level for phase continuity and power consistency are still under discussion in RAN4.

  - What factors determine the maximum duration?

- Proposed reply: downlink pathloss/RSRP and timing/frequency variation within the duration, the phase error/frequency error from PLL and PA output power stabilization in RF Tx chain;

  - Whether the maximum duration should be the same for different cases for both PUSCH and PUCCH?

- Proposed reply: yes, this should be same for PUSCH and PUCCH;

  - Whether the maximum duration is dependent on the modulation order of transmission, e.g., QPSK, 16QAM, 64QAM?

- Proposed reply: this maximum duration is not dependent on modulation order.

  - Whether the maximum duration is dependent on UL waveform (DFT-s-OFDM vs. OFDM)?

- Proposed reply: this maximum duration is not related with DFT-s-OFDM or CP-OFDM;

  - Whether the maximum duration is band specific?

- Proposed reply: this maximum duration is not band specific;

  - Besides the factors listed above, whether or not the maximum duration is further dependent on UE capabilities (e.g., multiple possible values for a given set of factor(s)), and if so, whether the UE should report such a duration

- Proposed reply: single maximum duration is preferred, otherwise this might cause more UE fragmentation and scheduling difficulties from network perspective.

References

- R4-2111706, LS on joint channel estimation for PUSCH and PUCCH, RAN1

Annex

3GPP TSG-RAN WG4 Meeting # 100-e 	R4-210xxxx

# Electronic Meeting, Aug. 16-27, 2021

Title:	Draft reply LS on NR coverage enhancement

Response to:	(R1-2106212) LS on joint channel estimation for PUSCH and PUCCH

Release:	Release 17

Work Item:	   NR_cov_enh

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:

Contact Person:

#### Name:	Fei, Xue

E-mail Address:	xue.fei25@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN4 would like to thank RAN1 for the (R1-2106212) LS on joint channel estimation for PUSCH and PUCCH. RAN4 has discussed the issues raised by RAN1 and concluded the following answers:

- For joint channel estimation, is there a maximum duration during which UE is able to maintain power consistency and phase continuity under certain tolerance level? If any, how long is it?

- Proposed reply: yes, there is a maximum duration, the tolerance level for phase continuity and power consistency are still under discussion in RAN4.

  - What factors determine the maximum duration?

- Proposed reply: downlink pathloss/RSRP and timing/frequency variation within the duration, the phase error/frequency error from PLL and PA output power stabilization in RF Tx chain;

  - Whether the maximum duration should be the same for different cases for both PUSCH and PUCCH?

- Proposed reply: yes, this should be same for PUSCH and PUCCH;

  - Whether the maximum duration is dependent on the modulation order of transmission, e.g., QPSK, 16QAM, 64QAM?

- Proposed reply: this maximum duration is not dependent on modulation order.

  - Whether the maximum duration is dependent on UL waveform (DFT-s-OFDM vs. OFDM)?

- Proposed reply: this maximum duration is not related with DFT-s-OFDM or CP-OFDM;

  - Whether the maximum duration is band specific?

- Proposed reply: this maximum duration is not band specific;

  - Besides the factors listed above, whether or not the maximum duration is further dependent on UE capabilities (e.g., multiple possible values for a given set of factor(s)), and if so, whether the UE should report such a duration

- Proposed reply: single maximum duration is preferred, otherwise this might cause more UE fragmentation and scheduling difficulties from network perspective.

2. Actions:

To RAN1 group.

ACTION: 	RAN4 respectfully asks RAN1 to take the above information into account.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting#101e 	   1st – 12th Nov 2021	Electronic Meeting