# 3GPP TSG-RAN WG4 Meeting # 100-e 	R4-2201960

Electronic Meeting, Jan 17th - 27th, 2022

Source: 	Samsung

Title: 	Inter-cell beam management in FeMIMO

Agenda Item:	6.19.3.2

Document for:	Discussion

Introduction

In previous RAN4 meeting, WF for inter-cell beam management has been approved in [1] as

Inter-cell beam management - Define known/unknown cell condition for non-serving cell configured for L1-RSRP measurements - Measurement accuracy for serving cell L1-RSRP can be reused for L1-RSRP measurement for non-serving cell for inter-cell beam management - The L1-RSRP measurement requirements for non-serving cells are only applicable to SSB withing the active DL BWP of the UE. - RAN4 develops UE requirements for at least Nmax = 1, where Nmax is the maximum number of RRC configured PCIs different from the serving cell for measurement/reporting. - RAN4 will further study to specify the requirements for other Nmax value taking RAN1 decision into account - RAN4 will further study timing offset assumptions between non-serving cell and serving cell for L1-RSRP measurement in future RAN4 meetings - Where non-serving cell refers to all the TRPs configured by gNB for inter-cell BM and serving cell(s) is the cell(s)/TRP(s) from which UE receive PDCCH/PDSCH. - RAN4 will further study the FFT implementation assumption for L1-RSRP measurement - RAN4 will further study the Rx beam assumption for L1 RSRP measurement - RAN4 will further study the measurement behaviour for L1-RSRP measurement for non-serving cell whether to perform the non-serving cell L1-RSRP measurement outside SMTC or within SMTC

In this contributions, analysis for above open issues of inter-cell beam management in FeMIMO has been provided.

Discussion

- In Rel-17, in order to further mTRP scenario, TCI switching to cells with different cell ID from serving cell is introduced. To support TCI switching to cells with different cell ID, L1-RSRP on SSB associated with cell with different cell ID are also introduced. Furthermore, RAN1 additionally agreed the indication of information for cells with different cell ID via serving cells, i.e,

Agreement On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, a CSI-SSB-ResourceSet configured for L1-RSRP measurement/reporting includes at least a set of SSB indices where PCI indices are associated with the set of SSB indices, respectively. The PCI indices refer to PCIs within the set of PCIs configured for inter-cell beam management or inter-cell multi-TRP. - The additionalInfo associated with SSB(s) with PCI(s) different from the serving cell agreed in RAN1 Agenda Item 8.1.2.2 is also applicable to inter-cell BM - Detailed signaling design is up to RAN2 - FFS (to be concluded in RAN1#107-e): Whether the above L1-RSRP measurement/reporting also includes group-based beam report for inter-cell mTRP Agreement - Center frequency, SCS, SFN offset are assumed to be the same for SSBs from the serving cell and the configured SSBs with PCI different from the serving cell for inter-cell multi TRP operation. - The information related to “SSB time domain position” for SSB with PCI different from the serving cell consists of [halfFrameIndex and] ssb-PositionsInBurst

- Therefore, the concept of cells with different cell ID under Rel-17 inter-cell beam management are different from non-serving cell concept in Rel-15/16 RRM requirements. It is better to align the terminology. In summary, the non-serving cell for L1-RSRP in previous RAN4 agreements are referred to configured SSBs with PCI different from serving cell for inter-cell multi TRP operation. With such clarifications, it is better to replace the “non-serving cell” with “configured SSB with PCI different from serving cell” in L1-RSRP discussions.

- Overall specification structure

To support L1-RSRP measurement for configured SSB with PCI different from serving cell in Rel-17 RAN4 specification, either introducing additional sub sections in section 9 or accommodating newly introduced L1-RSRP measurement requirements in existing section 9.5 can be considered as alternatives for specification structure.

For introducing new section for L1-RSRP measurement for non-serving cell, the similar structure as serving cell can be considered

9.12 L1-RSRP measurements for configured SSB with PCI different from serving cell 9.12.1 Introduction 9.12.2 Requirement applicability 9.12.3 Measurement reporting requirements 9.12.4 L1-RSRP measurement requirements 9.12.5 Measurement restriction 9.12.6 Scheduling availability

However, the drawback of introducing the new section is that without changing the title of existing section 9.5, i.e., L1-RSRP measurement for reporting, the differential between new section and section 9.5 is not very clear.

For performance part, i.e., accuracy requirements, it has been agreed to reuse the existing accuracy requirements for non-serving cell, to accommodate the measurement accuracy in existing section 10.1.19 and 10.1.20 can be considered, e.g., by introducing the below modifications for 10.1.19.1

##### Absolute Accuracy Unless otherwise specified, the requirements for absolute accuracy of SSB based L1-RSRP in this clause apply to all SSBs of the serving cell and SSB with PCI different from serving cell configured for L1-RSRP measurement.

We can also understand there may be other alternatives to specify the requirements for L1 RSRP measurement. Given March 2022 is the deadline for core requirements, it is strongly recommended RAN4 could agree on the specification structure in RAN4 101bis-e meeting taking the above suggestion into accounts. Relative CRs can be prepared in coming RAN4 Feb meeting in order to meet the schedule of FeMIMO WI core part.

Proposal 1: RAN4 agreed the specification structure for L1-RSRP measurement requirements for configured SSB with PCI different from serving cell in RAN4 101bis meeting taking the above suggestions into account

- Known/unkown cell condition

RAN4 has agreed to introduced known condition for configured SSB with PCI different from serving cell in previous RAN4 meeting. Also, RAN has limit the inter-cell beam management scenario to intra-frequency case. Therefore, the existing side condition for intra-frequency cell L3 measurement can be used as the condition whether such configured SSB with PCI different from serving cell are known or unknown. In current RAN4 specification, the condition for intra-frequency measurement is

The requirements in clause 9.2 apply, provided: - The cell being identified or measured is detectable. An intra-frequency cell shall be considered detectable when for each relevant SSB: - SS-RSRP related side conditions given in clauses 10.1.2 and 10.1.3 for FR1 and FR2, respectively, for a corresponding Band, - SS-RSRQ related side conditions given in clauses 10.1.7 and 10.1.8 for FR1 and FR2, respectively, for a corresponding Band, - SS-SINR related side conditions given in clauses 10.1.12 and 10.1.13 for FR1 and FR2, respectively, for a corresponding Band, - SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

In our understanding, different from cell identified in above requirements, serving cell has already provided the associated information for L1-RSRP measurement for configured SSB with different cell ID. As long as the measurement of such SSB with different PCI is detectable, such cell shall be specified as known cell. Otherwise, the SSB is unknown, For unknown SSBs, additional delay for identifying the cells can be considered based on the acknowledged information for SSB with different PCI for measurement period for L1 measurement period requirements.

Proposal 2: Existing condition for detected intra-frequency cell can be reused for known condition for L1-RSRP for SSBs with different PCI

Proposal 3: For unknown SSB with different PCI, RAN4 can further discuss whether to introduce additional delay performance for L1-RSRP.

UE measurement assumption and behaviour for L1-RSRP

RAN1 did not conclude either restrict non-serving measurement within SMTC or outside SMTC. Without stable RAN1 agreement, RAN4 can still slow start the discussion for measurement period for non-serving cell measurement. Actually, there were some agreements in previous RAN4 meeting, i.e.,

- In RAN4 #99e meeting, RAN4 discussed the UE behavior, performance and UE complexity for supporting non-serving cell measurement. RAN4 will further discuss the non-serving cell measurement in the context of L1/L2 centric inter-cell mobility considering the followings in the future RAN4 meetings: - For the case when the measurement RS from the non-serving cell is within SMTC, in FR1, legacy measurement behavior based on L3 measurement may be reused from RAN4 perspective. - RAN4 will continue to evaluate the impact on a longer measurement delay due to sharing of SMTC window by L1 and L3 measurements. - For the case when the measurement RS from the non-serving cell is outside SMTC and within SMTC, performance degradation or increased UE complexity may be expected - To consider concurrent serving cell PDSCH/PDCCH reception and neighbor SSB based L1 measurement outside SMTC, RAN4 needs more time to study on the impacts on performance degradation and UE complexity. - Note: existing UE feature simultaneousRxDataSSB-DiffNumerology is aimed for concurrent intra-frequency measurement on serving cell or neighbouring cell and PDCCH or PDSCH reception.

Based on above understanding, for FR1, if L1 measurement for SSB with different PCI can be restrict within SMTC, UE can perform L1-RSRP of SSB with different PCIs and L3 measurement simultaneously. Therefore, at least for FR1, RAN4 can start the discussions on specifying the L1-RSRP measurement requirements for the case SSB is fully overlapped with SMTC taking the existing serving cell L3 measurement behaviour as baseline.

For L1 measurement for SSB with different PCI outside SMTC in FR1, the trade-off between performance degradation caused by scheduling restriction and limitation of timing assumption between serving cell and other cells has to be considered.

For FR2, due to different Rx beam assumption for L1 measurement and L3 measurement, UE is not required to measure L1-RSRP and L3 measurement within SMTC. To avoid L3 measurement degradation, scaling factor has been introduced in Rel-15/16. Therefore, fore both FR1 and FR2, the existing requirements for L1 serving cell measurement can be reused as baseline for such case.

For L1 measurement for SSB with different PCI outside PCI in FR2, as similar as FR1, not only the timing difference assumption between cells and also the Rx beam assumption for L1 measurement and L3 measurement have to be considered.

Proposal 4, Existing serving cell L3 measurement behaviour in FR1 and L1 measurement requirements in FR2 can be used as baseline for L1-RSRP measurement for SSB with different PCI fully overlapped with SMTC.

Proposal 5: RAN4 can further discuss the case when SSB with different PCI not fully overlapped with SMTC by taking the scheduling restriction, measurement restriction, timing difference assumption and Rx beam assumption into account.

Conclusion

In this contributions, the following proposals have been provided

Proposal 1: RAN4 agreed the specification structure for L1-RSRP measurement requirements for configured SSB with PCI different from serving cell in RAN4 101bis meeting taking the above suggestions into account

Proposal 2: Existing condition for detected intra-frequency cell can be reused for known condition for L1-RSRP for SSBs with different PCI

Proposal 3: For unknown SSB with different PCI, RAN4 can further discuss whether to introduce additional delay performance for L1-RSRP.

Proposal 4, Existing serving cell L3 measurement behaviour in FR1 and L1 measurement requirements in FR2 can be used as baseline for L1-RSRP measurement for SSB with different PCI fully overlapped with SMTC.

Proposal 5: RAN4 can further discuss the case when SSB with different PCI not fully overlapped with SMTC by taking the scheduling restriction, measurement restriction, timing difference assumption and Rx beam assumption into account.

- References

[1] R4-2120321, WF on FeMIMO RRM requirements for inter-cell beam management, Samsung