3GPP TSG-RAN WG4 Meeting #101bis-e 												R4-2202250

Electronic Meeting, 17 – 26th January 2022

Agenda item:			5.1.2

Source:	Apple Inc.

Title:	WF on adding a 20 MHz channel opportunity (Channel and Sync raster point) in the 5925-5945 MHz range for n96 and n102

Document for:	Information

# Introduction

3GPP Rel-16 NR-U WI specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  While 5GHz is a well-known band for the unlicensed operation, 6GHz is a relative new band usage of which was approved recently in different regulatory regions. Following the publication of the 6GHz regulatory decision for the EU/CEPT countries is November 2020, 3GPP RAN#90 meeting decided to resume the corresponding WI [2] that aims at adding 3GPP support for the 6GHz band in that regulatory region. As per 3GPP RAN plenary guidance made during the RAN#93 meeting, RAN WG4 was tasked to define a new band for the unlicensed operation in the lower 6GHz band. In addition to that, RAN#94 meeting has also clarified that a dedicated lower 6GHz unlicensed band should be defined as 5925-6425MHz.

While the RAN plenary recommended to define the new lower unlicensed band as 5925-6425MHz, existing channel raster points does not include the raster for the first 20MHz channel in 5925-5945MHz. This document presents the way forward for enabling first 20MHz.

# Way forward

## Background

While the 3GPP TSG RAN recommended to define the new lower unlicensed band (n102) as 5925-6425MHz, existing channel raster points start at 5945MHz. For instance, Nref 797000 (see the highlighted text below) corresponds to 5955MHz center frequency of the 20MHz channel at 5945-5965MHz; and similarly, Nref 797668 corresponds to the 5965MHz center frequency of the 40MHz channel at 5945-5985MHz. While this is aligned with the available channels in the EU/CEPT region, it will effectively prohibit using first 20MHz in other countries/regions, such as UK, Morocco, UAE, etc. In fact, it is only EU/CEPT where this unlicensed band is limited to 5945-6425MHz, while in other countries it starts at 5925MHz. Furthermore, even Region 3 countries, such as Australia and New Zealand, define the starting point of this band at 5925MHz.

| n10217 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | TDD13 |
| --- | --- | --- | --- |

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 20 MHz | 797000, 798332, 799668, 801000, 802332, 803668, 805000, 806332, 807668, 809000, 810332, 811668, 813000, 814332, 815668, 817000, 818332, 819668, 821000, 822332, 823668, 825000, 826332, 827668 |
| 40 MHz | 797668, 800332, 803000, 805668, 808332, 811000, 813668, 816332, 819000, 821668, 824332, 827000 |
| 60 MHz | 798332, 799668, 803668, 805000, 809000, 810332, 814332, 815668, 819668, 821000, 825000, 826332 |
| 80 MHz | 799000, 804332, 809668, 815000, 820332, 825668 |

Furthermore, the same issue exists also for band n96 which also starts 5925MHz, but first channel raster point of which does not allow using first 20MHz.

## Way forward

-	In Rel-18, RAN WG4 will consider enabling first 20MHz, 5925-5945MHz, for both bands n102 and n96.

-	RAN WG4 will check potential raster and sync points.

-	The existing raster points shall not be changed.

-	RAN WG4 will check whether existing A-MPR values still can be re-used as they are.

-	No new NS values are assumed