3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2202266

Electronic Meeting, 17 – 25 January 2022

Agenda Item:	5.3

Source: 	ZTE

Title: 	WF on system parameters for 6GHz licensed band

Document for:	Approval

# 1.	Background

Contributions on system parameters for 6GHz licensed band are summarized as following:

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200149 | CATT | Discussion on system parameters for 6GHz licensed band |
| R4-2201304 | Xiaomi | Discussion on system parameters for 6G license band |
| R4-2201447 | ZTE Corporation | Discussion on system parameters for 6425-7125MHz |
| R4-2201504 | Huawei, HiSilicon | System parameters for 6GHz NR licensed band |
| R4-2201545 | Skyworks Solutions Inc. | 6GHz licensed band attributes based on RCC recommendations |
| R4-2201855 | Qualcomm Incorporated | Proposal: RAN4 to decide whether the defined 6 GHz licensed band should be specific to 6425 – 7125 MHz with possible NS, or a more general band 5925 – 7125 MHz with specific NS requirements for countries allowing only the subset. Proposal: Channel bandwidths for this band as 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 MHz. |
| R4-2201824 | ZTE Corporation | draft CR on introduction of 6GHz system parameters for 38.101-1 |
| R4-2201825 | ZTE Corporation | draft CR on introduction of 6GHz system parameters for 38.104 |
| R4-2201330 | Ericsson | Based on the above information, we would propose to specify at least the following channel BWs for this new 6GHz licensed band: 60, 70, 80, 90 and 100 MHz. And other channel BWs are not precluded. |

- Way forward

2.1	Frequency range

Agreement:

  - Option 1: Define a new band covering the 6425-7125 MHz frequency range

2.2 Band number

Agreement:

  - Band number could be discussed later on;

2.3 Channel bandwidth

Agreement:

  - 20, 30, 40, 50, 60, 70, 80, 90, 100;

  - FFS for 25, 35,45MHz

2.4 Channel raster

Candidate options:

  - Option 1: follow the legacy approach: 15kHz and 30kHz;

  - Option 2: based on 5MHz frequency block, FFS on the granularity of NR-ARFCN

2.5 Synchronization raster

This should depend on agreement of the minimum channel bandwidth and channel raster agreement, companies’ further view are welcome in 2nd round.