# 3GPP TSG-RAN WG4 Meeting #101bis-e	R4-2202355

# Electronic Meeting, 17th - 25th January, 2022

Source: 	vivo

Title: 	TP for TR 38.785: Addition of definitions and symbols to Chapter 3

Agenda Item:	6.15.1

Document for:	Approval

- Introduction

In this text proposal, definitions and symbols for SL enhancement were added on the latest version of TR 38.785[1].

- Reference

[1] TR 38.785 V0.4.0

# Annex: TP for channel arrangement in TR38.785

============================ Start of TP ============================

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Con-current operation: The simultaneous transmission and reception of sidelink and Uu interfaces, irrespective of TDM mode and FDM mode, while operation is agnostic of the service used on each interface.

Contiguous carriers: A set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block.

Contiguous resource allocation: A resource allocation of consecutive resource blocks within one carrier or across contiguously aggregated carriers. The gap between contiguously aggregated carriers due to the nominal channel spacing is allowed.

Contiguous spectrum: Spectrum consisting of a contiguous block of spectrum with no sub-block gaps.

Intra-band V2X operation: Operation of LTE V2X carrier and NR V2X carrier in the same operating band.

Intra-band V2X con-current operation: Operation of NR SL carrier and NR Uu carrier in the same licensed band.

Intra-band contiguous V2X operation: Operation of contiguous LTE V2X carrier and NR V2X carrier in the same operating band.

Intra-band contiguous V2X con-current operation: Operation of contiguous NR V2X carrier and NR Uu carrier in the same licensed band.

Inter-band con-current operation: Operation of NR Uu carrier and NR V2X carrier in different operating bands.

Intra-band V2X con-current operating UE: UE that supports the intra-band V2X con-current operation.

NR V2X UE/ V2X UE: UE that supports NR V2X/V2X features.

NR V2X carrier: Carrier for NR V2X service.

PC5: The interface for sidelink transmission.

PC2 V2X UE: V2X UE that supports 26 dBm power class.

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same UE. There may be multiple instances of sub-blocks within an RF bandwidth.

Sub-block bandwidth: The bandwidth of one sub-block.

Sub-block gap: A frequency gap between two consecutive sub-blocks within an RF bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation.

UE transmission bandwidth configuration: Set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE.

Vehicular UE: A UE embedded in a vehicle, permanently connected to an embedded antenna system that radiates externally for NR operating bands.

NOTE:	Vehicular UE does not refer to other UE form factors placed inside the vehicle.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔPPowerClass	Adjustment to maximum output power for a given power class

RB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔShift	Channel raster offset

TC	Allowed operating band edge transmission power relaxation

TC,c	Allowed operating band edge transmission power relaxation for serving cell c

∆SNRSL-Uu	Difference in the target SNR value between NR Uu and SL Uu

∆ILUL-DL	Difference in insertion loss on the UL received path for SL reception compare to the DL received path for NR Uu, and includes the difference of UL filter compared to the DL filter of Band duplexer and two switches on the UL path.

∆LCRB	Difference RB size between NR Uu RMC and NR SL RMC

BWChannel	Channel bandwidth

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low

BWChannel,max	Maximum channel bandwidth supported among all bands in a release

BWChannel_SL&UL	The total channel bandwidth for SL and UL

BWGB	max( BWGB,Channel(k) )

BWGB,Channel(k)	Minimum guard band of carrier k

BWgap		BWgap is the bandwidth of the gap between NRB1 and NRB2 possible allocations of SL CC1 and UL CC2 respectively.

BWDL	Channel bandwidth for DL

BWUL	Channel bandwidth for UL

BWinterferer	Bandwidth of the interferer

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

FC	RF reference frequency on the channel raster

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC,low	The Fc of the lowest carrier, expressed in MHz

FC,high	The Fc of the highest carrier, expressed in MHz

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset, low.

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset, high.

Fedge , low	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge,low = FC,low - Foffset,low.

Fedge, high	The higher edge of aggregated channel bandwidth, expressed in MHz. Fedge,high = FC,high + Foffset,high.

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FInterferer	Frequency of the interferer

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

Foffset	Frequency offset from FC_high to the higher edge or FC_low to the lower edge.

Foffset,high	Frequency offset from FC,high to the upper UE RF Bandwidth edge, or from FC,block, high to the upper sub-block edge

Foffset,low	Frequency offset from FC,low to the lower UE RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FREF,Shift	RF reference frequency for Supplementary Uplink (SUL) bands and for the uplink for all FDD bands

Fuw (offset)	The frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

Gpost connector	The supported post antenna connector gain

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

A-MPRBase		The default A-MPR value when no Gpost connector is declared

A-MPRStep		The weighting value of Gpost connector when Gpost connector is declared

Max()	The largest of given numbers

Min()	The smallest of given numbers

Physical resource block number

NRACLR	NR ACLR

NR_SLREFSENS	The REFSENS value defined for NR SL

NR_UuREFSENS 	The REFSENS value defined for NR Uu

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_alloc	The allocated RBs

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth NRB_agg=1jNRBj*2μj for carrier 1 to j, where μ is defined in TS 38.211

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks NRB,cj=NRBj*2μj for carrier j, where μ is defined in TS 38.211

NRB,largest BW	The largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in units of resource blocks

NRB,low	The transmission bandwidth configurations according to Table 5.3.2-1 for the lowest assigned component carrier in clause 5.3A.1

NRB,high	The transmission bandwidth configurations according to Table 5.3.2-1 for the highest assigned component carrier in clause 5.3A.1

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

NFV2X	Noise figure for V2X

PCMAX	The configured maximum UE output power

PCMAX, c	The configured maximum UE output power for serving cell c

PCMAX, f, c	The configured maximum UE output power for carrier f of serving cell c in each slot

PEMAX	Maximum allowed UE output power signalled by higher layers

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c

PInterferer	Modulated mean power of the interferer

Plargest BW	Power of the largest transmission bandwidth configuration of the component carriers in the bandwidth combination

PPowerClass	PPowerClass is the nominal UE power (i.e., no tolerance)

P-MPRc	Power Management Maximum Power Reduction for serving cell c

PRB	The transmitted power per allocated RB, measured in dBm

PUMAX	The measured configured maximum UE output power

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

PREFSENS_V2X	The REFSENS power for V2X

RBstart	Indicates the lowest RB index of transmitted resource blocks

RBStart,Low	RBStart,Low = max(1, floor(LCRB/2))

RBStart,High		RBStart,High = NRB – RBStart,Low – LCRB

RX_BW	The bandwidth for receiving

REFSENSV2X	REFSENS value defined for V2X

SNRV2X	Signal-to-Noise Ratio for V2X

T(PCMAX, f, c)	Tolerance for applicable values of PCMAX, f, c for configured maximum UE output power for carrier f of serving cell c

SSREF	SS block reference frequency position

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AGC	Automatic Gain Control

A-MPR	Additional Maximum Power Reduction

BLER	BLock Error Rate

BS	Base Station

CBW	Channel Bandwidth

CDF	Cumulative Distribution Function

CP-OFDM	Cyclic Prefix-OFDM

DMRS	Demodulation Reference Signal

DSRC	Dedicated Short-Range Communications

EIRP	Equivalent Isotropically Radiated Power

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

FDM	Frequency Division Multiplexing

FR1	Frequency Range 1

FR2	Frequency Range 2

HD	Half Duplex

ITS	Intelligent Transportation System

IM	Implementation Margin

LDPC	Low Density Parity Check

LTE	Long Term Evolution

LOS	Line-Of-Sight

MPR	Maximum Power Reduction

MBW	Measurement BandWidth

NF	Noise Figure

NLOS	Non-Line-Of-Sight

NR	New Radio

OLPC	Open Loop Power Control

PC	Power Control

PRB	Physical Resource Block

PIR	Packet Inter-Reception

PRR	Package Reception Ratio

ProSe	Proximity-based Services

PSCCH	Physical Sidelink Control CHannel

PSSCH	Physical Sidelink Shared CHannel

REFSENS	Reference Sensitivity

RF	Radio Frequency

SCS	Sub-Carrier Spacing

SINR	Signal to Interference plus Noise Ratio

SL	Sidelink

SNR	Signal-to-Noise Ratio

TDD	Time Division Duplex

TDM	Time Division Multiplexing

UE	User Equipment

UL	Uplink

V2V	Vehicle to Vehicle

V2X	Vehicle to Everything

VRU	Vulnerable Road User

============================ End of TP ============================