3GPP TSG-RAN WG4 Meeting # 101-bis-e 		                       R4-2202360

Electronic Meeting, January 17-25, 2022

Title: 	WF on switching time mask and sync issue for V2X intra-band con-current operation

Source: 	CATT

Agenda item:	6.15.3

Document for:	Approval

- Introduction

This WF will capture the agreements and potential open issues regarding switching time mask and sync issue for intra-band V2X con-current operation.

- Agreements in 1st round

In 1st round, the following agreements were made as below:

Issue 1-1-1: Clarification of same carrier and different carrier cases

Agreement:

  - Base on the agreements made in the last RAN4 meeting, RAN4 will define one time mask requirement for Case A and one time mask requirement for Case B and Case C

    - Case A: Same bandwidth with same carrier frequency

    - Case B: Different bandwidths with same carrier frequency

    - Case C:

      - Same bandwidth with different carrier frequency

      - Different bandwidth with different carrier frequency

  - Case A is considered as same carrier case while Case B and C are considered as different carrier case.

Issue 1-1-3: Whether to differentiate UL and DL if RAN4 agree on TA inclusion for switching time mask Agreement: No need to differentiate UL and DL if RAN4 agree on TA inclusion for switching time mask.

- Way forward

The way forwards for open issues are given as below:

Issue 1-1-2: Whether to include TA difference into switching time mask

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Further discussion is needed.

Issue 1-1-4: Switching time mask for same carrier case

- Proposals

  - Option 1: Consider the time mask for same carrier case in LGE paper R4-2200834.

  - Option 2: Consider the time mask for same carrier case in CATT paper R4-2200142.

  - Option 3: Consider the time mask for same carrier case in Xiaomi paper R4-2201502.

- Recommended WF

  - Further discuss whether to consider TA in time mask and then come back to detailed time mask.

    - Discuss both the real time TA and maximum/minimum TA of certain cell range for time mask requirement.

Issue 1-1-5: RF switching time and PSSCH/PUSCH preparation time

- Proposals

  - Option 1: Whether PSSCH/PUSCH preparation time can occur either sequentially or simultaneously with RF switching time can be up to UE implementation.

  - Option 2: The relative positioning of PSSCH/PUSCH preparation time and RF switching time should be considered in RRM session to define interruption requirements.

- Recommended WF

  - Whether to consider the relative positioning of PSSCH/PUSCH preparation time and RF switching time for interruption requirements can be left up to RRM session

Issue 1-1-6: Switching time mask for different carrier case

- Proposals

  - Option 1: Consider the time mask for different carrier case in LGE paper R4-2200834.

  - Option 2: Consider the time mask for different carrier case in CATT paper R4-2200142.

  - Option 3: Consider the time mask for different carrier case in Xiaomi paper R4-2201502.

- Recommended WF

  - Further discuss whether to consider TA in time mask and then come back to detailed time mask.

    - Discuss both the real time TA and maximum/minimum TA of certain cell range for time mask requirement.

    - Discuss whether a RF test for switching time is required

Issue 1-4-1: Capture note in TS or TR

- Proposals

  - Option 1: Capture the note for interference problem in TS 38.101-1

  - Option 2: Capture the note for interference problem in TR 38.785

- Recommended WF

  - Option 2 is agreeable based on majority of view.

- Reference

- R4-2202325, Email discussion summary for [101-e][125] NRSL_enh_Part_2, Moderator (CATT), RAN4#101-bis-e