3GPP TSG-RAN WG4 Meeting # 101-bis-e 													R4-2202598

Electronic Meeting, 17th – 25th January 2022

Agenda item:			6.10.2.2

Source:	vivo

Title:	WF on further RRM enhancement for NR and MR-DC – HO with PSCell

Document for:	Approval

# Introduction

This WF captured agreements during GTW session and email discussion. Open issues and candidate options after 2nd round discussion in email summary for thread “[101-bis-e][207] NR_RRM_enh2_2” for HO with PSCell are also captured.

The previous agreements on HO with PSCell can be found in [1-5].

# Agreements

## Sub-topic 2-2 Delay requirement design of HO with PSCell

Issue 2-2-2c-1: If both source PCell and source PSCell configured MOs which have the same SSB frequency and SCS as target PSCell, UE use the SMTC in the MO

- In HO with PSCell for NR-DC to NR-DC, if SMTC of target unknown PSCell is not configured in either targetcellSMTC-SCG-r16 or reconfigurationWithSync,

  - If both source PCell and source PSCell configured MOs which have the same SSB frequency and SCS as target PSCell, it is up to UE implementation which SMTC in the MOs are used.

- In HO with PSCell for EN-DC to EN-DC, if SMTC of target unknown PSCell is not configured in either targetcellSMTC-SCG-r16 or reconfigurationWithSync,

  - If both source PCell and source PSCell configured MOs which have the same SSB frequency and SCS as target PSCell, it is up to UE implementation which SMTC in the MOs are used.

Issue 2-2-8a: How the HO with PSCell delay requirements are specified

- For the parallel processing case of HO with PSCell, PSCell addition delay requirements and HO delay requirements are defined separately:

  - PSCell addition delay= TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms

  - HO delay = TRRC_delay +Tinterrupt = TRRC_delay +Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

- Note: Tprocessing can be further clarified base on outcome of Issue 2-2-3a and CR structure.

GTW Agreements.

- Agreement

  - Introduce extra margin Y ms for sequential processing case comparing to parallel processing case for UE SW processing and RF warm-up for [PCell handover] and PSCell addition/change

    - Y = [10] ms

    - Note: no extra interruption is required

## Sub-topic 2-4 Generic RACH assumption for HO with PSCell

Issue 2-4-1: 2 step and 4 step RACH for HO with PSCell

- No need to mention 2-step RA or 4-step RA in the requirement of HO with PSCell.

  - It can be revisited if issue is identified.

Issue 2-4-2: RACH occasion collision between Pcell and PSCell

- For RACH occasion collision between Pcell and PSCell,

  - For [EN-DC with target PSCell in FR1], adding clarification that additional uncertainty delay can be expected for PSCell RACH collision with PCell UL channels if the PSCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.1;

  - For [NE-DC with target PCell in FR1], adding clarification that additional uncertainty delay can be expected for PCell RACH collision with PSCell RACH if the PCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.1A;

  - Otherwise, if target PCell and target PSCell are on the different FRs for EN-DC or NR-DC, no need to consider RO collision issue.

Issue 2-4-4: CSI-RS based CFRA

- Do not consider CSI-RS based CFRA for handover with PSCell in this WI.

## Sub-topic 2-6 UE feature

Sub-topic description

Issue 2-6-1: UE feature group for HO with PSCell

- No dedicated UE capability to indicate whether UE can meet requirements.

  - R15/R16 UEs which support HO with PSCell are not required to meet R17 requirements for HO with PSCell.

# Open issues

## Sub-topic 2-2 Delay requirement design of HO with PSCell

Issue 2-2-3b: If UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in parallel

- Proposals

  - Option 1: (CATT, Apple, OPPO, ZTE, Huawei, Ericsson)

    - Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change)

  - Option 1a: (Qualcomm, vivo, Xiaomi, MTK, Apple, OPPO)

    - Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) + [X] ms.

    - X=5 (Qualcomm, Xiaomi, OPPO)

    - X=10 (MTK, Apple, Qualcomm)

    - X=FFS and can be different for different HO with PSCell scenarios (vivo)

  - Option 2a: (Intel, CMCC, [Nokia])

    - Tprocessing applies independently for PCell and PSCell.

    - FFS whether margin is needed

Issue 2-2-3d: Processing timeline for NR SA to EN-DC

- Proposals

  - Option 1 (vivo)

    - In R17, RAN4 will further discuss and conclude the feasibility and necessity of UE parallel processing in HO with PSCell from NR-SA to EN-DC for the case when network can provide SMTC of target unknown PSCell outside the container obtained from target E-UTRAN PCell, and if needed, send LS to RAN2 asking for the corresponding signalling design.

    - In R17, for HO with PSCell from NR-SA to EN-DC, RAN4 work on RRM requirements firstly assuming

    - parallel processing for the case when target PSCell is known, and

    - sequential processing for the case when SMTC of target unknown PSCell is provided to UE in the container obtained from target E-UTRAN PCell, and

    - parallel processing for the case when SMTC of target unknown PSCell is obtained by UE from the MOs of source PCell

  - Option 2 (Apple)

    - sequential processing is used for the case when SMTC of target unknown PSCell is provided to UE in the container obtained from target E-UTRAN PCell

    - otherwise, parallel processing is used (all the other cases shall assume parallel processing since UE doesn’t need to wait for new LTE PCell timing acquisition)

  - Option 3 (Qualcomm)

    - sequential processing shall be used

  - Option 4 (MTK)

    - If the SMTC of the target PSCell is configured in HO command:

    - UE follows the SMTC in the HO command, which, in this case, is based on the timing reference of target E-UTRAN PCell. Therefore, sequential processing should be performed to get the target PCell timing first.

    - If the SMTC of the target PSCell is not configured in HO command:

    - If UE is configured with source PCell MO:

      - UE follows the SMTC in this MO. Therefore, parallel processing can be performed.

    - If UE is not configured with source PCell MO:

      - UE assumes SSB has 5ms periodicity. Therefore, parallel processing can be performed.

  - Option 5 (Apple, Nokia)

    - Wait for RAN2 reply

## Sub-topic 2-5 Requirements for NR-U

Issue 2-5-1: Requirements for HO with PSCell for NR-U

- Proposals

  - Option 1 (CATT, MTK, Apple, Qualcomm, ZTE, Intel, vivo):

    - Postpone the requirement design of NR-U HO with PSCell until RAN4 completes the baseline requirement for HO with PSCell on licensed band.

  - Option 2 (Ericsson):

    - Requirements for HO with PSCell when PSCell is on NR-U shall be discussed in parallel with licensed carrier requirements and be specified in Rel-17

  - Option 3 (Apple, vivo):

    - Discuss requirements for NR-U in Rel-18.

# References

[1] R4-2120297	WF on further RRM enhancement for NR and MR-DC – HO with PSCell, vivo, RAN4 #101-e

[2] R4-2115337	WF on further RRM enhancement for NR and MR-DC – HO with PSCell, vivo, RAN4 #100-e

[3] R4-2108045	WF on further RRM enhancement for NR and MR-DC – HO with PSCell, vivo, RAN4 #99-e

[4] R4-2105787	WF on further RRM enhancement for NR and MR-DC – Handover with PSCell, Apple, RAN4#98bis-e

[5] R4-2103673	WF on further RRM enhancement for NR and MR-DC – Handover with PSCell, Apple, RAN4#98-e