3GPP TSG-RAN WG4 Meeting #101-e	R4-2117301

Electronic Meeting, Nov. 1 – Nov. 12, 2021

Agenda Item:	8.9.4.3

Source:	Apple

Title:	Discussion on the mobility requirement in FR2 HST RRM

Document for:	Discussion

# 1	Introduction

In Rel-17 NR support for high-speed train scenario in FR2 [1], potential RRM enhancement is proposed:

  - Study and identify RRM requirements impact and possible enhancement for

    - Idle/inactive mode cell reselection requirements enhancement

    - Connected mode requirements

      - Handover delay requirement

      - Measurement requirements including both L1 and SSB based L3 measurement

      - Beam management requirements including beam failure detection, candidate beam detection performance requirements

      - Other requirements if identified

[2,3] captures RRM related discussion in RAN4 100-e.

In this paper, we discuss the mobility requirement for idle/inactive state and connected state separately.

# 2	Discussion

Connected state mobility

Due to beam sweeping particularly for scenario B case, impact on mobility measurement is even higher. Therefore, for enhancement for short DRX cycle, enhancement like HST FR1 can be considered.

Way forward: Further consider the possibility of requirement enhancement for SMTC periodicity <= [40] ms - Option 1: M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1. - Option 2: Keep the 1.5 relaxation factor.

Connection Re-establishment to known cell

Way forward: FFS time to identify target NR cell for RRC connection re-establishment to known NR intra-frequency cell: - Option 1: MAX (TBD ms, 5 x N1 x TSMTC). - Option 3: Do not introduce any enhancements. - Other options are not precluded

Connection Re-establishment to unknown cell

Way forward: FFS time to identify target NR cell for RRC connection re-establishment to unknown NR intra-frequency cell: - Option 1: Keep current requirements with N1 scaling factor corresponding the number of RX beams: MAX (1000 ms, 10 x N1 x TSMTC) - Option 2: Do not introduce any enhancements. - Other options are not precluded

For RRC re-establishment procedure, the proposed enhancement for NR intra-cell identification is shown in Table I. N1 scaling factor can follow the general discussion of UE Rx beam. In this case, the FR2 known NR cell detection can be updated as shown in Table I.

For known NR cell, the requirement is proposed as MAX(400ms, 5 x N1 x TSMTC), where 400ms is the default value for FR2.

Table I: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | MAX(400ms, 5 x N1 x TSMTC) | MAX (1000 ms, 10 x N1 x TSMTC)) |
| < -8 | FR1 | N/A | 800 Note1 |
| < -8 | FR2 | N/A | 3520 Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

Proposal 1: M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1.

Proposal 2: Requirement to identify target NR cell for RRC connection re-establishment to known NR intra-frequency cell MAX (400 ms, 5 x N1 x TSMTC).

Proposal 3: Requirement to identify target NR cell for RRC connection re-establishment to unknown NR intra-frequency cell MAX (1000 ms, 10 x N1 x TSMTC).

Handover

For handover, the known cell for FR2 has been agreed in RAN4 100e meeting. It is straightforward to enhance scaling factor in the handover requirement based on the Rx beam discussion.

Proposal 4: Handover requirement can be enhanced depends on the number of Rx beam in general RRM requirement discussion.

Idle/inactive state mobility

In FR1 HST enhancement, reduced number of samples combined with additional parameter M2, M3 and M4 are introduced in DRX cycle < 320ms. When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2. Similar approach can be introduced in FR2 HST, as shown in the following table, where FR2 scaling factor N1 is introduced on top of FR1 HST table.

Table II: Tdetect,NR, TmeasureNR, and Tevaluate,NR when UE is configured with RRM enhancement for high speed

| DRX cycle length [s] | Scaling Factor (N1) | Tdetect,NR [s] (number of DRX cycles) | Tmeasure,NR [s] (number of DRX cycles) | Tevaluate,NR[s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
| 0.32 | TBD | 2.56 x M2 x N1 (8 x M2 x N1) | 0.32 x M3 x N1 (1 x M3 x N1) | 0.96 x M4 x N1 (3 x M4 x N1) |
| 0.64 | TBD | 5.12 x N1 (8 x N1) | 0.64 x N1 (1 x N1) | 1.92 x N1 (3x N1) |
| 1.28 | TBD | 8.96 x N1 (7 x N1) | 1.28 x N1 (1 x N1) | 3.84 x N1 (3 x N1) |
| 2.56 | TBD | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| NOTE 1: When SMTC <=40ms, M2=M3=M4=1; and when SMTC>40ms, M2=1.5, M3=M4=2 |  |  |  |  |

The value of scaling factor N1 should reuse the agreement on number of UE Rx beams in different deployment scenario, discussed in separate agenda item.

Proposal 5: For idle/inactive mode cell reselection enhancement, Use 320 ms DRX cycle as baseline for following analysis and requirement definition. The scaling factor N1 can reuse the general Rx beam agreement.

# 3	Summary

In this paper, we provide our view on mobility aspect of RRM enhancement for HST FR2.

Proposal 1: M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1.

Proposal 2: Requirement to identify target NR cell for RRC connection re-establishment to known NR intra-frequency cell MAX (400 ms, 5 x N1 x TSMTC).

Proposal 3: Requirement to identify target NR cell for RRC connection re-establishment to unknown NR intra-frequency cell MAX (1000 ms, 10 x N1 x TSMTC).

Proposal 4: Handover requirement can be enhanced depends on the number of Rx beam in general RRM requirement discussion.

Proposal 5: For idle/inactive mode cell reselection enhancement, Use 320 ms DRX cycle as baseline for following analysis and requirement definition. The scaling factor N1 can reuse the general Rx beam agreement.

# References

- RP-202118, New WID on NR support for high-speed train scenario in FR2, Samsung, Nokia, Sep 2020

- R4-2115334, Way forward for FR2 HST RRM (part 1), Nokia, Aug 2021

- R4-2115335, Way forward for FR2 HST RRM part 2, Samsung, Aug 2021