3GPP TSG-RAN4 Meeting #101-e	                                                         R4-2117343

Electronic meeting, Nov. 1 – 12, 2021

Title:	Discussion on UE positioning measurement requirements

Source:	CATT

Agenda Item:	5.1.4.1

Document for:	Discussion

1 Introduction

In RAN4 #100-e meeting, the remaining issues on the measurement period of UE positioning measurement were further discussed in the core part maintenance. The conclusions were captured in the approved WF [1] and there are still some open issues to be discussed in this meeting.

In this paper, we have some further discussions on the remaining issues and give our proposals.

2 Discussion

2.1 SRS/PRS proximity

In last meeting, the SRS/PRS proximity has been agreed but the UE behavior and requirements when the proximity condition is not met was not concluded. The candidate options are captured in [1] as below.

Candidate options: - Proposals for UE behaviour - Option 1 (CATT, OPPO, vivo, Nokia, HW) - The UE should still measure and report UE Rx-Tx measurement even if PRS/SRS proximity condition is not met - Option 2 (QC, Ericsson, Intel) - Up to UE implementation - Proposals for requirements - Option 1 (CATT, OPPO, Nokia, HW) - UE Rx-Tx measurement period requirements apply, but the UE Rx-Tx accuracy requirements may not apply - Option 2 (vivo, HW) - Both UE Rx-Tx measurement period and measurement accuracy requirements apply - A note may be needed that positioning accuracy may be degraded when PRS/SRS proximity condition is not met. - Option 3 (QC, Ericsson, Intel) - UE Rx-Tx time difference measurement requirements do not apply.

If the PRS/SRS proximity condition is not met, we think UE can still measure and report the measurement within the measurement period, so the measurement period requirements shall still apply. But the accuracy requirements may be not met due to lack of SRS resources. To move forward, we can also accept option 2 for requirements, since the accuracy requirements are possibly met depending on the UE implementation.

Proposal 1: If the PRS/SRS proximity condition is not met, UE can still measure and report the UE Rx-Tx time difference within the measurement period, and the measurement period requirements shall apply.

Proposal 2: If the PRS/SRS proximity condition is not met, the accuracy requirements are not applied or the accuracy requirements apply with adding a note that the accuracy may be degraded.

2.2 Measurement period with cell change

Issue 4-3-1: Measurement period requirements with cell change impacting SRS Candidate options: - Option 1 (CATT, OPPO, QC, HW, Ericsson, Nokia) - UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete. - Option 2 (vivo) - UE shall continue the on-going UE Rx-Tx time difference measurement, and the current measurement period and accuracy apply. Issue 4-3-2: Measurement period requirements with cell change not impacting SRS Candidate options: - Option 1a (CATT, HW) - No clarification is needed in core requirements in the specification - Option 1b (vivo, HW, CATT, Nokia) - UE shall continue the on-going UE Rx-Tx time difference measurement and the current measurement period and accuracy apply. - Option 2a (OPPO, QC, Intel, Ericsson, Nokia) - UE shall continue the on-going UE Rx-Tx time difference measurement, and longer measurement period is expected. - Option 2b (Ericsson) - UE shall continue the on-going UE Rx-Tx time difference measurement, and longer measurement period is expected. - In this case UE Rx-Tx time difference measurement period (TUERxTx,total,HO) shall be as follows: TUERxTx,total,HO = TUERxTx,Total + K*Teffect + THO TUERxTx,Total is the UE Rx-Tx time difference measurement period defined in section 9.9.4.5, TS 38.133. K is the number of times the handover occurs during TUERxTx,total,HO; Teffect is the largest Teffect,i among all the positioning frequency layers; THO THO is the time during which the UE Rx-Tx time difference measurement may not be possible due to handover; it can be up to Tinterrupt as defined in clause 6.1 of TS 38.133.

For the case of cell change impacting SRS, it is the same as handover. And it was specified in the current specification that the UE Rx-Tx time difference measurement period is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete. There is no need to define the situation redundantly.

For the case of cell change not impacting SRS, it is the same as non-HO serving cell change. It has been agreed that the UE shall continue and complete a UE Rx-Tx measurement while meeting UE Rx-Tx measurement accuracy requirements in clause 10.1.23, when a non-HO serving cell change occurs during the measurement, provided the cell change does not impact the configuration of the SRS used for the measurement. Since the UE behaviour has no difference with general cases, we think there is no need to capture it in the core requirements.

Proposal 3: When cell change impacting SRS occurs during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

Proposal 4: For the case of cell change not impacting SRS, UE shall continue the on-going UE Rx-Tx time difference measurement and the current measurement period and accuracy apply.

3 Summary

In this paper, the requirements of  UE positioning measurement are further discussed, and following proposals are given：

Proposal 1: If the PRS/SRS proximity condition is not met, UE can still measure and report the UE Rx-Tx time difference within the measurement period, and the measurement period requirements shall apply.

Proposal 2: If the PRS/SRS proximity condition is not met, the accuracy requirements are not applied or the accuracy requirements apply with adding a note that the accuracy may be degraded.

Proposal 3: When cell change impacting SRS occurs during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

Proposal 4: For the case of cell change not impacting SRS, UE shall continue the on-going UE Rx-Tx time difference measurement and the current measurement period and accuracy apply.

4 References

- R4-2115301	WF on UE PRS measurement requirements, Huawei