3GPP TSG-RAN WG4 Meeting #101-e                                                            R4-2117395

Electronic Meeting, November 1 - 12, 2021

Source: 	CATT

Title: 	Discussion on sweep time for unwanted emission testing

Agenda item:	4.1.4.1

Document for:	Approval

- Introduction

For unwanted emission testing, true RMS detection mode of measurement device is defined. While the sweep time for true RMS is not clarified in current specification.

In this contribution, we provide our views on general rule for sweep time setting.

- Discussion

In 3GPP BS RF specifications, for unwanted emission testing, such as ACLR, OBUE and spurious emissions, the detection mode is true RMS (Root Mean Square).  For example, True RMS is required for detection mode in the procedure for OBUE testing in section 6.6.4.42 in TS 38.141-1 shown in the following table.

##### 6.6.4.4.2 Procedure For BS type 1-H where there may be multiple TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.3.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested. 1) Connect the single-band connector or multi-band connector under test to measurement equipment as shown in annex D.1.1 for BS type 1-C or in annex D.3.1 for BS type 1-H. All connectors not under test shall be terminated. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity, efficiency and avoiding e.g. carrier leakage, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. The measurement device characteristics shall be: - Detection mode: True RMS.

As seen from above the procedure, a general rule for resolution bandwidth (RBW) was clarified, which indicates the RBW setting will affect the RMS measurement result. From our experience in measurement device setting, there are other settings will affect the RMS measurement result, such as sweep points for a frequency span, or sweep time for a frequency span. But the sweep time (RMS average duration) is not defined in the current specification.

For NR/LTE system using OFDM waveform which has high PAPR, the sweep time will impact the measurements results significantly. When the sweep time is too short, the result is more like the peak value rather than an average value. Usually the default sweep time (auto mode) of TE is setting from shortening test time perspective, since the TE does not have the idea of signal characteristics. It is an unnecessary risk to fail good equipment due to inappropriate seeping time setting. To capture specific signal characteristics, the sweep time should be carefully configured. For auto mode, if 1 GHz frequency span and 1MHz RBW are setting, the default sweep time[1] equals k*span/(RBW*RBW)=2*1e9/(1e6*1e6) =2ms, where k=2. For 15 kHz SCS, the one OFDM symbol length is 1/14ms = 71.4us. If measurement device collects 1001 sweep points, this means that the data in each point is collected at a rate of 1.998 us/point (2ms/1001 points), which is far smaller than one OFDM symbol length 71.4us. So for such little data in each point, it is impossible to average the impact of PAPR. We think the he data in each point should be collected at a rate of one OFDM symbol length/point.

Observation 1: Sweep time has critical impact for some emission test. However there is no definition on the sweep time for the concerning test in the current specification.

In FCC [1], there are some clarifications for sweep time configuration as following.

- If the device can be configured to transmit continuously (duty cycle ≥ 98%), set the (sweep time) > (number of points in sweep) × (symbol period) (e.g., by a factor of 10 × symbol period × number of points). Increasing the sweep time (i.e., slowing the sweep speed) will allow for averaging over multiple symbols - If the device cannot transmit continuously (duty cycle < 98%), a gated sweep shall be used when possible (i.e., gate triggered such that the analyzer only sweeps when the device is transmitting at full power), set the sweep time > (number of points in sweep) × (symbol period) but the sweep time shall always be maintained at a value that is less than or equal to the minimum transmission time. - If the device cannot be configured to transmit continuously (duty cycle < 98%) and a free-running sweep must be used, set the sweep time so that the averaging is performed over multiple on/off cycles by setting the sweep time > (number of points in sweep) × (transmitter period) (i.e., the transmit on-time + the off-time). The spectrum analyzer readings shall subsequently be corrected by [10 log (1/duty cycle)]. This assumes that the transmission period and duty cycle is relatively constant (duty cycle variation ≤ ±2%). - If the device cannot be configured to transmit continuously and a free-running sweep must be used, and if the transmissions exhibit a non-constant duty cycle (duty cycle variations > ±2%), set the sweep time so that the averaging is performed over the on-period by setting the sweep time > (symbol period) × (number of points), while also maintaining the sweep time < (transmitter on-time). The trace mode shall be set to max hold, since not every display point will be averaged only over just the on-time. Thus, multiple sweeps (e.g., 100) in maximum hold are necessary to ensure that the maximum power is measured.

In ECC recommendation 7401[2] “Unwanted emissions in the spurious domain”, recommends 6 in page 6 states as following.

6. that the appropriate spurious domain emission limits should be applicable to all services/equipment as detailed by Table 2. Unless the Peak Envelope Power (PEP) is explicitly quoted in this Recommendation the spurious domain emission limits specified from the transmitter into the antenna port are in terms of mean power. The mean power (P) of any spurious domain transmission from a burst transmitter is the mean power averaged over the burst duration;

During the discussion on LTE V2X UE co-existence spurious emission requirements [4], RAN4 reached the reached the agreement on the sweep time setting and RBW to improve measurement accuracy. The following was captured in TS 36.101 and TS 38.101.

NOTE 45:	Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set at least as (sweep points)*(symbol length) to improve the measurement accuracy.

Based on the discussion above, we propose to include the clarification on sweep time for RMS detection mode in the general part of unwanted emission requirements.

Proposal 1: it is proposed to include the following clarification on sweep time for true RMS detection mode in the general part of unwanted emission requirements.

Unless otherwise stated, for true RMS detection mode the sweep time shall be set at least as (sweep points)*(symbol length) to improve the measurement accuracy.

- Conclusion

This contribution provided the analysis on the general rules on sweep time for unwanted emissions testing. The following observations and proposals are concluded:

Observation 1: Sweep time has critical impact for some emission test. However there is no definition on the sweep time for the concerning test in the current specification.

Proposal 1: it is proposed to include the clarification on sweep time for true RMS detection mode in the general part of unwanted emission requirements.

Unless otherwise stated, for true RMS detection mode the sweep time shall be set at least as (sweep points)*(symbol length) to improve the measurement accuracy.

The corresponding draft CRs can be found in [5] and [6].

- References

- AgilentApplicationNote150.pdf, Agilent Spectrum Analysis Basics Application Note 150, Agilent

- ANSI C63.26-2015, FCC

- ERC Recommendation 74-01, Unwanted emissions in the spurious domain, ECC

- R4-1912424, LS on Sweep time for LTE V2X UE co-existence spurious emission, RAN4

- R4-2117396, Draft CR for TS 38.141-1 On sweep time for unwanted emission testing (Rel-15), CATT, RAN4#101-e

- R4-2117399, Draft CR for TS 38.141-2 On sweep time for unwanted emission testing (Rel-15), CATT, RAN4#101-e