3GPP TSG-RAN WG4 Meeting # 101-e                  	R4-2119010

# Electronic Meeting, November 01-12, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Impact on NR positioning RRM requirements

Agenda item:	8.21.2.4

Document for:	Discussion

# Introduction

In Rel-17, there are various positioning features to improve positioning accuracy, latency and resource efficiency as discussed in WF [2,3]. In this contribution, we discuss other features for NR positioning enhancements that has been discussed in RAN1/RAN2. Also, we additionally discuss RAN1 LS [4] on the first path PRS-RSRP requirements.

# Discussion

### 2.1	First path PRS-RSRP requirements (LS R1-2110627)

RAN1 has introduced first path PRS-RSRP measurement. It is a new measurement additional to other measurements like AoA or ToA to improve positioning estimation accuracy. A basic UE behavior is to read the power at the channel path where a UE detects the first time tap. The RX measuring DL-ToA can be reused for the first path detection, the additional discussion on UE behavior to read the first path PRS-RSRP is needed.

RAN4#100 Agreement on first path PRS-RSRP Wait for RAN1 outcome FFS: Requirements for first path PRS-RSRP and their feasibility RAN1#106b Agreement: For both UE-based and UE-assisted DL-AOD, the UE can be requested subject to UE capability to measure and report (for UE-assisted) the PRS RSRP of the first path - FFS: Details of measurement and reporting of PRS RSRP of the first path RAN1#106b Agreement: The measured path DL PRS RSRP for ith path delay is defined as the power of the received DL PRS signal configured for the measurement at the ith path delay of the channel response, and · path DL PRS RSRP for 1st path delay is the power corresponding to the first detected path · FFS: Whether the path RSRP measurement is normalized with PRS RSRP. · FFS: Whether the definition of the ith path delay (other than i=1) is required. · Note: UE may choose to use a time window to compute path DL PRS RSRP by UE implementation (there is no impact to specifications managed by RAN1 for this) · Note: This does not imply that the path delay has to be reported in DL-AoD positioning · Send LS to RAN4 to check the details of the definition and feedback if they identify any update is necessary.

Observation 1 : DL PRS RSRP for 1st path delay is the power corresponding to the first detected path. The RX measuring DL-ToA can be reused for the first path detection.

Proposal 1 : Additional behaviors of the first path RSRP measurements need to be studied to conduct performance study such as normalization by PRS-RSRP and time window. RAN4 further discuss how to define the parameters.

Proposal 2 : We prefer to focus on discussion about the first path RSRP requirement. Preclude requirement study on the ith path delay (other than i=1) as RAN#100 agreement (no requirements impact due to multipath/NLOS mitigation)

### 2.2	Positioning based on on-demand PRS

RAN4#100 Agreement on on-demand PRS Wait for agreements in other WGs FFS: RRM requirements impact due to on-demand PRS RAN1#106b Agreement: The following list of parameters is supported for UE-initiated and LMF initiated on-demand DL PRS request 1. Start/end time of DL PRS transmission 2. DL PRS resource repetition factor 3. Number of DL PRS resource symbols per DL PRS resource 4. DL-PRS CombSizeN 5. Number of DL PRS frequency layers 6. ON/OFF indicator (for LMF initiated request only) · FFS values for requested on-demand DL PRS parameters and whether parameters are resource-specific, TRP-specific, or PFL-specific

RAN1 had further discussion in on-demand PRS introduction, there are not much progress on the details of UE behaviors corresponding to on-demand PRS. We find that largely two behaviors are assumed in RAN1, each makes different requirement impacts.

Behavior 1 : On-demand PRS is switching ON/OFF of Rel-16 PRS up to an initiated request. This is similar to DL-PRS muting, but its switching is dynamic or semi-persistent up to on-demand.

Behavior 2 : On-demand PRS is dynamically or semi-persistently requested in resource-specific, TRP-specific, or PFL-specific manners

For behavior-1 case, the behavior will be very similar to the Rel-16 UE, the requirements may consider additionally the aperiodic DL-PRS switching on/off. We assume MG-based measurement is still valid for this case, RAN4 may try revisit the Rel-16 behaviors to make it applicable to Rel-17 on-demand PRS.

For behavior-2 case, this will turn out new behaviors. PRS transmission will be resource-specific out of MG period, and the accuracy will vary depending on resource configuration. RAN4 needs a new study for this case.

Observation 2 : RAN1 assumes largely two types of behaviors for on-demand PRS, each makes different requirement impacts.

Behavior 1 : On-demand PRS is switching ON/OFF of Rel-16 PRS up to an initiated request. This is similar to DL-PRS muting, but its switching is dynamic or semi-persistent up to on-demand.

Behavior 2 : On-demand PRS is dynamically or semi-persistently requested in resource-specific, TRP-specific, or PFL-specific manners

Proposal 3 : RAN4 starts to discuss how to handle performance requirements of the case that on-demand PRS is switching ON/OFF of Rel-16 PRS. RAN4 needs further information on on-demand PRS configuration.

### 2.3  SRS antenna switching

RAN4#100 agreemenet FFS: the impact of SRS antenna port switching impact on UE Rx-Tx measurement FFS: solution depends on the impact FFS: the impact of SRS antenna port switching impact on gNB Rx-Tx measurement and ULRTOA FFS: solution depends on the impact

We need to understand the issue more. First, we wonder if SRS antenna port needs to be switched for positioning measurement purpose. Only a single SRS antenna port is used for positioning measurements, so positioning SRS port control is different from sounding SRS purpose.

If SRS antenna port switches during the UE Rx-Tx timing measurement period, it impacts on accuracy, since SRS antenna port has a unique CS rotation, CS rotation makes different timing in an OFDM symbol. However, we don’t have clear understanding why SRS antenna port needs to be switched for positioning. In addition, positioning measurements is not supposed to be impacted by physical TX antenna switching due to RF circumstance like hand grip etc. Once a SRS antenna port is configured, one of UE physical TX antennas can transmit the configured SRS sequence without SRS antenna port switching.

Proposal 4 : A benefit of SRS antenna port switching for positioning purpose is unclear. Clarify if SRS antenna port needs to be switched for positioning measurement purpose.

# Conclusion

This contribution has discussed problems and possible approaches for NR positioning enhancement in Rel-17.

First path PRS-RSRP requirements (LS R1-2110627)

Observation 1 : DL PRS RSRP for 1st path delay is the power corresponding to the first detected path. The RX measuring DL-ToA can be reused for the first path detection.

Proposal 1 : Additional behaviors of the first path RSRP measurements need to be studied to conduct performance study such as normalization by PRS-RSRP and time window. RAN4 further discuss how to handle the parameters.

Proposal 2 : We prefer to focus on discussion about the first path RSRP requirement. Preclude requirement study on the ith path delay (other than i=1) as RAN#100 agreement (no requirements impact due to multipath/NLOS mitigation)

Positioning based on on-demand PRS

Observation 2 : RAN1 assumes largely two types of behaviors for on-demand PRS, each makes different requirement impacts.

Behavior 1 : On-demand PRS is switching ON/OFF of Rel-16 PRS up to an initiated request. This is similar to DL-PRS muting, but its switching is dynamic or semi-persistent up to on-demand.

Behavior 2 : On-demand PRS is dynamically or semi-persistently requested in resource-specific, TRP-specific, or PFL-specific manners

Proposal 3 : RAN4 starts to discuss how to handle performance requirements of the case that on-demand PRS is switching ON/OFF of Rel-16 PRS. RAN4 needs further information on on-demand PRS configuration.

SRS antenna switching

Proposal 4 : A benefit of SRS antenna port switching for positioning purpose is unclear. Clarify if SRS antenna port needs to be switched for positioning measurement purpose.

# References

[1] RP-210903, Revised WID on NR Positioning Enhancements, CATT, Intel Corporation, Ericsson

[2] R4-2115365 Way forward document for [100-e][236] NR_pos_enh_RRM_1, RAN4#100

[3] R4-2115367 WF on Rel-17 positioning enhancements RRM_2, RAN4#100

[4] R1-2110627  LS on definition of DL PRS path RSRP, RAN1#106b

[5] R1-2108639 LS on PRS measurement outside the measurement gap, RAN1#106