3GPP TSG-RAN WG4 Meeting # 101e	R4-2119441

# Electronic Meeting, Nov. 1-12, 2021

Source: 	Skyworks Solutions, Inc.

Title: 	Handling of maximum output power limit for PC2 inter-band combinations

Agenda Item:	7.29.2 UE RF requirements [NR_PC2_CA_R17_2BDL_2BUL-Core]

7.30.2 UE RF requirements [ENDC_UE_PC2_R17_NR_TDD -Core]

Document for:	Approval

# Introduction

In the previous RAN4 meetings, the group addressed allowing the power sum of the two CC to exceed the inter-band PC2 maximum power definition, if at least one of the bands is PC2. In this contribution, we clarify the implications of this and discuss the related requirement impacts.

# Discussion

## Possible PC2 inter-band Combinations

First, we want to clarify that this discussion is only for inter-band combinations: ENDC, NRCA, NEDC or NRDC, but not for intra-band combinations, since these relate to the band power class only.

Table 1, maps all the feasible PC2 inter-band combinations in Release 17, based on each of the band’s power class capability. The resulting peak power for each band, at maximum nominal power or extreme power, is provided and then compared with the PC2 maximum output power. The cases highlighted in green illustrate that the power remains within the current PC2 inter-band power class range.

Table 1: possible per band power class combinations in an inter-band PC2 combination

|  | maximum nominal power [dBm] |  |  | extreme power limit [dBm] |  |  | difference to PC2 inter-band [dB] | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| use case | Band1 | Band2 | Peak | Band1 | Band2 | Peak |  |  |
| - FDD/TDD PC3 + NR-U PC5 | 23 | 20 | 25 | 25 | 22 | 27 | -1 | Within the range of the PC2 maximum power no issue |
| - FDD/TDD PC3 + FDD/TDD PC3 | 23 | 23 | 26 | 25 | 25 | 28 | 0 |  |
| - FDD/TDD PC3 + TDD PC2 | 23 | 26 | 28 | 25 | 28 | 30 | 2 | Exceeds the range of the PC2 maximum power |
| - TDD PC2 + NR-U PC5 | 26 | 20 | 27 | 28 | 22 | 29 | 1 |  |
| - TDD PC2 + TDD PC2 | 26 | 26 | 29 | 28 | 28 | 31 | 3 | Should be for PC1.5 inter-band only thus not for Release 17 |
| - FDD/TDD PC3 + TDD PC1.5 | 23 | 29 | 30 | 25 | 31 | 32 | 4 |  |
| - TDD PC2 + TDD PC1.5 | 26 | 29 | 31 | 28 | 31 | 33 | 5 |  |
| - TDD PC1.5 + TDD PC1.5 | 29 | 29 | 32 | 31 | 31 | 34 | 6 |  |

From Table 1, several questions arise:

- Are NR PC3 + NR-U PC5 band combinations eligible for PC2 inter-band combination? The combined per-band power range fits in the +2/-3dB range.

- Should all combinations where the nominal power is 29dBm or above belong to a PC1.5 inter-band combination only, and thus not be allowed in Release 17?

- When we are discussing of allowing the peak power to exceed the power class maximum, is this for the combined nominal or extreme power?

To simplify the discussion and assess the requirements that may be affected, we believe it is essential that RAN4 achieves a common understanding of the above questions to recognise which combinations are already supported, in terms of power class definition, MSD, and duty cycle reporting, and which combinations requires one or the other aspects to be revised to be supported. Essentially, the objective is to determine which combination is optional, which combination is not, and which combination is not supported in Release 17.

In order to achieve this, we advance the following proposal for the scope of the discussion for the remainder of Release 17.

Proposal:

- NR PC3+PC3 case is de facto supported by PC2 inter-band.

- NR PC3 + NR-U PC5 inter-band combinations (1) are eligible for PC2 inter-band combinations and do not require revision of the requirements.

- Any combinations resulting in a peak nominal power equal to or exceeding 29dBm (5 to 8) can only be optionally supported with inter-band PC2 combinations with a maximum peak power of 30dBm which already applies for a PC3+PC2 case (3).

- Optionally, the maximum peak power is based on the sum of the defined per-band power class if it exceeds the PC2 power class range, but no more than 2dB (i.e. 30dBm) ( 3) and (4)

- The performance can be understood from each band power class and the 30dBm max peak power, but signalling may still be necessary, depending on the impact on requirements.

Assuming an agreement on the above proposal, it should be feasible to design:

- The PC2 inter-band PCmax equations for mandatory and optional support

- Additional MSD when max peak power exceeds 28dBm if required

- An interpretation of the max duty cycle UE declaration. For example, whether a UE reaching a nominal 29dBm peak power uses a default 25% or 50% overall duty cycle.

# Conclusions

In this contribution, we look into all the potential PC2 inter-band combination use cases in terms of individual band power class and make proposals to achieve a common understanding on the Release 17 scope for exceeding the current maximum power level of 28dBm.

Proposal:

- NR PC3+PC3 case is de facto supported by PC2 inter-band.

- NR PC3 + NR-U PC5 inter-band combinations (1) are eligible for PC2 inter-band combinations and do not require revision of the requirements.

- Any combinations resulting in a peak nominal power equal to or exceeding 29dBm (5 to 8) can only be optionally supported with inter-band PC2 combinations with a maximum peak power of 30dBm which already applies for a PC3+PC2 case (3).

- Optionally, the maximum peak power is based on the sum of the defined per-band power class if it exceeds the PC2 power class range, but no more than 2dB (i.e. 30dBm) ( 3) and (4)

- The performance can be understood from each band power class and the 30dBm max peak power, but signalling may still be necessary, depending on the impact on requirements.

# References

- R4-2111766 A way to increase UE maximum power for NR uplink inter band CA, Nokia, Nokia Shanghai Bell, RAN4#100e