3GPP TSG-RAN WG4 Meeting #101-e	               R4-2120083

E-meeting, 1st -12th Nov, 2021

Agenda item:	2

Source:	RAN4 Vice Chair (Samsung)

Title:	RAN4#101-e BSRF_Test_Demod Session Chair notes

Document for:	Information

Contents:

4	Rel-15 and previous release maintenance for LTE and NR	5

4.1	NR WIs (up to Rel-15)	5

4.1.2	UE EMC requirements	5

4.1.3	BS RF requirements	5

4.1.3.1	General	5

4.1.3.2	TX/RX requirements (38.104)	17

4.1.3.3	MSR specifications	21

4.1.4	BS conformance testing	30

4.1.4.1	General	30

4.1.4.2	Conducted conformance testing (38.141-1)	32

4.1.4.3	Radiated conformance testing (38.141-2)	32

4.1.4.4	eAAS specifications	37

4.1.5	BS EMC requirements	39

4.1.8	Demodulation and CSI requirements (38.101-4/38.104)	42

4.1.8.1	UE demodulation requirements	42

4.1.8.2	CSI requirements	44

4.1.8.3	BS demodulation requirements	45

4.1.10	Testability (38.810)	50

4.2	LTE WIs (up to Rel-15)	50

4.2.1 UE RF requirements	50

4.2.2	BS RF requirements	52

4.2.4	Demodulation performance requirements	57

4.2.4.1	UE demodulation and CSI requirements	57

4.2.4.2	BS demodulation requirements	58

5	Rel-16 maintenance for LTE and NR	60

5.1	NR WIs and TEI	60

5.1.1	NR-based access to unlicensed spectrum	60

5.1.1.3	BS RF and conformance testing	60

5.1.1.6	Demodulation and CSI requirements	61

5.1.1.6.1	General	61

5.1.1.6.2	UE demodulation and CSI requirements	61

5.1.1.6.3	BS demodulation requirements	61

5.1.2	Integrated Access and Backhaul for NR	62

5.1.2.1	RF requirements	63

5.1.2.2	RF conformance testing	64

5.1.2.2.1	General	64

5.1.2.2.2	Conducted conformance testing	64

5.1.2.2.3	Radiated conformance testing	65

5.1.2.4	EMC performance requirements	66

5.1.2.5	Demodulation and CSI requirements	66

5.1.2.5.1	General	66

5.1.2.5.2	IAB-DU performance requirements	66

5.1.2.5.3	IAB-MT performance requirements	67

5.1.3	Enhancements on MIMO for NR	67

5.1.3.2	Demodulation performance requirements	67

5.1.6	Other NR WIs and Rel-16 NR TEI	67

5.1.6.1	BS RF requirements	67

5.1.6.4	Demodulation and CSI requirements	68

5.1.6.4.1	UE demodulation requirements	68

5.1.6.4.2	CSI requirements	69

5.1.6.4.3	BS demodulation requirements	69

5.1.6.5	NR MIMO OTA test methods (38.827)	74

5.2	LTE WIs and TEI	74

5.2.1	BS RF requirements	74

5.2.4	Demodulation and CSI requirements	74

5.4	LS response for WP5D (RP-210747) on recommendations ITU-R M.2070 and ITU -R M.2071 on Unwanted Emissions of IMT-Advanced	74

6	Rel-17 maintenance for LTE and NR	77

6.1	Introduction of NR 47 GHz band	77

6.1.2	BS RF requirements and conformance (38.104/38.141-2)	77

6.1.4	Demodulation and CSI requirements	77

6.2	Introduction of channel bandwidths 35MHz and 45MHz for NR	78

6.2.2	BS RF requirements	78

6.2.4	UE demodulation and CSI requirements	78

6.8	Other WIs and Rel-17 TEI	78

6.8.1	BS RF requirements	78

6.8.4	Demodulation and CSI requirements	79

7	Rel-17 spectrum related WIs for NR	79

7.3	Introduction of 900 MHz spectrum to 5G NR applicable for Rail Mobile Radio	79

7.3.3	BS RF requirements	79

7.4	Introduction of 1900 MHz spectrum to 5G NR applicable for Rail Mobile Radio	81

7.4.3	BS RF requirements	81

7.36	Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n259	82

7.36.3	UE demodulation	82

8	Rel-17 non-spectrum related work items for NR	83

8.1	Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs	83

8.1.1	General	83

8.1.2	Performance requirements	85

8.1.2.1	Performance Requirements for FR1	85

8.1.2.2	Performance Requirements for FR2	86

8.1.2.3	MU assessment for FR1 and FR2	86

8.1.3	Testing methodologies	87

8.1.3.1	Testing parameters for Performance	87

8.1.3.2	Optimization of test methodologies	87

8.1.3.3	Channel model validation	87

8.2	Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)	88

8.2.1	General and work plan	88

8.2.2	SA test methodology	91

8.2.3	EN-DC test methodology	91

8.2.4	UE with multiple antennas test methodology	92

8.2.5	Others	94

8.5	NR repeater	94

8.5.1	General	94

8.5.1.1	System parameters	97

8.5.1.2	Repeater Class/Type	97

8.5.1.3	TDD repeater switching requirements	98

8.5.1.4	Others	99

8.5.2	Conductive RF core requirements	100

8.5.2.1	Transmitted power related requirements	104

8.5.2.2	Emission requirements	105

8.5.2.3	Others	106

8.5.3	Radiated RF core requirements	107

8.5.3.1	Transmitted power related requirements	108

8.5.3.2	Emission requirements	109

8.5.3.3	Others	110

8.5.4	EMC core requirements	110

8.6	Introduction of DL 1024QAM for NR FR1	112

8.6.1	General	112

8.6.2	UE RF requirements	114

8.6.3	BS TX RF requirements	115

8.6.4	BS RF conformance testing	116

8.6.5	Demodulation and CSI requirements	117

8.6.5.1	General	117

8.6.5.2	PDSCH requirements	118

8.6.5.3	CQI requirements	118

8.8	Enhancement for NR high speed train scenario in FR1	119

8.8.3	UE demodulation requirements (38.101-4)	119

8.8.3.2	PDSCH requirements for CA scenarios	121

8.9	NR support for high speed train scenario in FR2	123

8.9.2	High speed train deployment scenario in FR2	123

8.9.2.1	Deployment Scenario-A	124

8.9.2.2	Deployment Scenario-B	124

8.9.2.3	Others	125

8.9.5	Demodulation requirements	125

8.9.5.1	General	125

8.9.5.2	UE demodulation requirements	130

8.9.5.3	BS demodulation requirements	131

8.9.5.3.1	PUSCH requirements	131

8.9.5.3.2	PUSCH with UL timing adjustment requirements	132

8.9.5.3.3	PRACH requirements	132

8.12	Further enhancement on NR demodulation performance	133

8.12.1	General	133

8.12.2	UE demodulation and CSI requirements	138

8.12.2.1	MMSE-IRC receiver for inter-cell interference	138

8.12.2.1.1	PDSCH requirements	143

8.12.2.1.2	CSI requirements	145

8.12.2.2	MMSE-IRC receiver for intra-cell inter-user interference	146

8.12.2.3	CRS-IM receiver in scenarios with overlapping spectrum for LTE and NR	150

8.12.2.3.1	Receiver assumption	151

8.12.2.3.2	Necessity of Network assistant signaling	151

8.12.2.3.3	Test set-up	152

8.12.3	BS demodulation requirements	153

8.12.3.1	PUSCH demodulation requirements for FR1 256QAM	154

8.13	Solutions for NR to support non-terrestrial networks (NTN)	156

8.13.1	General	156

8.13.1.1	System parameters	160

8.13.1.2	NTN gNB Class/Type	161

8.13.1.3	Regulatory information	162

8.13.1.4	Others	163

8.13.2	Coexistence aspects	163

8.13.2.1	NTN coexistence scenarios and simulations	167

8.13.2.2	HAPS coexistence scenarios and simulations	169

8.13.3	BS RF requirements	169

8.13.3.1	TX requirements	175

8.13.3.2	RX requirements	176

8.13.4	UE RF requirements	177

8.13.4.1	TX requirements	177

8.13.4.2	RX requirements	177

8.16	Extending current NR operation to 71GHz	178

8.16.4	BS RF requirements	178

8.16.4.1	TX requirements	179

8.16.4.2	RX requirements	179

8.17	Enhancements to Integrated Access and Backhaul (IAB) for NR	180

8.17.1	General	180

8.17.2	RF requirements	181

8.17.2.1	Impact for Simultaneous operation of IAB child and parent links	181

8.17.2.2	Impact for Timing enhancement	182

8.17.2.3	Others	182

9	Rel-17 Study Items for NR	182

9.1	Study on enhanced test methods for FR2 in NR	182

9.1.1	Maintenance on objectives 1~6	182

9.1.1.1	MU assessment	184

9.1.1.2	Others	185

9.1.2	OTA test methods for UE RF, RRM and demodulation for 52.6~71GHz	185

9.1.2.1	General	187

9.1.2.1.1	Test system assumption	187

9.1.2.1.2	UE types	187

9.1.2.1.3	MU assessment	187

9.1.2.1.4	Others	187

9.1.2.2	Test methodology for UE RF	187

9.1.2.3	Test methodology for RRM	187

9.1.2.4	Test methodology for UE demodulation and CSI	187

10	Rel-17 Work Items for LTE	188

11	Liaison and output to other groups	188

12	Revision of the Work Plan	188

13	Any other business	188

4	Rel-15 and previous release maintenance for LTE and NR

4.1	NR WIs (up to Rel-15)

4.1.2	UE EMC requirements

4.1.3	BS RF requirements

Session Chair Note: R4-2118599/8600 moved to AI 4.1.1.

4.1.3.1	General

R4-2120781	Big CR for TS 37.104 Maintenance (Rel-14, CAT F)

Type: CR		For: Agreement
					37.104 v14.7.0	  CR-  rev  Cat: F (Rel-14)

					Source: MCC,Ericsson

Abstract:

Decision:		Agreed.

R4-2120788	Big CR for TS 37.104 Maintenance (Rel-15, CAT F)

Type: CR		For: Agreement
					37.104 v15.14.0	  CR-  rev  Cat: F (Rel-15)

					Source: MCC,Ericsson

Abstract:

Decision:		Agreed.

R4-2120789	Big CR for TS 37.104 Maintenance (Rel-16, CAT A)

Type: CR		For: Agreement
					37.104 v16.11.0	  CR-  rev  Cat: A (Rel-16)

					Source: MCC, Ericsson

Abstract:

Decision:		Agreed.

R4-2120790	Big CR for TS 37.104 Maintenance (Rel-17, CAT A)

Type: draftCR		For: Endorsement
					37.104 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: MCC, Ericsson

Abstract:

Decision:		Agreed.

R4-2120791	Big CR for TS 37.105 Maintenance (Rel-14, CAT F)

Type: CR		For: Agreement
	37.105 v14.7.0	  CR-  rev  Cat: F (Rel-14)

	Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120792	Big CR for TS 37.105 Maintenance (Rel-15, CAT F)

Type: CR		For: Agreement
					37.105 v15.14.0	  CR-  rev  Cat: F (Rel-15)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120793	Big CR for TS 37.105 Maintenance (Rel-16, CAT A)

Type: CR		For: Agreement
					37.105 v16.9.0	  CR-  rev  Cat: A (Rel-16)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120794	Big CR for TS 37.105 Maintenance (Rel-17, CAT A)

Type: CR		For: Agreement
					37.105 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120795	Big CR for TS 37.141 Maintenance (Rel-14, CAT F)

Type: CR		For: Agreement
	37.141 v14.7.0	  CR-  rev  Cat: F (Rel-14)

	Source: MCC, Ericsson

Abstract:

Decision:		Agreed.

R4-2120796	Big CR for TS 37.141 Maintenance (Rel-15, CAT F)

Type: CR		For: Agreement
					37.141 v15.14.0	  CR-  rev  Cat: F (Rel-15)

					Source: MCC, Ericsson

Abstract:

Decision:		Agreed.

R4-2120797	Big CR for TS 37.141 Maintenance (Rel-16, CAT A)

Type: CR		For: Agreement
					37.141 v16.11.0	  CR-  rev  Cat: A (Rel-16)

					Source: MCC, Ericsson

Abstract:

Decision:		Agreed.

R4-2120798	Big CR for TS 37.141 Maintenance (Rel-17, CAT A)

Type: CR		For: Agreement
					37.141 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: MCC, Ericsson

Abstract:

Decision:		Agreed.

R4-2120799	Big CR for TS 37.145-1 Maintenance (Rel-14, CAT F)

Type: CR		For: Agreement
	37.145-1 v14.10.0	  CR-  rev  Cat: F (Rel-14)

	Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120800 Big CR for TS 37.145-1 Maintenance (Rel-15, CAT F)

Type: CR		For: Agreement
					37.145-1 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120801	Big CR for TS 37.145-1 Maintenance (Rel-16, CAT A)

Type: CR		For: Agreement
					37.145-1 v16.8.0	  CR-  rev  Cat: A (Rel-16)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120802	Big CR for TS 37.145-1 Maintenance (Rel-17, CAT A)

Type: CR		For: Agreement
					37.145-1 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120803 Big CR for TS 37.145-2 Maintenance (Rel-15, CAT F)

Type: CR		For: Agreement
					37.145-2 v15.12.0	  CR-  rev  Cat: F (Rel-15)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120804	Big CR for TS 37.145-2 Maintenance (Rel-16, CAT A)

Type: CR		For: Agreement
					37.145-2 v16.9.0	  CR-  rev  Cat: A (Rel-16)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120805	Big CR for TS 37.145-2 Maintenance (Rel-17, CAT A)

Type: CR		For: Agreement
					37.145-2 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120809 Big CR for TS 38.104 Maintenance RF part (Rel-15, CAT F)

Type: CR		For: Agreement
					38.104 v15.15.0	  CR-  rev  Cat: F (Rel-15)

					Source: MCC, Ericsson

Abstract:

Decision:		Agreed.

R4-2120810	Big CR for TS 38.104 Maintenance RF part (Rel-16, CAT F)

Type: CR		For: Agreement
					38.104 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: MCC, Ericsson

Abstract:

Decision:		Agreed.

R4-2120811	Big CR for TS 38.104 Maintenance RF part (Rel-17, CAT F)

Type: CR		For: Agreement
					38.104 v17.3.0	  CR-  rev  Cat: F (Rel-17)

					Source: MCC, Ericsson

Abstract:

Decision:		Agreed.

R4-2120831 Big CR for TS 38.141-1 Maintenance RF part (Rel-15, CAT F)

Type: CR		For: Agreement
					38.141-1 v15.10.0	  CR-  rev  Cat: F (Rel-15)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120814	Big CR for TS 38.141-1 Maintenance RF part (Rel-16, CAT F)

Type: CR		For: Agreement
					38.141-1 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120815	Big CR for TS 38.141-1 Maintenance RF part (Rel-17, CAT A)

Type: CR		For: Agreement
					38.141-1 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120819 Big CR for TS 38.141-2 Maintenance RF part (Rel-15, CAT F)

Type: CR		For: Agreement
					38.141-2 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120820	Big CR for TS 38.141-2 Maintenance RF part (Rel-16, CAT A)

Type: CR		For: Agreement
					38.141-2 v16.9.0	  CR-  rev  Cat: A (Rel-16)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120821	Big CR for TS 38.141-2 Maintenance RF part (Rel-17, CAT A)

Type: CR		For: Agreement
					38.141-2 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120830	Big CR for TR 37.941 maintenance (Rel-16, CAT F)

Type: CR		For: Agreement
					37.941 v16.3.0	  CR-  rev  Cat: F (Rel-16)

					Source: MCC,Huawei

Abstract:

Decision:		Agreed.

------------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][301] BSRF_Maintenance, AI 4.1.3, 4.2.2, 5.1.6.1, 5.1.1.3, 5.2.1, 6.1.2, 6.8.1 – Johan Sköld

R4-2120597	Email discussion summary for [101-e][301] BSRF_Maintenance

Type: other		For: Information
					Source: Moderator (Ericsson)

Abstract:

Discussion:

Decision:		Revised to R4-2120718 (from R4-2120597).

R4-2120718	Email discussion summary for [101-e][301] BSRF_Maintenance

Type: other		For: Information
					Source: Moderator (Ericsson)

Abstract:

Discussion:

Decision:		Noted.

Conclusion after 2nd round

Existing t-doc

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2117201 | Draft CR to TS 38.104: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117202 | Draft CR to TS 38.104: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117203 | Draft CR to TS 38.104: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117204 | Draft CR to TS 37.104: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117205 | Draft CR to TS 37.104: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117206 | Draft CR to TS 37.104: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117207 | Draft CR to TS 37.104: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117208 | Draft CR to TS 37.141: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117209 | Draft CR to TS 37.141: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117210 | Draft CR to TS 37.141: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117211 | Draft CR to TS 37.141: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117212 | Draft CR to TS 38.141-1: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117213 | Draft CR to TS 38.141-1: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117214 | Draft CR to TS 38.141-1: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117215 | Draft CR to TS 38.141-2: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117216 | Draft CR to TS 38.141-2: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117217 | Draft CR to TS 38.141-2: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117218 | Draft CR to TS 37.105: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117219 | Draft CR to TS 37.105: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117220 | Draft CR to TS 37.105: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117221 | Draft CR to TS 37.105: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117222 | Draft CR to TS 37.145-1: Correction on tables for Band 23 coexistence and co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117223 | Draft CR to TS 37.145-1: Correction on tables for Band 23 coexistence and co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117224 | Draft CR to TS 37.145-1: Correction on tables for Band 23 coexistence and co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117225 | Draft CR to TS 37.145-1: Correction on tables for Band 23 coexistence and co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117226 | Draft CR to TS 37.145-2: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117227 | Draft CR to TS 37.145-2: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117228 | Draft CR to TS 37.145-2: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2119501 | Draft CR to TS 37.141: correction of the OBUE requirements applicability, Rel-15 | Huawei | Endorsed |  |
| R4-2119502 | Draft CR to TS 37.141: correction of the OBUE requirements applicability, Rel-16 | Huawei | Endorsed |  |
| R4-2119503 | Draft CR to TS 37.141: correction of the OBUE requirements applicability, Rel-17 | Huawei | Endorsed |  |
| R4-2117229 | Draft CR to TS 36.104: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117230 | Draft CR to TS 36.104: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117231 | Draft CR to TS 36.104: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117232 | Draft CR to TS 36.104: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117233 | Draft CR to TS 36.141: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117234 | Draft CR to TS 36.141: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117235 | Draft CR to TS 36.141: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117236 | Draft CR to TS 36.141: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117237 | Draft CR to TS 25.104: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Not pursued |  |
| R4-2117238 | Draft CR to TS 25.104: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Not pursued |  |
| R4-2117239 | Draft CR to TS 25.104: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Not pursued |  |
| R4-2117240 | Draft CR to TS 25.141: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Not pursued |  |
| R4-2117241 | Draft CR to TS 25.141: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Withdrawn |  |
| R4-2117242 | Draft CR to TS 25.141: Correction on tables for Band 23 co-location requirements | Nokia, Nokia Shanghai Bell | Withdrawn |  |
| R4-2119198 | Maintenance CR to TS 38.104: NR-U BS RF requirements | ZTE Corporation | Endorsed |  |
| R4-2119199 | Maintenance CR to TS 38.104: NR-U BS RF requirements | ZTE Corporation | Endorsed |  |
| R4-2119133 | CR to TR37.941: removal of outstanding [], Rel-16 | Huawei | Endorsed |  |
| R4-2118170 | CR to TS 38.104 - Fix to band n85 introduction | Ericsson | Agreed |  |
| R4-2120751 | CR to TR 38.803: Addition of array antenna model extension in subclause 5.2.3 | Ericsson | Postponed |  |
| R4-2119129 | Draft CR to TS 38.104: miscellaneous corrections, Rel-15 | Huawei | Not pursued |  |
| R4-2119130 | Draft CR to TS 38.104: miscellaneous corrections, Rel-16 | Huawei | Not pursued |  |
| R4-2119131 | Draft CR to TS 38.104: miscellaneous corrections, Rel-17 | Huawei | Endorsed | WI code correction: “NR_newRAT-Core” CR category correction: Cat. F |
| R4-2118086 | Draft CR to 38.104: BS OBUE Cat B requirements clarification | NEC | Endorsed |  |
| R4-2118087 | Draft CR to 38.104: BS OBUE Cat B requirements clarification | NEC | Endorsed |  |
| R4-2118088 | Draft CR to 38.104: BS OBUE Cat B requirements clarification | NEC | Endorsed |  |
| R4-2118089 | Draft CR to 38.141-1: BS OBUE Cat B requirements clarification | NEC | Endorsed |  |
| R4-2118090 | Draft CR to 38.141-1: BS OBUE Cat B requirements clarification | NEC | Endorsed |  |
| R4-2118091 | Draft CR to 38.141-1: BS OBUE Cat B requirements clarification | NEC | Endorsed |  |
| R4-2118092 | Draft CR to 38.141-2: BS OBUE Cat B requirements clarification | NEC | Endorsed |  |
| R4-2118093 | Draft CR to 38.141-2: BS OBUE Cat B requirements clarification | NEC | Endorsed |  |
| R4-2118094 | Draft CR to 38.141-2: BS OBUE Cat B requirements clarification | NEC | Endorsed |  |
| R4-2120629 | Maintenance CR for TS 37.104 section 6.6.1.3.1 and 6.6.1.4 | ZTE Corporation | Endorsed |  |
| R4-2119273 | Maintenance CR for TS 37.104 section 6.6.1.3.1 and 6.6.1.4 | ZTE Corporation | Endorsed |  |
| R4-2119274 | Maintenance CR for TS 37.104 section 6.6.1.3.1 and 6.6.1.4 | ZTE Corporation | Endorsed |  |
| R4-2120630 | Maintenance CR for TS 37.141 section 6.6.1.5.5 and 6.6.1.5.6 | ZTE Corporation | Endorsed |  |
| R4-2119276 | Maintenance CR for TS 37.141 section 6.6.1.5.5 and 6.6.1.5.6 | ZTE Corporation | Endorsed |  |
| R4-2119277 | Maintenance CR for TS 37.141 section 6.6.1.5.5 and 6.6.1.5.6 | ZTE Corporation | Endorsed |  |
| R4-2120682 | Maintenance CR for TS 37.105 Section 9.7.6.4.3.2 | ZTE Corporation | Endorsed |  |
| R4-2119279 | Maintenance CR for TS 37.105 Section 9.7.6.4.3.2 | ZTE Corporation | Endorsed |  |
| R4-2119280 | Maintenance CR for TS 37.105 Section 9.7.6.4.3.2 | ZTE Corporation | Endorsed |  |
| R4-2120632 | Maintenance CR for TS 37.145-1 section 6.6.6.5.2.5 and 6.6.6.5.2.6 | ZTE Corporation | Endorsed |  |
| R4-2119282 | Maintenance CR for TS 37.145-1 section 6.6.6.5.2.5 and 6.6.6.5.2.6 | ZTE Corporation | Endorsed |  |
| R4-2119283 | Maintenance CR for TS 37.145-1 section 6.6.6.5.2.5 and 6.6.6.5.2.6 | ZTE Corporation | Endorsed |  |
| R4-2120633 | Maintenance CR for TS 37.145-2 section 6.7.6.4.5.1.1 | ZTE Corporation | Endorsed |  |
| R4-2119285 | Maintenance CR for TS 37.145-2 section 6.7.6.4.5.1.1 | ZTE Corporation | Endorsed |  |
| R4-2119286 | Maintenance CR for TS 37.145-2 section 6.7.6.4.5.1.1 | ZTE Corporation | Endorsed |  |
| R4-2120756 | Draft CR to TS 38.141-1 with NRTC6 test configuration updates | Nokia, Nokia Shaghai Bell | Endorsed |  |
| R4-2119440 | Draft CR to TS 38.141-1 with NRTC6 test configuration updates | Nokia, Nokia Shaghai Bell | Endorsed |  |
| R4-2120752 | CR to TR 38.820: Addition of array antenna model extension in subclause 7.2 | Ericsson | Withdrawn |  |
| R4-2119187 | TP to TR 38.921: MR/LA BS UEM requirements | ZTE Corporation | Not pursued |  |

------------------------------------------------------------------------------------------------------------------------------------------------

Topic #1 BS RF maintenance for NR Rel-15

R4-2118459	CR to TR 38.803: Addition of array antenna model extension in subclause 5.2.3

Type: CR		For: Agreement
					38.803 v14.2.0	  CR-0016  rev  Cat: F (Rel-14)

					Source: Ericsson

Abstract:

A new section (subclause 5.2.3.2.4) is created to capture the array antenna model extension. Information related to the parameter definition, array antenna model extension, parameter values applicable for 1710 to 4990 MHz is included in the CR.

Decision:		Revised to R4-2120751 (from R4-2118459).

R4-2120751	CR to TR 38.803: Addition of array antenna model extension in subclause 5.2.3

Type: CR		For: Agreement
					38.803 v14.2.0	  CR-0016  rev  Cat: F (Rel-14)

					Source: Ericsson

Abstract:

A new section (subclause 5.2.3.2.4) is created to capture the array antenna model extension. Information related to the parameter definition, array antenna model extension, parameter values applicable for 1710 to 4990 MHz is included in the CR.

Decision:		Postponed.

Miscellaneous corrections on TS 38.104

R4-2119129	Draft CR to TS 38.104: miscellaneous corrections, Rel-15

Type: draftCR		For: Endorsement
					38.104 v15.15.0	  CR-  rev  Cat: F (Rel-15)

					Source: Huawei

Abstract:

Multiple miscellaneous corrections were identified to be required across the specification.

Decision:		Not pursued.

R4-2119130	Draft CR to TS 38.104: miscellaneous corrections, Rel-16

Type: draftCR		For: Endorsement
					38.104 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Huawei

Abstract:

Multiple miscellaneous corrections were identified to be required across the specification.

Decision:		Not pursued.

R4-2119131	Draft CR to TS 38.104: miscellaneous corrections, Rel-17

Type: draftCR		For: Endorsement
					38.104 v17.3.0	  CR-  rev  Cat: F (Rel-17)

					Source: Huawei

Abstract:

Multiple miscellaneous corrections were identified to be required across the specification.

Decision:		Endorsed.

4.1.3.2	TX/RX requirements (38.104)

Correction on Band 23 co-location requirements on TS 38.104

R4-2117201	Draft CR to TS 38.104: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					38.104 v15.15.0	  CR-  rev  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117202	Draft CR to TS 38.104: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					38.104 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements. Correct formatting errors in Table 5.3.5-1.

Decision:		Endorsed.

R4-2117203	Draft CR to TS 38.104: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					38.104 v17.3.0	  CR-  rev  Cat: F (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements. Correct formatting errors in Table 5.3.5-1.

Decision:		Endorsed.

Session chair Note: R4-211721~7217 moved to this AI from AI 4.1.4.

R4-2117212	Draft CR to TS 38.141-1: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					38.141-1 v15.10.0	  CR-  rev  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117213	Draft CR to TS 38.141-1: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					38.141-1 v16.9.0	  CR-  rev  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117214	Draft CR to TS 38.141-1: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					38.141-1 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117215	Draft CR to TS 38.141-2: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					38.141-2 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117216	Draft CR to TS 38.141-2: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					38.141-2 v16.9.0	  CR-  rev  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117217	Draft CR to TS 38.141-2: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					38.141-2 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

Correction on BS OBUE clarification for TS 38.104/TS 38.141-1/-2

R4-2118086	Draft CR to 38.104: BS OBUE Cat B requirements clarification

Type: draftCR		For: Endorsement
					38.104 v15.15.0	  CR-  rev  Cat:  (Rel-15)

					Source: NEC

Decision:		Endorsed.

R4-2118087	Draft CR to 38.104: BS OBUE Cat B requirements clarification

Type: draftCR		For: Endorsement
					38.104 v16.9.0	  CR-  rev  Cat:  (Rel-16)

					Source: NEC

Decision:		Endorsed.

R4-2118088	Draft CR to 38.104: BS OBUE Cat B requirements clarification

Type: draftCR		For: Endorsement
					38.104 v17.3.0	  CR-  rev  Cat:  (Rel-17)

					Source: NEC

Decision:		Endorsed.

Session chair Note: R4-2118089~8094 moved to this AI from AI 4.1.4

R4-2118089	Draft CR to 38.141-1: BS OBUE Cat B requirements clarification

Type: draftCR		For: Endorsement
					38.141-1 v15.10.0	  CR-  rev  Cat:  (Rel-15)

					Source: NEC

Decision:		Endorsed.

R4-2118090	Draft CR to 38.141-1: BS OBUE Cat B requirements clarification

Type: draftCR		For: Endorsement
					38.141-1 v16.9.0	  CR-  rev  Cat:  (Rel-16)

					Source: NEC

Decision:		Endorsed.

R4-2118091	Draft CR to 38.141-1: BS OBUE Cat B requirements clarification

Type: draftCR		For: Endorsement
					38.141-1 v17.3.0	  CR-  rev  Cat:  (Rel-17)

					Source: NEC

Decision:		Endorsed.

R4-2118092	Draft CR to 38.141-2: BS OBUE Cat B requirements clarification

Type: draftCR		For: Endorsement
					38.141-2 v15.11.0	  CR-  rev  Cat:  (Rel-15)

					Source: NEC

Decision:		Endorsed.

R4-2118093	Draft CR to 38.141-2: BS OBUE Cat B requirements clarification

Type: draftCR		For: Endorsement
					38.141-2 v16.9.0	  CR-  rev  Cat:  (Rel-16)

					Source: NEC

Decision:		Endorsed.

R4-2118094	Draft CR to 38.141-2: BS OBUE Cat B requirements clarification

Type: draftCR		For: Endorsement
					38.141-2 v17.3.0	  CR-  rev  Cat:  (Rel-17)

					Source: NEC

Decision:		Endorsed.

4.1.3.3	MSR specifications

Correction on Band 23 co-location requirements on TS 37.104/37.141/37.105/37.145

R4-2117204	Draft CR to TS 37.104: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					37.104 v14.7.0	  CR-  rev  Cat: F (Rel-14)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117205	Draft CR to TS 37.104: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					37.104 v15.14.0	  CR-  rev  Cat: A (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117206	Draft CR to TS 37.104: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					37.104 v16.11.0	  CR-  rev  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117207	Draft CR to TS 37.104: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					37.104 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117208	Draft CR to TS 37.141: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					37.141 v14.13.0	  CR-  rev  Cat: F (Rel-14)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117209	Draft CR to TS 37.141: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					37.141 v15.15.0	  CR-  rev  Cat: A (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117210	Draft CR to TS 37.141: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					37.141 v16.11.0	  CR-  rev  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117211	Draft CR to TS 37.141: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					37.141 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

Session Chair Note: R4-2117218~7228 moved to this AI from AI 4.1.4

R4-2117218	Draft CR to TS 37.105: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					37.105 v14.7.0	  CR-  rev  Cat: F (Rel-14)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117219	Draft CR to TS 37.105: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					37.105 v15.14.0	  CR-  rev  Cat: A (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117220	Draft CR to TS 37.105: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					37.105 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117221	Draft CR to TS 37.105: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					37.105 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117222	Draft CR to TS 37.145-1: Correction on tables for Band 23 coexistence and co-location requirements

Type: draftCR		For: Endorsement
					37.145-1 v14.10.0	  CR-  rev  Cat: F (Rel-14)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from tables for coexistence and co-location requirements.

Decision:		Endorsed.

R4-2117223	Draft CR to TS 37.145-1: Correction on tables for Band 23 coexistence and co-location requirements

Type: draftCR		For: Endorsement
					37.145-1 v15.11.0	  CR-  rev  Cat: A (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from tables for coexistence and co-location requirements.

Decision:		Endorsed.

R4-2117224	Draft CR to TS 37.145-1: Correction on tables for Band 23 coexistence and co-location requirements

Type: draftCR		For: Endorsement
					37.145-1 v16.8.0	  CR-  rev  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from tables for coexistence and co-location requirements.

Decision:		Endorsed.

R4-2117225	Draft CR to TS 37.145-1: Correction on tables for Band 23 coexistence and co-location requirements

Type: draftCR		For: Endorsement
					37.145-1 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from tables for coexistence and co-location requirements.

Decision:		Endorsed.

R4-2117226	Draft CR to TS 37.145-2: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					37.145-2 v15.12.0	  CR-  rev  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117227	Draft CR to TS 37.145-2: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					37.145-2 v16.9.0	  CR-  rev  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117228	Draft CR to TS 37.145-2: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					37.145-2 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

Correction on OBUE requirements applicability for TS 37.141

R4-2119501	Draft CR to TS 37.141: correction of the OBUE requirements applicability, Rel-15

Type: draftCR		For: Endorsement
					37.141 v15.15.0	  CR-  rev  Cat: F (Rel-15)

					Source: Huawei

Abstract:

During the work on the M.2070 recommendation updates for based on TS 37.141 OBUE requirements, it was observed that some of the requirements are omitted in the main text.

Decision:		Endorsed.

R4-2119502	Draft CR to TS 37.141: correction of the OBUE requirements applicability, Rel-16

Type: draftCR		For: Endorsement
					37.141 v16.11.0	  CR-  rev  Cat: A (Rel-16)

					Source: Huawei

Abstract:

During the work on the M.2070 recommendation updates for based on TS 37.141 OBUE requirements, it was observed that some of the requirements are omitted in the main text.

Decision:		Endorsed.

R4-2119503	Draft CR to TS 37.141: correction of the OBUE requirements applicability, Rel-17

Type: draftCR		For: Endorsement
					37.141 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Huawei

Abstract:

During the work on the M.2070 recommendation updates for based on TS 37.141 OBUE requirements, it was observed that some of the requirements are omitted in the main text.

Decision:		Endorsed.

Other corrections on TS 37.104

R4-2119272	Maintenance CR for TS 37.104 section 6.6.1.3.1 and 6.6.1.4

Type: draftCR		For: Endorsement
					37.104 v15.14.0	  CR-  rev  Cat: F (Rel-15)

					Source: ZTE Corporation

Decision:		Revised to R4-2120629 (from R4-2119272).

R4-2120629	Maintenance CR for TS 37.104 section 6.6.1.3.1 and 6.6.1.4

Type: draftCR		For: Endorsement
					37.104 v15.14.0	  CR-  rev  Cat: F (Rel-15)

					Source: ZTE Corporation

Decision:		Endorsed.

R4-2119273	Maintenance CR for TS 37.104 section 6.6.1.3.1 and 6.6.1.4

Type: draftCR		For: Endorsement
					37.104 v16.11.0	  CR-  rev  Cat: A (Rel-16)

					Source: ZTE Corporation

Decision:		Endorsed.

R4-2119274	Maintenance CR for TS 37.104 section 6.6.1.3.1 and 6.6.1.4

Type: draftCR		For: Endorsement
					37.104 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: ZTE Corporation

Decision:		Endorsed.

R4-2119275	Maintenance CR for TS 37.141 section 6.6.1.5.5 and 6.6.1.5.6

Type: draftCR		For: Endorsement
					37.141 v15.15.0	  CR-  rev  Cat: F (Rel-15)

					Source: ZTE Corporation

Decision:		Revised to R4-2120630 (from R4-2119275).

R4-2120630	Maintenance CR for TS 37.141 section 6.6.1.5.5 and 6.6.1.5.6

Type: draftCR		For: Endorsement
					37.141 v15.15.0	  CR-  rev  Cat: F (Rel-15)

					Source: ZTE Corporation

Decision:		Endorsed.

R4-2119276	Maintenance CR for TS 37.141 section 6.6.1.5.5 and 6.6.1.5.6

Type: draftCR		For: Endorsement
					37.141 v16.11.0	  CR-  rev  Cat: A (Rel-16)

					Source: ZTE Corporation

Decision:		Endorsed.

R4-2119277	Maintenance CR for TS 37.141 section 6.6.1.5.5 and 6.6.1.5.6

Type: draftCR		For: Endorsement
					37.141 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: ZTE Corporation

Decision:		Endorsed.

Other corrections on TS 37.105/37.145

Session Chair Note: R4-2119278~9286 moved to this AI from AI 4.1.4

R4-2119278	Maintenance CR for TS 37.105 Section 9.7.6.4.3.2

Type: draftCR		For: Endorsement
					37.105 v15.14.0	  CR-  rev  Cat: F (Rel-15)

					Source: ZTE Corporation

Decision:		Revised to R4-2120682 (from R4-2119278).

R4-2120682	Maintenance CR for TS 37.105 Section 9.7.6.4.3.2

Type: draftCR		For: Endorsement
					37.105 v15.14.0	  CR-  rev  Cat: F (Rel-15)

					Source: ZTE Corporation

Decision:		Endorsed.

R4-2119279	Maintenance CR for TS 37.105 Section 9.7.6.4.3.2

Type: draftCR		For: Endorsement
					37.105 v16.9.0	  CR-  rev  Cat: A (Rel-16)

					Source: ZTE Corporation

Decision:		Endorsed.

R4-2119280	Maintenance CR for TS 37.105 Section 9.7.6.4.3.2

Type: draftCR		For: Endorsement
					37.105 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: ZTE Corporation

Decision:		Endorsed.

R4-2119281	Maintenance CR for TS 37.145-1 section 6.6.6.5.2.5 and 6.6.6.5.2.6

Type: draftCR		For: Endorsement
					37.145-1 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: ZTE Corporation

Decision:		Revised to R4-2120632 (from R4-2119281).

R4-2120632	Maintenance CR for TS 37.145-1 section 6.6.6.5.2.5 and 6.6.6.5.2.6

Type: draftCR		For: Endorsement
					37.145-1 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: ZTE Corporation

Decision:		Endorsed.

R4-2119282	Maintenance CR for TS 37.145-1 section 6.6.6.5.2.5 and 6.6.6.5.2.6

Type: draftCR		For: Endorsement
					37.145-1 v16.8.0	  CR-  rev  Cat: A (Rel-16)

					Source: ZTE Corporation

Decision:		Endorsed.

R4-2119283	Maintenance CR for TS 37.145-1 section 6.6.6.5.2.5 and 6.6.6.5.2.6

Type: draftCR		For: Endorsement
					37.145-1 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: ZTE Corporation

Decision:		Endorsed.

R4-2119284	Maintenance CR for TS 37.145-2 section 6.7.6.4.5.1.1

Type: draftCR		For: Endorsement
					37.145-2 v15.12.0	  CR-  rev  Cat: F (Rel-15)

					Source: ZTE Corporation

Decision:		Revised to R4-2120633 (from R4-2119284).

R4-2120633	Maintenance CR for TS 37.145-2 section 6.7.6.4.5.1.1

Type: draftCR		For: Endorsement
					37.145-2 v15.12.0	  CR-  rev  Cat: F (Rel-15)

					Source: ZTE Corporation

Decision:		Endorsed.

R4-2119285	Maintenance CR for TS 37.145-2 section 6.7.6.4.5.1.1

Type: draftCR		For: Endorsement
					37.145-2 v16.9.0	  CR-  rev  Cat: A (Rel-16)

					Source: ZTE Corporation

Decision:		Endorsed.

R4-2119286	Maintenance CR for TS 37.145-2 section 6.7.6.4.5.1.1

Type: draftCR		For: Endorsement
					37.145-2 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: ZTE Corporation

Decision:		Endorsed.

4.1.4	BS conformance testing

4.1.4.1	General

------------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][302] NR_Conformance_Maintenance, AI 4.1.4 – Liehai Liu

R4-2120598	Email discussion summary for [101-e][302] NR_Conformance_Maintenance

Type: other		For: Information
					Source: Moderator (Huawei)

Abstract:

Discussion:

Decision:		Revised to R4-2120719 (from R4-2120598).

R4-2120719	Email discussion summary for [101-e][302] NR_Conformance_Maintenance

Type: other		For: Information
					Source: Moderator (Huawei)

Abstract:

Discussion:

Decision:		Noted.

R4-2120636	WF on sweep time for unwanted emission testing

Type: other		For: Approval
					Source: CATT

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2117395 | Discussion on sweep time for unwanted emission testing | CATT | Noted |  |
| R4-2117396 | draft CR for TS 38.141-1 On sweep time for unwanted emission testing (Rel-15) | CATT | Not Pursued |  |
| R4-2117399 | draft CR for TS 38.141-2 On sweep time for unwanted emission testing (Rel-15) | CATT | Not Pursued |  |
| R4-2117894 | General rule for resolution bandwidth 38.141-2 R15 | Huawei, HiSilicon | Endorsed |  |
| R4-2118727 | Draft CR to TS 37.145-1: ATC2b correction (rel-15) | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2118730 | Draft CR to TS 37.145-2: ATCR2b correction (rel-15) | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2118733 | Draft CR to TS 38.141-2: NRTC2 correction (rel-15) | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117885 | Correction on the test configuration for NC operation 37.141 R15 | Huawei, HiSilicon | Not Pursued |  |
| R4-2117886 | Correction on the test configuration for NC operation 37.141 R16 | Huawei, HiSilicon | Not Pursued |  |
| R4-2117888 | Correction on the test configuration for NC operation 37.145-1 R15 | Huawei, HiSilicon | Not Pursued |  |
| R4-2117889 | Correction on the test configuration for NC operation 37.145-1 R16 | Huawei, HiSilicon | Not Pursued |  |
| R4-2117891 | Correction on the test configuration for NC operation 37.145-2 R15 | Huawei, HiSilicon | Not Pursued |  |
| R4-2117892 | Correction on the test configuration for NC operation 37.145-2 R16 | Huawei, HiSilicon | Not Pursued |  |
| R4-2120634 | draftCR to 38.141-2: Addition of Plane Wave Synthesizer in OTA measurement system set-up | CAICT | Endorsed |  |
| R4-2120635 | CR on removal of Luant modem in radiated performance specification | Ericsson | Not Pursued |  |
| R4-2120758 | CR on clarification scope on Luant modem | Ericsson | Endorsed |  |
| R4-2120636 | WF on sweep time for unwanted emission testing | CATT | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

Topic #2 Sweep time for unwanted emission testing

R4-2117395	Discussion on sweep time for unwanted emission testing

Type: other		For: Approval
					Source: CATT

Decision:		Noted.

4.1.4.2	Conducted conformance testing (38.141-1)

CR for TS 38.141-1 on sweeping time

R4-2117396	draft CR for TS 38.141-1 On sweep time for unwanted emission testing (Rel-15)

Type: draftCR		For: Endorsement
					38.141-1 v15.10.0	  CR-  rev  Cat: F (Rel-15)

					Source: CATT

Decision:		Not pursued.

R4-2117397	draft CR for TS 38.141-1 On sweep time for unwanted emission testing (Rel-16)

Type: draftCR		For: Endorsement
					38.141-1 v16.9.0	  CR-  rev  Cat: A (Rel-16)

					Source: CATT

Decision:		Withdrawn.

R4-2117398	draft CR for TS 38.141-1 On sweep time for unwanted emission testing (Rel-17)

4.1.4.3	Radiated conformance testing (38.141-2)

CR for TS 38.141-2 on sweeping time

R4-2117399	draft CR for TS 38.141-2 On sweep time for unwanted emission testing (Rel-15)

Type: draftCR		For: Endorsement
					38.141-2 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: CATT

Decision:		Not pursued.

R4-2117400	draft CR for TS 38.141-2 On sweep time for unwanted emission testing (Rel-16)

Type: draftCR		For: Endorsement
					38.141-2 v16.9.0	  CR-  rev  Cat: A (Rel-16)

					Source: CATT

Decision:		Withdrawn.

R4-2117401	draft CR for TS 38.141-2 On sweep time for unwanted emission testing (Rel-17)

Type: draftCR		For: Endorsement
					38.141-2 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: CATT

Decision:		Withdrawn.

Resolution bandwith on 38.141-2

R4-2117894	General rule for resolution bandwidth 38.141-2 R15

Type: draftCR		For: Endorsement
					38.141-2 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

R4-2117895	General rule for resolution bandwidth 38.141-2 R16

Type: draftCR		For: Endorsement
					38.141-2 v16.9.0	  CR-  rev  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

R4-2117896	General rule for resolution bandwidth 38.141-2 R17

Type: draftCR		For: Endorsement
					38.141-2 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

Topic #1: OTA measurement system set-up

R4-2118538	Discussion on Verification of Co-Location Measurement by using PWS

Type: discussion		For: (not specified)
					Source: CAICT

Decision:		Noted.

R4-2118458	draftCR to 38.141-2: Addition of Plane Wave Synthesizer in OTA measurement system set-up

Type: draftCR		For: (not specified)
					38.141-2 v15.11.0	  CR-  rev  Cat:  (Rel-15)

					Source: CAICT

Abstract:

PWS chamber added to the corresponding annex E clauses on any suitable OTA chamber.

Decision:		Revised to R4-2120634 (from R4-2118458).

R4-2120634	draftCR to 38.141-2: Addition of Plane Wave Synthesizer in OTA measurement system set-up

Type: draftCR		For: (not specified)
					38.141-2 v15.11.0	  CR-  rev  Cat:  (Rel-15)

					Source: CAICT

Abstract:

PWS chamber added to the corresponding annex E clauses on any suitable OTA chamber.

Decision:		Endorsed.

R4-2118465	draftCR to 38.141-2: Addition of Plane Wave Synthesizer in OTA measurement system set-up

Type: draftCR		For: (not specified)
					38.141-2 v16.9.0	  CR-  rev  Cat:  (Rel-16)

					Source: CAICT

Abstract:

PWS chamber added to the corresponding annex E clauses on any suitable OTA chamber.

Decision:		Endorsed.

R4-2118473	draftCR to 38.141-2: Addition of Plane Wave Synthesizer in OTA measurement system set-up

Type: draftCR		For: (not specified)
					38.141-2 v17.3.0	  CR-  rev  Cat:  (Rel-17)

					Source: CAICT

Abstract:

PWS chamber added to the corresponding annex E clauses on any suitable OTA chamber.

Decision:		Endorsed.

Topic #4 NRTC2 correction

R4-2118733	Draft CR to TS 38.141-2: NRTC2 correction (rel-15)

Type: draftCR		For: Endorsement
					38.141-2 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Endorsed.

R4-2118734	Draft CR to TS 38.141-2: NRTC2 correction (rel-16)

Type: draftCR		For: Endorsement
					38.141-2 v16.9.0	  CR-  rev  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Endorsed.

R4-2118735	Draft CR to TS 38.141-2: NRTC2 correction (rel-17)

Type: draftCR		For: Endorsement
					38.141-2 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Endorsed.

Topic #5 Luant modem testing

R4-2119000	OTA BS with Luant modem testing

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view related to luant modem testing.

Decision:		Noted.

R4-2119001	CR on removal of Luant modem in radiated performance specification

Type: draftCR		For: Endorsement
					38.141-2 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

in this CR, the removel of Luant modem in radiated performance specification of BS is proposed

Decision:		Revised to R4-2120635 (from R4-2119001).

R4-2120635	CR on removal of Luant modem in radiated performance specification

Type: draftCR		For: Endorsement
					38.141-2 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

in this CR, the removel of Luant modem in radiated performance specification of BS is proposed

Decision:		Not pursued.

R4-2119002	CR on removal of Luant modem in radiated performance specification_R16

Type: draftCR		For: Endorsement
					38.141-2 v16.9.0	  CR-  rev  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

in this CR, the removel of Luant modem in radiated performance specification of BS is proposed

Decision:		Withdrawn.

R4-2119003	CR on removal of Luant modem in radiated performance specification_R17

Type: draftCR		For: Endorsement
					38.141-2 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

in this CR, the removel of Luant modem in radiated performance specification of BS is proposed

Decision:		Withdrawn.

R4-2119004	CR on clarification scope on Luant modem

Type: draftCR		For: Endorsement
					37.461 v15.4.0	  CR-  rev  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

In this CR, scope of Luant is clarified

Decision:		Withdrawn.

R4-2120758	CR on clarification scope on Luant modem

Type: draftCR		For: Endorsement
					37.461 v15.4.0	  CR-  rev  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

In this CR, scope of Luant is clarified

Decision:		Endorsed.

R4-2119005	CR on clarification scope on Luant modem_R16

Type: draftCR		For: Endorsement
					37.461 v16.0.0	  CR-  rev  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

In this CR, scope of Luant is clarified

Decision:		Endorsed.

4.1.4.4	eAAS specifications

Topic #3 Correction on test configuration for NC operation for TS 37.141/37.145

R4-2117885	Correction on the test configuration for NC operation 37.141 R15

Type: draftCR		For: Endorsement
					37.141 v15.15.0	  CR-  rev  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Decision:		Not pursued.

R4-2117886	Correction on the test configuration for NC operation 37.141 R16

Type: draftCR		For: Endorsement
					37.141 v16.11.0	  CR-  rev  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Decision:		Not pursued.

R4-2117887	Correction on the test configuration for NC operation 37.141 R17

Type: draftCR		For: Endorsement
					37.141 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Withdrawn.

R4-2117888	Correction on the test configuration for NC operation 37.145-1 R15

Type: draftCR		For: Endorsement
					37.145-1 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Decision:		Not pursued.

R4-2117889	Correction on the test configuration for NC operation 37.145-1 R16

Type: draftCR		For: Endorsement
					37.145-1 v16.8.0	  CR-  rev  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Decision:		Not pursued.

R4-2117890	Correction on the test configuration for NC operation 37.145-1 R17

Type: draftCR		For: Endorsement
					37.145-1 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Withdrawn.

R4-2117891	Correction on the test configuration for NC operation 37.145-2 R15

Type: draftCR		For: Endorsement
					37.145-2 v15.12.0	  CR-  rev  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Decision:		Not pursued.

R4-2117892	Correction on the test configuration for NC operation 37.145-2 R16

Type: draftCR		For: Endorsement
					37.145-2 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Decision:		Not pursued.

R4-2117893	Correction on the test configuration for NC operation 37.145-2 R17

Type: draftCR		For: Endorsement
					37.145-2 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Withdrawn.

Correction on ATC2b for TS 37.145

R4-2118727	Draft CR to TS 37.145-1: ATC2b correction (rel-15)

Type: draftCR		For: Endorsement
					37.145-1 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Endorsed.

R4-2118728	Draft CR to TS 37.145-1: ATC2b correction (rel-16)

Type: draftCR		For: Endorsement
					37.145-1 v16.8.0	  CR-  rev  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Endorsed.

R4-2118729	Draft CR to TS 37.145-1: ATC2b correction (rel-17)

Type: draftCR		For: Endorsement
					37.145-1 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Endorsed.

R4-2118730	Draft CR to TS 37.145-2: ATCR2b correction (rel-15)

Type: draftCR		For: Endorsement
					37.145-2 v15.12.0	  CR-  rev  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Endorsed.

R4-2118731	Draft CR to TS 37.145-2: ATCR2b correction (rel-16)

Type: draftCR		For: Endorsement
					37.145-2 v16.9.0	  CR-  rev  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Endorsed.

R4-2118732	Draft CR to TS 37.145-2: ATCR2b correction (rel-17)

Type: draftCR		For: Endorsement
					37.145-2 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Endorsed.

4.1.5	BS EMC requirements

------------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][303] NR_EMC, AI 4.1.2, 4.1.5, 8.5.4, 4.2.1(R4-2119126/9127/9128)– Wubin Zhou

R4-2120596	Email discussion summary for [101-e][303] NR_EMC

Type: other		For: Information
					Source: Moderator (ZTE)

Abstract:

Discussion:

Decision:		Revised to R4-2120720 (from R4-2120596).

R4-2120720	Email discussion summary for [101-e][303] NR_EMC

Type: other		For: Information
					Source: Moderator (ZTE)

Abstract:

Discussion:

Decision:		Noted.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2118064 | TP to TS38.114: Emission | ZTE Corporation | Approved |  |
| R4-2118228 | Updating TS38.114 to capture RAN4#101 agreements | ZTE Corporation | Approved |  |
| - R4-2120747 | - Draft CR to TS 36.124: correction of the frequency range for the RI test, Rel-15 | - Huawei | - Endorsed |  |
| - R4-2120748 | - Draft CR to TS 36.124: correction of the frequency range for the RI test, Rel-16 | - Huawei | Endorsed |  |
| - R4-2120757 | - Draft CR to TS 36.124: correction of the frequency range for the RI test, Rel-17 | - Huawei | Endorsed |  |
| - R4-2120637 | - TPs to TS 38.114 on RF Repeater EMC section 1 (Scope) and section 9 (Immunity) | - Ericsson LM | - Approved |  |
| - R4-2120639 | - TP to TS 38.114 – applicability overview | - Nokia, Nokia Shanghai Bell | - Approved |  |
| - R4-2120638 | - TP to TS 38.114 for sections 4.3, 5.3, 6.3, 6.4 | - Huawei | - Approved |  |
| - R4-2117787 | - Draft CR to TS 38.113: Radiated emission measurement uncertainty(R15) | - ZTE Corporation | - Not Pursued |  |
| - R4-2117788 | - Draft CR to TS 38.113: Radiated emission measurement uncertainty(R16) | - ZTE Corporation | - Withdrawn |  |

------------------------------------------------------------------------------------------------------------------------------------------------

Topic #2: NR BS EMC

R4-2117792	Discussion on highest measurement frequency and measurement uncertainty for NR BS radiated emission test

Type: discussion		For: Discussion
					Source: ZTE Corporation

Abstract:

In this contribution, we will continue the discussion about RE MU for NR BS.

Decision:		Noted.

R4-2117787	Draft CR to TS 38.113: Radiated emission measurement uncertainty(R15)

Type: draftCR		For: Endorsement
					38.113 v15.15.0	  CR-  rev  Cat: F (Rel-15)

					Source: ZTE Corporation

Abstract:

Add the uncertainty of radiated emission above 12.75 GHz.

The highest measurement frequency of radiated emission is limited to 26 GHz.

Decision:		Not pursued.

R4-2117788	Draft CR to TS 38.113: Radiated emission measurement uncertainty(R16)

Type: draftCR		For: Endorsement
					38.113 v16.5.0	  CR-  rev  Cat: A (Rel-16)

					Source: ZTE Corporation

Abstract:

Add the uncertainty of radiated emission above 12.75 GHz.

The highest measurement frequency of radiated emission is limited to 26 GHz.

Decision:		Withdrawn.

R4-2119132	Analysis of the updated IEC 61000-4-3 specification: upper frequency range for radiated immunity

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution we analyse the latest updates to the IEC 61000-4-3:2020 and related implications to the RAN4 EMC specifications.

Decision:		Noted.

4.1.8	Demodulation and CSI requirements (38.101-4/38.104)

4.1.8.1	UE demodulation requirements

R4-2120806	Big CR for TS 38.101-4 Maintenance (Rel-15, CAT F)

Type: CR		For: Agreement
					38.101-4 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: MCC,Intel

Abstract:

Decision:		Agreed.

R4-2120807	Big CR for TS 38.101-4 Maintenance (Rel-16, CAT F)

Type: CR		For: Agreement
					38.101-4 v16.6.0	  CR-  rev  Cat: F (Rel-16)

					Source: MCC,Intel

Abstract:

Decision:		Agreed.

R4-2120808	Big CR for TS 38.101-4 Maintenance (Rel-17, CAT A)

Type: CR		For: Agreement
					38.101-4 v17.2.0	  CR-  rev  Cat: A (Rel-17)

					Source: MCC, Intel

Abstract:

Decision:		Agreed.

------------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][317] Demod_Maintenance_UE, AI 4.18.1, 4.1.8.2, 5.1.6.4.1, 5.1.6.4.2, 5.1.3.2, 4.2.4.1, 6.1.4, 7.36.3 – Manasa Raghavan

R4-2120599	Email discussion summary for [101-e][317] Demod_Maintenance_UE

Type: other		For: Information
					Source: Moderator (Apple)

Abstract:

Discussion:

Decision:		Revised to R4-2120721 (from R4-2120599).

R4-2120721	Email discussion summary for [101-e][317] Demod_Maintenance_UE

Type: other		For: Information
					Source: Moderator (Apple)

Abstract:

Discussion:

Decision:		Noted.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2119030 | Draft CR Updates to LTE V2X PSBCH requirements in 36.101 (Rel-14) | Huawei, HiSilicon | Endorsed |  |
| R4-2119410 | draft CR to TR 38.847 UE performance requirements | Intel Corporation | Not pursued | New t-doc assigned for formal CR |
| R4-2120652 | LS on UE capability for supporting single DCI transmission schemes for multi-TRP | Apple | Approved |  |
| R4-2120651 | CR to TR 38.847 UE performance requirements | Intel | Agreed |  |
| R4-2120648 | Alignment of common test parameters | Rohde & Schwarz | Endorsed |  |
| R4-2120764 | Draft CR on CSI reference measurement channels | Intel Corporation | Endorsed | R15 CR |
| R4-2120765 | Draft CR on CSI reference measurement channels | Intel Corporation | Endorsed | R16 CR |
| R4-2117428 | Draft CR to 38.101-4 on Applicability for multi-TRxP test cases-R16 | Apple | Postponed |  |
| R4-2120649 | Draft CR Updates to NR V2X requirements in 38.101-4 (Rel-16) | Huawei, HiSilicon | Endorsed |  |
| R4-2120650 | CR: Applicability of FR2 demodulation requirements to n262 | Ericsson | Agreed |  |
| R4-2120754 | CR: Introduction of Noc power level for n259 PC5 | Ericsson | Agreed |  |

------------------------------------------------------------------------------------------------------------------------------------------------

Correction on common test parameters

R4-2118059	Alignment of common test parameters

Type: draftCR		For: Agreement
					38.101-4 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: Rohde & Schwarz

Decision:		Revised to R4-2120648 (from R4-2118059).

R4-2120648	Alignment of common test parameters

Type: draftCR		For: Agreement
					38.101-4 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: Rohde & Schwarz

Decision:		Endorsed.

R4-2118060	Alignment of common test parameters

Type: draftCR		For: Agreement
					38.101-4 v16.6.0	  CR-  rev  Cat: A (Rel-16)

					Source: Rohde & Schwarz

Decision:		Endorsed.

R4-2118061	Alignment of common test parameters

Type: draftCR		For: Agreement
					38.101-4 v17.2.0	  CR-  rev  Cat: A (Rel-17)

					Source: Rohde & Schwarz

Decision:		Endorsed.

4.1.8.2	CSI requirements

Correction on CSI reference measurement channel

R4-2117995	Draft CR on CSI reference measurement channels

Type: draftCR		For: Endorsement
					38.101-4 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: Intel Corporation

Decision:		Revised to R4-2120764 (from R4-2117995).

R4-2120764	Draft CR on CSI reference measurement channels

Type: draftCR		For: Endorsement
					38.101-4 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: Intel Corporation

Decision:		Endorsed.

R4-2117996	Draft CR on CSI reference measurement channels

Type: draftCR		For: Endorsement
					38.101-4 v16.6.0	  CR-  rev  Cat: A (Rel-16)

					Source: Intel Corporation

Decision:		Revised to R4-2120765 (from R4-2117996).

R4-2120765	Draft CR on CSI reference measurement channels

Type: draftCR		For: Endorsement
					38.101-4 v16.6.0	  CR-  rev  Cat: F (Rel-16)

					Source: Intel Corporation

Decision:		Endorsed.

R4-2117997	Draft CR on CSI reference measurement channels

Type: draftCR		For: Endorsement
					38.101-4 v17.2.0	  CR-  rev  Cat: A (Rel-17)

					Source: Intel Corporation

Decision:		Endorsed.

R4-2119442	Discussion on PDSCH scheduling impact on CSI requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

4.1.8.3	BS demodulation requirements

R4-2120812	Big CR for TS 38.104 Maintenance Demod part (Rel-16, CAT F)

Type: CR		For: Agreement
					38.104 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: MCC, Nokia

Abstract:

Decision:		Agreed.

R4-2120813	Big CR for TS 38.104 Maintenance Demod part (Rel-17, CAT F)

Type: CR		For: Agreement
					38.104	 v17.3.0  CR-  rev  Cat: F (Rel-17)

					Source: MCC, Nokia

Abstract:

Decision:		Agreed.

R4-2120816 Big CR for TS 38.141-1 Maintenance Demod part (Rel-15, CAT F)

Type: CR		For: Agreement
					38.141-1 v15.10.0	  CR-  rev  Cat: F (Rel-15)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120817	Big CR for TS 38.141-1 Maintenance Demod part (Rel-16, CAT F)

Type: CR		For: Agreement
					38.141-1 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120818	Big CR for TS 38.141-1 Maintenance Demod part (Rel-17, CAT A)

Type: CR		For: Agreement
					38.141-1 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: MCC, Huawei

Abstract:

Decision:		Agreed.

R4-2120822 Big CR for TS 38.141-2 Maintenance Demod part (Rel-15, CAT F)

Type: CR		For: Agreement
					38.141-2 v15.11.0	  CR-  rev  Cat: F (Rel-15)

					Source: MCC, Intel

Abstract:

Decision:		Withdrawn.

R4-2120823	Big CR for TS 38.141-2 Maintenance Demod part (Rel-16, CAT F)

Type: CR		For: Agreement
					38.141-2 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: MCC, Intel

Abstract:

Decision:		Agreed.

R4-2120824	Big CR for TS 38.141-2 Maintenance Demod part (Rel-17, CAT A)

Type: CR		For: Agreement
					38.141-2 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: MCC, Intel

Abstract:

Decision:		Agreed.

------------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][316] Demod_Maintenance_BS, AI 4.1.8.3, 5.1.6.4.3, 4.2.4.2, 5.1.1.6.3, 5.1.2.5 – Aijun Cao

R4-2120600	Email discussion summary for [101-e][316] Demod_Maintenance_BS

Type: other		For: Information
					Source: Moderator (ZTE)

Abstract:

Discussion:

Decision:		Withdrawn.

R4-2120668	Email discussion summary for [101-e][316] Demod_Maintenance_BS

Type: other		For: Information
					Source: Moderator (ZTE)

Abstract:

Discussion:

Decision:		Revised to R4-2120722 (from R4-2120668).

R4-2120722	Email discussion summary for [101-e][316] Demod_Maintenance_BS

Type: other		For: Information
					Source: Moderator (ZTE)

Abstract:

Discussion:

Decision:		Noted.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2119034 | Draft CR Cleanup the square brackets for LTE PUSCH conformance testing in 36.141 (Rel-15) | Huawei, HiSilicon | Endorsed | Mirror CRs R4-2119035/36 |
| R4-2119037 | Draft CR Cleanup the square brackets for LTE PUSCH performance requirements in 36.104 (Rel-15) | Huawei, HiSilicon | Endorsed |  |
| R4-2119038 | Draft CR Cleanup the square brackets for LTE PUSCH performance requirements in 36.104 (Rel-16) | Huawei, HiSilicon | Endorsed | Mirror CR R4-2119039 |
| R4-2117324 | Draft CR for TS 38.174: correction of the reference | CATT | Endorsed |  |
| R4-2119026 | draftCR on IAB-DU performance requirements in TS 38.174 | Huawei, HiSilicon | Endorsed |  |
| R4-2119392 | draftCR to TS 38.176-2 on manufacturer declarations | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2119393 | draftCR to TS 38.176-1 on applicability rules | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2117687 | draftCR for TS38.104 UL TA demod requirement (Rel-16) | Ericsson | Merged | Merged into the revision of R4-2119170 |
| R4-2118674 | Draft CR to 38.141-2: BS UL TA test condition AWGN level correction (8.2.5) | Keysight Technologies UK Ltd | Endorsed | Mirror CR R4-2118675 |
| R4-2119174 | DraftCR 38.141-2 Correction of BS demod requirements for HST(Rel-16) | Nokia, Nokia Shanghai Bell, Qualcomm Incorporated | Endorsed | Mirror CR: R4-2119258 |
| R4-2117414 | draft CR for TS 38.141-1 (Rel-15) correction on AWGN power level for PUCCH format 3 | NTT DOCOMO, INC. | Endorsed | Mirror CRs: R4-2117515/16 |
| R4-2120643 | Draft CR Updates to interlaced PF2 and PF3 performance requirements in 38.104 (Rel-17) | Huawei, HiSilicon | Endorsed | Revision of R4-2119040 |
| R4-2120644 | Correction on 2-step RACH BS demod vendor declaration for conducted BS demod requirements | Nokia, Nokia Shanghai Bell | Endorsed | Revision of R4-2118954 Mirror CR R4-2118955 |
| R4-2120645 | Correction on 2-step RACH BS demod vendor declaration for radiated BS demod requirements | Nokia, Nokia Shanghai Bell | Endorsed | Revision of R4-2118956 Mirror CR R4-2118957 |
| R4-2120646 | DraftCR 38.104 Correction of BS demod requirements for HST (Rel-16) | Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Ericsson, ZTE | Endorsed | Revision of R4-2119170 Mirror CR R4-2119256 |
| R4-2120647 | DraftCR 38.141-1 Correction of BS demod requirements for HST (Rel-16) | Nokia, Nokia Shanghai Bell, Qualcomm Incorporated | Endorsed | Revision of R4-2119172 Mirror CR R4-2119257 |

------------------------------------------------------------------------------------------------------------------------------------------------

Topic #4: NR demodulation requirements

Correction on AWGN power level

Session chair Note: R4-2117414 ~ 7416 moved to this AI from AI 4.1.4.

R4-2117414	draft CR for TS 38.141-1 (Rel-15) correction on AWGN power level for PUCCH format 3

Type: draftCR		For: Agreement
					38.141-1 v15.10.0	  CR-  rev  Cat: F (Rel-15)

					Source: NTT DOCOMO, INC.

Decision:		Endorsed.

R4-2117415	draft CR for TS 38.141-1 (Rel-16) editorial correction on AWGN power level for PUCCH format 3

Type: draftCR		For: Agreement
					38.141-1 v16.9.0	  CR-  rev  Cat: A (Rel-16)

					Source: NTT DOCOMO, INC.

Decision:		Endorsed.

R4-2117416	draft CR for TS 38.141-1 (Rel-17) editorial correction on AWGN power level for PUCCH format 3

Type: draftCR		For: Agreement
					38.141-1 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: NTT DOCOMO, INC.

Decision:		Endorsed.

4.1.10	Testability (38.810)

4.2	LTE WIs (up to Rel-15)

4.2.1 UE RF requirements

R4-2120785	Big CR for TS 36.124 Maintenance (Rel-15, CAT F)

Type: CR		For: Agreement
					36.124 v15.2.0	  CR-?  rev  Cat: F (Rel-15)

					Source:MCC,Nokia

Abstract:

Decision:		Agreed.

R4-2120786	Big CR for TS 36.124 Maintenance (Rel-16, CAT A)

Type: draftCR		For: Endorsement
					36.124 v16.1.0	  CR-?  rev  Cat: A (Rel-16)

					Source: MCC,Nokia

Abstract:

Decision:		Agreed.

Refer to Email discussion for [101-e][303] NR_EMC

Topic #1: UE EMC

Session Chair Note: R4-2119126/9127/9128 moved to this AI from AI 4.1.2; these draftCRs target for 36.124 instead of 38.124.

R4-2119126	Draft CR to TS 38.124: correction of the frequency range for the RI test, Rel-15

Type: draftCR		For: Endorsement
					38.124 v15.7.0	  CR-  rev  Cat: F (Rel-15)

					Source: Huawei

Abstract:

The E-UTRA UE EMC TS 36.124 specification is not following the IEC 61000-4-3 specification in terms of the frequency range applicable to the radiated immunity testing. This CR corrects this issue.

Decision:		Revised to R4-2120747

R4-2120747	Draft CR to TS 36.124: correction of the frequency range for the RI test, Rel-15

Type: draftCR		For: Endorsement
					36.124 v15.2.0	  CR-  rev  Cat: F (Rel-15)

					Source: Huawei

Abstract:

The E-UTRA UE EMC TS 36.124 specification is not following the IEC 61000-4-3 specification in terms of the frequency range applicable to the radiated immunity testing. This CR corrects this issue.

Decision:		Endorsed.

R4-2119127	Draft CR to TS 38.124: correction of the frequency range for the RI test, Rel-16

Type: draftCR		For: Endorsement
					38.124 v16.4.0	  CR-  rev  Cat: A (Rel-16)

					Source: Huawei

Abstract:

The E-UTRA UE EMC TS 36.124 specification is not following the IEC 61000-4-3 specification in terms of the frequency range applicable to the radiated immunity testing. This CR corrects this issue.

Decision:		Revised to R4-2120748 (from R4-2119127).

R4-2120748	Draft CR to TS 36.124: correction of the frequency range for the RI test, Rel-16

Type: draftCR		For: Endorsement
					36.124 v16.1.0	  CR-  rev  Cat: A (Rel-16)

					Source: Huawei

Abstract:

The E-UTRA UE EMC TS 36.124 specification is not following the IEC 61000-4-3 specification in terms of the frequency range applicable to the radiated immunity testing. This CR corrects this issue.

Decision:		Endorsed.

R4-2119128	Draft CR to TS 38.124: correction of the frequency range for the RI test, Rel-17

Type: draftCR		For: Endorsement
					38.124 v16.4.0	  CR-  rev  Cat: A (Rel-17)

					Source: Huawei

Abstract:

The E-UTRA UE EMC TS 36.124 specification is not following the IEC 61000-4-3 specification in terms of the frequency range applicable to the radiated immunity testing. This CR corrects this issue.

Decision:		Revised to R4-2120757 (from R4-2119128).

R4-2120757	Draft CR to TS 36.124: correction of the frequency range for the RI test, Rel-17

Type: draftCR		For: Endorsement
					36.124 v16.1.0	  CR-  rev  Cat: A (Rel-17)

					Source: Huawei

Abstract:

The E-UTRA UE EMC TS 36.124 specification is not following the IEC 61000-4-3 specification in terms of the frequency range applicable to the radiated immunity testing. This CR corrects this issue.

Decision:		Endorsed.

R4-2120641	Draft CR to TS 36.124: correction of the frequency range for the RI test, Rel-15

Type: draftCR		For: Endorsement
	36.124 v15.2.0	  CR-  rev  Cat: F (Rel-15)

   Source: Huawei

Abstract:

The E-UTRA UE EMC TS 36.124 specification is not following the IEC 61000-4-3 specification in terms of the frequency range applicable to the radiated immunity testing. This CR corrects this issue.

Decision:		Withdrawn.

R4-2120642	Draft CR to TS 36.124: correction of the frequency range for the RI test, Rel-16

Type: draftCR		For: Endorsement
	36.124 v16.1.0	  CR-  rev  Cat: A (Rel-16)

	Source: Huawei

Abstract:

The E-UTRA UE EMC TS 36.124 specification is not following the IEC 61000-4-3 specification in terms of the frequency range applicable to the radiated immunity testing. This CR corrects this issue.

Decision:		Withdrawn.

4.2.2	BS RF requirements

R4-2120777	Big CR for TS 36.104 Maintenance (Rel-14, CAT F)

Type: CR		For: Agreement
					36.104 v14.10.0	  CR-?  rev  Cat: F (Rel-14)

Source: MCC,CATT
Abstract:

Discussion:

Decision:		Agreed.

R4-2120778	Big CR for TS 36.104 Maintenance (Rel-15, CAT F)

Type: CR		For: Agreement
					36.104 v15.13.0	  CR-?  rev  Cat: F (Rel-15)

Source: MCC,CATT
Abstract:

Discussion:

Decision:		Agreed.

R4-2120779	Big CR for TS 36.104 Maintenance (Rel-16, CAT F)

Type: CR		For: Agreement
					36.104 v16.11.0	  CR-?  rev  Cat: F (Rel-16)

Source: MCC,CATT
Abstract:

Discussion:

Decision:		Agreed.

R4-2120780	Big CR for TS 36.104 Maintenance (Rel-17, CAT A)

Type: CR		For: Agreement
					36.104 v17.3.0	  CR-?  rev  Cat: A (Rel-17)

Source: MCC,CATT
Abstract:

Discussion:

Decision:		Agreed.

R4-2120787	Big CR for TS 36.141 Maintenance (Rel-14, CAT F)

Type: CR		For: Agreement
					36.104 v14.13.0	  CR-?  rev  Cat: F (Rel-14)

Source: MCC,Samsung
Abstract:

Discussion:

Decision:		Agreed.

R4-2120782	Big CR for TS 36.141 Maintenance (Rel-15, CAT F)

Type: CR		For: Agreement
					36.104 v15.14.0	  CR-?  rev  Cat: F (Rel-15)

Source: MCC,Samsung
Abstract:

Discussion:

Decision:		Agreed.

R4-2120783	Big CR for TS 36.141 Maintenance (Rel-16, CAT A)

Type: CR		For: Agreement
					36.104 v16.11.0	  CR-?  rev  Cat: A (Rel-16)

Source: MCC,Samsung
Abstract:

Discussion:

Decision:		Agreed.

R4-2120784	Big CR for TS 36.141 Maintenance (Rel-17, CAT A)

Type: CR		For: Agreement
					36.104 v17.3.0	  CR-?  rev  Cat: A (Rel-17)

Source: MCC,Samsung
Abstract:

Discussion:

Decision:		Agreed.

Refer to Email discussion for [101-e][301] BSRF_Maintenance

Topic #2 BS RF maintenance for LTE Rel-15

Corrections on Band 23 co-location requirements on TS 36.104/141/25.104/141

R4-2117229	Draft CR to TS 36.104: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					36.104 v14.10.0	  CR-  rev  Cat: F (Rel-14)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117230	Draft CR to TS 36.104: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					36.104 v15.13.0	  CR-  rev  Cat: A (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117231	Draft CR to TS 36.104: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					36.104 v16.11.0	  CR-  rev  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117232	Draft CR to TS 36.104: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					36.104 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117233	Draft CR to TS 36.141: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					36.141 v14.13.0	  CR-  rev  Cat: F (Rel-14)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from tables for co-location requirements. Location of Band 24 in the WA tables are also corrected.

Decision:		Endorsed.

R4-2117234	Draft CR to TS 36.141: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					36.141 v15.14.0	  CR-  rev  Cat: A (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117235	Draft CR to TS 36.141: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					36.141 v16.11.0	  CR-  rev  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117236	Draft CR to TS 36.141: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					36.141 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Endorsed.

R4-2117237	Draft CR to TS 25.104: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					25.104 v14.2.0	  CR-  rev  Cat: F (Rel-14)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Not pursued.

R4-2117238	Draft CR to TS 25.104: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					25.104 v15.5.0	  CR-  rev  Cat: A (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Not pursued.

R4-2117239	Draft CR to TS 25.104: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					25.104 v16.0.0	  CR-  rev  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from table for co-location requirements.

Decision:		Not pursued.

R4-2117240	Draft CR to TS 25.141: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					25.141 v14.3.0	  CR-  rev  Cat: F (Rel-14)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from tables for Home BS and co-location requirements.

Decision:		Not pursued.

R4-2117241	Draft CR to TS 25.141: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					25.141 v15.4.0	  CR-  rev  Cat: A (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from tables for Home BS and co-location requirements.

Decision:		Withdrawn.

R4-2117242	Draft CR to TS 25.141: Correction on tables for Band 23 co-location requirements

Type: draftCR		For: Endorsement
					25.141 v16.0.0	  CR-  rev  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Delete the entries for Band 23 from tables for Home BS and co-location requirements.

Decision:		Withdrawn.

4.2.4	Demodulation performance requirements

4.2.4.1	UE demodulation and CSI requirements

Refer to Email discussion for [101-e][317] Demod_Maintenance_UE

Correction on LTE V2X requirements for 36.101

R4-2119030	Draft CR Updates to LTE V2X PSBCH requirements in 36.101 (Rel-14)

Type: draftCR		For: Endorsement
					36.101 v14.20.0	  CR-  rev  Cat: F (Rel-14)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

R4-2119031	Draft CR Updates to LTE V2X PSBCH requirements in 36.101 (Rel-15)

Type: draftCR		For: Endorsement
					36.101 v15.16.0	  CR-  rev  Cat: A (Rel-15)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

R4-2119032	Draft CR Updates to LTE V2X PSBCH requirements in 36.101 (Rel-16)

Type: draftCR		For: Endorsement
					36.101 v16.11.0	  CR-  rev  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

R4-2119033	Draft CR Updates to LTE V2X PSBCH requirements in 36.101 (Rel-17)

Type: draftCR		For: Endorsement
					36.101 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

R4-2119513	Views on PDSCH grant on TRS slots for CSI Reporting Tests

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

4.2.4.2	BS demodulation requirements

Refer to Email discussion for [101-e][316] Demod_Maintenance_BS

Topic #1: LTE demodulation requirements

Clean up CR on LTE PUSCH requirements for TS 36.141/104

R4-2119034	Draft CR Cleanup the square brackets for LTE PUSCH conformance testing in 36.141 (Rel-15)

Type: draftCR		For: Endorsement
					36.141 v15.14.0	  CR-  rev  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

R4-2119035	Draft CR Cleanup the square brackets for LTE PUSCH conformance testing in 36.141 (Rel-16)

Type: draftCR		For: Endorsement
					36.141 v16.11.0	  CR-  rev  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

R4-2119036	Draft CR Cleanup the square brackets for LTE PUSCH conformance testing in 36.141 (Rel-17)

Type: draftCR		For: Endorsement
					36.141 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

R4-2119037	Draft CR Cleanup the square brackets for LTE PUSCH performance requirements in 36.104 (Rel-15)

Type: draftCR		For: Endorsement
					36.104 v15.13.0	  CR-  rev  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

R4-2119038	Draft CR Cleanup the square brackets for LTE PUSCH performance requirements in 36.104 (Rel-16)

Type: draftCR		For: Endorsement
					36.104 v16.11.0	  CR-  rev  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

R4-2119039	Draft CR Cleanup the square brackets for LTE PUSCH performance requirements in 36.104 (Rel-17)

Type: draftCR		For: Endorsement
					36.104 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

5	Rel-16 maintenance for LTE and NR

5.1	NR WIs and TEI

5.1.1	NR-based access to unlicensed spectrum

5.1.1.3	BS RF and conformance testing

Refer to Email discussion for [101-e][301] BSRF_Maintenance

Topic #3 BS RF maintenance for NR-U Rel-16

R4-2118906	Draft CR to TS 38.141-1 with NRTC6 test configuration updates

Type: CR		For: Endorsement
					38.141-1 v16.9.0	  CR-0250  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shaghai Bell

Decision: 		The document was withdrawn.

R4-2118907	Draft CR to TS 38.141-1 with NRTC6 test configuration updates

Type: CR		For: Endorsement
					38.141-1 v17.3.0	  CR-0251  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shaghai Bell

Decision: 		The document was withdrawn.

Correction for NR-U BS RF on TS 38.104

R4-2119198	Maintenance CR to TS 38.104: NR-U BS RF requirements

Type: draftCR		For: Endorsement
					38.104 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: ZTE Corporation

Decision:		Endorsed.

R4-2119199	Maintenance CR to TS 38.104: NR-U BS RF requirements

Type: draftCR		For: Endorsement
					38.104 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: ZTE Corporation

Decision:		Endorsed.

NRTC6 test configuration for TS 38.141-1

R4-2119439	Draft CR to TS 38.141-1 with NRTC6 test configuration updates

Type: draftCR		For: Endorsement
					38.141-1 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shaghai Bell

Decision:		Revised to R4-2120756 (from R4-2119439).

R4-2120756	Draft CR to TS 38.141-1 with NRTC6 test configuration updates

Type: draftCR		For: Endorsement
					38.141-1 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shaghai Bell

Decision:		Endorsed.

R4-2119440	Draft CR to TS 38.141-1 with NRTC6 test configuration updates

Type: draftCR		For: Endorsement
					38.141-1 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shaghai Bell

Decision:		Endorsed.

5.1.1.6	Demodulation and CSI requirements

5.1.1.6.1	General

5.1.1.6.2	UE demodulation and CSI requirements

5.1.1.6.3	BS demodulation requirements

Refer to Email discussion for [101-e][316] Demod_Maintenance_BS

Topic #2: NR-U demodulation requirements

R4-2119040	Draft CR Updates to interlaced PF2 and PF3 performance requirements in 38.104 (Rel-17)

Type: draftCR		For: Endorsement
					38.104 v17.3.0	  CR-  rev  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Revised to R4-2120643 (from R4-2119040).

R4-2120643	Draft CR Updates to interlaced PF2 and PF3 performance requirements in 38.104 (Rel-17)

Type: draftCR		For: Endorsement
					38.104 v17.3.0	  CR-  rev  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

5.1.2	Integrated Access and Backhaul for NR

R4-2120825	Big CR for TS 38.174 Maintenance (Rel-16, CAT F)

Type: CR		For: Agreement
					38.174 v16.4.0	  CR-  rev  Cat: F (Rel-16)

					Source: MCC,CATT

Decision:		Agreed.

R4-2120827	Big CR for TS 38.176-2 Maintenance (Rel-16, CAT F)

Type: CR		For: Agreement
					38.176-2 v16.1.0	  CR-  rev  Cat: F (Rel-16)

					Source: MCC, Nokia

Decision:		Agreed.

R4-2120826	Big CR for TS 38.176-1 Maintenance (Rel-16, CAT F)

Type: CR		For: Agreemnt
					38.176-1 v16.1.0	  CR-  rev  Cat: F (Rel-16)

					Source: MCC,Huawei

Decision:		Agreed.

R4-2120828	Big CR for TR 38.809 Maintenance (Rel-16, CAT F)

Type: CR		For: Agreemnt
					38.809 v16.4.0	  CR-  rev  Cat: F (Rel-16)

					Source: MCC,Samsung

Decision:		Agreed.

------------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][304] NR_IAB_Maintenance, AI 5.1.2.1, 5.1.2.2 – Golebiowski Bartlomiej

R4-2120601	Email discussion summary for [101-e][304] NR_IAB_Maintenance

Type: other		For: Information
					Source: Moderator (Nokia)

Abstract:

Discussion:

Decision:		Revised to R4-2120723 (from R4-2120601).

R4-2120723	Email discussion summary for [101-e][304] NR_IAB_Maintenance

Type: other		For: Information
					Source: Moderator (Nokia)

Abstract:

Discussion:

Decision:		Noted.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2119302 | draft CR to TR 38.809 - Addition of high PSD clause | Huawei | Note pursued |  |
| R4-2120662 | Draft CR to TR 38.809: Single PRB with high PSD transmission | Nokia, Nokia Shanghai Bell, Huawei | Endorsed | (Rev of R4-2117243) Huawei added as co-sourcing company |
| R4-2120663 | Draft CR to 38.809 on IAB conformance testing background | Samsung | Endorsed | (Rev of R4-2118229) |
| R4-2120664 | Draft CR to TS38.176-1 on TM and transmitter conformance testing | Samsung | Endorsed | (Rev of R4-2118230) |
| R4-2120665 | Draft CR to TS38.176-2 on TM and transmitter conformance testing | Samsung | Endorsed | (Rev of R4-2118231) |
| R4-2118999 | CR on removal of Luant modem in radiated performance specification | Ericsson | Not pursued |  |

------------------------------------------------------------------------------------------------------------------------------------------------

5.1.2.1	RF requirements

Single PRB with high PSD

R4-2117243	Draft CR to TR 38.809: Single PRB with high PSD transmission

Type: draftCR		For: Endorsement
					38.809 v16.4.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution provides the text proposal to record the agreement on single PRB with high PSD transmission into TR 38.809.

Decision:		Revised to R4-2120662 (from R4-2117243).

R4-2120662	Draft CR to TR 38.809: Single PRB with high PSD transmission

Type: draftCR		For: Endorsement
					38.809 v16.4.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell, Huawei

Abstract:

This contribution provides the text proposal to record the agreement on single PRB with high PSD transmission into TR 38.809.

Decision:		Endorsed.

R4-2119302	draft CR to TR 38.809 - Addition of high PSD clause

Type: draftCR		For: Agreement
					38.809 v16.4.0	  CR-  rev  Cat:  (Rel-16)

					Source: Huawei

Abstract:

After agreeing that the high PSD case s not applicable for the OAB-MT this captured in the technical report

Decision:		Not pursued.

5.1.2.2	RF conformance testing

5.1.2.2.1	General

R4-2118229	Draft CR to 38.809 on IAB conformance testing background

Type: draftCR		For: Endorsement
					38.809 v16.4.0	  CR-  rev  Cat: F (Rel-16)

					Source: Samsung

Decision:		Revised to R4-2120663 (from R4-2118229).

R4-2120663	Draft CR to 38.809 on IAB conformance testing background

Type: draftCR		For: Endorsement
					38.809 v16.4.0	  CR-  rev  Cat: F (Rel-16)

					Source: Samsung

Decision:		Endorsed.

5.1.2.2.2	Conducted conformance testing

R4-2118230	Draft CR to TS38.176-1 on TM and transmitter conformance testing

Type: draftCR		For: Endorsement
					38.176-1 v16.1.0	  CR-  rev  Cat: F (Rel-16)

					Source: Samsung

Decision:		Revised to R4-2120664 (from R4-2118230).

R4-2120664	Draft CR to TS38.176-1 on TM and transmitter conformance testing

Type: draftCR		For: Endorsement
					38.176-1 v16.1.0	  CR-  rev  Cat: F (Rel-16)

					Source: Samsung

Decision:		Endorsed.

5.1.2.2.3	Radiated conformance testing

R4-2118231	Draft CR to TS38.176-2 on TM and transmitter conformance testing

Type: draftCR		For: Endorsement
					38.176-2 v16.1.0	  CR-  rev  Cat: F (Rel-16)

					Source: Samsung

Decision:		Revised to R4-2120665 (from R4-2118231).

R4-2120665	Draft CR to TS38.176-2 on TM and transmitter conformance testing

Type: draftCR		For: Endorsement
					38.176-2 v16.1.0	  CR-  rev  Cat: F (Rel-16)

					Source: Samsung

Decision:		Endorsed.

IAB with luant modem testing

R4-2118998	IAB with luant modem testing

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view related to luant modem testing.

Decision:		Noted.

R4-2118999	CR on removal of Luant modem in radiated performance specification

Type: discussion		For: Approval
					38.176-2 v	  CR-  rev  Cat:  (Rel-16)

					Source: Ericsson

Abstract:

in this CR, the removel of Luant modem in radiated performance specification of IAB is proposed

Decision:		Not pursued.

5.1.2.4	EMC performance requirements

5.1.2.5	Demodulation and CSI requirements

Refer to Email discussion for [101-e][316] Demod_Maintenance_BS

5.1.2.5.1	General

Topic #3: IAB demodulation requirements

R4-2117324	Draft CR for TS 38.174: correction of the reference

Type: draftCR		For: Endorsement
					38.174 v16.4.0	  CR-  rev  Cat: F (Rel-16)

					Source: CATT

Decision:		Endorsed.

R4-2119392	draftCR to TS 38.176-2 on manufacturer declarations

Type: draftCR		For: Endorsement
					38.176-2 v16.1.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Endorsed.

R4-2119393	draftCR to TS 38.176-1 on applicability rules

Type: draftCR		For: Endorsement
					38.176-1 v16.1.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Endorsed.

5.1.2.5.2	IAB-DU performance requirements

R4-2119026	draftCR on IAB-DU performance requirements in TS 38.174

Type: draftCR		For: Endorsement
					38.174 v16.4.0	  CR-  rev  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

5.1.2.5.3	IAB-MT performance requirements

5.1.3	Enhancements on MIMO for NR

5.1.3.2	Demodulation performance requirements

Refer to Email discussion for [101-e][317] Demod_Maintenance_UE

Applicability for eMIMO multi-TRxP test cases

Session Chair Note: R4-2117428/7429 moved to this AI from AI 5.1.3.1

R4-2117428	Draft CR to 38.101-4 on Applicability for multi-TRxP test cases-R16

Type: draftCR		For: Endorsement
					38.101-4 v16.6.0	  CR-  rev  Cat: F (Rel-16)

					Source: Apple

Decision:		Postponed.

R4-2117429	Draft CR to 38.101-4 on Applicability for multi-TRxP test cases-R17

Type: draftCR		For: Endorsement
					38.101-4 v17.2.0	  CR-  rev  Cat: A (Rel-17)

					Source: Apple

Decision:		Withdrawn.

R4-2120652	LS on UE capability for supporting single DCI transmission schemes for multi-TRP

Type: LS out		For: Approval

To: RAN1; Cc: RAN2
					Source: Apple

Abstract:

Discussion:

Decision:		Approved.

5.1.6	Other NR WIs and Rel-16 NR TEI

5.1.6.1	BS RF requirements

Refer to Email discussion for [101-e][301] BSRF_Maintenance

Topic #4 BS RF maintenance for NR/LTE Rel-16

R4-2118460	CR to TR 38.820: Addition of array antenna model extension in subclause 7.2

Type: CR		For: Agreement
					38.820 v16.1.0	  CR-0004  rev  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

A new section (subclause 7.2.5) is added to capture the array antenna model extension. The model extension is required to study antenna geometries documented in subclause 7.2.3. The parameter sets relevant for the frequency range 7 to 24 GHz is still unde

Decision:		Postponed.

R4-2120752	CR to TR 38.820: Addition of array antenna model extension in subclause 7.2

Type: CR		For: Agreement
					38.820 v16.1.0	  CR-0004  rev  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

A new section (subclause 7.2.5) is added to capture the array antenna model extension. The model extension is required to study antenna geometries documented in subclause 7.2.3. The parameter sets relevant for the frequency range 7 to 24 GHz is still unde

Decision:		Withdrawn.

R4-2119133	CR to TR37.941: removal of outstanding [], Rel-16

Type: draftCR		For: Endorsement
					37.941 v16.3.0	  CR-  rev  Cat: F (Rel-16)

					Source: Huawei

Abstract:

There are still [] found in table 11.2.6.3-1, which shall be removed.

Decision:		Endorsed.

R4-2119187	TP to TR 38.921: MR/LA BS UEM requirements

Type: other		For: Approval
					Source: ZTE Corporation

Session Chair Note: draft CR/CR shall be used instead of TP for this TR.

Decision:		Not pursued.

5.1.6.4	Demodulation and CSI requirements

5.1.6.4.1	UE demodulation requirements

Refer to Email discussion for [101-e][317] Demod_Maintenance_UE

Update on NR V2X requirements

R4-2119041	Draft CR Updates to NR V2X requirements in 38.101-4 (Rel-16)

Type: draftCR		For: Endorsement
					38.101-4 v16.6.0	  CR-  rev  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Decision:		Revised to R4-2120649 (from R4-2119041).

R4-2120649	Draft CR Updates to NR V2X requirements in 38.101-4 (Rel-16)

Type: draftCR		For: Endorsement
					38.101-4 v16.6.0	  CR-  rev  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

R4-2119042	Draft CR Updates to NR V2X requirements in 38.101-4 (Rel-17)

Type: draftCR		For: Endorsement
					38.101-4 v17.2.0	  CR-  rev  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Endorsed.

5.1.6.4.2	CSI requirements

5.1.6.4.3	BS demodulation requirements

Refer to Email discussion for [101-e][316] Demod_Maintenance_BS

Correction on UL TA requirements for TS 38.104

R4-2117687	draftCR for TS38.104 UL TA demod requirement (Rel-16)

Type: draftCR		For: Endorsement
					38.104 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

Parameters correction and bracket removal

Decision:		Merged (with R4-2119170).

R4-2118674	Draft CR to 38.141-2: BS UL TA test condition AWGN level correction (8.2.5)

Type: draftCR		For: Endorsement
					38.141-2 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Keysight Technologies UK Ltd

Decision:		Endorsed.

R4-2118675	Draft CR to 38.141-2: BS UL TA test condition AWGN level correction (8.2.5)

Type: draftCR		For: Endorsement
					38.141-2 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Keysight Technologies UK Ltd

Decision:		Endorsed.

R4-2118954	Correction on 2-step RACH BS demod vendor declaration for conducted BS demod requirements

Type: draftCR		For: Agreement
					38.141-1 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Revised to R4-2120644 (from R4-2118954).

R4-2120644	Correction on 2-step RACH BS demod vendor declaration for conducted BS demod requirements

Type: draftCR		For: Agreement
					38.141-1 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Endorsed.

R4-2118955	Correction on 2-step RACH BS demod vendor declaration for conducted BS demod requirements

Type: draftCR		For: Agreement
					38.141-1 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Endorsed.

Correction on 2-step RACH requirements

R4-2118956	Correction on 2-step RACH BS demod vendor declaration for radiated BS demod requirements

Type: draftCR		For: Agreement
					38.141-2 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Revised to R4-2120645 (from R4-2118956).

R4-2120645	Correction on 2-step RACH BS demod vendor declaration for radiated BS demod requirements

Type: draftCR		For: Agreement
					38.141-2 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Endorsed.

R4-2118957	Correction on 2-step RACH BS demod vendor declaration for radiated BS demod requirements

Type: draftCR		For: Agreement
					38.141-2 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision:		Endorsed.

Correction on HST requirements

R4-2119170	DraftCR 38.104 Correction of BS demod requirements for HST

Type: draftCR		For: Endorsement
					38.104 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

Abstract:

Aligned PUSCH demodulation performance requirements with the final simullation summary [R4-2103972].

Removal of square brackets from UL TA requirements (Rel-16 only).

Removal of square brackets from PRACH requirements.

Decision:		Revised to R4-2120646 (from R4-2119170).

R4-2120646	DraftCR 38.104 Correction of BS demod requirements for HST

Type: draftCR		For: Endorsement
					38.104 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Ericsson, ZTE

Abstract:

Aligned PUSCH demodulation performance requirements with the final simullation summary [R4-2103972].

Removal of square brackets from UL TA requirements (Rel-16 only).

Removal of square brackets from PRACH requirements.

Decision:		Endorsed.

R4-2119171	DraftCR 38.104 Correction of BS demod requirements for HST

Type: draftCR		For: Endorsement
					38.104 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

Abstract:

Aligned PUSCH demodulation performance requirements with the final simullation summary [R4-2103972].

Removal of square brackets from UL TA requirements (Rel-16 only).

Removal of square brackets from PRACH requirements.

Decision: 		The document was withdrawn.

R4-2119172	DraftCR 38.141-1 Correction of BS demod requirements for HST

Type: draftCR		For: Endorsement
					38.141-1 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

Abstract:

Aligned PUSCH demodulation performance requirements with the final simullation summary [R4-2103972].

Removal of square brackets from PUSCH and PRACH requirements (Rel-16 only).

Decision:		Revised to R4-2120647 (from R4-2119172).

R4-2120647	DraftCR 38.141-1 Correction of BS demod requirements for HST

Type: draftCR		For: Endorsement
					38.141-1 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

Abstract:

Aligned PUSCH demodulation performance requirements with the final simullation summary [R4-2103972].

Removal of square brackets from PUSCH and PRACH requirements (Rel-16 only).

Decision:		Endorsed.

R4-2119173	DraftCR 38.141-1 Correction of BS demod requirements for HST

Type: draftCR		For: Endorsement
					38.141-1 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

Abstract:

Aligned PUSCH demodulation performance requirements with the final simullation summary [R4-2103972].

Removal of square brackets from PUSCH and PRACH requirements (Rel-16 only).

Decision: 		The document was withdrawn.

R4-2119174	DraftCR 38.141-2 Correction of BS demod requirements for HST

Type: draftCR		For: Endorsement
					38.141-2 v16.9.0	  CR-  rev  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

Abstract:

Aligned PUSCH demodulation performance requirements with the final simullation summary [R4-2103972].

Removal of square brackets from PRACH requirements.

Decision:		Endorsed.

R4-2119175	DraftCR 38.141-2 Correction of BS demod requirements for HST

Type: draftCR		For: Endorsement
					38.141-2 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

Abstract:

Aligned PUSCH demodulation performance requirements with the final simullation summary [R4-2103972].

Removal of square brackets from PRACH requirements.

Decision: 		The document was withdrawn.

R4-2119256	DraftCR 38.104 Correction of BS demod requirements for HST

Type: draftCR		For: Endorsement
					38.104 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Ericsson, ZT

Abstract:

Aligned PUSCH demodulation performance requirements with the final simullation summary [R4-2103972].

Removal of square brackets from UL TA requirements (Rel-16 only).

Removal of square brackets from PRACH requirements.

Decision:		Endorsed.

R4-2119257	DraftCR 38.141-1 Correction of BS demod requirements for HST

Type: draftCR		For: Endorsement
					38.141-1 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

Abstract:

Aligned PUSCH demodulation performance requirements with the final simullation summary [R4-2103972].

Removal of square brackets from PUSCH and PRACH requirements (Rel-16 only).

Decision:		Endorsed.

R4-2119258	DraftCR 38.141-2 Correction of BS demod requirements for HST

Type: draftCR		For: Endorsement
					38.141-2 v17.3.0	  CR-  rev  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

Abstract:

Aligned PUSCH demodulation performance requirements with the final simullation summary [R4-2103972].

Removal of square brackets from PRACH requirements.

Decision:		Endorsed.

5.1.6.5	NR MIMO OTA test methods (38.827)

R4-2120829	Big CR for TR 38.827 maintenance (Rel-16, CAT F)

Type: CR		For: Agreement
					38.827 v16.4.0	  CR-  rev  Cat: F (Rel-16)

					Source: MCC,vivo

Decision:		Agreed.

Refer to Email discussion for [101-e][325] NR_MIMO_OTA

R4-2118313	Draft CR to TR38.827: Measurement procedure update

Type: draftCR		For: Endorsement
					38.827 v16.4.0	  CR-  rev  Cat: F (Rel-16)

					Source: vivo

Decision:		Endorsed.

5.2	LTE WIs and TEI

5.2.1	BS RF requirements

5.2.4	Demodulation and CSI requirements

5.4	LS response for WP5D (RP-210747) on recommendations ITU-R M.2070 and ITU -R M.2071 on Unwanted Emissions of IMT-Advanced

------------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][315] LS_Response_BSRF, AI 5.4– Johan Sköld

R4-2120623	Email discussion summary for [101-e][315] LS_Response_BSRF

Type: other		For: Information
					Source: Moderator (TBA)

Abstract:

Discussion:

Decision:		Revised to R4-2120724 (from R4-2120623).

R4-2120724	Email discussion summary for [101-e][315] LS_Response_BSRF

Type: other		For: Information
					Source: Moderator (TBA)

Abstract:

Discussion:

Decision:		Noted.

R4-2120661 LS on work towards two new recommendations "Generic unwanted emission characteristics of base / mobile stations using the terrestrial radio interfaces of IMT-2020"

Type: LS out		For: Approval

To: TSG RAN; Cc: RAN5
	Source: Ericsson

Abstract:

Discussion:

Decision:		Revised to R4-2120769 (from R4-2120661).

R4-2120769 LS on work towards two new recommendations "Generic unwanted emission characteristics of base / mobile stations using the terrestrial radio interfaces of IMT-2020"

Type: LS out		For: Approval

To: TSG RAN; Cc: RAN5
	Source: Ericsson

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120660 | LS reply on Revision of Recommendations ITU-R M.2070 and ITU-R M.2071 on Unwanted Emissions of IMT-Advanced | Ericsson, Huawei, Nokia, ZTE | Approved |  |
| R4-2120769 | LS on work towards two new recommendations "Generic unwanted emission characteristics of base / mobile stations using the terrestrial radio interfaces of IMT-2020" | Ericsson | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

R4-2117244	Draft update of ITU-R recommendation M.2070, Chapters 2.5, 2.6 and 2.7

Type: other		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution provides the updates for Recommendations ITU R M.2070, Chapters 2.5, 2.6 and 2.7, for Revision 5 of Recommendation ITU-R M.2012

Decision:		Noted.

R4-2118116	LS reply on Revision of Recommendations ITU-R M.2070 and ITU-R M.2071 on Unwanted Emissions of IMT-Advanced

Type: LS out		For: Approval
					to TSG RAN
					Source: Ericsson, Huawei, Nokia, ZTE

Abstract:

The LS is a response to the request from WP5D on updating ITU-R M.2070. An annex will be added covering the updates for M.2070 (E-UTRA BS and MSR BS). The final response to WP5D will be put together by TSG RAN.

Decision:		Revised to R4-2120660 (from R4-2118116).

R4-2120660	LS reply on Revision of Recommendations ITU-R M.2070 and ITU-R M.2071 on Unwanted Emissions of IMT-Advanced

Type: LS out		For: Approval
					to TSG RAN
					Source: Ericsson, Huawei, Nokia, ZTE

Abstract:

The LS is a response to the request from WP5D on updating ITU-R M.2070. An annex will be added covering the updates for M.2070 (E-UTRA BS and MSR BS). The final response to WP5D will be put together by TSG RAN.

Decision:		Approved.

R4-2118117	Draft update of ITU-R recommendation M.2070, Chapters 1 and 2.1 to 2.4

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

The paper includes parts of the updates needed for M.2070, including operating bands (1) and parts of E-UTRA BS (2.1 to 2.4). Together with the other parts of M.2070, it is proposed to be attached as an Annex to the LS response on M.2070.

Decision:		Noted.

R4-2119134	Draft update of ITU-R recommendation M.2070, Chapters 3.1, 3.2, 3.3

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution, inputs to the updated ITU-R M.2070 recommendation are provided for sections 3.1 (Definitions), 3.2 (Symbols), 3.3 (OBUE requirements).

Decision:		Noted.

R4-2119186	Draft update of ITU-R recommendation M.2070, Chapters 3.5, 3.6 and 3.7

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

6	Rel-17 maintenance for LTE and NR

6.1	Introduction of NR 47 GHz band

6.1.2	BS RF requirements and conformance (38.104/38.141-2)

6.1.4	Demodulation and CSI requirements

Refer to Email discussion for [101-e][317] Demod_Maintenance_UE

Sub-topic 3-1 256QAM PDSCH demod requirements for 47GHz band

R4-2118012	Applicability of 256QAM PDSCH requirements for band n262

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2118685	Applicability of FR2 UE demodulation requirements for NR 47GHz band

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the applicability of the existing FR2 256QAM UE demodulation requirements to NR 47GHz band (n262).

Decision:		Noted.

R4-2118686	CR: Applicability of FR2 demodulation requirements to n262

Type: CR		For: Agreement
					38.101-4 v17.2.0	  CR-0273  rev  Cat: F (Rel-17)

					Source: Ericsson

Abstract:

This draft CR updates the test applicability for n262.

Decision:		Revised to R4-2120650 (from R4-2118686).

R4-2120650	CR: Applicability of FR2 demodulation requirements to n262

Type: CR		For: Agreement
					38.101-4 v17.2.0	  CR-0273  rev  Cat: F (Rel-17)

					Source: Ericsson

Abstract:

This draft CR updates the test applicability for n262.

Decision:		Agreed.

R4-2119023	Discussion on NR UE demodulation for 47GHz band

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119322	Extension of 256QAM PDSCH Demodulation Requirements to 47 GHz band

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2119410	draft CR to TR 38.847 UE performance requirements

Type: draftCR		For: Endorsement
					38.847 v17.0.0	  CR-  rev  Cat: F (Rel-17)

					Source: Intel Corporation

Decision:		Not pursued.

R4-2120651	CR to TR 38.847 UE performance requirements

Type: CR		For: Agreement
					38.847 v17.0.0	  CR-XXXX?  rev  Cat: F (Rel-17)

Abstract:

Discussion:

Decision:		Agreed.

6.2	Introduction of channel bandwidths 35MHz and 45MHz for NR

6.2.2	BS RF requirements

6.2.4	UE demodulation and CSI requirements

6.8	Other WIs and Rel-17 TEI

6.8.1	BS RF requirements

Refer to Email discussion for [101-e][301] BSRF_Maintenance

Topic #5 BS RF maintenance for NR/LTE Rel-17

R4-2118170	CR to TS 38.104 - Fix to band n85 introduction

Type: CR		For: Endorsement
					38.104 v17.3.0	  CR-0357  rev  Cat: F (Rel-17)

					Source: Ericsson

Abstract:

This CR is fixing a an issue after introducing band n85, support for NB-IoT in that band was missing

Decision:		Agreed.

6.8.4	Demodulation and CSI requirements

7	Rel-17 spectrum related WIs for NR

7.3	Introduction of 900 MHz spectrum to 5G NR applicable for Rail Mobile Radio

7.3.3	BS RF requirements

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][314] RAIL_900_1900MHz_BSRF_NWM, AI 7.3.3, 7.3.4– Michal Szydelko

R4-2120602	Email discussion summary for [101-e][314] RAIL_900_1900MHz_BSRF_NWM

Type: other		For: Information
					Source: Moderator (Huawei)

Abstract:

Discussion:

Decision:		Revised to R4-2120725 (from R4-2120602).

R4-2120725	Email discussion summary for [101-e][314] RAIL_900_1900MHz_BSRF_NWM

Type: other		For: Information
					Source: Moderator (Huawei)

Abstract:

Discussion:

Decision:		Noted.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120681 | TPs to TR 38.852: BS RF related agreements for RMR1900 | Huawei | Approved |  |
| R4-2120680 | TPs to TR 38.853: BS RF related agreements for RMR900 | Huawei | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

R4-2118164	RMR 900 MHz - BS RF

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses BS RF requirements aspects when introducing the new RMR 900MHz band

Decision:		Noted.

R4-2119136	Further discussion on the BS RF requirements for RMR900

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution, we provide further analysis for the BS RF requirements for RMR 900.

Decision:		Noted.

R4-2119231	On 900MHz RMR RAN4 BS RF requirements impact due to ECC Decision (20)

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

R4-2119135	Consideration of RMR900-specific requirements as regional requirements

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution, we provide further analysis of the TS 38.104, considering RMR900-specific requirements as regional requirements.

Session Chair Note: Move to this AI from AI 7.3.1

Decision:		Noted.

R4-2119137	TPs to TR 38.853: BS RF related agreements for RMR900

Type: pCR		For: Approval
					38.853 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

in this TP to TR38.853 we provide set of agreements from previous meetings.

Session Chair Note: Move to this AI from AI 7.3.1

Decision:		Revised to R4-2120680 (from R4-2119137).

R4-2120680	TPs to TR 38.853: BS RF related agreements for RMR900

Type: pCR		For: Approval
					38.853 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

in this TP to TR38.853 we provide set of agreements from previous meetings.

Session Chair Note: Move to this AI from AI 7.3.1

Decision:		Approved.

7.4	Introduction of 1900 MHz spectrum to 5G NR applicable for Rail Mobile Radio

7.4.3	BS RF requirements

Refer to Email discussion for [101-e][314] RAIL_900_1900MHz_BSRF_NWM

R4-2118167	RMR 1900 MHz - BS RF

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses BS RF requirements aspects when introducing the new RMR 1900MHz band

Decision:		Noted.

R4-2119139	Further discussion on the BS RF requirements for RMR1900

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution, we provide further analysis for the BS RF requirements for RMR 1900.

Decision:		Noted.

R4-2119233	On 1900MHz RMR RAN4 BS RF requirements impact due to ECC Decision (20)

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

R4-2119138	Consideration of RMR1900-specific requirements as regional requirements

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution, we provide further analysis of the TS 38.104, considering RMR1900-specific requirements as regional requirements.

Session Chair Note: Move to this AI from AI 7.4.1

Decision:		Noted.

R4-2119140	TPs to TR 38.852: BS RF related agreements for RMR1900

Type: pCR		For: Approval
					38.852 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

in this TP to TR38.852 we provide set of agreements from previous meetings.

Session chair Note: Move to this AI from AI 7.4.4

Decision:		Revised to R4-2120681 (from R4-2119140).

R4-2120681	TPs to TR 38.852: BS RF related agreements for RMR1900

Type: pCR		For: Approval
					38.852 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

in this TP to TR38.852 we provide set of agreements from previous meetings.

Session chair Note: Move to this AI from AI 7.4.4

Decision:		Approved.

7.36	Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n259

7.36.3	UE demodulation

Refer to Email discussion for [101-e][317] Demod_Maintenance_UE

R4-2118687	CR: Introduction of Noc power level for n259 PC5

Type: CR		For: Agreement
					38.101-4 v17.2.0	  CR-0274  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

This CR add Noc power level for n259 PC5.

Decision:		Revised to R4-2120754 (from R4-2118687).

R4-2120754	CR: Introduction of Noc power level for n259 PC5

Type: CR		For: Agreement
					38.101-4 v17.2.0	  CR-0274  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

This CR add Noc power level for n259 PC5.

Decision:		Agreed.

8	Rel-17 non-spectrum related work items for NR

8.1	Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs

8.1.1	General

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][325] NR_MIMO_OTA, AI 5.1.6.5, 8.1– Xuan Yi

R4-2120603	Email discussion summary for [101-e][325] NR_MIMO_OTA

Type: other		For: Information
					Source: Moderator (CAICT)

Abstract:

Discussion:

Decision:		Revised to R4-2120726 (from R4-2120603).

R4-2120726	Email discussion summary for [101-e][325] NR_MIMO_OTA

Type: other		For: Information
					Source: Moderator (CAICT)

Abstract:

Discussion:

Decision:		Noted.

R4-2120684	WF on NR MIMO OTA

Type: other		For: Approval
					Source: vivo, CAICT

Abstract:

Discussion:

Decision:		Approved.

R4-2120685	TP to TS 38.151 on Clarification of NF Compensation for FR2 PSP

Type: pCR		For: Approval
	38.151 v0.5.0	  CR-  rev  Cat:  (Rel-17)

Source: Keysight Technologies

Abstract:

Discussion:

Decision:		Withdrawn.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2118299 | 3GPP TS 38.151 v0.6.0 | vivo | Approved | draft TS |
| R4-2118302 | TP to TS 38.151 on updated structure | vivo | Approved | pCR |
| R4-2118313 | Draft CR to TR38.827:Measurement procedure update | vivo | Approved | draftCR |
| R4-2119377 | Clarification of NF Compensation for FR2 PSP | Keysight Technologies UK Ltd | Approved | pCR |
| R4-2119379 | Beam Specific Reference Channel Emulation Curves for Validation Purposes for FR1 CDL-C UMa | Keysight Technologies UK Ltd, Spirent Communications, CMCC, CAICT | Approved | discussion |
| R4-2120684 | WF on NR MIMO OTA | vivo, CAICT | Approved | other |
| R4-2120683 | Updated time plan for FR1 lab alignment and requirement development | CAICT, OPPO | Approved | discussion |

------------------------------------------------------------------------------------------------------------------------------------------------

R4-2117682	FR2 MIMO OTA test points clarification

Type: discussion		For: Approval
					Source: MediaTek Beijing Inc.

Abstract:

Proposal1: Clarify FR2 MIMO OTA test point definition is needed.

Proposal2: Confirm FR2 MIMO OTA test point definition concept is “Alt2 by the example”, and further clarify it in TR38.827 by some manners.

Decision:		Noted.

R4-2117972	Discussion on FR1 MIMO OTA Lab Alignment Timeline

Type: discussion		For: Decision
					Source: Apple

Decision:		Noted.

R4-2118299	3GPP TS 38.151 v0.6.0

Type: draft TS		For: Approval
					38.151 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision:		Approved.

R4-2118302	TP to TS 38.151 on updated structure

Type: pCR		For: Approval
					38.151 v0.5.0	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision:		Approved.

R4-2118303	Views on lab alignment activiy

Type: discussion		For: Approval
					Source: vivo

Decision:		Noted.

R4-2119377	Clarification of NF Compensation for FR2 PSP

Type: pCR		For: Approval
					38.151 v0.5.0	  CR-  rev  Cat:  (Rel-17)

					Source: Keysight Technologies UK Ltd

Decision:		Approved.

8.1.2	Performance requirements

8.1.2.1	Performance Requirements for FR1

R4-2117849	Discussion on FR1 MIMO OTA test campaign

Type: discussion		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2118300	Discussion on FR1 MIMO OTA performance

Type: discussion		For: Approval
					Source: vivo

Decision:		Noted.

R4-2118604	Updated time plan for FR1 lab alignment and requirement development

Type: discussion		For: (not specified)
					Source: CAICT, OPPO

Decision:		Revised to R4-2120683 (from R4-2118604).

R4-2120683	Updated time plan for FR1 lab alignment and requirement development

Type: discussion		For: (not specified)
					Source: CAICT, OPPO

Decision:		Approved.

R4-2118607	PAD candidates information for FR1 MIMO OTA lab alignment

Type: discussion		For: (not specified)
					Source: CAICT

Decision:		Noted.

R4-2118902	For lab alignment

Type: discussion		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2118904	Views on PAD management

Type: discussion		For: Approval
					Source: OPPO

Decision:		Revised to R4-2120753 (from R4-2118904).

R4-2120753	Views on PAD management

Type: discussion		For: Approval
					Source: OPPO

Decision:		Noted.

8.1.2.2	Performance Requirements for FR2

R4-2118142	On FR2 MIMO OTA performance requirements

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2118143	Summary results for alignment of FR2 MIMO OTA

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Decision:		Noted.

8.1.2.3	MU assessment for FR1 and FR2

R4-2118301	Discussion on preliminary FR1 MIMO OTA MU assessment

Type: discussion		For: Approval
					Source: vivo

Decision:		Noted.

R4-2119540	on FR1 measurement uncertianty

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision:		Noted.

8.1.3	Testing methodologies

R4-2119378	Maximum DL Power for NR FR2 MIMO

Type: discussion		For: Approval
					Source: Keysight Technologies UK Ltd

Decision:		Noted.

R4-2117181	Consideration on Probe#3 of FR2 MIMO OTA

Type: discussion		For: Approval
					Source: CMCC

Decision:		Withdrawn.

8.1.3.1	Testing parameters for Performance

8.1.3.2	Optimization of test methodologies

8.1.3.3	Channel model validation

R4-2118587	Validation limits for FR1 CDL-C UMa channel model

Type: discussion		For: (not specified)
					Source: CAICT, CMCC

Decision:		Noted.

R4-2118903	The reference curves of channel model validation

Type: discussion		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2119052	Discussion on FR2 channel model validation

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119093	Channel Model Spatial Validation Pass/Fail limits

Type: other		For: Approval
					Source: Spirent Communications

Abstract:

This contribution contains pass/fail limits for the following:

FR1:	CDL-C-UMa with 2 beams

CDL-C-UMi with 1 beam

Each will contain data for:

Spatial Correlation

Temporal Correlation

PDP (low band, high band)

V/H

Decision:		Noted.

R4-2119253	On MIMO channel validation results

Type: discussion		For: (not specified)
					Source: Xiaomi

Decision:		Withdrawn.

R4-2119379	Beam Specific Reference Channel Emulation Curves for Validation Purposes for FR1 CDL-C UMa

Type: discussion		For: Approval
					Source: Keysight Technologies UK Ltd, Spirent Communications, CMCC, CAICT

Decision:		Approved.

R4-2119541	initial channel validation results

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119558	FR1 MIMO OTA channel model validation results

Type: discussion		For: (not specified)
					Source: CAICT

Decision:		Noted.

8.2	Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)

R4-2119176	TR 38.834 v0.2.0

Type: draft TR		For: Approval
					38.834 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: OPPO

Abstract:

TR 38.834 v020

Decision:		Approved.

8.2.1	General and work plan

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][326] FR1_TRP_TRS_Part1, AI 8.2.1, 8.2.2, 8.2.3– Ruixin Wang

R4-2120604	Email discussion summary for [101-e][326] FR1_TRP_TRS_Part1

Type: other		For: Information
					Source: Moderator (vivo)

Abstract:

Discussion:

Decision:		Revised to R4-2120727 (from R4-2120604).

R4-2120727	Email discussion summary for [101-e][326] FR1_TRP_TRS_Part1

Type: other		For: Information
					Source: Moderator (vivo)

Abstract:

Discussion:

Decision:		Noted.

R4-2120689	WF on FR1 TRP TRS

Type: other		For: Approval
					Source: vivo

Abstract:

Discussion:

Decision:		Approved.

R4-2120690	Reply LS to CTIA on NR phantoms

Type: LS out		For: Approval

To: CTIA Certification
	Source: vivo

Abstract:

Discussion:

Decision:		Withdrawn.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2119176 | TR 38.834 v0.2.0 | OPPO | Approved |  |
| R4-2120687 | 3GPP TS 38.161 v0.0.1 skeleton | vivo, Apple | Approved |  |
| R4-2120686 | TP to TR 38.834 on general aspects and band parameters | vivo | Approved |  |
| R4-2120688 | TP to TR 38.834 on SA test setup and calibration procedure | vivo | Approved |  |
| R4-2120689 | WF on FR1 TRP TRS | vivo | Approved |  |
| R4-2120690 | Reply LS to CTIA on NR phantoms | vivo | Withdrawn |  |

------------------------------------------------------------------------------------------------------------------------------------------------

R4-2117292	on how to treat the UE manufacturing impacts

Type: discussion		For: (not specified)
					Source: Huawei Tech.(UK) Co.. Ltd

Decision:		Withdrawn.

R4-2117850	Discussion on hand phantom and wide range band

Type: discussion		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2117975	Views on frequency test points and manufacturing tolerances

Type: discussion		For: Decision
					Source: Apple

Decision:		Noted.

R4-2118304	TP to TR 38.834 on general aspects and band parameters

Type: pCR		For: Approval
					38.834 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision:		Revised to R4-2120686 (from R4-2118304).

R4-2120686	TP to TR 38.834 on general aspects and band parameters

Type: pCR		For: Approval
					38.834 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision:		Approved.

R4-2118307	Discussion and Reply LS to CTIA on NR phantoms

Type: discussion		For: Approval
					Source: vivo

Decision:		Noted.

R4-2118309	3GPP TS 38.161 v0.0.1 skeleton

Type: draft TS		For: Approval
					38.161 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: vivo, Apple

Decision:		Revised to R4-2120687 (from R4-2118309).

R4-2120687	3GPP TS 38.161 v0.0.1 skeleton

Type: draft TS		For: Approval
					38.161 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: vivo, Apple

Decision:		Approved.

R4-2118898	Consideration on hand phantoms with different hand grip sizes

Type: discussion		For: Approval
					Source: OPPO

Decision:		Noted.

8.2.2	SA test methodology

R4-2117182	Proposals of NR Band n28,n41 and n79 Test Configuration for TRP

Type: discussion		For: Approval
					Source: CMCC

Decision:		Noted.

R4-2118305	TP to TR 38.834 on SA test setup and calibration procedure

Type: pCR		For: Approval
					38.834 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision:		Revised to R4-2120688 (from R4-2118305).

R4-2120688	TP to TR 38.834 on SA test setup and calibration procedure

Type: pCR		For: Approval
					38.834 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision:		Approved.

8.2.3	EN-DC test methodology

R4-2117851	Discussion on power splitting of ENDC

Type: discussion		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2117973	Views on EN-DC test configurations

Type: discussion		For: Decision
					Source: Apple

Decision:		Noted.

R4-2118148	On EN-DC test methodology

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2118306	Discussion on EN-DC power splitting

Type: discussion		For: Approval
					Source: vivo

Decision:		Noted.

R4-2118899	Consideration on power configuration for EN-DC

Type: discussion		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2119252	further discussion on EN-DC power setting

Type: discussion		For: (not specified)
					Source: Xiaomi

Decision:		Noted.

R4-2119542	on EN-DC power splitting of FR1 TRP-TRS test

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision:		Noted.

8.2.4	UE with multiple antennas test methodology

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][327] FR1_TRP_TRS_Part2, AI 8.2.4, 8.2.5– Qifei Liu

R4-2120605	Email discussion summary for [101-e][327] FR1_TRP_TRS_Part2

Type: other		For: Information
					Source: Moderator (OPPO)

Abstract:

Discussion:

Decision:		Revised to R4-2120728 (from R4-2120605).

R4-2120728	Email discussion summary for [101-e][327] FR1_TRP_TRS_Part2

Type: other		For: Information
					Source: Moderator (OPPO)

Abstract:

Discussion:

Decision:		Noted.

R4-2120693	WF on FR1 TRP TRS for UE with multi-antenna

Type: other		For: Approval
					Source: OPPO

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120693 | WF on TRP TRS for UE with multi-antenna v05 | OPPO | Approved |  |
| R4-2120691 | Revision of R4-2119590 TP to TR 38.834 on performance metrics | CAICT | Approved |  |
| R4-2119178 | TP to TR38.834 on UE coordinate system | Rohde & Schwarz, vivo | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

R 4-2117183	Consideration on Tx Antenna Switching Test Methodology

Type: discussion		For: Approval
					Source: CMCC

Decision:		Noted.

R4-2117974	Views on transmit antenna selection test methodology

Type: discussion		For: Decision
					Source: Apple

Decision:		Noted.

R4-2118308	Discussion on UE with Tx and Rx switching

Type: discussion		For: Approval
					Source: vivo

Decision:		Noted.

R4-2118900	Consideration on test method for UE with TAS function on

Type: discussion		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2119543	on TAS ON test

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision:		Noted.

8.2.5	Others

R4-2118901	Consideration on test time reduction for EN-DC

Type: discussion		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2119178	TP to TR 38.834 on Annex A – UE coordinate system

Type: pCR		For: Approval
					38.834 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: ROHDE & SCHWARZ, vivo

Decision:		Approved.

R4-2119590	TP to TR 38.834 on performance metrics

Type: pCR		For: (not specified)
					38.834 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: CAICT

Decision:		Revised to R4-2120691 (from R4-2119590).

R4-2120691	TP to TR 38.834 on performance metrics

Type: pCR		For: (not specified)
					38.834 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: CAICT

Decision:		Approved.

8.5	NR repeater

8.5.1	General

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][305] NR_Repeater_General, AI 8.5.1– Valentin Gheorghiu

R4-2120606	Email discussion summary for [101-e][305] NR_Repeater_General

Type: other		For: Information
					Source: Moderator (Qualcomm)

Abstract:

Discussion:

Decision:		Revised to R4-2120729 (from R4-2120606).

R4-2120729	Email discussion summary for [101-e][305] NR_Repeater_General

Type: other		For: Information
					Source: Moderator (Qualcomm)

Abstract:

Discussion:

Decision:		Noted.

GTW discussion on 4th Nov

Issue 3-2: TDD switching Requirement Definition

- Proposals

  - Option 1: Do not define “end-to-end” delay, just define switching (power ramp-up/-down) requirement

  - Option 2: Introduce “end-to-end” switching delay requirement

  - Option 3: Others

GTW discussion:

Huawei: We are fine with option 1. If we only define power ramp-up/-down, we still need to have the reference for timing.

CATT:  We are missing concept one is TDD switching time requirement another way is transient period. If we want to ensure the switch on/off on Tx/Rx chain, then transient period not enough.

Ericsson: We agree with option 1, the requirement means the transient period from on to off or vice versa.  We also need to define the on/off power/gain.

CMCC: We prefer option 3. We only define the requirement, not specific to any delay or switching.

Nokia: We need to have reference timing which can be signal input timing. Group delay can be included as part of requirements.

QC: We should have reference timing which can be the signal into Repeater input port. For specific devices, we can further discuss test design in conformance case with some exception.

Ericsson: If BS and UE can meet the time mask and on/off power requirements self, what would be impact on Repeater on this requirement. For group delay, we agree not have explicit requirement.

Huawei: We don’t think explicit Group delay requirements needed. The requirement can be similar as UTRA.

Nokia: We don’t have dedicated group delay requirements, but can be part of factor considered in the requirement.

CMCC: The repeater even can work with larger time with ramp up/off compared to transient period without performance degradation in Network. We proposed to define test cases without timing requirements, just defining off gain/power, and the starting point of downlink burst and uplink burst (the starting time of the signal come to the input port of repeater).

E///:  We have different understanding on the group delay agreement, we can consider typical value/declared value when we conduct the conformance test.

Nokia: We seems CMCC’s proposals aligned with our proposal. We should a requirements covering all components.

QC: We have similar comments as Nokia. We still need to have requirements how long repeater to power ramp on. For group delay, if larger group delay allowed, that can be covered by conformance by declaration or fixed value or this need to be covered by switching requirements.

CATT: We need to verify EVM for this requirement. We share same view as QC for group delay. We can further discuss this in conformance part.

Ericsson: If we fix one value, the largest value need to be chosen which make the requirements meaningless. We should separate the discussion on switching requirements for RF chain and group delay requirements.

Huawei: We have different cases, signal directed to UE, or through single/multiple repeaters to UE.

Nokia: We can take some restriction how the requirements applied considering deployment scenario.

E///: We can further discuss how to capture such limitation.

Agreements:

Including switching ([power ramp-up/-down]) time requirements

-FFS whether group delay need to be considered in conformance test and/or in core requirements

-Not precluded the scenarios with large group delay e.g. by declaration basis

-Timing reference as [the signal to repeater input port]

Off power/gain requirements also need to be included

FFS for EVM need to included or not

R4-2120653	WF on system parameters for repeaters

Type: other		For: Approval
					Source: CATT

Abstract:

Discussion:

Decision:		Approved.

R4-2120654	WF on Repeater Classes/Types

Type: other		For: Approval
					Source: Nokia

Abstract:

Discussion:

Decision:		Approved.

R4-2120655	WF on Repeater Switching Requirements and Power Transients

Type: other		For: Approval
					Source: Qualcom

Abstract:

Discussion:

Decision:		Approved.

R4-2120656	WF on Specification Drafting

Type: other		For: Approval
					Source: CMCC

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120653 | WF on system parameters for repeaters | CATT | Approved |  |
| R4-2120654 | WF on Repeater Classes/Types | Nokia | Approved |  |
| R4-2120655 | WF on Repeater Switching Requirements and Power Transients | Qualcomm | Approved |  |
| R4-2120656 | WF on Specification Drafting | CMCC | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

8.5.1.1	System parameters

R4-2118245	Passband definition

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Proposed updated passband definition

Decision:		Noted.

R4-2118467	Discussion on multi-band repeater requirements

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2118742	Multi-band operation of NR repeaters

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

8.5.1.2	Repeater Class/Type

R4-2117316	Discussion on repeater class

Type: other		For: Approval
					Source: CATT

Decision:		Noted.

R4-2117723	Discussion on repeater classes

Type: discussion		For: Decision
					Source: CMCC

Decision:		Noted.

R4-2118468	Discussion on repeater classes

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2118743	Identifying classes/types for NR-Repeaters

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

R4-2119304	Repeater reference points and class definitions

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discussion n the last open issues for repeater class definitions and also the interface diagrams

Decision:		Noted.

R4-2119305	TP to TS 38.106 Repeater reference points and class definitions

Type: pCR		For: Approval
					38.106 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

TP to the technical specification defining the repeater reference points and the class definitions.

Decision:		Postponed.

8.5.1.3	TDD repeater switching requirements

R4-2117317	Discussion on TDD repeater requirements

Type: other		For: Approval
					Source: CATT

Decision:		Noted.

R4-2117724	Discussion on TDD related requirements

Type: discussion		For: Decision
					Source: CMCC

Decision:		Noted.

R4-2118238	Repeaters TDD switching considerations

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Proposals to resolve TDD switching

Decision:		Noted.

R4-2118469	Discussion on TDD repeater switching requirements

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2118725	about Repeater TDD Switching requirement

Type: discussion		For: Agreement
					Source: Keysight Technologies UK Ltd

Decision:		Noted.

R4-2118909	TDD repeater switching requirements

Type: other		For: Approval
					Source: Nokia, Nokia Shaghai Bell

Decision:		Noted.

R4-2119308	Repeater switching requirements

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discuss and make proposals for the remaining open issues around the repeater switching requirements

Decision:		Noted.

8.5.1.4	Others

R4-2117731	TP to TS 38.106 classes definition, DL output power, frequency stability and spurious requirements for FR1

Type: draft TR		For: Endorsement
					38.106 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: CMCC

Decision:		Postponed.

R4-2118470	Discussion on general principle of repeater requirements

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2118908	Discussion on NR Repeaters core specification drafting and work split

Type: other		For: Discussion
					Source: Nokia, Nokia Shaghai Bell

Decision:		Noted.

8.5.2	Conductive RF core requirements

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][306] NR_Repeater_RF_Part1, AI 8.5.2– Chunxia Guo

R4-2120607	Email discussion summary for [101-e][306] NR_Repeater_RF_Part1

Type: other		For: Information
					Source: Moderator (CMCC)

Abstract:

Discussion:

Decision:		Revised to R4-2120730 (from R4-2120607).

R4-2120730	Email discussion summary for [101-e][306] NR_Repeater_RF_Part1

Type: other		For: Information
					Source: Moderator (CMCC)

Abstract:

Discussion:

Decision:		Noted.

GTW discussion on 4th Nov

Issue 1-3-1: ALC testing metric with 10dB increased power compared with maximum value

- Proposals

  - Option 1:  output power and unwanted emission requirements. (CATT, Ericsson)

    - Unwanted emission requirements include OBUE, ACLR and spurious emission.

  - Option 2:  output power and EVM. (CMCC, ZTE)

  - Option 3:  output power, EVM, OBUE and ACLR (Nokia)

  - Option 4: ALC performance when driven above the maximum value should be limited to ensuring acceptable interference but not signal quality. (Qualcomm)

- GTW discussion

Ericsson: We think Unwanted emission including OBUE, ACLR, spurious emission and output power ; but also ok to consider OBUE, ACLR, output power.

Huawei: For +10 dB case, we think EVM can be exception case but can be discussed on conformance phase. We would like to exclude spurious emission considering test time.

Nokia: We also have concern on spurious emission. Do we consider +10 dB considered as normal operation or extreme condition? We can respect majority to exclude EVM if this is the case.

CMCC: We support to include EVM. Under low gain case with larger input power, almost all requirements can be trustable, but for EVM should be verified considering non-linearity.  We are ok not considering spurious emission.

Commscope: OBUE, Spurious emission should be considered.  We don’t think EVM need to be verified during ALC status. If OBUE can be satisfied, the EVM should also be satisfied.

ZTE: Spurious emission not needed, this can be verified by other requirements. OBUE is fine for us. We share similar view as CMCC, EVM still needed.

CATT: We proposed the same approach as UTRA. We don’t think EVM need to be included.

CMCC: We have RF filter which OBUE can be passed, but can’t resolve the issue for EVM. Last PA in non-linear region which also can bring issue for EVM. Now we are discussing out of band emission not within passband.

CATT: Tx gain will be decreased to maintain same output power which means linearity can be guarantee.

Commscope: There is close relationship between OBUE and EVM which means EVM not really necessary for this test. We don’t think ALC is normal condition. EVM requirements shall be verified by normal conditions. For OBUE close to the pass channel, filter not works.

Huawei: We should differentiate conformance and core requirements.

Agreement:

For ALC core requirements, including below requirements OBUE, ACLR , output power, spurious emission and EVM requirements

FFS whether spurious emission and EVM requirements need to be test under ALC test condition which can be further discussed in conformance phase

Issue 2-1-2: inside passband ACLR?

- Proposals

  - Option 1: no inside passband ACLR requirements for both DL and UL (CATT, Ericsson, ZTE)

  - Option 2: DL inside passband ACLR is based on manufacturer’s declaration (CMCC)

  - Option 3: at least OBUE requirements are specified inside passband (Nokia)

  - Option 4: the case for inside passband ACLR is perhaps not very strong considering the levels are very similar to the expected gain and NF performance of the repeater. (Huawei)

- GTW discussion:

CMCC: We can accept option 1 under the condition EVM is verified with all the carriers in the passband are transmitting simultaneously.

Huawei: We are OK, but test condition is conformance issue.

Nokia: We are ok with option 1, but we would like to include OBUE for inside passband concerning system performance impact. We may have cases which only occupy partial channel for transmitted signal.

Ericsson: We are ok with option 1.  In BS specification, we don’t have requirements for ACLR and emission within band. EVM and NF can be used to verified within passband. For CMCC proposed condition, we need to refine the wording.

Nokia: We still need to address the inband issue with partial channel transmission case.

ZTE: We also have same understanding as Nokia. Configurable passband is one of candidate objective on Rel-18 smart repeater discussion, and how to ensure emission on empty carrier?

Agreement:

No inside passband ACLR requirements for both DL and UL

-Further discuss in conformance phase, EVM test condition with all the ‘carriers’ in the passband are transmitting simultaneously.

Issue 2-3-1: inside passband OBUE

- Proposals

  - Option 1: no inside passband OBUE requirements (CATT, Ericsson)

  - Option 2: reusing BS OBUE requirements for both DL and UL(ZTE)

  - Option 3: at least OBUE requirements are specified inside passband (Nokia)

- GTW discussion:

Ericsson: Configurable passband, means the configured passband not include the empty carriers.  For UL performance, OBUE can be useful or not for such case? We would like to see more analysis for the necessary of including such requirements.

ZTE: RF filter can’t be changed.

Nokia: We are open to further discus what kind of requirements required i.e. in-band emission from UE specification.

Huawei: We think NF limit is similar as OBUE limit.

Ericsson: For DL, and UL can be separate for the discussion?

ZTE: This applied for both DL and UL.

Commscope: NF can be covered by OBUE requirements; NF not suitable parameter for this case. NF in UL can be depending on deployment scenario.

Nokia: We think applied for both DL and UL.

Huawei: We think further discussion required for both DL and UL.

Agreement: FFS whether inside passband OBUE requirements or other requirements needed for DL and UL for the case with non-full passband transmission

Issue 3-1-1: EVM limits

- Proposals

  - Option 1: 3.5% (optional and further discuss the possibility of lower value) and 8%, further discuss the usefulness of 17.5%(Ericsson)

  - Option 2: [17.5%], 8% and 3.5%(CMCC)

  - Option 3: 3.5% for repeater supporting 256QAM and 8% for repeater supporting QPSK, 16QAM and 64QAM(ZTE)

  - Option 4: Home repeater class could potentially be used to describe the acceptable QPSK only deployment scenario. Setting a 64QAM system repeater target between 3.5 to 5% 3.5%-5% seems achievable. (Huawei)

  - Option 3: same EVM-% linked together with all modulation schemes as specified for gNBs and UEs. Besides, allow repeater manufacturer to declare highest supported modulation. (Nokia)

  - Option 4: Support of 256 QAM should be a capability declared by the manufacturer for conformance testing. (Qualcomm)

  - Option 5: RAN4 starting assumption is repeater EVM equal to BS or UE EVM. Evaluate performance under this condition to determine impact. (Qualcomm)

- GTW Discussion:

Nokia: We would like to take modulation orders together with EVM limit.

ZTE: 3.5% for repeater supporting 256QAM and 8% for repeater supporting QPSK, 16QAM and 64QAM. Not sure we need to half PI BPSK.

Agreement:

3.5% EVM limit with supporting upper to 256QAM modulation order

8% EVM limit with supporting upper to 64QAM modulation order

Support of 3.5% should be a capability declared by the manufacturer

Issue 3-2-2: NF or equivalent requirements and corresponding test method

- Proposals

  - Option 1: NF by NF analyzer (CMCC, ZTE)

  - Option 2: NF only applies for RF repeater based on analog design and is only limited for FR1 (Keysight)

  - Option 3: perform EVM conformance test with minimum input power (Ericsson, ZTE, Huawei)

Pin = -174 + 10*log10(CBW) + NF -20*log10(EVM)

- GTW discussion:

Huawei: We believe option 3 can be used; but of course in-band emission can be another alternative.

ZTE: Do we need to extend or narrow down th candidate options?

Ericsson: Option 4: Absolute maximum output power with no input within part of passband.

Commscope: NF output power or maximum output power is preferred and more feasible to test. OBUE can also be used in UL.

ZTE: Without input signal, then noise figure will be dominating by first LNA component, not sure whether that’s typical scenario in the field.

Commscope: We don’t think option 3 suitable. We prefer option 4.

Keysight: We are fine removing option 2 together with option 1 removed.

Nokia: Can we go with option 4?

CMCC: For option 4, does mean there is no input signal, or with input signal which partially within passband, partially out of passband?

Huawei: If inside passband OBUE introduced, then option 4 preferred; otherwise option 3 preferred.

Ericsson: We are ok for both option 3 and option 4. Either no input signal, or only only within passband with partially allocation. We can further discuss later for that.

ZTE: We have concern on option 4, whether Repeater gain can supply sufficient gain for link budget with minimum receiver power.

Nokia: if we go with option 4, then no further discussion needed for inside passband OBUE.

Huawei: It’s still FFS whether inside OBUE needed or not.

Agreement: NF can be covered by the equivalent requirements with below options:

  - Option 1: Perform EVM conformance test with minimum input power

  - Option 2: Absolute maximum output power with no input within part of passband e.g. inside passband OBUE

  - Only one option should be selected in the end from RAN4 core requirements aspect

R4-2120666	WF on repeater conducted requirements

Type: other		For: Approval
					Source: CMCC

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120666 | WF on repeater conducted requirements | CMCC | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

8.5.2.1	Transmitted power related requirements

R4-2117318	Discussion on NR repeater conducted output power requirement

Type: other		For: Approval
					Source: CATT

Decision:		Noted.

R4-2117412	Views on NR  repeater output power related requirements for FR1

Type: other		For: Approval
					Source: NTT DOCOMO, INC.

Decision:		Noted.

R4-2117725	Discussion on repeater power related conducted requirements

Type: discussion		For: Decision
					Source: CMCC

Decision:		Noted.

R4-2118085	Discussion on NR repeater conducted output power requirements

Type: other		For: Approval
					Source: NEC

Decision:		Noted.

R4-2118239	Repeater conducted output power

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Remaining output power issues

Decision:		Noted.

R4-2118744	Conducted power related requirements consideration for NR-Repeaters

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

R4-2119207	Discussions on power related requirements for conducted repeater

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2119306	Discuss FR1 conducted Power requirement.

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discuss the format and the accuracy requirement for the FR1 conducted power requirements.

Decision:		Noted.

R4-2119307	TP to TS 38.106 - Repeater conducted output power

Type: pCR		For: Approval
					38.106 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

TP to the technical specification defining the repeater conducted power requirements and associated definitions.

Decision:		Postponed.

8.5.2.2	Emission requirements

R4-2117320	Discussion on NR repeater conducted emission requirements

Type: other		For: Approval
					Source: CATT

Decision:		Noted.

R4-2117726	Discussion on repeater emission related conducted requirements

Type: discussion		For: Decision
					Source: CMCC

Decision:		Noted.

R4-2118240	Repeater FR1 emissions requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Remaining emissions issues

Decision:		Postponed.

R4-2118745	Repeater conducted unwanted emissions

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

R4-2119208	Discussions on emission related requirements for conducted repeater

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2119309	FR1 emissions ACLR

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discuss the potential conducted ALCR requirements both inside and outside the passband

Decision:		Noted.

8.5.2.3	Others

R4-2117321	Discussion on NR repeater FR1 ACRR requirement

Type: other		For: Approval
					Source: CATT

Decision:		Noted.

R4-2117722	Discussion on other RF conducted requirements for NR repeater

Type: discussion		For: Decision
					Source: CMCC

Decision:		Noted.

R4-2118241	Repeater FR1 other

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

EVM, ACRR, out of band gain etc. Issues

Decision:		Noted.

R4-2118724	about NF measurement for Repeater requirement

Type: discussion		For: Information
					Source: Keysight Technologies UK Ltd

Decision:		Noted.

R4-2118910	Signal quality and OOB gain considerations for FR1 NR Repeaters

Type: other		For: Approval
					Source: Nokia, Nokia Shaghai Bell

Decision:		Noted.

R4-2119209	Discussions on other requirements of conducted repeater

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2119214	Repeater conducted and power-related requirements

Type: discussion		For: Approval
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2119310	Repeater EVM requirements

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discuss the repeater EVM requirement concerning the open issues around QPSK only repeaters and general repeater EVM values.

Decision:		Noted.

R4-2119311	Repeater Noise requirements

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discuss the potential repeater noise requirements, how its related to inside passband ALCR and also EVM.

Decision:		Noted.

8.5.3	Radiated RF core requirements

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][307] NR_Repeater_RF_Part2, AI 8.5.3– Richard Kybett

R4-2120608	Email discussion summary for [101-e][307] NR_Repeater_RF_Part2

Type: other		For: Information
					Source: Moderator (Huawei)

Abstract:

Discussion:

Decision:		Revised to R4-2120731 (from R4-2120608).

R4-2120731	Email discussion summary for [101-e][307] NR_Repeater_RF_Part2

Type: other		For: Information
					Source: Moderator (Huawei)

Abstract:

Discussion:

Decision:		Noted.

R4-2120667	WF on Repeater radiated requirements

Type: other		For: Approavl
					Source: Huawei

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120667 | WF on Repeater radiated requirements | Huawei | Aproved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

8.5.3.1	Transmitted power related requirements

R4-2117319	Discussion on NR repeater radiated output power requirement

Type: other		For: Approval
					Source: CATT

Decision:		Noted.

R4-2117413	Views on NR  repeater output power related requirements for FR2

Type: other		For: Approval
					Source: NTT DOCOMO, INC.

Decision:		Noted.

R4-2117727	Discussion on repeater power related radiated requirements

Type: discussion		For: Decision
					Source: CMCC

Decision:		Noted.

R4-2118242	Repeater FR2 output power

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Remaining output power issues

Decision:		Noted.

R4-2118746	Radiated power related requirements consideration for NR-Repeaters

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

R4-2119210	Discussions on power related requirements for radiated repeater

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2119312	FR2 Output power requirements

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discuss the format and the accuracy requirement for the FR2 radiated power requirements.

Decision:		Noted.

R4-2119313	TP to TS 38.106 - Repeater radiated output power

Type: pCR		For: Approval
					38.106 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

TP to the technical specification defining the repeater conducted power requirements and associated definitions.

Decision:		Postponed.

8.5.3.2	Emission requirements

R4-2117729	Discussion on repeater emission related radiated requirements

Type: discussion		For: Decision
					Source: CMCC

Decision:		Noted.

R4-2118243	Repeater FR2 unwanted emissions

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Remaining emissions issues

Decision:		Noted.

R4-2118747	Radiated unwanted emissions for FR2 repeaters

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

R4-2119211	Discussions on emission related requirements for radiated repeater

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2119314	FR2 Emissions ACLR

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discuss the potential radiated ALCR requirements both inside and outside the passband

Decision:		Noted.

8.5.3.3	Others

R4-2117322	Discussion on NR repeater FR2 ACRR requirement

Type: other		For: Approval
					Source: CATT

Decision:		Noted.

R4-2117728	Discussion on other RF radiated requirements for NR repeater

Type: discussion		For: Decision
					Source: CMCC

Decision:		Noted.

R4-2118244	Repeater FR2 other

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

EVM, ACRR, out of band gain etc. Issues

Decision:		Noted.

R4-2118911	Signal quality and OOB gain considerations for FR2 NR Repeaters

Type: other		For: Approval
					Source: Nokia, Nokia Shaghai Bell

Decision:		Noted.

R4-2119212	Discussions on other requirements of radiated repeater

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

8.5.4	EMC core requirements

Refer to Email discussion for [101-e][303] NR_EMC

Topic #4: NR Repeaters EMC

R4-2117585	TPs to TS 38.114 on RF Repeater EMC section 1 (Scope) and section 9 (Immunity)

Type: pCR		For: Approval
					38.114 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson LM

Abstract:

Text Proposal to TS 38.114 on Scope and Immunity

Decision:		Revised to R4-2120637 (from R4-2117585).

R4-2120637	TPs to TS 38.114 on RF Repeater EMC section 1 (Scope) and section 9 (Immunity)

Type: pCR		For: Approval
					38.114 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson LM

Abstract:

Text Proposal to TS 38.114 on Scope and Immunity

Decision:		Approved.

R4-2118053	Discussion on test conditions and performance assessment for NR  repeater EMC tests

Type: discussion		For: Discussion
					Source: ZTE Corporation

Abstract:

In this contribution, we first summarize the progress/WFs of NR RF repeaters and then give some discussion on its EMC test conditions and performance assessment.

Decision:		Noted.

R4-2118064	TP to TS38.114: Emission

Type: pCR		For: Approval
					38.114 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Abstract:

In this contribution, we provide a TP for TS 38.114 Clause 8 based on skeleton.

Decision:		Approved.

R4-2118228	Updating TS38.114 to capture RAN4#101 agreements

Type: draft TS		For: Agreement
					38.114 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Abstract:

To reflect the approved TP into TS 38.114 v0.1.0.

Decision:		Approved.

R4-2119099	TP to TS 38.114 – applicability overview

Type: pCR		For: Approval
					38.114 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

TP to TS 38.114, Section 7.

Decision:		Revised to R4-2120639 (from R4-2119099).

R4-2120639	TP to TS 38.114 – applicability overview

Type: pCR		For: Approval
					38.114 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

TP to TS 38.114, Section 7.

Decision:		Approved.

R4-2119474	TP to TS 38.114 for sections 4.3, 5.3, 6.3, 6.4

Type: pCR		For: Approval
					38.114 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

In this contribution, we provide TP to TS 38.114 (NR Repeater EMC) for sections 4.3, 5.3, 6.3, 6.4.

Decision:		Revised to R4-2120638 (from R4-2119474).

R4-2120638	TP to TS 38.114 for sections 4.3, 5.3, 6.3, 6.4

Type: pCR		For: Approval
					38.114 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

In this contribution, we provide TP to TS 38.114 (NR Repeater EMC) for sections 4.3, 5.3, 6.3, 6.4.

Decision:		Approved.

8.6	Introduction of DL 1024QAM for NR FR1

8.6.1	General

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][308] NR_DL1024QAM_BSRF, AI 8.6.1, 8.6.3, 8.6.4, 8.6.2– Thomas Chapman

R4-2120609	Email discussion summary for [101-e][308] NR_DL1024QAM_BSRF

Type: other		For: Information
					Source: Moderator (Ericsson)

Abstract:

Discussion:

Decision:		Revised to R4-2120732 (from R4-2120609).

R4-2120732	Email discussion summary for [101-e][308] NR_DL1024QAM_BSRF

Type: other		For: Information
					Source: Moderator (Ericsson)

Abstract:

Discussion:

Decision:		Noted.

GTW on 3rd Nov

Issue 1-1: EVM for 1024QAM

- Proposals

  - Option 1: 2.5% for all classes (Nokia, Nokia Shanghai Bell, Ericsson, Verizon, KDDI, SoftBank, NTT DOCOMO, AT&T, SK Telecom, T-Mobile USA, Intel, NEC, Qualcomm) – 13 companies

  - Option 2: 2.5% for LA and MR classes; 2.8% for WA class (ZTE) -1 company

  - Option 3: 2.8% for all BS classes (Huawei, HiSilicon, CATT, China Unicom) – 4 companies

- Discussion

- Huawei: With option 1 Tx EVM 2.5%, larger power back-off required; option 3 which requires less power back off. We think option 3 also supporting 2.5% with in the range.

- ZTE: Better to clarify how much power back-off required with option 1 Tx EVM 2.5%? That’s the reason we proposed option 2.

- CATT: Companies are arguing we should maintain same requirements as LTE. But the channel bandwidth, SU are different between NR and LTE, which bring difficulty to support 2.5%.

- AT&T: We already have similar note as LTE which power back-off can be manufacture declaration basis. We need to respect the fact power back-off allowed to achieve 2.5% to have performance gain.

- SoftBank: We agree with AT&T, we need to make decision since companies now repeated same comments for several meetings.

- Nokia: For WA class, there is no power limitation which means no issue for power-back off. And based on evaluation, 2.5% preferred to achieve more performance gain.

- China Unicom: We share same concern on power back-off raised by Huawei. We are also fine to go with option 2 for comprise.

- Nokia: This feature only applied for UE which closed to BS, why we have power back-off issue? There are some flexibility to allow power back-off for WA BS classes. Option 1 supported by several operators also supported by several infra-vendors.

- Ericsson: Reducing Tx EVM is helpful to have high SINR which bring more benefits to enable DL 1024QAM with performance gain.

- T-Mobile USA: We share same view as AT&T and Ericsson, Nokia. Option 2 is not a good comprise for us. For Fixed wireless access usage, we would like to support 2.5% for WA as well.

- Huawei: If DL 1024QAM configured, then power back-off need to enable for whole channel which may bring impact on cell edge UEs.

- ZTE: We observed power back-off required with 2.5% EVM.

- CATT: We share similar view as Huawei, power back-off impact all the scheduled UE in the cell.

- Nokia: There are many varied factors impact on power back-off which belongs to implemenation issue.

- Tentative Agreement:

DL 1024QAM with 2.5% EVM for all BS classes,

  - Further discuss in conformance test part whether any limitation needed for power back-off during test

Comeback in final round to make final agreement for this issue by end of this meeting.

R4-2120659	WF on 1024QAM conformance requirements

Type: other		For: Approval
					Source: CATT

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120770 | CR for TS 38.104: 1024QAM | Huawei, HiSilicon | Agreed |  |
| R4-2118780 | Draft CR to TS 38.141-1: Introduction of 1024 QAM in FR1 Ericsson | Ericsson | Postponed | Conformance CRs will be discussed next meeting |
| R4-2120658 | UE RX RF requirements for 1024QAM for NR FR1 | Huawei, HiSilicon | Agreed |  |
| R4-2120659 | WF on 1024QAM conformance requirements | CATT | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

8.6.2	UE RF requirements

RMC for RF

R4-2117392	UE RX RF requirements for 1024QAM for NR FR1

Type: other		For: Approval
					Source: CATT

Decision:		Noted.

R4-2117876	CR for TS 38.101-1: 1024QAM

Type: CR		For: Agreement
					38.101-1 v17.3.0	  CR-0944  rev  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Revised to R4-2120658 (from R4-2117876).

R4-2120658	CR for TS 38.101-1: 1024QAM

Type: CR		For: Agreement
					38.101-1 v17.3.0	  CR-0944  rev  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Agreed.

8.6.3	BS TX RF requirements

BS Tx EVM requirements

R4-2118915	BS TX EVM requirement for 1024QAM

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell, Ericsson, Verizon, KDDI, SoftBank, NTT DOCOMO, AT&T, SK Telecom, T-Mobile USA

Decision:		Noted.

R4-2117393	BS TX RF requirements for 1024QAM for NR FR1

Type: other		For: Approval
					Source: CATT

Decision:		Noted.

R4-2117874	BS RF requirements for support of 1024QAM

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2118027	Discussion on Tx EVM requirements for DL 1024QAM in FR1

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2118084	Discussion on BS EVM requirements for 1024QAM

Type: other		For: Approval
					Source: NEC

Decision:		Noted.

R4-2119192	Discussion on BS requirements for NR 1024QAM

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

Simulation results

R4-2117873	Link simulation for support of 1024QAM

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2118913	Simulation results for 1024QAM

Type: other		For: Approval
					Source: Nokia, Nokia Shaghai Bell

Decision:		Noted.

CR for introduction 1024QAM

R4-2117875	CR for TS 38.104: 1024QAM

Type: CR		For: Agreement
					38.104 v17.3.0	  CR-0355  rev  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Revised to R4-2120657 (from R4-2117875).

R4-2120657	CR for TS 38.104: 1024QAM

Type: CR		For: Agreement
					38.104 v17.3.0	  CR-0355  rev  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Revised to R4-2120770 (from R4-2120657).

R4-2120770	CR for TS 38.104: 1024QAM

Type: CR		For: Agreement
					38.104 v17.3.0	  CR-0355  rev  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Agreed.

8.6.4	BS RF conformance testing

R4-2117394	BS test requirements for 1024QAM for NR FR1

Type: other		For: Approval
					Source: CATT

Decision:		Noted.

R4-2118914	DL 1024QAM FR1 impact to NR BS conformance specification

Type: other		For: Approval
					Source: Nokia, Nokia Shaghai Bell

Decision:		Noted.

R4-2118780	Draft CR to TS 38.141-1: Introduction of 1024 QAM in FR1

Type: draftCR		For: Endorsement
					38.141-2 v17.3.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

BS RF conformance requirements are added for transmit signal quality test procedure for 1024 QAM where power back off procedure and applicablity are included.

Decision:		Postponed.

8.6.5	Demodulation and CSI requirements

8.6.5.1	General

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][318] NR_DL1024QAM_Demod_NWM, AI 8.6.5- Jiakai Shi

R4-2120610	Email discussion summary for [101-e][318] NR_DL1024QAM_Demod_NWM

Type: other		For: Information
					Source: Moderator (Ericsson

Abstract:

Discussion:

Decision:		Revised to R4-2120733 (from R4-2120610).

R4-2120733	Email discussion summary for [101-e][318] NR_DL1024QAM_Demod_NWM

Type: other		For: Information
					Source: Moderator (Ericsson

Abstract:

Discussion:

Decision:		Noted.

R4-2120699	WF on DL1024QAM demodulation and CQI reporting requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120699 | WF on DL1024QAM demodulation and CQI reporting requirements | Ericsson | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

R4-2118688	Work plan for UE demodulation requirements for WI DL 1024QAM for NR FR1

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution proposes the work plan for UE demodulation requirements for WI DL 1024QAM for NR FR1

Decision:		Noted.

R4-2117430	On UE demod and CQI reporting requirements with 1024QAM

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

8.6.5.2	PDSCH requirements

R4-2118026	Discussion on PDSCH requirements for DL 1024QAM in FR1

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2118689	UE demodulation requirements for DL 1024QAM

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the scope of UE demodulation requirements for DL 1024QAM WI.

Decision:		Noted.

R4-2119024	Discussion on PDSCH demodulation requiremetns for NR DL FR1 1024QAM

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119323	Discussion on PDSCH Performance Requirements for FR1 1024QAM

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

8.6.5.3	CQI requirements

R4-2118690	CQI reporting requirements for DL 1024QAM

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the scope of CQI reporting requirements for DL 1024QAM WI.

Decision:		Noted.

R4-2119025	Discussion on CQI requirements for NR DL FR1 1024QAM

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

8.8	Enhancement for NR high speed train scenario in FR1

8.8.3	UE demodulation requirements (38.101-4)

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][319] NR_HST_FR1_Demod, AI 8.8.3– Xiaoran Zhang

R4-2120611	Email discussion summary for [101-e][319] NR_HST_FR1_Demod

Type: other		For: Information
					Source: Moderator (CMCC)

Abstract:

Discussion:

Decision:		Revised to R4-2120734 (from R4-2120611).

R4-2120734	Email discussion summary for [101-e][319] NR_HST_FR1_Demod

Type: other		For: Information
					Source: Moderator (CMCC)

Abstract:

Discussion:

Decision:		Noted.

GTW discussion on 5th Nov

Issue 1-1: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme

Agreement: Combined option 3 as following table

| Scenario | UE is capable of: | Applicability for UE capable of HST-SFN advanced receiver | Applicability for UE not capable of HST-SFN advanced receiver |
| --- | --- | --- | --- |
| 1 | (Test 1) FDD 15 + TDD 30 and (Tet 3) TDD 30 + TDD 30 | HST-SFN for (Test 1) HST-DPS for (Test 3) | HST-DPS for (Test 1) and (Test 3) |
| 2 | (Test 2) FDD 15 + FDD 15 and (Test 3) TDD 30 + TDD 30 | HST-SFN for (Test 2) HST-DPS for (Test 3) | HST-DPS for (Test 2) and (Test 3) |
| 3 | Only (Test 1) FDD 15 + TDD 30 | HST-SFN and HST-DPS | HST-DPS |
| 4 | Only (Test 2) FDD 15 + FDD 15 only | HST-SFN and HST-DPS | HST-DPS |
| 5 | Only (Test 3) FDD 30 + TDD 30 | HST-SFN and HST-DPS | HST-DPS |

Issue 1-2: UE capability for HST-SFN CA

  - Option 1: (Intel) define a new Rel-17 UE capability to indicate the support of HST-SFN CA

    - 1a (CMCC): the granularity of the capability is a per-UE

    - 1b (ZTE): the granularity of the capability is per band combination

    - 1c (Huawei): the granularity of the capability is per band

  - Option 2 (Apple, Ericsson, DOCOMO): Do not introduce additional UE capability for HST-SFN CA.

GTW discussion:

Apple: If such capability signalling introduced in Rel-17, does that mean this feature will be supported from Rel-17?

If we introduce for HST_SFN CA case, does this mean we will open a door for other advanced receiver with CA cases in the future?

Ericsson: We have same question as Apple. This related to release independent discussion.

DoCoMO: We have same questions as Apple/Ericsson. Considering LTE experience, advanced receiver capability signalling apply for both single carrier and CA case as perUE basis features.

CMCC: For release independent, introducing new capability signalling doesn’t not mean this feature can’t be supported from earlier release. Similar story happened in Rel-16 HST demodulation, RAN2 has mechanism for that. Rel-16 HST capability signalling only applied for single carrier case.

Huawei: We think release independent and capability signalling can be discussed separately. In history, we already have the case to have separate capability signalling for single carrier and CA case for advanced receiver. LTE and NR are different RATs, we can’t always follow LTE case.

Ericsson: We think release independent and capability signalling are related. If this requirement only applied for Rel-17, then existing signalling can be applied. If this is from Rel-15/16 as early implementation, then new UE capability signalling maybe needed to avoid legacy UE can’t pass this test case. I think RRM delegates also discuss similar thing and the decision should be decided together.

Intel: We don’t think we have the case that UE support in single carrier ad not supporting in CA case.

In real network, there are commercial UEs which already support CA cases, with new capability signalling will bring the issue to NW.

Huawei: We think introducing new capability signalling bring benefits. We have some demand from our products plan.

DoCoMO: We have similar view as E///. We are fine to apply release independent from Rel-15 for HST-SFN CA case without new capability signalling. We prefer to take same as LTE.

CMCC: On RRM session discussion, we have similar discussion; but RRM scope and Demod scope are different. We can discuss separately for RRM issues and demodulation issues and in the end we can draw conclusion based on the input from RRM and demod.

Ericsson: From NW side, this capability signalling is per UE capability; once UE informed to NW, NW assume UE can support this featue regardless of number of  carriers single carrier or CA cases.

Huawei: We want NW can discriminate these different capability UEs with new capability signalling.

ZTE: We prefer to introduce new capability.

Intel: We can’t discriminate UEs since commercial UEs already existing in the network. The safest way is no new capability signalling.

FFS whether new capability signalling needed or not

Further discuss: How to treat existing commercial UEs which can support HST-SFN CA cases, but can’t report new capability signalling

Issue 1-3: Applicability rule between single carrier and CA

Tentative agreements pending on further checking from QC and Ericsson:

Define the applicability rule between single carrier and CA as UE can skip single carrier test case if it explicitly passes the corresponding CA test case for the same SCS. The existing applicability rules for single carrier requirements would still apply.

  - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.

    - UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling

    - UE need to pass Rel-16 FDD HST single tap test.

    - UE can skip Rel-16 TDD and Rel-15 HST single tap test

  - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.

    - UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16

    - UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16

    - UE need to pass Rel-16 FDD HST single tap test.

    - UE can skip Rel-16 TDD and Rel-15 HST single tap tests

R4-2120700	WF on FR1 HST demodulation

Type: other		For: Approval
					Source: CMCC

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120700 | WF on FR1 HST demodulation | CMCC | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

8.8.3.2	PDSCH requirements for CA scenarios

R4-2117431	Discussion on PDSCH CA Requirements in HST

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2117698	Discussion on FR1 HST UE demodulation for CA scenario

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2117707	Views on HST CA tests for FR1

Type: discussion		For: Discussion
					Source: NTT DOCOMO, INC.

Decision:		Noted.

R4-2118007	Views on FR1 HST CA PDSCH performance requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2118118	Updated simulation results for HST-SFN joint transmission for CA scenario

Type: discussion		For: Information
					Source: CMCC

Decision:		Noted.

R4-2118430	Discussion on PDSCH requirements for CA scenarios

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2118683	PDSCH demodulation requirements for CA with HST-SFN scenario

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the open issues of the PDSCH demodulation requirements for CA with HST-SFN scenario.

Decision:		Noted.

R4-2119015	Discussion on PDSCH CA scenarios for NR UE HST FR1 performance requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119016	Simulation results on PDSCH CA scenarios for NR UE HST FR1 performance requirements

Type: other		For: Information
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119544	Views on FR1 HST PDSCH CA Tests

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2118682	Summary for FR1 HST demodulation results

Type: other		For: Information
					Source: Ericsson

Abstract:

This spread sheet summarizes the simulation results of FR1 HST demodulation requirements.

Decision:		Noted.

8.9	NR support for high speed train scenario in FR2

8.9.2	High speed train deployment scenario in FR2

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][320] NR_HST_FR2_Scenario, AI 8.9.2– Jackson(He) Wang

R4-2120612	Email discussion summary for [101-e][320] NR_HST_FR2_Scenario

Type: other		For: Information
					Source: Moderator (Samsung)

Abstract:

Discussion:

Decision:		Revised to R4-2120735 (from R4-2120612).

R4-2120735	Email discussion summary for [101-e][320] NR_HST_FR2_Scenario

Type: other		For: Information
					Source: Moderator (Samsung)

Abstract:

Discussion:

Decision:		Noted.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120701 | TP to TR 38.854 on Deployment Scenario Analysis for FR2 HST | Samsung | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

R4-2118227	TP to TR 38.854 on Deployment Scenario Analysis for FR2 HST

Type: pCR		For: Approval
					38.854 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, Beijing Jiaotong University

Decision:		Revised to R4-2120701 (from R4-2118227).

R4-2120701	TP to TR 38.854 on Deployment Scenario Analysis for FR2 HST

Type: pCR		For: Approval
					38.854 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, Beijing Jiaotong University

Decision:		Approved.

8.9.2.1	Deployment Scenario-A

R4-2118431	Discussion on NR FR2 HST scenario-A

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2119021	Discussion on NR FR2 HST deployment Scenario-A

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119385	On HST FR2 Deployment Scenario-A

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

8.9.2.2	Deployment Scenario-B

R4-2118432	Discussion on NR FR2 HST scenario-B

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2119022	Discussion on NR FR2 HST deployment Scenario-B

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119386	On HST FR2 Deployment Scenario-B

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

R4-2119479	On HST FR2 Deployment Scenario-B

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

8.9.2.3	Others

R4-2119321	Discussion on Issues related to TO/FO Jumps in FR2 HST Deployment Scenarios

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

8.9.5	Demodulation requirements

8.9.5.1	General

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][321] NR_HST_FR2_Demod, AI 8.9.5– Yunchuan Yang

R4-2120613	Email discussion summary for [101-e][321] NR_HST_FR2_Demod

Type: other		For: Information
					Source: Moderator (Samsung)

Abstract:

Discussion:

Decision:		Withdrawn.

R4-2120631	Email discussion summary for [101-e][321] NR_HST_FR2_Demod

Type: other		For: Information
					Source: Moderator (Samsung)

Abstract:

Discussion:

Decision:		Revised to R4-2120736 (from R4-2120631).

R4-2120736	Email discussion summary for [101-e][321] NR_HST_FR2_Demod

Type: other		For: Information
					Source: Moderator (Samsung)

Abstract:

Discussion:

Decision:		Noted.

GTW discussion on 5th Nov

Issue 1-1-1: Channel model for Uni-directional RRH deployment

Candidate options:

  - Option 1(Nokia, Intel):

    - RAN4 to consider the configuration where the train is moving from the serving beam (i.e., in the same direction with RRH antenna panel orientation) as a basis for HST FR2 requirements in uni-directional deployments.

    - If uni-directional deployment in Scenario-A where the train moves towards serving beam is found to be impractical, change the uni-directional channel mode to match the scenario where the train is moving from the serving beam.

cosθt=-Ds_offset+vtDmin2+Ds_offset+vt2,  0<t≤Dsv    (eq. 1)

cosθt=cosθt modDsv,  t>Ds/v  ,   (eq. 2)

where 0 ≤Ds_offset<Ds   (eq. 3)

  - Option 2(Samsung, Ericsson, Huawei, Qualcomm): Following previous agreement

GTW Discussion:

Nokia: In our study, moving toward serving beam may have challenge compared to moving away from beams; meanwhile we agree no difference from demodulation performance perspective.

Samsung: Similar situation may also face for another option and from demodulation perspective, no difference.

Ericsson: From demodulation perspective, no difference.

Agreement: Following previous agreement.

Issue 1-1-2: Channel model for Bi-directional RRH deployment

GTW discussion:

Agreement:  Option 2(a) for for FR2 HST Bi-directional RRH deployment

Issue 1-1-3:  Whether to incorporate propagation delay in DL demodulation channel model?

Candidate options:

  - Option 1(ZTE): To consider propagation delay for DL demodulation, the following model can be considered for propagation delay for DL demodulation

  - Option 2(Ericsson, Samsung, Nokia): Not considering  propagation delay in DL demodulation channel model

  - Option 3(Huawei): Pending on the RRM session discussion

GTW discussion:

Qualcomm: We share some concern as Huawei, we should model propagation delay and delay jump into channel modelling since it will impact receiver requirements even it can be covered by RRM requirements.

Huawei: Time/Doppler frequency tracking are important from receiver performance side. If not incorporated delay jump into channel modelling for demodulation, then why we need to define uni-directional performance requirements?

Intel: Delay jump is realistic in real deployment for uni-directional, however this belongs to RRM scope other than demodulation part. We can further wait RRM decision.

Ericsson: We don’t need to model any absolute delay jump into demodulation as traditional case; other we are wondering test feasibility for OTA test set-up.

Samsung: Delay jump only happens when RRH switching; this is same for FR1 HST scenario. When delay jumps happen, TCI switch will be happened which clearly belongs to RRM scope and we didn’t schedule PDSCH during this TCI switch period.

ZTE: For FR2, propagation delay/delay jump have much impact on FR2 other than FR1, we share same view as Huawei and QUALCOMM.

Samsung: What’s the Propagation delay impact, during this test cases/channel modelling, only single path modelled; no multiple Taps existing which means no relative delay between paths.

Qualcomm: How to guarantee demodulation perspective within RRM requirements?

Nokia: We are hearing companies keeping discussion on RRM related procedures; we didn’t hear any issue from RRM session discussion; we would like to further discuss in RRM session for this potential issue if needed.

Samsung: For DL in RRM session, RRM already draw conclusion in PSS/SSS is robust enough to cover the performance.

ZTE: PSS/SSS using time correlation detection, the performance is robust enough.

Ericsson: We should separate discussion on RRM scope and demodulation scope. In FR1 HST DPS test cases, during test case which require TCI switching and also require L1-RSRP reporting and depending on RSRP reporting, TE may trigger TCI switching at wrong position (not the centre point) this will impact on the demodulation performance; we brought the issue and all the companies we should avoid such situation and focused on demodulation requirements and not to schedule PDSCH in first TRS from TCI switching perioid. In FR1 HST demodulation, we try not to include RRM impact on demodulation part. We should try to verify the receiver performance separately.

QC: Propagation delay difference includes the delay jump.

Agreement:

-TCI switching belongs RRM scope, no need to be modelling and considering in demodulation requirements

-Demodulation requirements will not verify the PDSCH performance during TCI switching period.

Issue 1-1-5: Channel model for UL timing adjustment requirement

Candidate options:

  - Option 1 (CATT, Samsung):

  - Option 2 (Ericsson):

- Δt = (4.6*t/7.2) mod 4.6

    - If the TA test includes the timing jump associated with Scenario A uni-directional, enable that optionally the BS is informed when the timing jump occurs.

    - If there is a TA model including the timing jump change, then BS should declare whether they support Scenario A uni-directional and the associated requirement; for BS that do not support then the previously agreed TA requirement should be applicable.

  - Option 3 (Huawei):

    - Discuss timing jump issue later when conclusions are made in RRM session. If large timing jump cannot be verified in RRM session, the channel model for the UL timing case should be changed to involve the timing jump, e.g.

  - Option 4 (Nokia, Intel, Huawei):

    - Discuss if a new requirement and setup is needed for testing of UL timing at beam change between non-collocated RRHs based on the results of RRM discussion.

GTW discussion:

Nokia: This test introduced from LTE HST, with one moving UE together with one static UE under RRC connected state with TAC. If we follow same mechanism as LTE, then we can follow LTE approach if we want to follow new mechanism from RRM, it will be a separate test case.

Intel: One shot time adjustment belongs to UE side issue; this requirement was used to verify TAC which should similar as LTE.

Ericsson: We don’t need any new BS requirements for one shot automatous TA adjustment.

Agreements:

The baseline assumption: following same approach as existing PUSCH UL timing adjustment requirement (option 1)

  - Further checking RRM conclusion on UL timing at beam change between non-collocated RRHs

GTW discussion on Nov 10th

Issue 1-1-3:  Whether to incorporate propagation delay/delay jump in DL demodulation channel model

GTW discussion:

QUALCOMM: We don’t think RRM conclusion applicable for this issue.

Agreement

- No propagation delay and delay jump modelling in channel model for DL PDSCH demodulation.

Issue 1-1-5: Channel model for UL timing adjustment requirement

GTW discussion:

QC: The new mechanism has adopted in RRM session.

Samsung: This new mechanism didn’t impact UL demodulation requirements.

Agreement: Following same approach as existing PUSCH UL timing adjustment demodulation requirement specified in TS 38.104

R4-2120702	WF on general and UE demodulation requirement for FR2 HST

Type: other		For: Approval
					Source: Samsung

Abstract:

Discussion:

Decision:		Revised to R4-2120775 (from R4-2120702).

R4-2120775	WF on general and UE demodulation requirement for FR2 HST

Type: other		For: Approval
					Source: Samsung

Abstract:

Discussion:

Decision:		Approved.

R4-2120703	WF on BS demodulation requirement for FR2 HST

Type: other		For: Approval
					Source: Nokia, Samsung

Abstract:

Discussion:

Decision:		Approved.

R4-2120704	Simulation assumption for PDSCH requirement for FR2 HST

Type: other		For: Approval
					Source: Intel

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120775 | WF on general and UE demodulation requirement for FR2 HST | Samsung | Approved |  |
| R4-2120703 | WF on BS demodulation requirement for FR2 HST | Nokia, Samsung | Approved |  |
| R4-2120704 | Simulation assumption for PDSCH requirement for FR2 HST | Intel | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

R4-2117591	Simulation summary for Rel-17 FR2 HST

Type: other		For: Information
					Source: Samsung

Decision:		Noted.

R4-2117592	View on demodulation requirement for Rel-17 FR2 HST

Type: discussion		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2118234	FR2 HST Demodulation general issues

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Issues common for BS and UE

Decision:		Noted.

R4-2119320	Discussion on FR2 HST UE Demodulation Performance Requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2119388	On HST FR2 Channel Modeling

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

R4-2119409	General aspects of HST FR2 demodulation performance requirements introduction

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

8.9.5.2	UE demodulation requirements

R4-2117593	Discussion and simulation results of PDSCH requirement for Rel-17 FR2 HST

Type: discussion		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2117694	Discussion on UE demodulation for FR2 HST

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2118008	Views on FR2 HST PDSCH performance requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2118434	Discussion on UE demodulation requirements for HST_FR2

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2118684	UE demodulation requirements for HST FR2

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the UE demodulation requirements for HST FR2.

Decision:		Noted.

R4-2119020	Discussion on UE demodulation requirements for FR2 HST

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

8.9.5.3	BS demodulation requirements

R4-2119387	On HST FR2 BS Demod CR work split

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

R4-2119472	On HST FR2 BS Demod CR work split

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

8.9.5.3.1	PUSCH requirements

R4-2117409	Discussion on PUSCH demodulation requirements for FR2 HST

Type: other		For: Approval
					Source: CATT

Decision:		Noted.

R4-2117594	Discussion and simulation results of PUSCH requirement for Rel-17 FR2 HST

Type: discussion		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2118009	Views on FR2 HST PUSCH performance requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2118235	FR2 HST PUSCH requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Proposals to resolve outstanding PUSCH issues

Decision:		Noted.

R4-2119017	Discussion on PUSCH demodulation requirements for FR2 HST

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119389	On HST FR2 PUSCH Demodulation Requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

8.9.5.3.2	PUSCH with UL timing adjustment requirements

R4-2117410	Discussion on UL TA demodulation requirements for FR2 HST

Type: other		For: Approval
					Source: CATT

Decision:		Noted.

R4-2117595	Discussion and simulation results of UL timing adjustment requirement for Rel-17 FR2 HST

Type: discussion		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2118010	Views on FR2 HST UL TA performance requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2118236	Ul timing adjustment requirement

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Proposal for new TA model

Decision:		Noted.

R4-2119018	Discussion on PUSCH UL TA demodulation requirements for FR2 HST

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119390	On HST FR2 PUSCH with UL Timing Adjustment Requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

8.9.5.3.3	PRACH requirements

R4-2117411	Discussion on PRACH demodulation requirements for FR2 HST

Type: other		For: Approval
					Source: CATT

Decision:		Noted.

R4-2117596	Discussion and simulation results of PRACH requirement for Rel-17 FR2 HST

Type: discussion		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2118011	Views on FR2 HST PRACH performance requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2118237	FR2 HST PRACH requirement

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

PRACH proposed requirement

Decision:		Noted.

R4-2119019	Discussion on PRACH demodulation requirements for FR2 HST

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119391	On HST FR2 PRACH Requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

8.12	Further enhancement on NR demodulation performance

8.12.1	General

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][322] NR_perf_enh2_Demod_Part1, AI 8.12.1, 8.12.2.3– Shan Yang

R4-2120614	Email discussion summary for [101-e][322] NR_perf_enh2_Demod_Part1

Type: other		For: Information
					Source: Moderator (China Telecom)

Abstract:

Discussion:

Decision:		Revised to R4-2120737 (from R4-2120614).

R4-2120737	Email discussion summary for [101-e][322] NR_perf_enh2_Demod_Part1

Type: other		For: Information
					Source: Moderator (China Telecom)

Abstract:

Discussion:

Decision:		Noted.

GTW Discussion on Nov 3rd

Topic #3: Signalling aspects for CRS-IM

Issue 3-1: Parameters needed for LLR weighting

Summary of 1st round discussion:

- Proposals on the needed information for LLR weighting:

  - Option 1: The presence and location of interference CRS (Ericsson, Intel, Nokia)

  - Option 2: The presence of interference CRS (China Telecom, HW, CMCC)

  - Option 3: The presence, location and sequence of interference CRS (Apple, MTK, Intel, HW, QC)

  - Note: the presence information includes the presence of LTE cell, MBSFN configuration, CRS muting information, and the CRS location information includes LTE carrier frequency, bandwidth, v-shift, CRS port number

- Discussion:

Apple: There are different UE implementations; one no need to CRS sequence information which rely on power detection; another required CRS sequence.

Huawei: For scenario 1, if UE always can assume same information from CRS-Rate matching pattern information of severing cell.

We would like to check whether RAN4 group can reach the consensus for basic/reference receiver assumption with common understanding.

Intel: we share similar view as Apple, we shouldn’t mandate/restrict UE implementation. We define minimum requirements with receiver agonistic way.

CMCC: We believe LLR weighting with energy detection is reasonable.

QC: We share similar view with Apple, more available information, less power consumption required with robust performance. We support option 3. We observed performance loss with mismatched information if CRS not present with CRS-IM receiver.

Nokia: Does companies agree with the note? Assuming energy detection to define requirements not precluded UE apply advanced receiver to achieve better performance. Have interference other than CRS interference will improve the performance instead of degrading the performance.

China Telecom: We think the information all needed, but we need to further discuss how to achieve such information case by case. We don’t plan to restrict UE implementation, usually we define reference receiver assumption to define minimum performance requirements. With energy detection, the UE complexity lower than other ways. It’s feasible to do power comparison among symbols.

MTK: We share similar view with Apple, Qualcomm. UE need to implement CRS-IM, and NW only need to provide NWA signalling. Not sure whether power difference detection trustable to get CRS port number.

Ericsson:  We didn’t specify any specific receiver implementation into specification, we only define minimum performance requirements in receiver agonistic way. We don’t expect any other additional information.

China Telecom: 2nd symbol can be used to transmit PDCCH which not feasible to compare symbol 0 and 1 we can compare the power in symbol 8 and 9.

Huawei: If high load on interference cell, then no need to enable CRS_IM receiver.

Apple: We believe all the information in the set required besides LTE cell presence.

Qualcomm: We believe CRS location information also required regardless how these come from.

Intel: We need to focus on which required, then we can discuss how to get them. For reference receiver assumption, we think baseline receiver should think all information available and other advanced receiver maynot require some or full information.

China Telecom: We should decouple which information needed and how obtain such information separately. The CRS location including v_shift and number of ports, in our paper, we explain how to obtain such information by UE power detection. Can we clarify whether CRS sequence used in simulation for the results submitted by companies?

Agreement:

Enable CRS-IM receiver (LLR weighting), below parameters/information needed:

- The presence of CRS information including: the presence of LTE cell, MBSFN configuration, [CRS muting information] if configured

- CRS location information including: LTE carrier frequency, bandwidth, v-shift, CRS port number

- FFS CRS sequence information needed or not which including: Cell ID, [slot number within radio frame information]

FFS how UE can obtain above parameters which can be discussed in case by case manner also depending on deployment scenarios with below candidate alternatives

- UE detection

- NWA signaling including existing signaling or new dedicated signaling

- Following some specific assumption under certain conditions/scenarios

Ericsson: What’s the detection method referred here?

Huawei: We are discussion detection with power detection assumption.

Intel: In our simulation, cell ID/CRS sequence information used for CE of CRS.

Apple/QC: If advanced receiver allowed, CRS sequence information required. CRS sequence information also require slot offset information.

China Telecom: If we have 8 neighbour cells informed, does UE need to power detection for all the cells? If so, the complexity increased compared to without the sequence which can deal more interference cells by power detection.

Huawei: Regarding sub-frame boundary alignment, this is for 15kHz and sync scenario with same TDD pattern.

Intel: For CRS sequence generation, slot number information also required. The complexity can be reduced with aware such information instead of blind detection per slot.

Apple: We think v-shift, CRS port number required for CRS location information, and this information already including CRS-RM information. For sequence, same view as Intel and QC. If such information aware by UE, then depending UE implementation to decide how apply LLR weighting with CE.

Agreement：

Companies are encouraged to bring simulation results for LLR weighting with and without CRS sequence. Based on performance comparison, RAN4 plan to draw conclusion whether CRS sequence information needed or not in Jan 2022 RAN4 meeting.

Interested companies can bring simulation result for power difference between symbols with and without CRS REs) for the detection of number of CRS ports.

- Further discuss the assumption of interference cell PDSCH loading

The complexity and power consumption impact also need to be considered when RAN4 make decision.

Companies continued to discuss the candidate options for the implementation of LLR weighting receiver

Apple: we have concern on 2nd proposal. It is possible but do we need to enforce UE do that? We should not impact the processing timeline.

Intel: Different companies have different assumption for LLR weighting i.e. LLR scaling as output equalization, or adjustment on equalization. First we need to align the receiver assumption. We also need to consider the complexity.

QC: We agree with Apple and Intel.

China Telecom: For proposal 2, we don’t think power detection will bring additional complexity, the only effort was the power comparison.

Huawei: We agree with China Telecom. Anyway power detection always required.

QC: Detecting the pRB of interference would require power comparison with RE level basis.

Issue 3-2: Assumptions on the network configuration

Summary of 1st round discussion:

  - On CRS muting in scenario 1 and 2:

    - Feedback from operators and infra vendors:

- Option 1: CRS muting is not configured (China Telecom, Nokia, CMCC, Huawei, E///)

    - Feedback from chipset and UE vendors:

- Option 2: Not to have any assumption for NW configuration (MTK)

- Need clarification from operators and infra vendors on the probability of configuring CRS muting (Apple, Intel, QC)

Discussion:

QC: We would like to get operators’ feedback whether this can be guarantee there always no CRS muting in network.

Intel: We also need to check operators and infra vendors and if specific assumption applied, then assumption need to be captured into specification when defining requirements.

MTK: We think it’s quite risky to think always no CRS muting in real field; but we would like respect the feedback from operators and infra-vendors’ view.

Apple: How to guarantee CRS muting always not deployed especially for scenario 2?

China Telecom: Our response for scenario 1, CRS muting not deployed. But we would like to hear feedback from others.

Ericsson: From NW point of view, we can’t guarantee CRS muting enable or not enable? We can clarify the assumption in the specification.

Nokia: For scenario 1, CRS muting is not a common scenario which not need to be taking to account for defining Ran4 requirements.

ZTE: We share similar view as Ericsson, we can’t guarantee for this, this implementation basis.

CMCC:  From CMCC perspective, CRS muting not deployed in my network. This should be the most typical scenario.

Huawei: We agree with CMCC.

QC: List some certain assumptions as default assumption; if not aligned with assumption, then NW can indicate to UE.

R4-2120705	WF on CRS-IM receiver in scenarios with overlapping spectrum for LTE and NR

Type: other		For: Approval
					Source: China Telecom

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120706 | Updated work plan for Further enhancement on NR demodulation performance WI | China Telecom | Approved |  |
| R4-2120705 | WF on CRS-IM receiver in scenarios with overlapping spectrum for LTE and NR | China Telecom | Approved |  |
| R4-2117189 | Draft TR 38.833 v0.2.0: Further enhancement on NR demodulation performance | China Telecom | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

R4-2117188	Updated work plan for Further enhancement on NR demodulation performance WI

Type: Work Plan		For: Approval
					Source: China Telecom

Decision:		Revised to R4-2120706 (from R4-2117188).

R4-2120706	Updated work plan for Further enhancement on NR demodulation performance WI

Type: Work Plan		For: Approval
					Source: China Telecom

Decision:		Approved.

R4-2117189	Draft TR 38.833 v0.2.0: Further enhancement on NR demodulation performance

Type: draft TR		For: Approval
					38.833 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: China Telecom

Abstract:

For email approval after the meeting, i.e., to implement the TPs approved during the meeting.

Decision:		Approved.

8.12.2	UE demodulation and CSI requirements

8.12.2.1	MMSE-IRC receiver for inter-cell interference

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][323] NR_perf_enh2_Demod_Part2, AI 8.12.2.1, 8.12.2.2– Belov Dmitry

R4-2120615	Email discussion summary for [101-e][323] NR_perf_enh2_Demod_Part2

Type: other		For: Information
					Source: Moderator (Intel)

Abstract:

Discussion:

Decision:		Revised to R4-2120738 (from R4-2120615).

R4-2120738	Email discussion summary for [101-e][323] NR_perf_enh2_Demod_Part2

Type: other		For: Information
					Source: Moderator (Intel)

Abstract:

Discussion:

Decision:		Noted.

GTW discussion on 5th Nov

Issue 1-3-1: Deployment

Candidate options:

- Option 1 (Apple, Qualcomm, MediaTek, Docomo, Huawei): Only consider homogeneous network deployment

- Option 2 (CMCC, China Telecom, Nokia, Intel, [Ericsson]): Include HetNet deployment

  - CMCC: Cover different configuration under HomoNet and HetNet such as Rx configuration, propagation condition and so on

  - China Telecom: Cover HomNet and HetNet tests for test cases with different Rx number

  - Nokia: 4 Rx for heterogenous while 2Rx and 4 Rx or 2 Rx only for homogenous

  - Intel: Consider the following set of requirements

    - HomoNet: All SSBs are in the same time/frequency resources, TDL-C channel model, 2 and 4 Rx

    - HetNet: Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources, TDL-A channel model, 2 and 4 Rx

    - To reduce the test effort, we can define the applicability rule that verification for one of scenarios is sufficient.

GTW discussion:

China Telecom: We would like to cover deployment scenarios from Homogenous and HeNet scenario. We are fine covering different scenarios with different TDL channel model.

CMCC: HetNet scenario is important for us and observed interference in real deployment. We are open to Intel’s proposals for channel modelling and SSB configuration. We are open to define some test applicable rule if this help to save test effort from UE side.

Qualcomm: From UE perspective, nothing changes between different scenarios. We didn’t see the need to have duplicated test cases.

Huawei: From UE receiver side, it’s agnostic for scenarios; the HetNet scenario, INR may larger than SNR which not proper to introduce test case.

Apple: We share similar view as QC and Huawei. INR under HetNet is higher, which may not suitable for defining requirements. 
Ericsson: We prefer to minimum test cases; we can consider some different test parameter combination for different scenarios e.g. MCS, channel model and test point to reduce test effort.

Intel: We agree the observation from Apple and QUALCOMM, from test coverage aspect, it’s also important to cover different deployment scenario. We can consider covering different scenarios with different combinations.

Apple: We are wondering how to introduce test applicable rules among Homogenous and HetNet scenarios.

Nokia:

Agreement: Introducing test cases with different parameters for Homogenous scenario and HetNet scenario with minimized test cases:

- One test case applied for Homogenous for each duplex mode and 2Rx/4Rx

- One test case applied for HetNet for each duplex mode and 2Rx/4Rx

- If UE supporting both TDD and FDD with same Rx number, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode

Issue 3-1-1: Rank of interference PDSCH for rank of target UE is 2

Candidate options:

- Option 1 (Apple, CMCC, China Telecom, Intel, Qualcomm, MediaTek, ZTE): Only Rank 2 (Target UE) + Rank 2 (Co-schedule UE)

- Option 2 (Ericsson – First Priority): Rank 2 + Variable Rank

- Option 3 (Ericsson – Second Priority, Huawei): Rank 2 + Rank 1 and Rank 2 + Rank 2

GTW discussion:

Agreement: Option 1.

Issue 3-2-4: Signal power assumptions

Candidate options:

- Option 1 (Apple, Intel, Qualcomm, Ericsson, Huawei): Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal

- Option 2 (CMCC, China Telecom): For the signal power assumption, the average total signal power should be the same for each paired UE regardless of the rank allocation.

GTW discussion:

China Telecom: Option 1 means co-scheduled UE will have 3dB power lower than target UE with Rank2 + Rank1.

Apple: This is related to the assumption used for results captured into TP.

Issue 3-2-5: SNR assumptions

Candidate options:

- Option 1 (CMCC, Qualcomm): SNR = STargetUE/N

  - Qualcomm: Regardless of the option chosen, the performance is not going to change. It’s just a matter of how we capture it in the spec. In the end, RAN4 is defining the requirements for target UE. So, in our opinion, we should define the requirement based on target UE SNR only and define the power ratio between the two UEs as discussed in Issue 3-2-4. We believe that this way will be clearer to the reader of the spec.

- Option 2 (Apple, CMCC, China Telecom, Intel, Ericsson, Huawei): SNR = (STargetUE+ SInterfUE)/N

GTW discussion:

QC: We are proposing based on option 1 which friendly for the readers of specification since we are defining UE performance requirements.

Intel: We think from Test aspect, we will observe the total received signal and total received power. We can check the test set-up in LTE phase.

Ericsson: For LTE phase, we use option 2 for simultaneously transmission test. And we prefer option 2 considering SNR definition.

Apple: It’s difficult to separate power allocation for two UEs since this come from together in BS transmission side.

Qualcomm: This not really necessary comes from same direction.

Agreement: Option 2 with further working on the clarification when defining performance requirements into specification.

GTW discussion on Nov 10th

Topic #1: MMSE-IRC receiver for inter-cell interference – Demodulation requirements

Sub-topic 1-1: Common test parameters

Issue 1-1-1: Network type

- Option 1 (Apple, Qualcomm, MediaTek, ZTE, Huawei): Only consider synchronized network

- Option 2 (CMCC, China Telecom, Nokia): Include FDD asynchronized network type in FR1.

  - Option 2A (CMCC, China Telecom, Nokia): Reuse LTE time and frequency offset configuration as the starting point (i.e. 0.33 ms and 0 Hz for cell 1 and 0.67 ms and 0 Hz for cell 1)

  - Option 2B (Ericsson, Nokia): Time offset relative to serving cell is 0.5 slot + half a OFDM symbol.

GTW discussion:

China Telecom: We can’t always assume sync scenario in FDD; the performance impact need to be studied, we believe performance gain still valid under such scenario.

Nokia: We can start with option 1 and further study option 2.

Ericsson: We are fine to go with option 1 as baseline. In real network deployment perspective, async can’t precluded.

CMCC: We would like to check time offset set-up for option 2.

Agreement:

Option 1 as baseline assumption, interested companies are encouraged to bring results for async scenario under the baseline assumption of MMSE-IRC receiver.

  - For asynchronized scenario, reusing LTE configuration of time/frequency offset as starting point.

Topic #3: MMSE-IRC receiver for intra-cell inter-user interference

Sub-topic 3-1: Inter-user interference modeling

Issue 3-1-2: Precoder selection for interference UE

- Option 1 (Apple, Intel, Qualcomm, MediaTek, China Telecom): Select the precoder to ensure orthogonality

- Option 2 (CMCC, Ericsson, ZTE, Huawei): Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure it is not equal to PMI matrix of target UE.

- Option 3 (China Telecom, Intel): Cover both in different cases. e.g. Random for rank 1+1 and orthogonal for rank 2+2

GTW discussion:

Huawei: under option 2, larger performance gain can be observed; also this can’t preclude in network configuration.

QC: Based on the collected results, option 1 has larger performance gain compared to option 2.

China Telecom: We observe option 1 has large performance gain; we think more realistic with option 1.

CMCC: We have different observation, option 2 has larger gain with MMSE-IRC over MMSE.

MTK: We have same observation as QC and China Telecom.

Intel: Based on my review on collected results, option 2 can’t achieve peak throughput under option 2 under some cases and SINR is higher under option 2.

Apple: Option 1 is more practical case. With random precoder, the performance is not robust. We have similar observation as Intel.

Huawei: Option 1 is ideal assumption for network configuration.

Nokia: In general Network will neither using orthogonality nor random precoding in reality we allow control level interference. Both options are possible, but we think option 1 has better performance. We can consider option 3 to cover both cases.

Qualcomm: in RAN4 we follow RAN1 specification design of precoder, in reality network can enable more optimized precoder to improve the performance. In this test, we don’t enable PMI reporting.

Apple: In practical, network will try to improve performance with co-scheduling UEs.

Ericsson: For target UE precoding, we use random precoding; This also not practical in real network.

Intel: For TDD, SRS can be used to decide the precoder.

Sub-topic 3-2: PDSCH parameters

Issue 3-2-2: Propagation condition

- Option 1 (Apple, CMCC, Intel for random PMI, Qualcomm, MediaTek): Only TDLA30-10

- Option 2 (China Telecom, Intel for orthogonal PMI, Huawei): Only TDLC300-100

- Option 3 (Ericsson, Intel): Rank 1 with TDLC300-100 and Rank 2 with TDLA30-10.

GTW discussion:

Agreement: Option 3

R4-2120707	WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC

Type: other		For: Approval
					Source: Intel

Abstract:

Discussion:

Decision:		Approved.

R4-2120708	WF on CSI requirements for inter-cell interference MMSE-IRC

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion:

Decision:		Approved.

R4-2120709	WF on MMSE-IRC receiver for intra-cell inter-user interference

Type: other		For: Approval
					Source: Huawei

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-2120707 | WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC | Intel Corporation | Approved |
| R4-2120707 | WF on CSI requirements for inter-cell interference MMSE-IRC | Ericsson | Approved |
| R4-2120709 | WF on MMSE-IRC receiver for intra-cell inter-user interference | Huawei, HiSilicon | Approved |
| R4-2120710 (Revision of R4-2117435) | TP to TR 38.833: Interference Modeling for intra-cell inter-user interference | Apple | Approved |
| R4-2120711 (Revision of R4-2117738) | TP to TR 38.833: Summary of link level evaluation for inter-user interference suppression for MU-MIMO | CMCC | Approved |
| R4-2120712 (Revision of R4-2118002) | TP to TR 38.833: Link level simulation results for inter-user interference suppression for MU-MIMO | Intel Corporation | Approved |
| R4-2120713 (Revision of R4-2118412) | draftTP to TR38.833: receiver structure for intra-cell inter-user IRC | Ericsson | Approved |
| R4-2120714 (Revision of R4-2118864) | TP to TR 38.833: Conclusion for phase I evaluation on inter-user interference suppression for MU-MIMO scenario | China Telecom | Approved |
| R4-2120715 (Revision of R4-2119049) | TP: Introduction of simulation assumptions for intra cell inter user MMSE-IRC receiver | Huawei, HiSilicon | Approved |
| R4-2120716 (Revision of R4-2119402 | TP to TR 38.833 Scenario for inter-user interference suppression for MU-MIMO | MediaTek inc. | Approved |

------------------------------------------------------------------------------------------------------------------------------------------------

8.12.2.1.1	PDSCH requirements

R4-2117432	Discussion on PDSCH requirements in intercell interference scenarios

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2117642	On general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we have provided our views on various open issues with relation to demodulation requirements for inter-cell interference. We make proposals concerning Network type, Propagation Condition and deployment assumptions for homo- and hetero

Decision:		Noted.

R4-2117735	Discussion on R17 demodulation enhancement for inter-cell interference suppressing

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2117736	Simulation results for inter-cell interference suppressing

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2117994	Views on MMSE-IRC receiver for inter-cell interference test

Type: discussion		For: Discussion
					Source: NTT DOCOMO, INC.

Decision:		Noted.

R4-2117998	Discussion on PDSCH demodulation MMSE-IRC requirements for scenario with inter-cell interference

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2117999	Summary of PDSCH simulation results for inter-cell interference suppression

Type: other		For: Information
					Source: Intel Corporation

Decision:		Noted.

R4-2118406	Remaining issues on PDSCH requirement for inter-cell interference

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses PDSCH requirements for inter-cell IRC

Decision:		Noted.

R4-2118408	Simulation results on PDSCH performance for inter-cell interference

Type: other		For: Information
					Source: Ericsson

Abstract:

This contribution submits our simulation results for PDSCH demodulation for inter-cell IRC

Decision:		Noted.

R4-2118861	On PDSCH requirements for UE MMSE-IRC receiver for inter-cell interference suppression

Type: discussion		For: Discussion
					Source: China Telecom

Decision:		Noted.

R4-2119043	Discussion on PDSCH demodulation requirements for inter-cell MMSE-IRC receiver

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119044	Simulation results for PDSCH demodulation requirements for inter-cell MMSE-IRC receiver

Type: other		For: Information
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119403	Discussion on the PDSCH requirements for scenarios with inter-cell inter-user interference

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision:		Noted.

R4-2119545	Views on Inter-cell Interference PDSCH Tests

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

8.12.2.1.2	CSI requirements

R4-2117433	Discussion on CSI reporting requirements in intercell interference scenarios

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2117641	On CSI requirements for inter-cell interference MMSE-IRC

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we have provided our views on various open issues with relation to CSI requirements and the Interference model. We make proposals concerning PMI reporting requirements and interference model.

Decision:		Noted.

R4-2117737	Discussion on CSI reporting related issues for inter-cell interference MMSE-IRC

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2118000	Discussion on CSI reporting requirements with MMSE-IRC for scenario with inter-cell interference

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2118407	Remaining issues on CSI reporting requirements for inter-cell interference

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses CSI reporting requirements for inter-cell IRC

Decision:		Noted.

R4-2118409	Simulation results on CSI reporting for inter-cell interference

Type: other		For: Information
					Source: Ericsson

Abstract:

This contribution submits our simulation results for CSI reporting for inter-cell IRC

Decision:		Noted.

R4-2118862	On CSI requirements for UE MMSE-IRC receiver for inter-cell interference suppression

Type: discussion		For: Discussion
					Source: China Telecom

Decision:		Noted.

R4-2119045	Discussion on CSI tests for inter-cell MMSE-IRC receiver

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119404	Discussion on the CSI requirements for scenarios with inter-cell inter-user interference

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision:		Noted.

R4-2119548	Views on Inter-cell Interference CSI Reporting Tests

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

8.12.2.2	MMSE-IRC receiver for intra-cell inter-user interference

R4-2117434	Discussion on PDSCH requirements in MU-MIMO scenarios

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2117435	TP to TR 38.833: Interference Modeling for intra-cell inter-user interference

Type: pCR		For: Approval
					38.833 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision:		Revised to R4-2120710 (from R4-2117435).

R4-2120710	TP to TR 38.833: Interference Modeling for intra-cell inter-user interference

Type: pCR		For: Approval
					38.833 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision:		Approved.

R4-2117738	TP to TR 38.833: Summary of link level evaluation for inter-user interference suppression for MU-MIMO

Type: draft TR		For: Endorsement
					38.833 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: CMCC

Decision:		Revised to R4-2120711 (from R4-2117738).

R4-2120711	TP to TR 38.833: Summary of link level evaluation for inter-user interference suppression for MU-MIMO

Type: draft TR		For: Endorsement
					38.833 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: CMCC

Decision:		Approved.

R4-2118001	Discussion on MMSE-IRC requirements for scenario with intra-cell inter-user interference

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2118002	TP to TR 38.833: Link level simulation results for inter-user interference suppression for MU-MIMO

Type: pCR		For: Approval
					38.833 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: Intel Corporation

Decision:		Revised to R4-2120712 (from R4-2118002).

R4-2120712	TP to TR 38.833: Link level simulation results for inter-user interference suppression for MU-MIMO

Type: pCR		For: Approval
					38.833 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: Intel Corporation

Decision:		Approved.

R4-2118410	Remaining issues on MMSE-IRC receiver for intra-cell inter-user interference

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses PDSCH requirements for MU-MIMO

Decision:		Noted.

R4-2118411	Simulation results on PDSCH performance for intra-cell inter-user interference

Type: other		For: Information
					Source: Ericsson

Abstract:

This contribution submits our simulation results for PDSCH demodulation for intra-cell inter-user IRC

Decision:		Noted.

R4-2118412	draftTP to TR38.833: receiver structure for intra-cell inter-user IRC

Type: draft TR		For: Endorsement
					38.833 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

This draft TP submits the receiver structure of MU-MIMO receiver

Decision:		Revised to R4-2120713 (from R4-2118412).

R4-2120713	draftTP to TR38.833: receiver structure for intra-cell inter-user IRC

Type: draft TR		For: Endorsement
					38.833 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

This draft TP submits the receiver structure of MU-MIMO receiver

Decision:		Approved.

R4-2118863	Views on UE MMSE-IRC receiver for intra-cell inter-user interference suppression

Type: discussion		For: Discussion
					Source: China Telecom

Decision:		Noted.

R4-2118864	TP to TR 38.833: Conclusion for phase I evaluation on inter-user interference suppression for MU-MIMO scenario

Type: pCR		For: Approval
					38.833 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: China Telecom

Decision:		Revised to R4-2120714 (from R4-2118864).

R4-2120714	TP to TR 38.833: Conclusion for phase I evaluation on inter-user interference suppression for MU-MIMO scenario

Type: pCR		For: Approval
					38.833 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: China Telecom

Decision:		Approved.

R4-2119046	Discussion on intra cell inter-user MMSE-IRC receiver

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119047	Simulation results for intra cell inter-user MMSE-IRC receiver

Type: other		For: Information
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119048	Summary of simulation results for intra cell inter-user MMSE-IRC receiver

Type: other		For: Information
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119049	TP: Introduction of simulation assumptions for intra cell inter user MMSE-IRC receiver

Type: pCR		For: Approval
					38.833 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Revised to R4-2120715 (from R4-2119049).

R4-2120715	TP: Introduction of simulation assumptions for intra cell inter user MMSE-IRC receiver

Type: pCR		For: Approval
					38.833 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Approved.

R4-2119402	TP to TR 38.833 Scenario for inter-user interference suppression for MU-MIMO

Type: pCR		For: Approval
					38.833 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: MediaTek inc.

Decision:		Revised to R4-2120716 (from R4-2119402).

R4-2120716	TP to TR 38.833 Scenario for inter-user interference suppression for MU-MIMO

Type: pCR		For: Approval
					38.833 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: MediaTek inc.

Decision:		Approved.

R4-2119405	Discussion on the PDSCH requirements for scenarios with intra-cell inter-user interference

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision:		Noted.

R4-2119549	Views on Intra-cell Inter-user Interference Scenarios

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

8.12.2.3	CRS-IM receiver in scenarios with overlapping spectrum for LTE and NR

R4-2119050	Discussion on open issues for CRS-IM receiver

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119051	Simulation results for CRS-IM receiver

Type: other		For: Information
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119092	On Implementation aspects for interference cell CRS-RM

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we have provided our views on the open issues with relation to implementation aspects for interference cell CRS-RM. The intention is to highlight the need for a solution to the interference cell CRS-RM issue and add one additional pos

Decision:		Noted.

R4-2119406	Discussion on CRS interference handling in NR

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision:		Noted.

R4-2119550	Views on CRS Interference Mitigation in NR

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

8.12.2.3.1	Receiver assumption

R4-2117436	Discussion on CRS interference mitigation in NR

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2117639	On NR UE Receiver Assumptions for CRS IM

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we have provided our views on various open issues with relation to CRS-IM receiver baseline requirements. We propose to use Scenario 1 with LLR Weighting as baseline for CRS-IM requirement definition.

Decision:		Noted.

R4-2118003	CRS-IM receiver assumptions for scenarios with overlapping spectrum for LTE and NR

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2118396	Discussion on receiver assumption for CRS interference handling

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discuss left open issues related to the receiver assumption

Decision:		Noted.

R4-2118865	Discussion on the receiver assumption for CRS-IM requirement definition

Type: discussion		For: Discussion
					Source: China Telecom

Decision:		Noted.

8.12.2.3.2	Necessity of Network assistant signaling

R4-2117437	Discussion on Network Assistance for CRS-IM in NR

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2117640	On Necessity of Network assistant signalling

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we have provided our views on various open issues with relation to the need for Network Assistance signaling for CRS-IM scenarios. We make proposal to not define requirements for NWA and not include misdetection in the requirements if

Decision:		Noted.

R4-2117739	Discussion on the network assistant signaling necessity for CRS-IM

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2118004	Network assistance signaling for CRS-IM receiver for scenarios with overlapping spectrum for LTE and NR

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2118397	Discussion on the necessity of network assistance signling

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discuss the necessity of network assistance signling

Decision:		Noted.

R4-2118866	Discussion on the necessity of network assistance signalling and UE capability signalling for CRS-IM

Type: discussion		For: Discussion
					Source: China Telecom

Decision:		Noted.

8.12.2.3.3	Test set-up

R4-2117740	Discussion on the test setup for CRS-IM

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2118005	Test setup for CRS-IM receiver requirements for scenarios with overlapping spectrum for LTE and NR

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2118398	Discussion on the test set-up for CRS interference handling

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discuss the detailed test setup for phase II defining requirement

Decision:		Noted.

R4-2118867	Discussion on the test setup for CRS-IM reqquirement definition

Type: discussion		For: Discussion
					Source: China Telecom

Decision:		Noted.

8.12.3	BS demodulation requirements

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][324] NR_perf_enh2_Demod_Part3, AI 8.12.3- Tricia Li

R4-2120616	Email discussion summary for [101-e][324] NR_perf_enh2_Demod_Part3

Type: other		For: Information
					Source: Moderator (Huawei)

Abstract:

Discussion:

Decision:		Revised to R4-2120739 (from R4-2120616).

R4-2120739	Email discussion summary for [101-e][324] NR_perf_enh2_Demod_Part3

Type: other		For: Information
					Source: Moderator (Huawei)

Abstract:

Discussion:

Decision:		Noted.

R4-2120717	WF for FR1 PUSCH with 256QAM performance requirements

Type: other		For: Approval
					Source: Huawei

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120717 | WF for FR1 PUSCH with 256QAM performance requirements | Huawei | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

8.12.3.1	PUSCH demodulation requirements for FR1 256QAM

R4-2117391	Simulation results for PUSCH 256QAM performance requirement

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2117527	Simulation results for PUSCH demodulation requirements for FR1 256QAM

Type: discussion		For: Information
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we have provided our simulation results for PUSCH demodulation requirements for FR1 256QAM.

Decision:		Noted.

R4-2117590	Simulation results for PUSCH with 256QAM

Type: discussion		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2117685	Discussion on FRC for PUSCH 256QAM

Type: other		For: Discussion
					Source: Ericsson

Abstract:

FRC table for PUSCH 256QAM

Decision:		Noted.

R4-2117686	Simulation result for PUSCH 256QAM

Type: other		For: Information
					Source: Ericsson

Abstract:

Simulation results for all agreed bandwidth and SCS

Decision:		Noted.

R4-2117741	Simulation results for PUSCH 256QAM performance

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2118006	Simulation results for FR1 256QAM PUSCH demodulation requirements

Type: other		For: Information
					Source: Intel Corporation

Decision:		Noted.

R4-2118435	Simulation results for FR1 UL 256QAM demodulation requirement

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2118868	Simulation results on PUSCH FR1 256QAM demodulation requirements

Type: discussion		For: Discussion
					Source: China Telecom

Decision:		Noted.

R4-2119027	Simulation results for PUSCH demodulation requirements for FR1 UL 256QAM

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Revised to R4-2120627 (from R4-2119027).

R4-2120627	Simulation results for PUSCH demodulation requirements for FR1 UL 256QAM

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119028	Summary of simulation results for PUSCH requirements for FR1 UL 256QAM

Type: discussion		For: Information
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119029	Considerations for CR splitting for FR1 PUSCH 256QAM performance requirements

Type: other		For: Information
					Source: Huawei, HiSilicon

Decision:		Noted.

8.13	Solutions for NR to support non-terrestrial networks (NTN)

8.13.1	General

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][309] NTN_Solutions_Part1, AI 8.13.1- Dorin Panaitopol

R4-2120617	Email discussion summary for [101-e][309] NTN_Solutions_Part1

Type: other		For: Information
					Source: Moderator (Thales)

Abstract:

Discussion:

Decision:		Revised to R4-2120740 (from R4-2120617).

R4-2120740	Email discussion summary for [101-e][309] NTN_Solutions_Part1

Type: other		For: Information
					Source: Moderator (Thales)

Abstract:

Discussion:

Decision:		Noted.

GTW Discussion on 2nd Nov

Topic #1 system parameter

Issue 1-1-1: NTN Satellite Band Prefix

- Proposals

  - Option 1: “n”

  - Option 2: “s”

- Discussion:

Ericsson: We are ok with option 1 with a note in the table.

Qualcomm: We support option 1 with a note.

ZTE: We share same view as E/// and Qualcomm, ‘n’ means newRat better to use this to avoid confusion.

Nokia: We are ok OP1 with a note.

CATT: We are OK for option 1, but not sure whether note still needed since new spec will be introduced for NTN operation in RAN4.

Huawei: We are ok with option 1, but still need to improve the wording in above which show as a example.

Thales: We can further work on the note and the table in email discussion.

Agreement:

Using ‘n’ as prefix, companies are continued the effort on the “note” and table for introduction of NTN satellite bands.

Topic #2: NTN Satellite gNB Class/Type

Issue 2-1-1: Satellite NTN BS Type 1-C

- Proposals

  - Option 1: non-AAS architecture (1-C) is confirmed as being out of scope of the Rel-17 NTN work.

- Discussion:

- Huawei: we can remove 1-C from Rel-17 NTN Work.

- Agreement: non-AAS architecture (1-C) is confirmed as being out of scope of the Rel-17 NTN work.

Issue 2-1-2: Satellite NTN BS Type 1-O

- Proposals

  - Option 1: To support the BS type 1-O in Rel-17; BS type 1-O (AAS architecture) consideration for NTN gNB is confirmed to be included in Rel-17.

  - Option 2: Further check the progress on BS type 1-O in Nov. 2021 RAN4 meeting and decide if BS type 1-O to be further considered in Rel-17.

  - Option 3: Further check the progress on BS type 1-O in Jan. 2022 RAN4 meeting and decide if BS type 1-O to be further considered in Rel-17.

- Discussion:

ZTE: We think 1-O shall be supported in Rel-17, there are much commonality among 1-H and 1-O, we didn’t see much difference; we can check the status in Jan.2022 RAN4 meeting.

Huawei: Both 1-O and 1-H are AAS based on architecture, we shall support both of them, we can contribute to complete the work in time.

Ericsson: We are fine to include 1-O at this moment and check the status by end of WI core part.

Thales: We are fine to include 1-O. There is another issue 2-1-4 can be discussed together.

CATT: No strong view whether include 1-O in Rel-17 or Rel-18 but worry about the progress including spec drafting, requirements introduction.

Agreement:

Further check the progress on BS type 1-O in Jan. 2022 RAN4 meeting and decide if BS type 1-O to be further considered in Rel-17.

For Jan 2022 meeting, RAN4 can consider to have dedicated AIs for BS Type 1-O requirements.

No need to consider BS 1-O and 1-H as package from RAN4 requirements introduction perspective.

Issue 2-2-1: Satellite NTN gNB Class – Number of Classes in Rel-17

- Proposals

  - Option 1: RAN4 should introduce only one NTN BS class in Rel-17

    - Note: same requirements or priority per one class

  - Option 2: RAN4 should introduce two NTN BS class in Rel-17

    - Note: e.g. one for LEO, one for GEO

  - Option 3: RAN4 should introduce three NTN BS class in Rel-17

    - Note: e.g. one for LEO@600, one for LEO@1200, one for GEO

  - Option 4: RAN4 should introduce four NTN BS class in Rel-17

    - Note: e.g. one for LEO@600, one for LEO@1200, one for MEO, one for GEO

- Discussion:

E///: In previous meeting agreement, we consider 3 classes as candidate classes and further dicuss and check the requirements; and pending on the difference from requirements perspective, we can conclude whether these classes needed ot not.

ZTE:We share same view as E///. The NF between GEO and LEO is different, that’s one possible requirement which has difference; other requirements dynamic range, ICS also need to be further considered.

CATT: We share same view as E///; we observed MCL difference between different statellites around 5-10 dB. We need to further check the requirements whether there is a need for these classes.

Nokia: Similar view as previous companies; we believe there is a need to introduce different classes.

Thales: Maximum output power depending on many details on satellite side, it maybe not feasible to introduce power limitation on NTN BS side. We concern about the workload; we should focus on essential part if strong demand for separate classes i.e. maximum 2 classes.

Hughes: We agree with Thales to focus on single class. What’s the purpose and benefits of introducing such classes.

CATT: Multiple power classes can be introduced for wide area BS.

E///: No need to always refer to BS 38.104 for class introduction. We suggest these as starting point and further check the requirements.

ZTE: BS classes associated with different RF requirements. For NTN co-channel co-existence study current not considered, the power limitation on NTN BS including whether needed or not pending on further discussion.

Thales: We agree with E/// and ZTE previous comments. The satellite to ground altitude can be varied, how to cover these different values. We may consider the co-existence TN bands is TDD or FDD; this may lead to different requirements for ACLR/ACS.

ZTE: We consider most strigent requirements across all cases in band agonistic way.

Agreement:

It’s FFS whether separate NTN gNB classes needed or not for Rel-17 which pending on further check on the RF requirements.

- If no difference observed from RAN4 RF requirements perspective, then only single NTN BS class will be introduced as wide area BS.

- All NTN BS classes can be potentially considered equivalent as to Wide Area BS (e.g. if all classes have the same requirements).

- At least introduce NTN BS class with wide coverage

The Classes intended to be used for differentiate the RF requirements.

Below candidate NTN gNB class can be considered as starting point:

- GEO, LEO@600, LEO@1200

- FFS whether need to LEO@600, LEO@1200 can be merged as single class

Topic #4: NTN TR and TS Titles and Contents

Issue 4-1-1: Titles and Scope of NTN NR TR and TS (general) – candidate proposals for (Satellite payload + feeder link + GW + Non-NTN infrastructure gNB)

- Proposals

  - Option 1: Companies to declare preference for (payload + feeder link + GW + Non-NTN infrastructure gNB) naming, please answer only with YES for your preferences.

| Company | Satellite Access Node | Satellite BS | Satellite gNB | Satellite Node B | NTN Satellite gNB | Satellite Assisted Access Node [added after 100-e] |
| --- | --- | --- | --- | --- | --- | --- |
| ZTE | Yes |  |  |  |  |  |

- Discussion:

E///: Fine with “Satellite Access Node” or “Satellite Assisted Acess Node”. Concern including BS which bring confusion.

Nokia: Fine with option 1 or new one from Ericsson.

CATT: RAN4 focused on equipments to introduce requirements other than logic node.

Thales: In Rel-17, NTN payload is transparent; but for future this may be changed. We are ok for all the options.

Ericsson: Option 1 not exclude any possible architecture.

Nokia: Using Node maybe different compared to traditional BS naming, but this can be used with future proof manner.

Agreement: “Satellite Access Node” agreed to use for RAN4 requirements and spec title for the box of Satellite payload + feeder link + GW + Non-NTN infrastructure gNB.

R4-2120669	Way Forward on NTN_solutions_Part1

Type: other		For: Approval
					Source: THALES

Abstract:

Discussion:

Decision:		Revised to R4-2120771 (from R4-2120669).

R4-2120771	Way Forward on NTN_solutions_Part1

Type: other		For: Approval
					Source: THALES

Abstract:

Discussion:

Decision:		Revised to R4-2120774 (from R4-2120771).

R4-2120774	Way Forward on NTN_solutions_Part1

Type: other		For: Approval
					Source: THALES

Abstract:

Discussion:

Decision:		Approved.

R4-2120776	Draft TR for 38.863 v0.1.0

Type: drafTR 	For: Approval

38.863 v 0.1.0

Source: Samsung

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120774 | Way Forward on NTN_solutions_Part1 | THALES | Approved |  |
| R4-2120759 | draft TP to TR 38.863: Operating bands and channel arrangements | ZTE Corporation | Approved | pCR on TR 38.863 |
| R4-2120760 | TP for 38.863 on system parameters on satellite bands | Huawei | Approved | pCR on TR 38.863 |
| R4-2120761 | TP to TR 38.863 - Regulatory aspects | Ericsson | Approved | pCR on TR 38.863 |
| R4-2120762 | TP to TR 38.863: node class, RF RX (6.2) | Huawei | Approved | pCR on TR 38.863 |
| R4-2120763 | TP for 38.863 on NTN UE transmission characteristics | Huawei | Approved | pCR on TR 38.863 |

------------------------------------------------------------------------------------------------------------------------------------------------

8.13.1.1	System parameters

R4-2117377	On open issue for NTN system parameters

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2118147	Discussion on NTN system parameters

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2118159	NTN - System parameters

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses some system parameters issues left from previous meetings

Decision:		Noted.

R4-2118613	On NTN System parameters

Type: discussion		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

R4-2118716	TP for 38.863 on system parameters on satellite bands

Type: pCR		For: Approval
					38.863 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Revised to R4-2120760 (from R4-2118716).

R4-2120760	TP for 38.863 on system parameters on satellite bands

Type: pCR		For: Approval
					38.863 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Approved.

R4-2119200	Further discussion on system parameters for NTN

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2119204	draft TP to TR 38.863:Operating bands and channel arrangements

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Revised to R4-2120759 (from R4-2119204).

R4-2120759	draft TP to TR 38.863: Operating bands and channel arrangements

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Approved.

8.13.1.2	NTN gNB Class/Type

R4-2117378	Furhter discussion on NTN BS class/type

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2117730	NTN gNB Class

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2118614	On NTN gNB ClassType

Type: discussion		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

R4-2119141	Further discussion on the NTN gNB Class/Type

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution, we provide further inputs to the discussion on the NTN gNB Class/Type.

Decision:		Noted.

R4-2119201	Further discussion on NTN gNB class and type

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2119592	On the Definition of NTN gNB Classes

Type: discussion		For: Discussion
					Source: THALES

Abstract:

RAN4 should introduce only one NTN BS class.

Decision:		Noted.

8.13.1.3	Regulatory information

R4-2118157	NTN - Regulatory information - TP to TR 38.863

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

This TP is suggesting text for the Regulatory section in TR 38.863

Decision:		Revised to R4-2120761 (from R4-2118157).

R4-2120761	NTN - Regulatory information - TP to TR 38.863

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

This TP is suggesting text for the Regulatory section in TR 38.863

Decision:		Approved.

8.13.1.4	Others

R4-2117379	Furhter discussion on NTN specificaiton

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2117380	draft LS on NTN architecture

Type: LS out		For: Approval
					to RAN3
					Source: CATT

Decision:		Noted.

R4-2118156	NTN - General

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses some NTN general aspects issues left from previous meetings

Decision:		Noted.

R4-2119299	NTN MEO Scenarios

Type: discussion		For: Agreement
					Source: Hughes/EchoStar

Decision:		Noted.

R4-2119553	NTN NR UE Technical Specification Discussion

Type: discussion		For: Discussion
					Source: THALES

Abstract:

RAN4 should not consider a new specification for NTN UE, since most requirements are the same as for TN UE.

Decision:		Noted.

8.13.2	Coexistence aspects

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][310] NTN_Solutions_Part2, AI 8.13.2- Yiran Jin

R4-2120624	Email discussion summary for [101-e][310] NTN_Solutions_Part2

Type: other		For: Information
					Source: Moderator (Samsung)

Abstract:

Discussion:

Decision:		Revised to R4-2120741 (from R4-2120624).

R4-2120741	Email discussion summary for [101-e][310] NTN_Solutions_Part2

Type: other		For: Information
					Source: Moderator (Samsung)

Abstract:

Discussion:

Decision:		Noted.

R4-2120670	WF on [310] NTN_Solutions_Part2

Type: other		For: Approval
					Source: Samsung

Abstract:

Discussion:

Decision:		Approved.

Agreement: RAN4 agreed no further effort on NTN to NTN (satellite to satellite) co-existence scenarios in Rel-17 NTN WI.

R4-2120671	Simulation assumptions for NTN co-existence

Type: other		For: Approval
					Source: Samsung

Abstract:

Discussion:

Decision:		Approved.

R4-2120672	Simulation assumptions for HAPS co-existence

Type: other		For: Approval
					Source: Nokia

Abstract:

Discussion:

Decision:		Approved.

R4-2120749	Summary of NTN co-existence study

Type: other		For: Approval
					Source: Samsung

Abstract:

Discussion:

Decision:		Approved.

R4-2120750	Summary of HAPS co-existence study

Type: other		For: Approval
					Source: Nokia

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120670 | WF on [310] NTN_Solutions_Part2 | Samsung | Approved |  |
| R4-2120671 | Simulation assumptions for NTN co-existence | Samsung, CATT | Approved |  |
| R4-2120672 | Simulation assumptions for HAPS co-existence | Nokia | Approved |  |
| R4-2120749 | Summary of NTN co-existence study | Samsung | Approved |  |
| R4-2120750 | Summary of HAPS co-existence study | Nokia | Approved |  |
| R4-2120772 | Draft text proposal to update TR 38.863 NTN related RF and co-existence aspects | Samsung | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

R4-2117778	Draft text proposal to update TR 38.863 NTN related RF and co-existence aspects

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung

Abstract:

draft text proposal to update TR 38.863 with:

1. skeleton of the TR

2. contents of Chapter 1, 2 and 6

3. contents of Annex A and B

Decision:		Revised to R4-2120673 (from R4-2117778).

R4-2120673	Draft text proposal to update TR 38.863 NTN related RF and co-existence aspects

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung

Abstract:

draft text proposal to update TR 38.863 with:

1. skeleton of the TR

2. contents of Chapter 1, 2 and 6

3. contents of Annex A and B

Decision:		Revised to R4-2120772 (from R4-2120673).

R4-2120772	Draft text proposal to update TR 38.863 NTN related RF and co-existence aspects

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung

Abstract:

draft text proposal to update TR 38.863 with:

1. skeleton of the TR

2. contents of Chapter 1, 2 and 6

3. contents of Annex A and B

Decision:		Approved.

R4-2118162	NTN - LS to RAN2 on NTN UE assumptions for TN-NTN coexistence studies

Type: LS out		For: Approval
					to RAN2
					Source: Ericsson

Abstract:

This is a LS to inform RAN2 about important assumptions taken in RAN4 regarding NTN UE locations

Decision:		Noted.

R4-2119294	On the S-band NTN-NTN Coexistence

Type: discussion		For: Decision
					Source: Hughes/EchoStar

Decision:		Noted.

8.13.2.1	NTN coexistence scenarios and simulations

R4-2117383	Further discussion on NTN co-existence simulation

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2117384	Co-existence simulation results for NTN <--> TN scenarios

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2117745	Simulation result for coexistence study on NR to support non-terrestrial networks

Type: discussion		For: Information
					Source: Xiaomi

Decision:		Noted.

R4-2117777	NR-NTN co-ex assumption and ACIR results

Type: discussion		For: Approval
					Source: Samsung

Decision:		Noted.

R4-2118145	Coexistence simulation assumptions and results for NTN

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2118158	NTN - Simulation first results

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution provides our simulation results based on the agreed assumptions

Decision:		Revised to R4-2120626 (from R4-2118158).

R4-2120626	NTN - Simulation first results

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution provides our simulation results based on the agreed assumptions

Decision:		Noted.

R4-2118715	Initial NTN simulation Results and discussion on ACLR and ACS

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2119202	Further discussion on simulation assumptions for NTN

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2119203	Set 1 Simulation results for NTN coexistence study

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2119300	NTN-NR MEO Scenarios and Characteristics

Type: discussion		For: Agreement
					Source: Hughes/EchoStar

Decision:		Noted.

R4-2119546	Correction of some Simulation Parameters for NTN Coexistence Scenarios

Type: discussion		For: Discussion
					Source: THALES

Abstract:

In this contribution it is proposed to correct some of the simulation parameters used as simulation assumptions for NTN coexistence studies.

Decision:		Noted.

R4-2119552	NTN coexistence simulation results

Type: discussion		For: Decision
					Source: MediaTek (Chengdu) Inc.

Decision:		Noted.

R4-2119554	On NTN-NTN Coexistence Analysis

Type: discussion		For: Discussion
					Source: THALES

Abstract:

RAN4 should remove NTN-NTN coexistence for LEO-LEO, LEO-GEO and GEO-GEO scenarios in S-band.

Decision:		Noted.

R4-2119557	TN-NTN Coexistence Results for Phase 1

Type: discussion		For: Discussion
					Source: THALES, Magister Solutions Ltd

Abstract:

This contribution presents Phase 1 simulation results for TN-NTN coexistence.

Decision:		Revised to R4-2120628 (from R4-2119557).

R4-2120628	TN-NTN Coexistence Results for Phase 1

Type: discussion		For: Discussion
					Source: THALES, Magister Solutions Ltd

Abstract:

This contribution presents Phase 1 simulation results for TN-NTN coexistence.

Decision:		Noted.

8.13.2.2	HAPS coexistence scenarios and simulations

R4-2118146	Coexistence simulation assumptions and results for HAPS

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2118615	HAPS simulation assumptions and results for coexistence study

Type: discussion		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

R4-2118714	Discussion on HAPS simulation assumptions

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision:		Noted.

8.13.3	BS RF requirements

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][311] NTN_Solutions_Part3, AI 8.13.3, 8.13.4- Yuexia Song

R4-2120618	Email discussion summary for [101-e][311] NTN_Solutions_Part3

Type: other		For: Information
					Source: Moderator (CATT)

Abstract:

Discussion:

Decision:		Revised to R4-2120742 (from R4-2120618).

R4-2120742	Email discussion summary for [101-e][311] NTN_Solutions_Part3

Type: other		For: Information
					Source: Moderator (CATT)

Abstract:

Discussion:

Decision:		Noted.

R4-2120674	WF on Tx RF requirements for satellite access node

Type: other		For: Approval
					Source: CATT

Abstract:

Discussion:

Decision:		Approved.

R4-2120675	WF on Rx RF requirements for satellite access node

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion:

Decision:		Approved.

R4-2120676	WF on NTN UE RF requirements

Type: other		For: Approval
					Source: Huawei

Abstract:

Discussion:

Decision:		Approved.

GTW Discussion on 2nd Nov

Sub-topic 1-2 Tx requirements for Satellite BS

Issue 1-2-1: Base station output power

- Proposals

  - Option 1: Define different power classes for GEO/LEO/MEO/HEO etc. the following framework is proposed.

| NTN BS class | Prated,c,AC |
| --- | --- |
| NTN BS class A (TBD) | (Note) |
| NTN BS class B (LEO1200) | ≤ TBD dBm |
| NTN BS class C (LEO600) | ≤ TBD dBm |
| NOTE: FFS |  |

  - Option 2: Other, please specify.

- GTW discussion:

ZTE: One possible way was leaving to manufacture declaration basis considering the status which hrequired NTN co-channel co-existence. We would like to hear Sattlite Operators feedback whether GEO/LEO can be operated on the same frequency/channel.

Hughes/Omnispace: This is possible, but strictly follow ITU-R policy.

E///:  We didn’t any need to specify the power limitation in RAN4 but want to hear the feedback from Satellite industry.

Thales: We don’t think any need to specify the power limitation in RAN4 for satellite access node.

Hughes: We agree with Thales.

Huawe i: This is applied for all classes with different orbit types. Can we rely on regulatory or some other ways in real field to ensure no performance degradation?

Ercisson: Our understanding this applied for all NTN BS classes.

Thales: These applied for all satellite types including GEO, LEO. The distance from orbit to UE much larger than existing TN wide-area BS class.

ZTE: Share same view with Ericsson.

Agreement:

The power limitation on “satellite access node” are manufacture declaration basis, no limitation in RAN4 specification.

Some background information from regulatory can be considered to be included in the TR for information.

Issue 1-2-2: RE power dynamic range

- Proposal

  - Option 1: The current RE power dynamic range can be reused for Satellite BS

  - Option 2: not to define the RE power dynamic range requirement.

- GTW discussion:

ZTE: We proposed option 2. For NTN BS, the beam width is wide, the receiver SINR is quite low; not sure whether still need to introducing such requirements. We are not sure this requirement helpful for co-channel co-existence of different beams.

Thales: Different FRF can be deployed and can be implementation basis.

E///: We prefer to include it at current moment and open to further discuss whether this requirement needed or not.

Huawei: We need to further discuss why need to remove this requirement as this is one of esstial requirements.

CATT: We didn’t see strong reason to remove this requirement.

Thales: This requirement applied for Satellite Access Node?

Agreement:

Further discuss the necessity of introducing of RE power dynamic range requirement

The current RE power dynamic range can be reused for Satellite Access Node if introduced

Issue 1-2-3: total power dynamic range

- Proposal

  - Option 1: The current total power dynamic range requirement can be reused.

    - If the co-existence simulations indicate that there is no need to change the ACLR, SU can be reused from NR.

  - Option 2: Other, please specify.

- GTW discussion:

Huawei: Using previous criteria to specify total power dynamic range which scaled based on total transmission RB configuration.

Thales: Based on our simulation, the ACLR can be lower than existing one in TN BS specification.

Agreement:

The current total power dynamic range requirement can be reused.

- If the co-existence simulations indicate that there is no need to change the ACLR, SU can be reused from NR.

Issue 1-2-4: Frequency error

- Proposal

  - Option 1: The current requirement for WA BS can be reused, i.e. 0.05ppm.

  - Other, please specify.

- GTW discussion:

Agreements: The current requirement for WA BS can be reused, i.e. 0.05ppm.

Issue 1-2-5: Modulation quality (EVM)

- Proposal

  - Option 1: The current requirement can be reused for QPSK and 16 QAM for satellite BS.

    - 64QAM and 256QAM is not considered in Rel-17.

  - Option 2: others, please specify

- GTW discussion:

ZTE: The received SINR is quite low which cannot support 64QAM and 256QAM; we support option 1.

Ericsson: We think 64QAM still needed.

Huawei: We share same view as Ericsson. We would like to further check the possibility of high modulation orders.

QC: We have similar view as Ericsson/Hauwei. Without power limitation for BS, we may still have possibility to support high modulation orders. We would like to check with satellite companies what’s the current status.

Thales:  For 256QAM is not possible, and 64QAM have possibility in FR2, but quite challenge in FR1. Usually 32QAM supported in satellite.

CATT: We can exclude 256QAM based on feedback from Thales.

Apple: This is applied for DL or UL?

Agreement:

For Satellite Access Node DL transmission:

- The current requirement can be reused for QPSK and 16 QAM

- FFS whether 64QAM can be supported in FR1

- 256QAM is not supported by Satellite Access node in Rel-17.

Issue 1-2-6: Time alignment error

- Proposal

  - Option 1: The TAE requirement is not needed for satellite BS in Rel-17.

    - CA feature is not considered at this stage

  - Option 2: TAE requirement for MIMO is reused for satellite BS and FFS on TAE for CA.

- GTW discussion:

Thales: CA is out of Rel-17 NTN WI scope.

CATT: For MIMO case, the channel between satellite and UE is rice channel which not possible to support MIMO feature.

Omnispace: It’s possible to support MIMO pending antenna implementation on satellite. We don’t want to remove this.

ZTE: It’s not harm to have such requirement for MIMO case if this scenario still possible.

Huawei: We prefer not to preclude MIMO case. If we consider multi-user case, we think still have the possibility; we would like to clarify the scenario mentioned here.

T-Mobile USA: This specific for MU-MIMO or SU-MIMO or both of them?

Thales: We believe MIMO included in future release.

Agreement:

CA is out of Rel-17 NTN WI scope for RAN4 requirements

FFS whether MIMO cases supported in Rel-17 NTN WI scope.

Issue 1-2-7: ACLR

- Proposal

  - Option 1: RAN4 decides which scenario the ACLR should be based on, NTN-NTN or NTN-TN.

  - Option 2: The ACLR of the NTN Satellite node shall be no higher than 25 dB according to NTN – TN scenario simulation.

  - Option 3: Others, please specify

- GTW discussion:

ZTE: We should rely on what’s the scenarios we covered now to draw conclusion considering the progress.

Ericsson: NTN-NTN is discussed under another email thread, and same story for ACLR/ACS.

Thales: Based on our simulation, ACLR can be lower than 25dB i.e. 20dB, 15dB.

QC: NTN-NTN is 2nd phase, not meaning low priority; we should introduce requirements based on worst cases.

Thales: We would like to further discuss whether NTN-NTN need to be covered.

Sub-topic 1-3 Rx requirements for Satellite BS

Issue 1-3-1: FRC

- Proposals

  - Option 1: The FRCs specified in TS 38.104 shall be re-used to specify the satellite node Rx requirements. Specifically the following FRCs and DMRS pattern is re-used.

|  | BS Channel BW | Subcarrier spacing | Number of PRBS |
| --- | --- | --- | --- |
| G-FR1-A1-1 | 5 MHz | 15 kHz | 25 |
| G-FR1-A1-2 | 5 MHz | 30 kHz | 11 |
| G-FR1-A1-3 | 10 MHz | 60 kHz | 11 |
| G-FR1-A1-4 | 20 MHz | 15 kHz | 106 |
| G-FR1-A1-5 | 20 MHz | 30 kHz | 51 |
| G-FR1-A1-6 | 20 MHz | 60 kHz | 24 |

  - Option 2: Other FRC’s are defined for satellite BS to address the low SNR operating point.

- GTW discussion:

Agreement: taking option 1 as starting point.

Issue 1-3-2: Noise figure

- Proposals

  - Option 1: Consider the satellite access node’s noise figure as a whole and use the following as starting point.

    - Satellite companies are encouraged to input on this.

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 19 | 1.1 | 1.1 |
| G_Rx (dBi) | 51 | 30 | 30 |
| NF (dB) | 7.4 | 4.3 | 4.3 |

  - Option 2: Other, please specify.

- GTW discussion:

Ericsson: We think we should focus on NF for whole Satellite Access Node.

Thales: This value from TR 38.821 for initial study, in reality NF can be lower than this.

ZTE: Based on the comment from Thales, this value is feasible from implemantion.

CATT: We share same view as E///, we shall focus on the whole satellite acess node.

Agreements: Option 1

Issue 1-3-3: Implementation margin

- Proposals

  - Option 1: reuse the implementation margin for TN in the corresponding frequency. E.g. 2dB IM for S band and 1.5dB IM for L band.

  - Option 2: Other, please specify.

- GTW discussion:

ZTE: We think 2dB used as IM for REFSENS requirements.

Ericsson: We proposed 2dB as TN used before, not sure where 1.5dB come from?

Agreement: Reusing margin from TN as [2dB].

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-21120674 | WF on Tx RF requirements for satellite access node | CATT | Approved |  |
| R4-21120675 | WF on Rx RF requirements for satellite access node | Ericsson | Approved |  |
| R4-21120676 | WF on NTN UE RF requirements | Huawei | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

R4-2119556	On the Variation of NTN ACLR and NTN ACS Parameters

Type: discussion		For: Discussion
					Source: THALES

Abstract:

The purpose of the contribution is to show how the NTN ACLR and NTN ACS Parameters can be varied, depending on the scenario type.

Decision:		Noted.

8.13.3.1	TX requirements

R4-2117381	Further discussion on Tx requirements for NTN BS

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2118160	NTN - Satellite Node - Tx requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses Satellite node - Tx requirements

Decision:		Noted.

R4-2119205	Discussion on Tx requirements of satellite gNB

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

8.13.3.2	RX requirements

R4-2117382	Further discussion on Rx requirements for NTN BS

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2118161	NTN - Satellite Node - Rx requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses Satellite node - Rx requirements

Decision:		Noted.

R4-2119142	TP to TR 38.863: node class, RF RX (6.2)

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

In this contribution, we provide TP to TR38.863 for section 6.2, based on WF agreements from meetings RAN4#100-e, and RAN4#99-e.

Decision:		Revised to R4-2120762 (from R4-2119142).

R4-2120762	TP to TR 38.863: node class, RF RX (6.2)

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

In this contribution, we provide TP to TR38.863 for section 6.2, based on WF agreements from meetings RAN4#100-e, and RAN4#99-e.

Decision:		Approved.

R4-2119206	Discussion on Rx requirements of satellite gNB

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

8.13.4	UE RF requirements

R4-2117746	Discussion on UE RF requirements for NR to support non-terrestrial networks

Type: other		For: Approval
					Source: Xiaomi

Decision:		Noted.

8.13.4.1	TX requirements

R4-2117332	Further discussion on UE Tx RF requirements for NTN

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2118718	TP for 38.863 on NTN UE transmission characteristics

Type: pCR		For: Approval
					38.863 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Revised to R4-2120763 (from R4-2118718).

R4-2120763	TP for 38.863 on NTN UE transmission characteristics

Type: pCR		For: Approval
					38.863 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision:		Approved.

8.13.4.2	RX requirements

R4-2117333	Further discussion on UE Rx RF requirements for NTN

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2118717	Discussion on UE RF requirements for satellite access

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision:		Noted.

8.16	Extending current NR operation to 71GHz

8.16.4	BS RF requirements

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][312] NR_exto71GHz_BSRF, AI 8.16.4- Toni lahteensuo

R4-2120619	Email discussion summary for [101-e][312] NR_exto71GHz_BSRF

Type: other		For: Information
					Source: Moderator (Nokia)

Abstract:

Discussion:

Decision:		Revised to R4-2120743 (from R4-2120619).

R4-2120743	Email discussion summary for [101-e][312] NR_exto71GHz_BSRF

Type: other		For: Information
					Source: Moderator (Nokia)

Abstract:

Discussion:

Decision:		Noted.

R4-2120677	WF on BS RF Tx requirements

Type: other		For: Approval
					Source: Nokia

Abstract:

Discussion:

Decision:		Approved.

R4-2120678	WF on BS RF Rx requirements and CR worksplit

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120677 | WF on BS RF Tx requirements | Nokia, Nokia Shanghai Bell | Approved |  |
| R4-2120678 | WF on BS RF Rx requirements and CR worksplit | Ericsson | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

8.16.4.1	TX requirements

R4-2117247	Proposals on BS transmitter requirements for extending current NR operation to 71 GHz

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution provides further proposals on BS transmitter requirements for extending current NR operation to 71 GHz according to the approved WF and the findings in the corresponding study item as recorded in TR 38.808.

Decision:		Noted.

R4-2117389	Discussion on the BS TX RF requirements for 52.6-71GHz

Type: other		For: Approval
					Source: CATT

Decision:		Noted.

R4-2118461	On BS RF transmitter requirements for the frequency range 52 to 71 GHz

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we present an overview of BS transmitter requirements and some proposals necessary to progress the work related to defining RF core requirements for the NR extension up to 71 GHz. To further stimulate the discussion draft specificatio

Decision:		Noted.

R4-2119190	Discussion on BS Tx requirements for 52.6-71GHz

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

8.16.4.2	RX requirements

R4-2117248	Proposals on BS receiver requirements for extending current NR operation to 71 GHz

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution provides further proposals on BS receiver requirements for extending current NR operation to 71 GHz according to the approved WF and the findings in the corresponding study item as recorded in TR 38.808.

Decision:		Noted.

R4-2117390	Discussion on the BS RX RF requirements for 52.6-71GHz

Type: other		For: Approval
					Source: CATT

Decision:		Noted.

R4-2118462	On BS RF receiver requirements for the frequency range 52 to 71 GHz

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we continue the discussion from [4] on receiver requirements relevant for the NR extension to support up to 71 GHz. The contribution presents an overview of base station receiver requirements and some proposals to further progress the

Decision:		Noted.

R4-2119191	Discussion on BS Rx requirements for 52.6-71GHz

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

8.17	Enhancements to Integrated Access and Backhaul (IAB) for NR

8.17.1	General

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][313] NR_eIAB, AI 8.17.1, 8.17.2- Yankun Li

R4-2120620	Email discussion summary for [101-e][313] NR_eIAB

Type: other		For: Information
					Source: Moderator (Samsung)

Abstract:

Discussion:

Decision:		Revised to R4-2120744 (from R4-2120620).

R4-2120744	Email discussion summary for [101-e][313] NR_eIAB

Type: other		For: Information
					Source: Moderator (Samsung)

Abstract:

Discussion:

Decision:		Noted.

R4-2120679	WF on RF specification impact for eIAB

Type: other		For: Approval
					Source: Samsung

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120679 | WF on RF specification impact for eIAB | Samsung | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

8.17.2	RF requirements

8.17.2.1	Impact for Simultaneous operation of IAB child and parent links

R4-2118232	Simultaneous operation on IAB-node’s child and parent links

Type: other		For: Approval
					Source: Samsung

Decision:		Noted.

R4-2118436	Discussion on power imbalance for simultaneous operation of IAB

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2118736	RF requirements for simultaneous IAB-MT and IAB-DU operation

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision:		Noted.

R4-2118979	RF impact analysis for simultaneous DU and MT operation

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on generic RAN4 work relating to the objectives focusing the timing aspect.

Decision:		Noted.

8.17.2.2	Impact for Timing enhancement

R4-2118233	Timing enhancement on Rel-17 IAB

Type: other		For: Approval
					Source: Samsung

Decision:		Noted.

R4-2118437	Discussion on timing issues for simultaneous operation of IAB

Type: other		For: Approval
					Source: ZTE Corporation

Decision:		Noted.

R4-2118912	On IAB timing error

Type: other		For: Approval
					Source: Nokia, Nokia Shaghai Bell

Decision:		Noted.

R4-2118980	IAB MT /DU case 6 timing

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on RF impact on simultaneous operation of DU and MT.

Decision:		Noted.

8.17.2.3	Others

9	Rel-17 Study Items for NR

9.1	Study on enhanced test methods for FR2 in NR

9.1.1	Maintenance on objectives 1~6

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][328] FR2_enhTestMethods_Part1, AI 4.1.10, 9.1.1- Anatoliy (Toliy) Ioffe

R4-2120621	Email discussion summary for [101-e][328] FR2_enhTestMethods_Part1

Type: other		For: Information
					Source: Moderator (Apple)

Abstract:

Discussion:

Decision:		Revised to R4-2120745 (from R4-2120621).

R4-2120745	Email discussion summary for [101-e][328] FR2_enhTestMethods_Part1

Type: other		For: Information
					Source: Moderator (Apple)

Abstract:

Discussion:

Decision:		Noted.

R4-2120697	TP on CFFdeltaNF

Type: pCR		For: Approval
					38.884 v1.1.0	  CR-  rev  Cat:  (Rel-17)

Source: Rohde &Schwarz, Keysight

Abstract:

Discussion:

Decision:		Approved.

R4-2120768	Draft TR38.884 Study on enhanced test methods for FR2 NR UEs v1.2.0

Type: drafTR 	For: Approval

38.884 v1.2.0

Source: Apple, vivo, Intel

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2118963 | UL MIMO EVM simulations for TE dual-receiver using the overlapping PUSCH DMRS configuration | Anritsu Limited | Approved |  |
| R4-2120697 | TP on CFFdeltaNF | Rohde &Schwarz, Keysight | Approved |  |
| R4-2120695 | TP to TR 38.884 on general aspects | vivo | Approved |  |
| R4-2120696 | Preliminary MU assessment of CFFDNF and CFFNF methodologies | Keysight Technologies UK Ltd, Rohde & Schwarz | Approved |  |
| R4-2120768 | Draft TR38.884 Study on enhanced test methods for FR2 NR UEs v1.2.0 | Apple, vivo, Intel | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

9.1.1.1	MU assessment

R4-2117854	Preliminary assessment of MU on FR2 test time reduction

Type: discussion		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2117971	Preliminary uncertainty assessment for the high DL power and low UL power objective

Type: discussion		For: Decision
					Source: Apple

Decision:		Noted.

R4-2119177	MU analysis and applicability for CFFDNF with black-box approach

Type: discussion		For: Approval
					38.884 v	  CR-  rev  Cat:  (Rel-17)

					Source: ROHDE & SCHWARZ

Decision:		Revised to R4-2120694 (from R4-2119177).

R4-2120694	MU analysis and applicability for CFFDNF with black-box approach

Type: discussion		For: Approval
					38.884 v	  CR-  rev  Cat:  (Rel-17)

					Source: ROHDE & SCHWARZ

Decision:		Noted.

R4-2119380	Preliminary MU assessment of CFFDNF and CFFNF methodologies

Type: pCR		For: Approval
					38.884 v1.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Keysight Technologies UK Ltd, Rohde & Schwarz

Decision:		Revised to R4-2120696 (from R4-2119380).

R4-2120696 Preliminary MU assessment of CFFDNF and CFFNF methodologies

Type: pCR		For: Approval
					38.884 v1.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Keysight Technologies UK Ltd, Rohde & Schwarz

Decision:		Approved.

9.1.1.2	Others

R4-2118316	TP to TR 38.884 on general aspects

Type: pCR		For: Approval
					38.884 v1.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision:		Revised to R4-2120695 (from R4-2118316).

R4-2120695	TP to TR 38.884 on general aspects

Type: pCR		For: Approval
					38.884 v1.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision:		Approved.

R4-2118963	UL MIMO EVM simulations for TE dual-receiver using the overlapping PUSCH DMRS configuration

Type: discussion		For: Approval
					Source: Anritsu Limited

Decision:		Approved.

R4-2119381	On Black-Box Correlation Factor NF Test Methodology

Type: discussion		For: Approval
					Source: Keysight Technologies UK Ltd

Decision:		Noted.

9.1.2	OTA test methods for UE RF, RRM and demodulation for 52.6~71GHz

-----------------------------------------------------------------------------------------------------------------------------------------------

Email discussion for [101-e][329] FR2_enhTestMethods_Part2_NWM, AI 9.1.2- Aida Vera Lopez

R4-2120622	Email discussion summary for [101-e][329] FR2_enhTestMethods_Part2_NWM

Type: other		For: Information
					Source: Moderator (Intel)

Abstract:

Discussion:

Decision:		Revised to R4-2120746 (from R4-2120622).

R4-2120746	Email discussion summary for [101-e][329] FR2_enhTestMethods_Part2_NWM

Type: other		For: Information
					Source: Moderator (Intel)

Abstract:

Discussion:

Decision:		Noted.

R4-2120698	WF on OTA test methods for 52.6~71GHz

Type: other		For: Approval
	Source: Intel

Abstract:

Discussion:

Decision:		Revised to R4-2120767 (from R4-2120698).

R4-2120767	WF on OTA test methods for 52.6~71GHz

Type: other		For: Approval
	Source: Intel

Abstract:

Discussion:

Decision:		Approved.

Conclusion after 2nd round

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2120767 | WF on OTA test methods for 52.6~71GHz | Intel | Approved |  |

------------------------------------------------------------------------------------------------------------------------------------------------

R4-2118869	On 60GHz OTA testing for vehicular UE

Type: discussion		For: Discussion
					Source: LG Electronics Finland

Decision:		Noted.

9.1.2.1	General

9.1.2.1.1	Test system assumption

9.1.2.1.2	UE types

R4-2118314	Discussion on UE types for B52.6GHz test methods development

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2119511	UE types for FR2-2 OTA test methods

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

9.1.2.1.3	MU assessment

9.1.2.1.4	Others

9.1.2.2	Test methodology for UE RF

R4-2118315	Discussion on Test methods for B52.6GHz UE RF

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

9.1.2.3	Test methodology for RRM

9.1.2.4	Test methodology for UE demodulation and CSI

R4-2119512	FR2-2 OTA test methods for UE demodulation

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

10	Rel-17 Work Items for LTE

11	Liaison and output to other groups

12	Revision of the Work Plan

13	Any other business