3GPP TSG-RAN4 Meeting #101-e			R4-2117021

Electronic Meeting, 01 November – 12 November, 2021

3GPP TSG-RAN WG2 Meeting #115-e		R2-2109196

Electronic, 16th Aug. – 27th Aug. 2021

Title:	LS to RAN4 on eDRX

Response to:	-

Release:	Rel-17

Work Item:	NR_redcap-Core

Source:	RAN2

To:	RAN4

Cc:

# Contact Person:

# Name:	                Chenli

E-mail Address:	 Chenli5g@vivo.com

# 1. Overall Description:

RAN2 discussed eDRX features for any kind of UEs (either RedCap or non-RedCap), and achieved the following conclusions:

Agreements:

- eDRX feature is optional for any UE (including RedCap and non-RedCap UEs).

- RAN2 considers the configuration as an invalid case, where INACTIVE eDRX cycle is configured but IDLE eDRX cycle is not configured. FFS whether to capture this restriction in RAN2 spec.

- RAN2 considers the configuration as invalid case, where INACTIVE eDRX cycle is longer than IDLE eDRX cycle. FFS whether to capture this restriction in RAN2 spec.

- The maximum PTW length is 40.96s when IDLE eDRX cycle is longer than 10.24s.

- The minimum PTW length is 1.28s and the step length/granularity of PTW length is 1.28 when IDLE eDRX cycle is longer than 10.24s.

- When IDLE eDRX cycle is longer than 10.24s, PH calculation formula defined in LTE is re-used, i.e.

PH_CN:  H-SFN mod TeDRX,_CN,H= (UE_ID_H mod TeDRX_CN,H)

-  where TeDRX_CN,H is equal to IDLE eDRX cycle.

- When IDLE eDRX cycle is longer than 10.24s, CN PTW_end calculation formula defined in LTE is re-used, i.e.

PTW_end is radio frame satisfying SFN = (PTW_start + L*100 - 1) mod 1024,

- where L is PTW length configured by upper layers.

Working assumption:

- When IDLE eDRX cycle is longer than 10.24s, CN PTW_start calculation formula defined in LTE is re-used as the baseline, as below. FFS whether CN PTW_start position could be configurable by network. Note: this formula would be revisited if INACTIVE eDRX cycle can be above 10.24s

PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation: SFN = 1024/N* ieDRX, where - ieDRX = floor(UE_ID_H /TeDRX,H) mod N - FFS N = 4 or 8, FFS if N can take other values

# 2. Actions:

# To RAN WG4

# RAN2 kindly request RAN4 to take the above information into account during the following work, and study/specify the corresponding requirements inside PTW, when IDLE eDRX>10.24s, and provide feedback, if any.

# 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #116-e		1-12 Nov. 2021				e-Meeting

TSG RAN WG2 Meeting #117-e		TBD							e-Meeting