3GPP TSG-RAN4 Meeting #101-e			R4-2117022

Electronic Meeting, 01 November – 12 November, 2021

3GPP TSG RAN WG2#115-e	R2-2109197

e-Meeting, 16th - 27th August, 2021

Title:	LS on RRM Relaxations for RedCap

Response to:

Release:	Rel-17

Work Item:	NR_redcap-Core

Source:	RAN2

To:	RAN4

Cc:

Contact person:	Yiru Kuang, kuangyiru@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN2 have discussed the RRM relaxations and achieved the following agreements on RRM relaxation criteria.

Agreements in RAN2#113bis-e: - At least for RRC idle/inactive, a measurement-based R17 stationarity criterion can be configured separately from R16 low-mobility criterion for R17 UEs supporting the feature. - Network can configure R17 stationarity criterion/criteria together with a not-at-cell-edge criterion, to trigger RRM relaxations in RRC Idle/Inactive for R17 UEs supporting the feature. FFS whether the R16 not-at-cell-edge thresholds can be reused or separate R17 not-at-cell-edge thresholds are needed. Agreements in RAN2#114-e: - An RSRP/RSRQ based stationarity criterion (Working Assumption: the same as in idle/inactive) can be configured for UEs in RRC Connected. If the criterion is met, this is reported to the network (FFS how/when). It is FFS whether, based on this, besides possibly reconfiguring RRM measurements (up to network implementation), the network can enable RRM measurement relaxation (FFS whether same method as in Idle/Inactive). - Subscription based relaxation criteria will not be considered in Rel-17 RRM relaxation. - Reuse R16 low mobility criterion, as part or whole of Rel-17 stationary criterion in RRC_IDLE/INACTIVE. When NW configures both Rel-17 stationary criterion and Rel-16 low mobility criterion, NW configures different Rel-17 thresholds (i.e., SSearchDeltaP_stationary/TSearchDeltaP_stationary) from Rel-16 (SSearchDeltaP / TSearchDeltaP). How to configure the criterion (e.g. more stringent) is left to NW implementation (i.e. no specification impact to RAN2). - Postpone the following discussion until RAN4 defines RRM relaxation method for Rel-17: When NW configures both R16/R17 relaxation criteria and the UE fulfills both, UE performs: - Option 1) UE performs Rel-17 RRM relaxation method - Option 2) It is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation - When NW configures Rel-17 RRM relaxation for RRC_IDLE/INACTIVE, Rel-17 stationary criterion is mandatory, and Rel-17 not-at-cell-edge criterion is optional configuration. Agreements in RAN2#115-e: - Do not introduce nor reuse not-at-cell-edge threshold for R17 RRC_CONNECTED UEs. - Do not introduce beam change based criterion in Rel-17. - The network provides the configuration of stationarity criterion to the UE via dedicated signalling (e.g. RRCReconfiguration message) in RRC_CONNECTED. - Introduce separate Rel-17 not-at-cell-edge threshold, and the new threshold is only associated with Rel-17 stationary criterion (if configured). If configured with a not-at-cell-edge criterion, the R17 stationary criterion can only be configured together with the R17 not-at-cell-edge criterion, not with the R16 one.

RAN2 respectfully asks RAN4 to take the above agreements into account.

For RRC_IDLE/INACTIVE, RAN4 is asked to study and define the corresponding Rel-17 RRM relaxation method.

For RRC_CONNECTED, RAN2 will continue discussing the details.

# 2	Actions

To RAN4 group:

ACTION: 	RAN2 respectfully asks RAN4 to take the above agreements into account.

For RRC_IDLE/INACTIVE, RAN4 is asked to study and define the corresponding Rel-17 RRM relaxation method.

# 3	Dates of next TSG-RAN WG2 meetings

TSG-RAN2 Meeting #116-e		1-12 November 2021			Electronic Meeting

TSG-RAN2 Meeting #117-e		17-26 January 2022			Electronic Meeting