3GPP TSG-RAN4 Meeting #101-e			R4-2119598

Electronic Meeting, 01 November – 12 November, 2021

3GPP TSG RAN WG1 Meeting #106bis-e	R1-2110698

E-meeting, October 11th – 19th, 2021

Title:	Reply LS on SCell dropping issue of CA

Response to:	R1-2108701/R4-2114950

Release:	Rel-17

Work Item:	NR_RF_FR1_enh-Core

Source:	RAN WG1

To:	RAN WG4

Cc:	RAN WG5

Contact Person:

#### Name:	Frank Long

E-mail Address:	frank.longyi {@ }huawei.com

Attachments:	-

1. Overall Description:

RAN1 thanks RAN4 for the LS on SCell dropping issue of CA. Regarding the questions raised in the LS, RAN1 answers are as follows.

Question 1: Whether UE drop Scell power according to the priority rule defined in 38.213 is considered as an issue from RAN1 perspective.

Answer: RAN1 understands that SCell dropping due to power limitation can occur. But RAN1 doesn’t see changes to the priority rule defined in TS 38.213 as a way to address such issue since RAN1 has not identified any issue on the priority rules in TS 38.213.

Question 2: Whether UE drop Scell power according to the priority rule defined in 38.213 has been addressed from 16 or 17? If not, what expected solution(s) are?

Answer: No until this RAN1 meeting, RAN1 is not working to modify the power scaling priority rule in Rel-17 either, and understands that RAN4 is discussing potential RAN4 specification updates.

Question 3: If the problem above is solved in RAN4 specifications with solution by higher layer configuration, e.g. introduce additional UE-specific configuration of power limits on Pcmax,f,c for each CC to prevent SCell dropping (see e.g. R4-2112826 or R4-2114551 for details), is there any expected RAN1 spec impact or possible conflict with UE behaviour defined in RAN1 specifications?

Answer: Regarding the solution examples, there is no RAN1 specification impact by introducing additional UE-specific configuration of power limits on Pcmax,f,c for each CC to prevent SCell dropping. For other aspects, including potential conflict with current RAN1 specification, a definite answer can only be provided when the detailed solution is provided by RAN4.

2. Actions:

To: RAN4

ACTION: 	RAN1 respectfully ask RAN4 to take the above answers into account in their further work.

3. Date of Next TSG-RAN1 Meetings:

3GPP RAN1#107-e		11 - 19 November 2021   	Online

3GPP RAN1#107-bis-e		17 - 25 January 2022   	Online