3GPP TSG-RAN WG4 Meeting # 101-e 	R4-2120314

# Electronic Meeting, Nov. 1-12, 2021

Title:	LS on further agreements on RLM and BFD relaxation for UE Power Saving enhancements

Response to:

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact Person:

#### Name:	Yanliang SUN, Althea Huang

#### E-mail Address:	 yanliang.sun@vivo.com, Althea.Huang@MEDIATEK.COM

Attachments:

1. Overall Description:

In RAN4 #100e, LS R4-2115439 on criteria for RLM/BFD relaxation is sent to RAN2. In RAN4 #101e, further agreements are achieved in RAN4 as follows.

For the low mobility criterion,

- Intra-frequency L3 RSRP measurement of serving cell based on SSB is used for low mobility criteria evaluation.

- For low mobility criterion, the threshold on RSRP variation and the time period over which the RSRP variation is evaluated for relaxed RLM/BFD measurement are configured by network.

  - Thresholds for R16 low mobility criterion and R17 low mobility criterion can be configured separately.

For the good serving cell quality criterion,

- Reuse the existing method to evaluate “downlink radio link quality” for RLM/BFD

  - hypothetical BLER with corresponding PDCCH parameters is used to evaluate good serving cell quality criterion

- The good serving cell quality criteria for RLM is based on an offset X dB and Qx, while Qx is derived from PDCCH transmission parameters.

For the configuration of the relaxation criteria,

- When both criteria are configured, UE is allowed to enter relaxation only when both criteria are fulfilled.

RAN4 respectfully ask RAN2 take above agreements into considerations and further discuss on corresponding RRC signalling for the above aspects agreed in RAN4

2. To RAN WG2 group.

ACTION: 	RAN4 respectfully asks RAN2 to take RAN4 agreements in consideration and specify the corresponding signaling.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #101e-bis	17-25, Jan, 2022	Online