3GPP TSG-RAN WG4 Meeting # 101-e	R4-2117439

# Electronic Meeting, November 01-12, 2021

Agenda item:	8.19.3.2

Source:	Apple Inc.

Title:	Discussion on RRM requirements for inter-cell beam management

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#100-e RRM requirements for FeMIMO were discussed was discussed and way forward [1] was agreed. The agreements for inter-cell beam management are:

Inter-cell beam management - RAN4 needs to specify the intra-frequency L1-RSRP measurement requirements for non-serving cells - RAN4 needs to specify the intra-frequency L1-RSRP measurement accuracy requirements for non-serving cells - For non-serving L1-RSRP measurement of single panel FR2 UE, requirements will be applied if UE only measure L1-RSRP from one single cell at a time. - To guarantee UE’s mobility performance, RAN4 shall agree that PCell/PSCell’s L3-RSRP measurement delay shall not be impacted by NSC measurements. FFS UE measurement behaviour for L1-RSRP. - RAN4 will further study if UE only performs L1-RSRP measurements on the identified non-serving cell(s)

In this contribution we present our views on RRM requirements for inter-cell beam management.

# 2. Discussion

## L1-RSRP Requirements for Non-Serving cells

RAN4 needs to specify SSB based L1-RSRP measurements for non-serving cells for inter-cell beam management. Under the category of L1-RSRP measurements, we propose to specify:

- Measurement period

- Measurement restriction

- Scheduling restriction

Proposal #1: RAN4 should discuss and define (1) Measurement period, (2) Measurement restriction and (3) Scheduling restriction for SSB based L1-RSRP measurements on neighbor cell.

In R15 the L1-RSRP measurement requirements are only applicable for SSB and CSI-RS in the active BWP. Similarly, the SSB based L1-RSRP measurement requirements on non-serving cells shall only be when the SSB is within the active DL BWP.

Proposal #2: The L1-RSRP measurement requirements for non-serving cells are only applicable to SSB withing the active DL BWP of the UE.

The non-serving cells are configured for L1-RSRP measurements in CSI report config. Also, there is no agreement in RAN1 that UE has already identified and measured the non-serving cell configured for L1-RSRP measurement.

Observation #1: There is no agreement in RAN1 that the non-serving cell configured for L1-RSRP measurement is already identified and measured by the UE.

We might first need to define a condition for known cell and then further discuss the measurement requirements.

Proposal #3: Define known cell condition for non-serving cell configured for L1-RSRP measurements.

The known cell condition for non-serving cell L1-RSRP measurement can be defined as follows:

In FR1 the cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds.

In FR2  the cell is known if, during the last 5 seconds –

- the UE has sent a valid measurement report for the non-serving cell and

- One of the SSBs measured from the non-serving cell being configured remains detectable

If the non-serving cell is unknown, then the measurement period would be extended by the time needed for cell identification, measurement and report. Since this is an intra-frequency measurement without gap, the time for cell identification and measurement can be based on intra-frequency cell identification requirements without gap.

In FR1-FDD: Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms

In FR1 TDD and FR2: Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Proposal #4: For unknown cell case, the measurement period is extended by the time needed for intra-frequency cell identification and measurement.

If the non-serving cell configured for L1-RSRP measurement is known to the UE, the UE still doesn’t know the configuration like periodicity of the SSB. Restricting the L1-RSRP measurement to SMTC would increase overhead and delay of these measurements. For L1-RSRP measurements on the non-serving cell, the UE needs to be configured with this information. The UE can be configured with:

Option 1: Configure the periodicity of the non-serving cell as part of the CSI-ReportConfig

Option 2: Define a new SMTC configuration for L1 measurement.

The new SMTC config for L1-RSRP measurement could only include the information needed like periodicity if the SSB.

Proposal #5: For non-serving cell L1-RSRP measurement:

Option 1: UE is configured with non-serving cell SSB periodicity as part of the CSI-ReportConfig

Option 2: Define a new SMTC configuration for L1 measurement.

Measurement period

The measurement period for neighbor cell L1-RSRP measurements could be based on the existing serving cell L1-RSRP measurements for FR1 and FR2.

Proposal #6: Define SSB based L1-RSRP measurement period for neighbor cell similar to existing serving cell requirements.

The measurement period on neighbor cell for FR1 and FR2 could be defined as:

Measurement period TL1-RSRP_Neighbor_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Neighbor_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_neighbor) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_neighbor)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB_neighbor is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 are configured; otherwise K = 1.5. |  |

Measurement period TL1-RSRP_Neighbor_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Neighbor_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB_neighbor) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB_Neighbor)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB_neighbor is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Where,

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N= 8.

With the assumption that periodicity of SSB on non-serving cell is known, either via new SMTC or explicit configurations, the definition of P can be based on what we currently have for L1-RSRP measurements for the serving cell, where TSSB is replaced by TSSB_Nonserving .

For example, in FR1, if neighbor cell measurement is not limited to SMTC, not overlapping with SSB of serving cell

-	P=11-TSSB_NeighborMRGP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

Measurement restriction

We need to define measurement restriction on SSB based L1-RSRP measurements, if the SSB from non-serving cell is on the same OFDM symbol as SSB or CSI-RS from serving cell for other L1 measurements as follows:

For FR1, when the SSB from neighbor cell for L1-RSRP measurement is in the same OFDM symbol as CSI-RS from serving cell for RLM, BFD, CBD or L1-RSRP measurement,

- If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

- If SSB and CSI-RS have different SCS,

  - If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

  - If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

For FR1, when the SSB from neighbor cell for L1-RSRP measurement is in the same OFDM symbol as SSB from serving cell for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure SSB from both serving and neighbor cell without any restriction.

For FR2, when the SSB from neighbor cell for L1-RSRP measurement on one CC is in the same OFDM symbol as CSI-RS or SSB from serving cell for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

For FR2, there is no measurement restriction allowed when the network configures mixed numerology between SSB from neighbor cell for L1-RSRP measurement on one FR2 band and CSI-RS or SSB from serving cell for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, provided that UE is capable of independent beam management on this FR2 band pair.

Proposal #7: Define Measurement restriction on SSB based L1-RSRP measurements for non-serving cell, if the SSB from non-serving cell is on the same OFDM symbol as SSB or CSI-RS from serving cell for other L1 measurements.

Scheduling Restrictions

For SSB based L1-RSRP measurements on non-serving cell, the following scheduling availability need to be defined:

- Scheduling availability of UE performing L1-RSRP measurement on with a same subcarrier spacing as PDSCH/PDCCH on FR1

- Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

- Scheduling availability of UE performing L1-RSRP measurement on FR2

- Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA

Proposal #8: Define scheduling availability for UE performing L1-RSRP measurement on neighbor cell.

# 3. Conclusion

In this paper, we present our views on RRM requirements for inter-cell beam management. Our observations and proposals are captured below:

Proposal #1: RAN4 should discuss and define (1) Measurement period, (2) Measurement restriction and (3) Scheduling restriction for SSB based L1-RSRP measurements on neighbor cell.

Proposal #2: The L1-RSRP measurement requirements for non-serving cells are only applicable to SSB withing the active DL BWP of the UE.

Observation #1: There is no agreement in RAN1 that the non-serving cell configured for L1-RSRP measurement is already identified and measured by the UE.

Proposal #3: Define known cell condition for non-serving cell configured for L1-RSRP measurements.

Proposal #4: For unknown cell case, the measurement period is extended by the time needed for intra-frequency cell identification and measurement.

Proposal #5: For non-serving cell L1-RSRP measurement:

Option 1: UE is configured with non-serving cell SSB periodicity as part of the CSI-ReportConfig

Option 2: Define a new SMTC configuration for L1 measurement.

Proposal #6: Define SSB based L1-RSRP measurement period for neighbor cell similar to existing serving cell requirements.

Proposal #7: Define Measurement restriction on SSB based L1-RSRP measurements for non-serving cell, if the SSB from non-serving cell is on the same OFDM symbol as SSB or CSI-RS from serving cell for other L1 measurements.

Proposal #8: Define scheduling availability for UE performing L1-RSRP measurement on neighbor cell.

# Reference

- R4-2115355, “WF for FeMIMO RRM”, Samsung.