3GPP TSG RAN WG4 Meeting #101-e	R4-2117666

Online, 1st – 12th Nov. 2021

Agenda item:	5.1.6.2.1

Source: 	SoftBank Corp.

Title: 	Discussion on the power imbalance requirements for Type-2 UEs

Document for:	Approval

Introduction

In RAN4#100-e meeting, the WF for Type 2 UE Rx Imbalance Requirement was approved [1]. In this paper, we show our view for the options in the WF.

Discussion

The following are the excerpt of the WF [1].

Proposal 1: The table format is agreeable. There are some options on how to specify the requirements for Type 2 UE with DL carrier power imbalance.

- Option 1a:

| Carriers | Power in transmission bandwidth configuration (dBm) | channel bandwidth | Frequency relationship |
| --- | --- | --- | --- |
| Wanted carrier | REFSENS + 14 dB | BWwanted ≤ BWanother | Place two DL carriers as close as possible |
| Another carrier with overlapping DL bands | Power of wanted carrier + 31.5 dB |  |  |
| Wanted carrier | REFSENS + 14 dB | BWwanted > BWanother |  |
| Another carrier with overlapping DL bands | Power of wanted carrier + 31.5 – 10*log10(BWwanted /BWanother) dB |  |  |

- Option 1b:

| Carriers | Power in transmission bandwidth configuration (dBm) | channel bandwidth | Frequency relationship |
| --- | --- | --- | --- |
| Wanted carrier | REFSENS + 1 dB | BWwanted ≤ BWanother | Place two DL carriers as close as possible |
| Another carrier with overlapping DL bands | Power of wanted carrier + 25 dB |  |  |
| Wanted carrier | REFSENS + 1 dB | BWwanted > BWanother |  |
| Another carrier with overlapping DL bands | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) dB |  |  |

- Option 2 (Gap between DL carriers ≥ 50MHz):

| Carriers | Power in transmission bandwidth configuration (dBm) | channel bandwidth | Frequency relationship |
| --- | --- | --- | --- |
| Wanted carrier | REFSENS + 1 dB | BWwanted ≤ BWanother | max (5/2*another DL BW, 50MHz) |
| Another carrier with overlapping DL bands | Power of wanted carrier + 25 dB |  |  |
| Wanted carrier | REFSENS + 1 dB | BWwanted > BWanother |  |
| Another carrier with overlapping DL bands | Power of wanted carrier + 25 dB – 10*log10(BWwanted /(5*min(BWanother, 20MHz))) |  |  |

- Option 3: Combination of option 1 and option 2

- Option 4: Other parameters are not precluded.

Firstly, the above requirements are applied to the UE that supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands, defined in the UE capability signaling interBandMRDC-WithOverlapDL-Bands-r16. Our understanding is that currently DC_20_n28, 42_n77/n78 and DC_48_n77 fit this condition in the band combinations already specification is completed. With regard to DC_42_n77/78 and DC_48_n77, the Option 1a/1b indicates that two DL carriers are placed contiguously.

Observation 1: For DC_42_n77/78 and DC_48_n77, the Option 1a/1b indicates that two DL carriers are placed contiguously.

It is noted that the the UE capability signaling interBandContiguousMRDC is applied to DC_42_n77/78 and DC_48_n77. According to the explanation of this IE in TS38.306, the UE supports intra-band non-contiguous (NG)EN-DC/NE-DC requirements if the field is absent.

Observation 2: For DC_42_n77/78 and DC_48_n77, the UE supports intra-band non-contiguous EN-DC requirements as a default considering the UE capability signaling interBandContiguousMRDC.

From the above observations, it seems to be strange that only Option 1a/1b is specified to DC_42_n77/78 and DC_48_n77 and it is straightforward that the concept Option 2, DL carriers are placed non-contiguously, should be included in the requirements for those band combinations. In addition, the following figure indicates the spectrum allocation of DC_42_n77/n78 in Japan [2]. It is found that the gap between DL carriers in each operator is more than 50MHz. From this point of view, the conditions of Option 2 are suitable for the real deployment.

Figure 2-1: The spectrum allocation of DC_42_n77/n78 in Japan

Considering it, our proposal is as follows.

Proposal: The concept of Option 2, two DL carriers are placed non-contiguously and gap between DL carriers ≥ 50MHz, should be included in the requirements.

Conclusions

In this contribution, we shared our view for specifying the requirements for Type 2 UE with DL carrier power imbalance.

Observation 1: For DC_42_n77/78 and DC_48_n77, the Option 1a/1b indicates that two DL carriers are placed contiguously.

Observation 2: For DC_42_n77/78 and DC_48_n77, the UE supports intra-band non-contiguous EN-DC requirements as a default considering the UE capability signaling interBandContiguousMRDC.

Proposal: The concept of Option 2, two DL carriers are placed non-contiguously, should be included in the requirements.

- References

[1]	R4-2114905, WF on Type 2 UE RX Imbalance Requirement, Huawei, HiSilicon, Qualcomm, RAN4#100-e

[2] R4-2117306, Motivation for supporting non-colocated scenarios for band 42 and n77/n78, SoftBank, RAN4#101-e